Chapter 3.
Peripheral System Configuration

  - Input/Output Ports

POTATO has three I/O ports, each allocated to their own address in the Special Function Registers (SFR); in addition, the input/output direction of ports 1 and 3 is determined by the corresponding data direction register (PnDDR). Port 1 is also used as a serial interface/next-generation game machine interface, and as a pulse generator output, and is controlled by a functional control register (P1FCR). In addition, port 7 is also provided as an input-only port.

When a reset is executed, all ports are set to input mode, and the port latch bits are set to "0."

The following Special Function Registers must be manipulated in order to use the input/output ports:

Port 1 (P1)		P1		P1DDR		P1FCR

Port 3 (P3)		P3		P3DDR		P3INT		EXT

Port 7 (P7)		P7	(input only)

Note:

- Caution is required when reading port data from an I/O port, because some instructions read the port latch, and some instructions read the data that is being applied to the port. The instructions listed below read the port latch data. Refer to the diagram below.

BPC, DBNZ, INC, DEC, SET1, CLR1, NOT1

| Fig. 3-1-1. Data Path When Executing BPC, DBNZ, INC, DEC, SET1, CLR1, and NOT1 |  | Fig. 3-1-2. Data Path When Executing an Instruction Other Than BPC, DBNZ, INC, DEC, SET1, CLR1, and NOT1 |
| --- | --- | --- |

    - Port 1

Port 1 can be used for input/output for the VMS serial interface, and for pulse generator output. Port 1 can also be used for input/output for the next-generation game machine interface. Only SIO (P10 to P15) and the pulse generator output (P17) can be used from a game program. Use only bit manipulation instructions to access this register. For details on the SIO output, refer to Chapter 3, section 3.5, "Serial Interface;" for details on the pulse generator output, refer to Chapter 3, section 3.3, "Timer 1 (T1)."

- Port 1 Latch (P1)

| Port 1 | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | P17 | P16 | P15 | P14 | P13 | P12 | P11 | P10 |
| Function | Pulse Output | TEST | SCK1 | SB1 | SO1 | SK0 | SB0 | SO0 |

- Port 1 Data Direction Register (P1DDR)

The Port 1 Data Direction Register is a write-only register that corresponds to each data latch bit.    It is essential to note that if a bit manipulation instruction, the INC instruction, the DEC instruction, or the DBNZ instruction is used on a write-only register, all bits other than the specified bit will be set to "1."    The following instructions are used on P1DDR:

	MOV		MOV @

	ST		ST @

	POP

| Symbol | Address | R/W | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PIDDR | 145H | W | P17DDR | P16DDR | P15DDR | P14DDR | P13DDR | P12DDR | P11DDR | P10DDR |
| After a reset |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit name | Function |
| --- | --- |
| P17DDR (bit 7) to P10DDR (bit 0) | I/O control |
|  | 0: Input mode 1: Output mode |

P17DDR (bit 7):	P17 I/O control

|

P10DDR (bit 0): 	P10 I/O control

These bits switch the data I/O direction for each bit of port 1 (P17 to P10) between output mode (1) and input mode (0). When a bit is set to "1," the corresponding bit from P17 to P10 enters output mode; when a bit is set to "0," the corresponding bit from P17 to P10 enters input mode.

Example: When P17DDR = 1, P17 is in output mode.

- Port 1 Function Control Register (P1FCR)

The Port 1 Function Control Register is a write-only register. It is essential to note that if a bit manipulation instruction, the INC instruction, the DEC instruction, or the DBNZ instruction is used on a write-only register, all bits other than the specified bit will be set to "1." The following instructions are used on P1FCR:

	MOV		MOV @

	ST		ST @

	POP

| Symbol | Address | R/W | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PIFCR | 146H | W | P17FCR | P16FCR | P15FCR | P14FCR | P13FCR | P12FCR | P11FCR | P10FCR |
| After a reset |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit name | Function |
| --- | --- |
| P17FCR (bit 7) | P17 control function |
|  | 0: Port data (P17) output 1: PWM output |
| P16FCR (bit 6) | P16 control function |
|  | 0: Port data (P16) output 1: Buzzer (BUZ) output |
| P15FCR (bit 5) | P15 control function |
|  | 0: Port data (P15) output 1: Serial interface clock (SCK1) output |
| P14FCR (bit 4) | P14 control function |
|  | 0: Port data (P14) output 1: Serial interface data (SB1) input/output |
| P13FCR (bit 3) | P13 control function |
|  | 0: Port data (P13) output 1: Serial interface data (SO1) output |
| P12FCR (bit 2) | P12 control function |
|  | 0: Port data (P12) output 1: Serial interface clock (SCK0) output |
| P11FCR (bit 1) | P11 control function |
|  | 0: Port data (P11) output 1: Serial interface data (SB0) input/output |
| P10FCR (bit 0) | P10 control function |
|  | 0: Port data (P10) output 1: Serial interface data (SO0) output |

P17FCR (bit 7):		P17 function selection

This pin selects either PWM (1) or port data (0) for the P17 output. When this bit is set to "1," P17 outputs the logical sum of the PWM signal and the port latch data.    When this bit is set to "0," P17 outputs the port latch data.

P16FCR (bit 6):		P16 function selection

This pin selects either the buzzer (1) or port data (0) for the P16 output. When this bit is set to "1," P16 outputs the logical sum of the buzzer signal and the port latch data. When this bit is set to "0," P16 outputs the port latch data.

P15FCR (bit 5):		P15 function selection

This pin selects either the serial clock (1) or port data (0) for the P15 output. When this bit is set to "1," P15 outputs the logical sum of the serial interface clock (SCK1) and the port latch data. When this bit is set to "0," P15 outputs the port latch data.

P14FCR (bit 4):		P14 function selection

This pin selects either serial data (1) or port data (0) for the P14 output. When this bit is set to "1," P14 outputs the logical sum of the serial interface data (SB1) and the port latch data. When this bit is set to "0," P14 outputs the port latch data.

Note that serial interface data can always be input.

P13FCR (bit 3):		P13 function selection

This pin selects either serial data (1) or port data (0) for the P13 output. When this bit is set to "1," P13 outputs the logical sum of the serial interface data (SO1) and the port latch data. When this bit is set to "0," P13 outputs the port latch data.

P12FCR (bit 2):		P12 function selection

This pin selects either the serial clock (1) or port data (0) for the P12 output.    When this bit is set to "1," P12 outputs the logical sum of the serial interface clock (SCK0) and the port latch data.    When this bit is set to "0," P12 outputs the port latch data.

P11FCR (bit 1):		P11 function selection

This pin selects either serial data (1) or port data (0) for the P11 input/output.    When this bit is set to "1," P11 outputs the logical sum of the serial interface data (SB0) and the port latch data. In bus mode, however, this bit is an input/output (SB0). When this bit is set to "0," P11 outputs the port latch data.

Note that serial interface data can always be input.

P10FCR (bit 0):		P10 function selection

This pin selects either serial data (1) or port data (0) for the P13 output. When this bit is set to "1," P10 outputs the logical sum of the serial interface data (SO0) and the port latch data. When this bit is set to "0," P10 outputs the port latch data.

Note on Writing Game Programs for the VMS:

Game programs for the VMS must perform the following processing:

	When the VMS is operating on a standalone basis (SIO is not being used)

1.	Monitor the 5V detection in port 7.

2.	Once the 5V signal has been detected, change bits 2 and 5 of port 1 to port data output mode, and then output a "0" (zero) from each bits.

Once the above processing is completed, restore the settings that were in effect before the above processing was performed.

Notes:

- When using one of Port 1's independent functions, it is necessary to set the port latch that corresponds to that function to "0." For example, when using PWM, set P17 = 0.

- The port latch data is read by the following instructions: BPC, DBNZ, INC, DEC, SET1, CLR1 and NOT1.    Other instructions read the data that is applied to the port.

Fig. 3-1-3.    Port 1 Block Diagram

    - Port 3

Port 3 is an input-only port that is used for the VMS direction keys, buttons (A, B, MODE), and VMS SLEEP button.

- Port 3 Latch (P3)

| Symbol | Address | R/W |  | Bit 7 | Bit 6 | Bit 5 |  | Bit 4 |  | Bit 3 |  | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P3 | 14CH | R/W |  | P37 | P36 | P35 |  | P34 |  | P33 |  | P32 | P31 | P30 |
| Function |  |  | SLEEP |  | MODE | Button B | Button A |  | RIGHT |  | LEFT |  | DOWN | UP |
| After reset |  |  | 0 |  | 0 | 0 | 0 |  | 0 |  | 0 |  | 0 | 0 |

Because bits 0 through 7 of port 3 are programmable and can be pulled up through software, it is necessary for the user program to substitute ones for P3. when a button is pressed, the corresponding bit goes to "0."

Note:

Regarding the direction keys, design game programs so that opposing directions (up-down, left-right) cannot be pressed simultaneously.

- Port 3 Data Direction Register (P3DDR)	14DH

The above register is set by the internal system program.

This register cannot be accessed from a game program.

- Port 3 Interrupt Control Register (P3INT)

| Symbol | Address | R/W |  | Bit 7 | Bit 6 | Bit 5 |  | Bit 4 |  | Bit 3 |  | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P3INT | 14EH | R/W |  | - | - | - |  | - |  | - |  | P32INT | P31INT | P30INT |
| After reset |  |  | H |  | H | H | H |  | H |  | 0 |  | 0 | 0 |

| Bit name | Function |
| --- | --- |
| P32INT (bit 2) | Port 3 interrupt control flag |
|  | 0: Prohibits cancellation of HOLD mode when an interrupt is generated by port 3. 1: Enables cancellation of HOLD mode when an interrupt is generated by port 3. |
| P31INT (bit 1) | Port 3 interrupt source flag |
|  | 0: No interrupt source 1: Interrupt source |
| P30INT (bit 0) | Port 3 interrupt request flag |
|  | 0: Interrupt request prohibited 1: Interrupt request enabled |

P32INT (bit 2):	Port 3 interrupt selection flag

This flag selects whether port 3 is to be used for interrupt source generation or not.

P32INT = 0: Does not generate interrupts on port 3.

P32INT = 1: Generates interrupts on port 3.

P31INT (bit 1):	Port 3 interrupt source flag

This flag has meaning only when the P32INT flag is set. This flag is used to monitor the presence/absence of port 3 interrupt sources.

This flag is set (1) when an interrupt source is generated on port 3, and does not change if no interrupt source is generated. This flag must be reset by software.

P30INT (bit 0):	Port 3 interrupt request enable control

This bit enables (1)/disables (0) interrupt requests due to the generation of an interrupt source on port 3.

P30INT = 0:	Disables interrupt requests on port 3.

P30INT = 1:	Enables interrupt requests to vector address 004BH when an interrupt source is generated on port 3 (when P31INT = 1).

Fig. 3-1-4.    Port 3 Block Diagram

    - Port 7

Port 7 is an input-only port that is used for checking low voltage in the VMS and for checking the connection with the next-generation game machine. (Refer to Fig. 3-1-8.)

- Port 7 (P7)

| Symbol | Address | R/W |  | Bit 7 | Bit 6 | Bit 5 |  | Bit 4 |  | Bit 3 |  | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P7 | 15CH | R |  | - | - | - |  | - |  | P73 |  | P72 | P71 | P70 |
| Function |  |  | - |  | - | - | - |  | ID1 |  | ID0 |  | Low voltage | 5V detection |
| After reset |  |  | H |  | H | H | H |  | 0 |  | 0 |  | 1 | 0 |

internal pull-up resistor transistors set the initial input state of bits 7 through 0 of port 7 to "1." When a button is pressed, the corresponding bit goes to "0."

Fig. 3-1-8.    Port 7 Block Diagram