  - Flash EEPROM

    - Overview

POTATO has an internal 128K flash EEPROM (Electrically Erasable Programmable ROM) that can operate with a single power supply.

    - Functions

- Programming and read operations possible with a single power supply

- Capacity: 131072  8 bits: Data area

- Programmable/erasable in block (page) units

1 block = 128 bytes (= 1 page)

- Number of times overwriting is possible

50,000 times/page (Ta = 25°C)  (with memory management by program)

- Built-in step-up voltage circuit for writing

- Overwriting end detection function (detected within subroutine call in the internal BIOS program)

Toggle bit method

Data polling method

- Batch erase of software possible

    - Accessing the Data Area EEPROM

The data area can be written by using a PROM writer, and can be read and written by calling a subroutine in the microcontroller's internal BIOS program.

      - Reading/Writing Data in a Program

Loading the internal BIOS programs makes it possible to easily access the data area EEPROM.

BIOS programs that are used:	FM_WRT_EX (writes data to the data area EEPROM)

FM_VRF_EX (Verifies writing data in the data area EEPROM)

FM_PRD_EX (reads the data in the data area EEPROM)

Data area EEPROM space:	128  8 bits  1024 pages

When accessing (reading or writing) the data area EEPROM, load and use the target internal BIOS programs from within the program as subroutine calls ("callr" instruction and "callf" instruction).

- FM_WRT_EX

This subroutine is loaded in order to write data to the data area EEPROM.

Loading this subroutine writes one page (128  8 bits). The following settings must be made beforehand in order to load this subroutine.

(Items that should be set beforehand)

1	Specification of the EEPROM write start address (17-bit specification)

Lower address (8 bits):	Specify in 7FH in RAM (Bank-1).

Upper address (8 bits):	Specify in 7EH in RAM (Bank-1).

Bank address (1 bit):	Specify in bit 0 of 7DH in RAM (Bank-1).

When setting the lower address, remember that it is not possible to write data so that it spans two pages.

Because this writing operation is performed one page (128 bytes) at a time, specify zeroes ("0") for bits 0 through 6 of the lower address.

2.	Setting the data that is to be written in EEPROM

	Set the data that is to be written in EEPROM in RAM at addresses 80H through 0FFH ahead of time.

3.	Specification of the method for detecting the end of the EEPROM writing operation

	There are two methods for detecting the end of the EEPROM writing operation; specify which of those methods is to be used.

Toggle bit method:	Bit 0 of 7CH in RAM (Bank-1) = 0

Data polling method:	Bit 0 of 7CH in RAM (Bank-1) = 1

	Specify which of the above two methods is to be used through bit 0 of 7CH in RAM.

- FM_VRF_EX

This subroutine is used to verify that the data that was written in the data area EEPROM was written correctly.

Loading this subroutine compares one page of data (128 x 8 bits) written in EEPROM with the original data (the data in addresses 80H through 0FFH in RAM (Bank-1)).

	When all 128 bytes are correct:	Accumulator (ACC) = 00H

	When even one of the 128 bytes is correct:	Accumulator (ACC)  00H

This subroutine must be executed after the data has been completely written to EEPROM, but before the data in addresses 80H through 0FFH in ROM has been overwritten.

(Necessary data for loading)

1.	Specification of the EEPROM read start address (17-bit specification)

(1) Lower address (8 bits):	Specify in 7FH in RAM (Bank-1).

(2) Upper address (8 bits):	Specify in 7EH in RAM (Bank-1).

(3) Bank address (1 bit):	Specify in bit 0 of 7DH in RAM (Bank-1).

When setting the lower address, remember that it is not possible to read data spanning two pages.

2.	EEPROM data and the comparison data

	This subroutine verifies that the data that was written in EEPROM was written correctly. The data that is compared to the EEPROM data is the data in addresses 80H through 0FFH in RAM (Bank-1), starting from the data in address 80H.

3.	End of the EEPROM reading operation

(When comparison results do not match

When the lower 7 bits of the address are "7FH"

- FM_PRD_EX

This subroutine is used to read data from the data area EEPROM.

Loading this subroutine reads one page (128 x 8 bits). The following settings must be made beforehand in order to load this subroutine.

(Items that should be set beforehand)

1.	Specification of the EEPROM read start address (17-bit specification)

(1) Lower address (8 bits):	Specify in 7FH in RAM (Bank-1).

(2) Upper address (8 bits):	Specify in 7EH in RAM (Bank-1).

(3) Bank address (1 bit):	Specify in bit 0 of 7DH in RAM (Bank-1).

When setting the lower address, remember that it is not possible to read data that spans two pages.

Because this reading operation is performed one page (128 bytes) at a time, specify zeroes ("0") for bits 0 through 6 of the lower address.

2.	Data that was read from EEPROM

The data that is read from EEPROM is written in addresses 080H through 0FFH in RAM (Bank-1).

Note:

When accessing the data area EEPROM, disable all interrupts before making subroutine calls of the microcomputer's internal OS program. In some cases interrupts cannot be accepted while accessing the data area EEPROM, which can lead to misoperation.

    - Accessing the Program Area EEPROM

The program area is created in the EEPROM. As a rule, when POTATO is shipped, the entire program area is filled with "FFH." The method provided for the user in order to write program data in the program area is described below.

      - Writing with a PROM Writer

A third-party PROM writer and Sega's Conversion Board (W86F8716Q) can be used to easily write the program area.

(Refer to Chapter 3, section 3.10.3.2, "Writing/Reading with a PROM Writer.")

Note that using this method after the board has been installed does carry a risk of causing problems with other circuits.