  - Port Interrupt Functions

    - Overview

In addition to its digital I/O function, port 3 can be used to generate interrupts or release HOLD mode. This function can be used to implement a "key-on wakeup" function that releases HOLD mode when a key switch is pressed.

A port interrupt can be implemented through port 3.

    - Function

In addition to its digital I/O function, port 3 also has the following functions:

- Generates an interrupt when it detects a low-level signal.

- Releases HOLD mode when it detects a low-level signal.

After HOLD mode is released, the internal RC oscillation is adopted for the system clock.

In order to use the port interrupt function, it is necessary to manipulate the Special Function Registers shown below.

For port 3 interrupt:

	P3		P3DDR		P3INT

	IE

    - Circuit Configuration

Fig. 3-8-1.  Port 3 Interrupt Circuit Block Diagram

    - Related Registers

- Port 3 Interrupt Control Register (P3INT)

For details, refer to Chapter 3, section 3.1.2, "Port 3 Interrupt Control Register."

| Symbol | Address | R/W | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P3INT | 14EH | R/W | - | - | - | - | - | P32INT | P31INT | P30INT |
| After reset |  |  | H | H | H | H | H | 0 | 0 | 0 |

| Bit name | Function |
| --- | --- |
| P32INT (bit 2) | Port 3 interrupt control flag |
|  | 0: Interrupts through port 3 and HOLD mode release through port 3 disabled. 1: Interrupts through port 3 and HOLD mode release through port 3 enabled. |
| P31INT (bit 1) | Port 3 interrupt source flag |
|  | 0: No interrupt source 1: Interrupt source |
| P30INT (bit 0) | Port 3 interrupt request enable |
|  | 0: Interrupt request enabled. 1: Interrupt request disabled. |

- Master interrupt enable control register (IE)

For details, refer to chapter 3, section 3.8.3, "Master Interrupt Enable Control Register."

| Symbol | Address | R/W | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IE | 108H | R/W | IE7 | - | - | - | - | - | IE1 | IE0 |
| After reset |  |  | 0 | H | H | H | H | H | 0 | 0 |

| Bit name | Function |  |  |  |
| --- | --- | --- | --- | --- |
| IE7 (bit 7) | Master interrupt enable control (high level, low level) |  |  |  |
|  | 0: All interrupt requests disabled. 1: All interrupt requests enabled. |  |  |  |
| IE1 (bit 1) | INT0, 1 interrupt priority control |  |  |  |
| IE0 (bit 0) | IE1 | IE0 | INT1 priority level | INT0 priority level |
|  | 0 1 X | 0 0 1 | Highest level Low level Low level | Highest level Highest level Low level |

    - Description of Operation

- Port 3 interrupts

¬	Set bit 2 of the port 3 interrupt control register (P3INT) to "1." This selects the port 3 interrupt.

Through a Special Function Register, select the pin in port 3 (P37 through P30) on which the low-level signal is to be detected.

The following conditions must be met in order to accept a port 3 interrupt:

- The corresponding bit in the Port 3 Control Register (P3DDR) must be set to input mode.

P3mDDR = 0	(m = 0 to 7)

- The corresponding bit in the Port 3 Register (P3) must be set.

P3n = 1	(n = 0 to 7)

®	If a low-level signal is detected, the interrupt source is set to "1." If the interrupt request enable flag has been set, an interrupt request is generated, and if the master interrupt enable flag has been set, control branches to vector address 004BH.

¯	If the two conditions described in item 2 above are met while in HALT mode, HALT mode is released and control branches to vector address 004BH.

°	If the two conditions described in item 2 above are met while in HOLD mode, HOLD mode is released and control branches to vector address 004BH. In this case, the internal RC oscillation is selected for the system clock.

    - State Transitions

- State transitions in HOLD mode

This flowchart applies to port 3 interrupts.

Note:

- When releasing HOLD mode through P3, set other individual interrupt request enable flags to "0."

- HALT mode in state transition

This flowchart applies to port 3 interrupts.

    - Program Example

This example applies to port 3 interrupts.

- Program

This program releases HOLD mode without branching to an interrupt routine when a low-level signal is detected on P37.

- Example application circuit

Fig. 3-8-2.  Application Circuit Example