  - Subroutine Instructions

CALL  a12  (Near absolute subroutine call)

| Instruction code | 0 0 0a11 1a10a9a8 a7a6a5a4a3a2a1a0 08H0FH,18H~1FH |
| --- | --- |
| Number of bytes | 2 |
| Number of cycles | 2 |
| Function | (PC)  (PC) + 2,(SP) (SP) + 1,((SP)) (PC7~0),(SP) (SP) + 1, ((SP)) (PC15~8),(PC110)  a12 |
| Affected flags |  |
| Interrupt acceptance | Permitted at 2nd cycle |

Description:

This instruction increments the Program Counter (PC) twice, increments the Stack Pointer (SP), and then stores the lower byte of the PC in the address in data memory (RAM) that is specified by the SP. This instruction then increments the Stack Pointer (SP) again, and stores the upper byte of the PC in the address in RAM specified by the SP. Finally, this instruction then transfers the data a11 through a0 to bits 11 through 00 of the PC.

Example 1:

The value of label LA is 0F0EH.

|  |  |  | PC | Instruction code | SP | RAM 20H | RAM 21H |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | MOV | #01FH,SP | 0FFAH | 23061FH | 1FH | - | - |
|  | CALL | LA | 0FFDH | 1F0EH | 21H | FFH | 0FH |
| LA: | INC | ACC | 0FOEH | 6300H | 21H | FFH | 0FH |
|  | RET |  | 0F10H | A0H | 1FH | FFH | 0FH |
|  | NOP |  | 0FFFH | 00H | 1FH | FFH | 0FH |

Example 2:

The value of label LA is 1F0EH.

|  |  |  | PC | Instruction code | SP | RAM 20H | RAM 21H |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | MOV | #01FH,SP | 0FFBH | 23061FH | 1FH | - | - |
|  | CALL | LA | 0FFEH | 1F0EH | 21H | 00H | 10H |
| LA: | INC | ACC | 1F0EH | 6300H | 21H | 00H | 10H |
|  | RET | 1F10H | A0H | 1FH | 00H | 10H |  |
|  | INC | ACC | 1000H | 6300H | 1FH | 00H | 10H |

CALLF  a16  (Far absolute subroutine call)

| Instruction code | 0 0 1 0 0 0 0 0 a15a14a13a12a11a10a9a8 a7a6a5a4a3a2a1a0 20H |
| --- | --- |
| Number of bytes | 3 |
| Number of cycles | 2 |
| Function | (PC)  (PC) + 3,(SP)←(SP) + 1,((SP)) (PC7~0),(SP)(SP) + 1, ((SP))(PC158),(PC)a16 |
| Affected flags |  |
| Interrupt acceptance | Permitted at 2nd cycle |

Description:

This instruction increments the Program Counter (PC) three times, increments the Stack Pointer (SP), and then stores the lower byte of the PC in the address in data memory (RAM) that is specified by the SP. This instruction then increments the Stack Pointer (SP) again, and stores the upper byte of the PC in the address in RAM specified by the SP. Finally, this instruction then transfers the data a15 through a0 to bits 15 through 00 of the PC.

Example 1:

The value of label LA is 0F0EH.

|  |  |  | PC | Instruction code | SP | RAM 20H | RAM 21H |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | MOV | #01FH,SP | 0FF9H | 23061FH | 1FH | - | - |
|  | CALLF | LA | 0FFCH | 200F0EH | 21H | FFH | 0FH |
| LA: | INC | ACC | 0F0EH | 6300H | 21H | FFH | 0FH |
|  | RET |  | 0F10H | A0H | 1FH | FFH | 0FH |
|  | NOP |  | 0FFFH | 00H | 1FH | FFH | 0FH |

Example 2:

The value of label LA is 0F0EH.

|  |  |  | PC | Instruction code | SP | RAM 20H | RAM 21H |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | MOV | #01FH,SP | 0FFAH | 23061FH | 1FH | - | - |
|  | CALLF | LA | 0FFDH | 200F0EH | 21H | 00H | 10H |
| LA: | INC | ACC | 0F0EH | 6300H | 21H | 00H | 10H |
|  | RET |  | 0F10H | A0H | 1FH | 00H | 10H |
|  | INC | ACC | 1000H | 6300H | 1FH | 00H | 10H |

CALLR  r16  (Far relative subroutine call)

| Instruction code | 0 0 0 1 0 0 0 0 r7r6r5r4r3r2r1r0 r15r14r13r12r11r10r9r8 10H |
| --- | --- |
| Number of bytes | 3 |
| Number of cycles | 4 |
| Function | (PC)  (PC) + 3,(SP) (SP) + 1,((SP))(PC7~0),(SP) (SP) + 1, ((SP))(PC15~8),(PC)(PC)  1 + r16 |
| Affected flags |  |
| Interrupt acceptance | Permitted at 4th cycle |

Description:

This instruction increments the Program Counter (PC) three times, increments the Stack Pointer (SP), and then stores the lower byte of the PC in the address in data memory (RAM) that is specified by the SP. This instruction then increments the Stack Pointer (SP) again, and stores the upper byte of the PC in the address in RAM specified by the SP. Finally, this instruction then decrements the PC, adds the data r15 through r0 to the contents of the PC, and transfers the result to the PC.

Example 1:

The value of label LA is 1100H.

|  |  |  | PC | Instruction code | SP | RAM 20H | RAM 21H |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | MOV | #01FH,SP | 0FF9H | 23061FH | 1FH | - | - |
|  | CALLR | LA | 0FFCH | 100201H | 21H | FFH | 0FH |
| LA: | INC | ACC | 1100H | 6300H | 21H | FFH | 0FH |
|  | RET |  | 1102H | A0H | 1FH | FFH | 0FH |
|  | NOP |  | 0FFFH | 00H | 1FH | FFH | 0FH |

Example 2:

The value of label LA is 1100H.

|  |  |  | PC | Instruction code | SP | RAM 20H | RAM 21H |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | MOV | #01FH,SP | 0FFCH | 23061FH | 1FH | - | - |
|  | CALLR | LA | 0FFDH | 100101H | 21H | 00H | 10H |
| LA: | INC | ACC | 1100H | 6300H | 21H | 00H | 10H |
|  | RET |  | 1102H | A0H | 1FH | 00H | 10H |
|  | INC | ACC | 1000H | 6300H | 1FH | 00H | 10H |

RET (Return for subroutine)

| Instruction code | 1 0 1 0 0 0 0 0 A0H |
| --- | --- |
| Number of bytes | 1 |
| Number of cycles | 2 |
| Function | (PC15~8)  ((SP)),(SP)(SP)  1,(PC7~0)((SP)),(SP)(SP)  1 |
| Affected flags |  |
| Interrupt acceptance | Permitted at 2nd cycle |

Description:

This instruction transfers the contents of the address in data memory (RAM) that is specified by the Stack Pointer (SP) to the upper byte of the Program Counter (PC). This instruction then decrements the SP, transfers the contents of the address in RAM that is specified by the SP to the lower byte of the Program Counter (PC), and then decrements the SP again.

Example 1:

The value of label LA is 0F0EH.

|  |  |  | PC | Instruction code | SP | RAM 20H | RAM 21H |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | MOV | #01FH,SP | 0FF9H | 23061FH | 1FH | - | - |
|  | CALLF | LA | 0FFCH | 200F0EH | 21H | FFH | 0FH |
| LA: | INC | ACC | 0F0EH | 6300H | 21H | FFH | 0FH |
|  | RET |  | 0F10H | A0H | 1FH | FFH | 0FH |
|  | NOP |  | 0FFFH | 00H | 1FH | FFH | 0FH |

Example 2:

The value of label LA is 0F0EH.

|  |  |  | PC | Instruction code | SP | RAM 20H | RAM 21H |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | MOV | #01FH,SP | 0FFAH | 23061FH | 1FH | - | - |
|  | CALLF | LA | 0FFDH | 200F0EH | 21H | 00H | 10H |
| LA: | INC | ACC | 0F0EH | 6300H | 21H | 00H | 10H |
| RET |  |  | 0F10H | A0H | 1FH | 00H | 10H |
|  | INC | ACC | 1000H | 6300H | 1FH | 00H | 10H |

RETI(Return for interrupt)

| Instruction code | 1 0 1 1 0 0 0 0 B0H |
| --- | --- |
| Number of bytes | 1 |
| Number of cycles | 2 |
| Function | (PC15~8)  ((SP)),(SP)(SP)  1,(PC7~0)((SP)),(SP)(SP)  1 |
| Affected flags |  |
| Interrupt acceptance | Not permitted |

Description:

This instruction transfers the contents of the address in data memory (RAM) that is specified by the Stack Pointer (SP) to the upper byte of the Program Counter (PC). This instruction then decrements the SP, transfers the contents of the address in RAM that is specified by the SP to the lower byte of the Program Counter (PC), decrements the SP again, and then restarts the interrupt acceptance function that was disabled when an interrupt was accepted.

Example 1:

|  |  | PC | Instruction code |  |
| --- | --- | --- | --- | --- |
| NOP |  | 0FFAH | 00H |  |
| NOP |  | 0FFBH | 00H |  |
| MOV | #001H,ACC | 0FFCH | 230001H | External interrupt 0 generated |
| INC | ACC | 0003H | 6300H |  |
| RET1 |  | 0005H | B0H |  |
| NOP |  | 0FFFH | 00H |  |

Example 2:

|  |  | PC | Instruction code |  |
| --- | --- | --- | --- | --- |
| NOP |  | 0FFCH | 00H |  |
| MOV | #00EH,B | 0FFDH | 23020EH | External interrupt 1 generated |
| INC | ACC | 0013H | 6300H |  |
| RET1 |  | 0015H | B0H |  |
| INC | ACC | 1000H | 6300H |  |

  - Bit Manipulation Instructions

CLR1  d9,b3  (Clear direct bit)

| Instruction code | 1 1 0d8 1b2b1b0 d7d6d5d4d3d2d1d0 C8H~CFH,D8H~DFH |
| --- | --- |
| Number of bytes | 2 |
| Number of cycles | 1 |
| Function | (d9,b3)  0 |
| Affected flags |  |
| Interrupt acceptance | Permitted |

Description:

This instruction resets the bit specified by b2 to b0 of the address in data memory (RAM) or the Special Function Register that is specified by d8 to d0.

Example 1

|  |  | ACC |  |  |
| --- | --- | --- | --- | --- |
| MOV | #001H,ACC | 01H | 0000 | 0001B |
| CLR1 | ACC,0 | 00H | 0000 | 0000B |

Example 2:

|  |  | RAM 7FH |  |  |
| --- | --- | --- | --- | --- |
| MOV | #001H,07FH | 01H | 0000 | 0001B |
| CLR1 | 07FH,0 | 00H | 0000 | 0000B |

Notes:

- When this instruction is applied to one of the ports P1 or P3, the port latch of that port is selected; the external signal that is applied to that port is not selected. Furthermore, applying this instruction to port P7 does not change its status.

SET1  d9,b3  (Set direct bit)

| Instruction code | 1 1 1d8 1b2b1b0 d7d6d5d4d3d2d1d0 E8H~EFH, F8H~FFH |
| --- | --- |
| Number of bytes | 2 |
| Number of cycles | 1 |
| Function | (d9,b3)  1 |
| Affected flags |  |
| Interrupt acceptance | Permitted |

Description:

This instruction sets the bit specified by b2 to b0 of the address in data memory (RAM) or the Special Function Register that is specified by d8 to d0.

Example 1:

|  |  | ACC |  |  |
| --- | --- | --- | --- | --- |
| MOV | #000H,ACC | 00H | 0000 | 0000B |
| SET1 | ACC,7 | 80H | 1000 | 0000B |

Example 2:

|  |  | RAM 7FH |  |  |
| --- | --- | --- | --- | --- |
| MOV | #001H,07FH | 01H | 0000 | 0001B |
| SET1 | 07FH,6 | 41H | 0100 | 0001B |

Notes:

- When this instruction is applied to one of the ports P1 or P3, the port latch of that port is selected; the external signal that is applied to that port is not selected. Furthermore, applying this instruction to port P7 does not change its status.

NOT1  d9,b3  (Not direct bit)

| Instruction code | 1 0 1d8 1b2b1b0 d7d6d5d4d3d2d1d0 A8H~AFH, B8H~BFH |
| --- | --- |
| Number of bytes | 2 |
| Number of cycles | 1 |
| Function | (d9,b3)  (d9,b3) |
| Affected flags |  |
| Interrupt acceptance | Permitted |

Description:

This instruction inverts the bit specified by b2 to b0 of the address in data memory (RAM) or the Special Function Register that is specified by d8 to d0.

Example 1:

|  |  | ACC |  |  |
| --- | --- | --- | --- | --- |
| MOV | #000H,ACC | 00H | 0000 | 0000B |
| NOT1 | ACC,7 | 80H | 1000 | 0000B |
| NOT1 | ACC,7 | 00H | 0000 | 0000B |

Example 2:

|  |  | RAM 7FH |  |  |
| --- | --- | --- | --- | --- |
| MOV | #001H,07FH | 01H | 0000 | 0001B |
| NOT1 | 07FH,6 | 41H | 0100 | 0001B |
| NOT1 | 07FH,6 | 01H | 0000 | 0001B |

Notes:

- When this instruction is applied to one of the ports P1 or P3, the port latch of that port is selected; the external signal that is applied to that port is not selected. Furthermore, applying this instruction to port P7 does not change its status.

  - Miscellaneous Instruction

NOP  (No operation)

| Instruction code | 0 0 0 0 0 0 0 0 00H |
| --- | --- |
| Number of bytes | 1 |
| Number of cycles | 1 |
| Function |  |
| Affected flags |  |
| Interrupt acceptance | Permitted |

Description:

This instruction consumes one machine cycle.

  - Macro Instruction

CHANGE   label name (or address)

Description:

This is POTATO's own macro instruction.

(1)		When executed in internal program mode

	Switches from internal program mode  external program mode

	The program counter is set to the external program address specified by the label or address.

(2)	When executed in external program mode

	Switches from external program mode  internal program mode (when LDCEXT = 0)

	The program counter is set to the internal program address specified by the label or address.

(Note:	Even if the CHANGE instruction is executed in external program mode when LDCEXT = 1, the system does not enter internal program mode. In actuality, it jumps to the address that was specified by the CHANGE instruction in the external program.)

(3)	The program mode switch is made after executing any special macro instructions.

(4)	Interrupts are not accepted while this macro is executing.