  - VMS Work RAM

    - Overview

There are two banks of 256 bytes provided as communications buffers in the new-generation game machine dedicated interface. As long as a data transfer is not being performed on the LM-BUS, these buffers can also be accessed as work RAM.

Whether a data transfer is being conducted with the new generation game machine can be determined by referencing the ASEL flag in the VSEL register. (A data transfer is in progress when ASEL = "1.") Note that a normal data transfer cannot be guaranteed if this work RAM is accessed while a data transfer is being performed with a new generation game machine.

    - Work RAM Control Registers

VMS Control Register (VSEL)

| Symbol | Address | R/W | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VSEL | 163H | R/W | - | - | - | INCE | - | - | SIOSEL | ASEL |
| After reset |  |  | H | H | H | 0 | H | H | 0 | 0 |

Game programs can access only bit 4. A bit manipulation instruction must be used.

INCE (bit 4):	VTRBF address counter automatic increment

This bit controls the automatic incrementing of the address counter when writing/reading VTRBF from the CPU side.

When this bit is set to "1," the address counter is automatically incremented by one after VTRBF has been accessed from the CPU side. When this bit is set to "0," the current address is saved after access.

SIOSEL (bit 1):	P1 port usage selection control

This bit controls the selection of whether the P1 port (P10 to P15) is to be used as a normal I/O port and as I/O pins for a synchronous serial interface, or as the new game machine dedicated interface. Always set this bit to "0" in game program mode.

ASEL (bit 0):	VTRBF address input select control

This bit controls the selection of access to VTRBF that is used as a buffer for the VMS and the new generation game machine dedicated interface.

Always set this bit to "0" in game program mode. If this bit is set to "1," access from the MPU to VTRBF is not possible.

- Work RAM access address (VRMAD1, 2)

| Symbol | Address | R/W | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VRMAD1 | 164H | R/W | VRMAD7 | VRMAD6 | VRMAD5 | VRMAD4 | VRMAD3 | VRMAD2 | VRMAD1 | VRMAD0 |
| After reset |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Symbol | Address | R/W | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VRMAD2 | 165H | R/W | - | - | - | - | - | - | - | VRMAD8 |
| After reset |  |  | H | H | H | H | H | H | H | 0 |

These registers set the address that is to be accessed from the CPU side in work RAM (VTRBF).

VRMAD1 is the lower 8 bits of the address; VRMAD2 switches the bank.

When the VSEL bit is set to "1," VRMAD is incremented each time that VTRBF is accessed.

- Send/receive buffer (VTRBF)

| Symbol | Address | R/W | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VTRBF | 166H | R/W | VTRBF7 | VTRBF6 | VTRBF5 | VTRBF4 | VTRBF3 | VTRBF2 | VTRBF1 | VTRBF0 |
| After reset |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

This register is used to access the data in the address specified by VRMAD.

When the CPU writes to this register, the data is written to the address specified by VRMAD. when the CPU reads this register, it reads the data from the address that was specified by VRMAD.

If the VSEL bit is set to "1," VRMAD is incremented automatically each time that this register is accessed.

    - Accessing Work RAM

When accessing VMS work RAM, the address that is to be accessed is stored in the VRMAD1 and 2 registers. An application accesses work RAM by storing the address value in VRMAD1 and 2 and then reading or writing the VTRBF register.

It is important to note that the VRMAD1 and 2 registers have an auto-increment function. For details, refer to the next item.

    - Notes on Using the Address Register for Work RAM

Fig. 3-9-1 illustrates an access to work RAM.

When a game program accesses work RAM, the address value is specified by the VRMAD1 and 2 registers.

It is essential to note that if the INCE flag in the VSEL register has been set to "1," the value in VRMAD is automatically incremented each time that game program work RAM is accessed. (No distinction is made between reads and writes.)

When INCE = 1

VTRBF

(work RAM)

VRMAD1, 2

(system address register)

VTRBF

(work RAM)

VTRBF register

MPU

data1

Address value prior to access

data1

data1

Address value after access

(address prior to access + 1)

When INCE = 0

VRMAD1, 2

(system address register)

VTRBF register

MPU

data1

Address value prior to access

Address value after access

(no change)

Fig. 3-9-1.  VMS Work RAM Access