  - System Clock Generation Function

    - Overview

POTATO has three internal oscillation circuits for use as system clock generation circuits: the main clock oscillation circuit, the sub-clock oscillation circuit, and the RC oscillation circuit. Of these, the RC oscillation circuit has an internal resistor (R) and capacitor (C), and does not require any external circuitry. The selection of one of these three clocks as the system clock is made through software.

Note that, in actual practice, battery consumption is high when the main clock oscillation circuit and the RC oscillation circuit are used, so select the sub-clock as the system clock whenever the other circuits are not needed.

    - Functions

- This function generates the system clock, which is the foundation of the execution of instructions by the microcomputer.

- One of two clocks (sub-clock oscillation or RC oscillation) can be selected as the system clock through software. Game programs should not use the main clock ascillation.

- This function generates the base timer clock.

- Main clock oscillation and RC oscillation can be halted by software instructions.
(This makes it possible to conserve battery power.)

- This function generates system clock 1 (S1), which is the foundation for operation of circuit blocks that still operate in HALT mode, and system clock 2 (S2), which is the foundation for operation of circuit blocks that stop operating in HALT mode.

- In HOLD mode, main clock oscillation, sub-clock oscillation, and RC oscillation are all stopped.

In order to control the system clock, it is necessary to manipulate the following Special Function Registers:

- OCR

- PCON

    - Circuit Configuration

Fig. 4-2-1 shows the configuration of the system clock generation circuit.

- Main clock oscillation circuit······ ¬

This circuit is made to oscillate by connecting a ceramic oscillation circuit to the CF1 and CF2 pins. If the main clock is not to be used, connect CF1 to VDD and leave the CF2 pin open.

- Sub-clock oscillation circuit······

This circuit is made to oscillate by connecting a crystal oscillation circuit (32.768kHz typ.) to the XT1 and XT2 pins.

If the sub-clock is not to be used, connect XT1 to VDD and leave the XT2 pin open.

- Internal RC oscillation circuit······ ®

This circuit is made to oscillate by a resistor (R) and capacitor (C) that are built into the microcomputer. After a reset or the release of HOLD, the system runs according to this clock.

- System clock selector······ ¯

Bits 4 and 5 of the Oscillation Control Register (OCR) are used to select either the sub-clock oscillation circuit or the RC oscillation circuit as the system clock source. Game programs should not use the main clock ascillation.

- System clock generation circuit······ °

System clocks 1 and 2 are generated from the clock source that was selected by the system clock selector. System clock 1 (S1) runs when executing instructions and when in HALT mode. System clock 2 (S2) runs when executing instructions. When in HOLD mode, both S1 and S2 stop.

- Oscillation Control Register (OCR)······ ±

This register controls the start and stop of oscillation by the main clock oscillation circuit and the RC oscillation circuit, switches the system clock source, and controls the cycle time.

- Power Control Register (PCON)······ ²

This register sets the standby state (HOLD/HALT mode).

Fig. 4-2-1.  System Clock Generation Circuit Block Diagram

- Status of each block during reset, HALT, and HOLD

Table 4-2-1.  Status of Each Block During Standby

| Block | State |  |  |
| --- | --- | --- | --- |
|  | During reset | During HALT | During HOLD |
| Main clock oscillation circuit | Oscillates | Status when power is suddenly applied | Stopped |
| Internal RC oscillation circuit | Oscillates | Status when power is suddenly applied | Stopped |
| Sub-clock oscillation circuit | Stopped | Status when power is suddenly applied | Stopped |
| System clock oscillation circuit | Running | Running | Stopped |

Note:

- After a reset or after HOLD is released, the internal RC oscillation clock is automatically selected as the system clock.

    - Related Registers

- Oscillation Control Register

| Symbol | Address | R/W | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OCR | 10EH | R/W | OCR7 | - | OCR5 | OCR4 | - | - | OCR1 | OCR0 |
| After reset |  |  | 0 | H | 0 | 0 | H | H | 0 | 0 |

| Bit name | Function |  |  |
| --- | --- | --- | --- |
| OCR7 (bit 7) | System clock generation circuit control |  |  |
|  | 0: Cycle time source is 1/12 of the oscillating frequency 1: Cycle time source is 1/6 of the oscillating frequency |  |  |
| OCR5 (bit 5) OCR4 (bit 4) | System clock selection |  |  |
|  | OCR5 | OCR4 | System clock |
|  | 0 0 1 1 | 0 1 0 1 | Internal RC oscillation Do not use Sub-clock (crystal oscillation) Do not use After reset or HOLD release: RC oscillation |
| OCR1 (bit 1) | Internal RC oscillation circuit control |  |  |
|  | 0: Internal RC oscillation circuit operation start/in progress 1: Internal RC oscillation circuit stopped |  |  |
| OCR0 (bit 0) | Main clock oscillation circuit control |  |  |
|  | 0: Main clock oscillation circuit operation start/in progress 1: Main clock oscillation circuit stopped |  |  |

OCR7 (bit 7):	System clock generation circuit control

This bit controls whether the cycle time is to be 1/12 of the source oscillation frequency, or 1/6. When this bit is set to "1," the cycle time is implemented as 1/6 of the source oscillation frequency; when this bit is set to "0," the cycle time is implemented as 1/12 of the source oscillation frequency.

In the VMS, this bit should be set as shown below:

* Be sure to set '1' when using the sub-clock.

| System clock | OCR7 |
| --- | --- |
| Main clock (CF oscillation) | OCR7=1 |
| Internal RC oscillation | OCR7=0/1 |
| Sub-clock (crystal oscillation) | OCR7=1 |

OCR5 (bit 5):	System clock selection

OCR4 (bit 4):	These bits select the system clock. After a reset or after the release of HOLD mode, internal RC oscillation is selected automatically.

| OCR5 | OCR4 | System clock |
| --- | --- | --- |
| 0 | 0 | Internal RC oscillation |
| 0 | 1 | Do not use |
| 1 | 0 | Sub-clock (crystal oscillation) |
| 1 | 1 | Do not use |

OCR1 (bit 1):	Internal RC oscillation circuit control

This bit stops (1)/starts the internal RC oscillation circuit. When this bit is set to "1," the internal RC oscillation circuit stops; when this bit is set to "0," the internal RC oscillation circuit starts or continues to run.

OCR0 (bit 0):	Main clock oscillation circuit control

This bit stops (1)/starts the main clock oscillation circuit. The main clock is not used by the VMS, so always set this bit to '1'.

Notes:

- An adequate amount of time must be provided when starting the oscillation of the main clock. When the main clock is stopped, the RC clock is running, the RC (or sub-) clock is selected as the system clock, and you wish to switch the system clock to the main clock, start the main clock first and then wait an adequate amount of time before actually switching the clock.

- An adequate amount of time at least equal to the time required for starting the main clock (200us) must be provided when starting the oscillation of the sub-clock (200us). When switching the system clock source from RC oscillation to the sub-clock after releasing a reset or releasing HOLD mode, start sub-clock oscillation first and then wait an adequate amount of time before actually switching the clock.

- Power Control Register (PCON)

| Symbol | Address | R/W | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PCON | 107H | R/W | - | - | - | - | - | - | PCON1 | PCON0 |
| After reset |  |  | H | H | H | H | H | H | 0 | 0 |

| Bit name | Function |
| --- | --- |
| PCON1 (bit 1) | HOLD mode control |
|  | 0: 1: Set HOLD mode |
| PCON0 (bit 0) | HALT mode control |
|  | 0: 1: Set HALT mode |

PCON1 (bit 1):	HOLD mode control

This bit selects the standby state. When this bit is set to "1," the microcomputer enters HOLD mode and, once all oscillation circuits have stopped, the system stops. When HOLD mode is released, this bit is automatically reset. Note that setting this bit to "0" does not change the standby state.

There are three methods for releasing HOLD mode:

- Reset

- Applying the specified level to the P70/INT0 or P71/INT1 pin

- Port 3 interrupt source

PCON0 (bit 0):	HALT mode control

This bit selects the standby state. When this bit is set to "1," the microcomputer enters HALT mode, the program stops at the address where the HALT was executed, and the oscillation circuits maintain their current state. HALT mode can be released an interrupt.

When HALT mode is released, this bit is automatically reset. Note that setting this bit to "0" does not change the standby state.

When HALT mode is in effect, system clock 2 (S2) stops.

    - System Clock Operation Mode

There are three system clocks:

- Internal RC oscillation clock

After a reset, when the power is turned on, or when HOLD mode is released, this clock is set as the system clock. Even if there are no external oscillation circuits, the microcomputer runs using just this clock.

- Main clock

The unit enters fast processing mode whenever the main clock is used, but this increases battery consumption by a factor of ten compared to internal RC oscillation. The main clock should not be used in game programs.

- Sub-clock

This is a slow processing mode that is used in order to reduce current consumption and make backup power last longer.

When operating in sub-clock mode, the main clock and the internal RC oscillation clock can be stopped in order to further reduce current consumption.

Fig. 4-2-2 shows the state transition diagram for the microcomputer when it enters HALT or HOLD mode.

It is important to note that if the main clock or the sub-clock is specified as the system clock in an application circuit that does not have an external main clock oscillation circuit or sub-clock oscillation circuit, the microcomputer will cease to operate.

RC OSC	: Internal RC oscillation circuit	MAIN	: Main clock oscillating frequency

MAIN OSC	: Main clock oscillation circuit	SUB	: Sub-clock oscillating frequency

SUB OSC	: Sub-clock oscillation circuit	PCON0	: Power control register bit 0 (HALT control)

Oscillating	: Oscillating state	PCON1	: Power control register bit 1 (HOLD control)

Stopped	: Stopped state	OCR0	: Oscillation control register bit 0

S1	: System clock 1	OCR1	: Oscillation control register bit 1

S2	: System clock 2	OCR4	: Oscillation control register bit 4

RC	: Internal RC oscillating frequency	OCR5	: Oscillation control register bit 5

Fig. 4-2-2.  Clock Operation Mode Transition Diagram

*1)	Before switching the system clock to the sub-clock, allow enough time to ensure that the oscillation of the sub-clock has stabilized. For details on the oscillation stabilization time for the sub-clock (32.768kHz crystal oscillation), refer to the most recent "Semiconductor News."

*2)	Before switching the system clock to the main clock, allow enough time to ensure that the oscillation of the main clock has stabilized. For details on the oscillation stabilization time for the main clock, refer to the most recent "Semiconductor News."