# 2

Chapter 2.

Assembler Option Specification

This chapter explains how to use the assembler options to specify and control the operation of M86K. In the MS-DOS version, all options begin with the assembler option characters "-" or "/", and in the UNIX version all options begin with the assembler option character "-". In either case, no distinction is made between upper- and lower-case letters for the option specification letter. For example, "-I' and "-i" are interpreted as having the same meaning.

  - Specification for Not Distinguishing between Upper- and Lower-case Letters in Identifiers

Option

-I

If this switch is specified, the assembler makes no distinction between upper-case and lower-case letters in user-defined identifiers (labels, macro names, symbols). If this switch is not specified, a distinction is made between the upper- and lower-case forms of each letter. the effect of this switch is limited to user-defined identifiers, and does not apply to mnemonics or SFRs.

  - Specification for Outputting Debugging Information

Option

-D

If this switch is specified, the assembler does not output the symbol information and source line information in the object file. When debugging an object file that lacks this information, source line mode cannot be used. If this switch is not specified, both types of information are output in the object file, and source line mode can be used for debugging.

  - Specification for Not Optimizing Branching Instructions

Option

-J

This switch can be specified when assembling source code that includes pseudo instructions that require optimization (JMPO, CALLO, BRO); specifying this switch suppresses the optimization operation. As a result, all pseudo instructions are interpreted as 3-byte instructions regardless of the jump destination. If this switch is not specified when assembling source code that includes pseudo instructions that require optimization, the optimization operation is performed. If there are no pseudo instructions that require optimization, the operation of the assembler is the same, whether this switch is specified or not.

  - Specification for Suppressing the Copyright Notice

Option

-N

If this switch is specified, the copyright notice, etc., is not displayed when the assembler is started up. This switch is used to keep the display screen "clean" by suppressing all unnecessary display information, aside from error messages, when starting up the assembler from a utility such as "make".

  - Reserved Word File Specification

Option

-R

The character string that starts with the first character that follows this switch and ends with the last character before the first subsequent space character that is encountered is indicated to the assembler as the name of the reserved word file. For example, assume that the following specification is made:

m86k -rm86krsvd.rwd source.asm,,source.lst

In this case, the name of the reserved word file is m86krsvd.rwd. This specification takes priority over the environment variable M86KRSVDFILE.

  - Work Buffer Size Specification

Option

-P

If a numeric value is specified after this switch, that value is adopted as the size of the assembler's internal work buffer. The work buffer is an area that is used in order to increase processing speed when the assembler is registering and expanding macros, and is allocated in main memory when the assembler is started up. The default size is 4096 bytes. This is not likely to prove to be inadequate in the case of a typical source program. However, if the buffer size is too small, the assembler displays the following error message and interrupts processing:

no more PARAMETER buffer (123) 45

(The two digits at the end of the message are internal information, and may vary.) If this type of message is displayed, use this switch to specify a larger buffer size and then repeat the assembly process. For example, if the following is specified:

m86k -p8192 source.asm

Then the work buffer size will be 8192 bytes. Only a decimal value can be specified, and must be specified directly after the switch character "P" with no intervening space. Furthermore, if only the switch is specified, with no valid number, the buffer size is unchanged and remains at its default of 4096.

  - Option List Display

Option

-?

If this switch is specified, the assembler displays the following list of options that can be used, and then terminates execution. Note that if this switch is specified, execution terminates, regardless of what other options were specified.

Usage: m86k [option] source,[object],[list],[xref]

option:

/Ddonotmakelocalsymboltableandsourcelineattributesinobjectfile

/Iignorecaseforuserdefinedsymbol

/Jdonottrytooptimize

/Nskipdisplayingcopyrightmessage

/Psizeparameterbuffer size in decimal

/Rfileread ‘file’ as reserved word file