# 1

Chapter 1.

Environment Variables

  - Environment Variables That Are Used by the Development Support Tools for the L86K Series

The L86K series development support tools use the environment variables described below.

PATH	:	This variable defines the search path.  This is defined by a format that is added to the previously defined PATH.

| Name | File that is searched for |
| --- | --- |
| M86K | Searches for the reserved word file m86krsvd.rwd in the directory defined by PATH. |
| L86K | Searches for the reserved word definition symbol file lc86k.lib in the directory defined by PATH. |
| CGR86K | Searches for the default character generator data file (DEFAULT.CDF for he LC864000 Series, DEFAULT.GGR in all other cases) in the directory defined by PATH. |

CHIPNAME	:	This variable defines the name of the chip (or series) that is the target of processing.

| Name | Description |
| --- | --- |
| M86K | Defines the name of the chip that is the target of the assembly operation. Any CHIP pseudo instructions (refer to Part 2, "Assembler," Chapter 6, "Pseudo Instructions," under the item " CHIP Pseudo Instructions") that are written in the source program are ignored. This environment variable is referenced when assembling a source program that contains no chip pseudo instructions. If the ROM size field in the chip name (the last two digits in the chip name) is "00," the ROM size is not checked, and the assembly operation proceeds on the assumption that there are 64K of ROM. |
| SU86K | Defines the name of the chip for which option data is to be created. The ROM size field in the chip name (the last two digits in the chip name) is ignored. |
| CGR86K | Defines the name of the chip for which a character generator data file is to be created. The ROM size field in the chip name (the last two digits in the chip name) is ignored. |

When the last two digits in the chip name for each series are "00," that name does not exist as an actual chip. Such chip names are defined for convenience sake as having the largest ROM size in their respective series.  Using such a chip name for assembly is valuable when a user wants to know the size of a user-created program.

M86KRSVDFILE	:	This variable defines the name of the reserved word file.

| Name | File that is searched for |
| --- | --- |
| M86K | Defines the file name and the directory where the reserved word file is stored. If this environment variable is not defined, m86krsvd.rwd is used as the default file name, and m86krsvd.rwd is searched for in the sequence described in PATH. |

M86KWORKFILE	:	This variable defines the work file name.

| Name | File that is searched for |
| --- | --- |
| M86K | If the work memory that is dynamically allocated by M86K while the assembly operation is in progress is too large to fit in main memory, or if there is no EMS memory, or if all of the EMS memory has been used, this variable specifies the name of a work file that can be used as a type of extended memory. The name can be specified with a drive name and a path name (for the MS-DOS version), or a path name (for the UNIX version). |

TMP	:	This variable defines the directory where the work file is stored.

| Name | File that is searched for |
| --- | --- |
| M86K | If it is necessary for M86K to create a work file (refer to the item, "M86KWORKFILE") and the environment variable M86KWORKFILE is not defined, a work file is created in the directory specified by this environment variable. If this environment variable is not defined, the work file is created in the current directory. In all of these cases, however, the file name is fixed to m86kwork.tmp. |

    - Setting the Environment Variables (MS-DOS Version)

The SET command is used to set the environment variables in MS-DOS. For details on the SET command, refer to the MS-DOS manual.

Example: Setting the default chip name to LC866200

A> SET CHIPNAME=LC866200

    - Setting the Environment Variables (UNIX Version)

The setenv command is used to set the environment variables in UNIX. For details on the setenv command, refer to the UNIX manual.

Example: Setting the default chip name to LC866200

host% setenv CHIPNAME LC866200

Note:

UNIX versions are provided for the following tools only: M86K (assembler), L86K (linkage loader) and LIB86K (library manager).