Appendix 1. Variable Bit Length Pulse Generator

	Cycle Tp for Main Interval P

Tp = 2[BIT] Ttc

	Total Low Level Pulse Width [sigma]T[1] within Main Interval P

TL = (2[BIT] [T1LC] /256[T1HC])  Ttc

*T1HC and T1LC are decimal expressions.

*[T1HC] is the valid bit value.

This section provides a supplemental explanation for the above equations, which were presented in section 3.3.5, "Circuit Configuration and Description of Operation."

[BIT] represents the bit length that is set. The number of Sub-Intervals T within a Main Interval P is determined by the bit length, which is set by the Timer 1 High Reload Register (T1HR) and the Timer 1 Low Reload Register. However, T1LR should be set to 00H. If the settings are for a 9-bit pulse generator, [BIT] = 9 and the cycle for the Main Interval P would be Tp = 29 * Ttc = 512Ttc. Because Sub-Interval T consists of 256Ttc (Ttc: pulse signal clock cycle), Sub-Interval T is repeated twice within Main Interval P. (Refer to the table.)

| (deleted) Bit length | Count of Sub-Interval T repetitions | Pulse generator bit length settings (binary) |  | (deleted) Low level pulse width settings (binary) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1HR | T1LR | T1HR upper bits | T1LR lower bits |
| 16 | 256 | 0000 0000 | 0000 0000 | XXXX XXXX | XXXX XXXX |
| 15 | 128 | 1000 0000 | 0000 0000 | XXXX XXXX | XXXX XXX0 |
| 14 | 64 | 1100 0000 | 0000 0000 | XXXX XXXX | XXXX XX00 |
| 13 | 32 | 1110 0000 | 0000 0000 | XXXX XXXX | XXXX X000 |
| 12 | 16 | 1111 0000 | 0000 0000 | XXXX XXXX | XXXX 0000 |
| 11 | 8 | 1111 1000 | 0000 0000 | XXXX XXXX | XXX0 0000 |
| 10 | 4 | 1111 1100 | 0000 0000 | XXXX XXXX | XX00 0000 |
| 9 | 2 | 1111 1110 | 0000 0000 | XXXX XXXX | X000 0000 |

(X: 0 or 1)		                                              Shaded areas indicate valid bits.

[T1LR] indicates the value of the Timer 1 Low Compare Data Register (T1LC), and sets a common low level pulse width for all Sub-Intervals. If [T1LC] = 3, a 3Ttc low level pulse is added to all Sub-Intervals, and the resulting low level pulse width within a Main Interval becomes 6Ttc (when [T1HC] = 00).

[T1HC] shows the valid value of the Timer 1 High Compare Data Register (T1HC), and sets a common low level pulse width for all Sub-Intervals.

The following figure shows the position of the Sub-Intervals to which low level pulses are added by [T1HC]. When [T1HC] = 1, a low level pulse of 1Ttc is added to the intervals labeled "1." When [T1HC] = 6, a low level pulse of 1Ttc is added to the intervals labeled "2" and "4."

| 9 bits | 1 |  | Small interval: 2 |
| --- | --- | --- | --- |

| 10 bits | 2 | 1 | 2 |  | Small interval: 4 |
| --- | --- | --- | --- | --- | --- |

| 11 bits | 4 | 2 | 4 | 1 | 4 | 2 | 4 |  | Small interval: 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 12 bits | 8 | 4 | 8 | 2 | 8 | 4 | 8 | 1 | 8 | 4 | 8 | 2 | 8 | 4 | 8 |  | Small interval: 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 16 bits | 128 | 64 | 128 | 32 | 128 | 64 | 128 | 16 | 128 | 64 | 128 | 32 |  | 32 | 128 | 32 | 128 | 64 | 128 |  | Small interval: 256 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

In the preceding example, [T1HC] = 0, but when [T1HC] = 1, the low level pulse width within Main Interval P is 7Ttc, as shown below. (In the case of a 9-bit pulse generator, the number of valid bits is "1.")

Next, consider the case of an 11-bit pulse generator.

[BIT] = 11, and the cycle of Main Interval P is TP = 211  Ttc = 2048Ttc. Because Sub-Interval T consists of 256Ttc (Ttc: pulse generator clock cycle), Sub-Interval T is repeated 8 times within Large Interval P.

The following figure shows the change in the output waveform due to T1HC when T1LC is constant.

When T1LC = 10 (0AH) and [T1HC] = 0, 10Ttc of low level pulses are output in all Sub-Intervals.

As shown in "B" in the figure, when [T1HC] = 3, 11Ttc of low level pulses are generated within three Sub-Intervals within one Main Interval.

As shown in "C" in the figure, when [T1HC] = 4, 11Ttc of low level pulses are generated within four Sub-Intervals. Furthermore, as shown in "D" in the figure, when [T1HC] = 5, 11Ttc of low level pulses are generated within five Sub-Intervals.

Appebdix 2 List of Symbols

| Symbol | Address | Name | Initial Value | Page |
| --- | --- | --- | --- | --- |
| RAM (BANK0) | 00H-0FFH | Data Memory | XXXXXXXX (Saved upon reset) | - |
| RAM (BANK1) | 00H-0FFH | Data Memory | XXXXXXXX (Saved upon reset) | - |
| ACC | 100H | Accumulator | 00000000 | 2-8 |
| PSW | 101H | Program Status Word | 00000000 | 2-9 |
| B | 102H | B Register | 00000000 | 2-8 |
| C | 103H | C Register | 00000000 | 2-8 |
| TRL | 104H | Table Reference Register Low Byte | 00000000 | 2-11 |
| TRH | 105H | Table Reference Register High Byte | 00000000 | 2-11 |
| SP | 106H | Stack Pointer | XXXXXXXX | 2-10 |
| PCON | 107H | Power Control Register | HHHHHH00 | 4-12 |
| IE | 108H | Master Interrupt Enable Control Register | 0HHHHH00 | 3-82, 4-4 |
| IP | 109H | Interrupt Priority Ranking Control Register | 0X000000 | 4-4 |
| OCR | 10EH | Oscillation Control Register | 0H00HH00 | 4-10 |
| T0CNT | 110H | Timer/Counter 0 Control Register | 00000000 | 3-9 |
| T0PRR | 111H | Timer 0 Prescaler Data Register | 00000000 | 3-13 |
| T0L | 112H | Timer 0 Low Register | 00000000 | 3-13 |
| T0LR | 113H | Timer 0 Low Reload Register | 00000000 | 3-13 |
| T0H | 114H | Timer 0 High Register | 00000000 | 3-14 |
| T0HR | 115H | Timer 0 High Reload Register | 00000000 | 3-14 |
| T1CNT | 118H | Timer 1 Control Register | 00000000 | 3-27 |
| T1LC | 11AH | Timer 1 Low Compare Data Register | 00000000 | 3-29 |
| T1L | 11BH | Timer 1 Low Register | 00000000 | 3-29 |
| T1LR |  | Timer 1 Low Reload Register | 00000000 | 3-29 |
| T1HC | 11CH | Timer 1 High Compare Data Register | 00000000 | 3-30 |
| T1H | 11DH | Timer 1 High Register | 00000000 | 3-29 |
| T1HR |  | Timer 1 High Reload Register | 00000000 | 3-30 |
| MCR | 120H | Mode Control Register | 00000000 | 3-71 |
| STAD | 122H | Start Address Register | 00000000 | 3-72 |
| CNR | 123H | Character Number Register | H0000000 | 3-74 |
| TDR | 124H | Time Division Register | HH000000 | 3-74 |
| XRBK | 125H | Bank Address Register | H0000000 | 3-74 |
| VCCR | 127H | LCD Contrast Control Register | 00000000 | 3-75 |
| SCON0 | 130H | SIO0 Control Register | H0H00000 | 3-50 |
| SBUF0 | 131H | SIO0 Buffer | 00000000 | 3-53 |
| SBR | 132H | SIO Baud Rate Generator Register | 00000000 | 3-53 |
| SCON1 | 134H | SIO1 Control Register | 00H00000 | 3-52 |
| SBUF1 | 135H | SIO1 Buffer | 00000000 | 3-53 |
| P1 | 144H | Port 1 Latch | 00000000 | 3-2 |
| P1DDR | 145H | Port 1 Data Direction Register | 00000000 | 3-2 |
| P1FCR | 146H | Port 1 Function Control Register | 00000000 | 3-2 |
| P3 | 14CH | Port 3 Latch | 00000000 | 3-5 |

Appebdix 2 List of Symbols (continued)

| Symbol | Address | Name | Initial Value | Page |
| --- | --- | --- | --- | --- |
| P3DDR | 14DH | Port 3 Data Direction Register | 00000000 | 3-5 |
| P3INT | 14EH | Port 3 Interrupt Control Register | HHHHH000 | 3-5 |
| P7 | 15CH | Port 7 Latch | HHHHXXXX | 3-7 |
| I01CR | 15DH | External Interrupt 0, 1 Control Register | 00000000 | 3-79 |
| I23CR | 15EH | External Interrupt 2, 3 Control Register | 00000000 | 3-81 |
| ISL | 15FH | Input Signal Selection Register | HH000000 | 3-12 |
| BTCR | 17FH | Base Timer Control Register | 00000000 | 3-44 |
| RAM(XRAM) (BANK0) | 180H-1FBH | Display RAM | XXXXXXXX (Saved upon reset) | 3-70 |
| RAM(XRAM) (BANK1) | 180H-1FBH |  |  |  |
| RAM(XRAM) (BANK2) | 180H-185H |  |  |  |
| VSEL | 163H | VMS Control Register | HHH0HH00 | 3-90 |
| VRMAD1 | 164H | Work RAM Access Address 1 | 00000000 | 3-91 |
| VRMAD2 | 165H | Work RAM Access Address 2 | HHHHHHH0 | 3-91 |
| VTRBF | 166H | Send/Receive Buffer | XXXXXXXX | 3-91 |
| VLREG | 167H | Length registration | 00000000 | - |

Appendix 3. List of Instructions

| Mnemonic | Instruction code | Bytes |  | Cycles | Operation |  | PSW |  |  | Page |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | CY |  | AC | OV |  |
| ADD #i8 | 1 0 0 0 0 0 0 1 i7 i6 i5 i4 i3 i2 i1 i0 | 2 | 1 |  | (A )i | 〇 |  | 〇 | 〇 | 5-9 |
| ADD d9 | 1 0 0 0 0 0 1 d8 d7 d6 d5 d4 d3 d2 d1 d0 | 2 | 1 |  | (A) d9) | 〇 |  | 〇 | 〇 | 5-10 |
| ADD @Rj | 1 0 0 0 0 1 j1 j0 | 1 | 1 |  | (A) Rj)) j:0, 1, 2, 3 | 〇 |  | 〇 | 〇 | 5-11 |
| ADDC #i8 | 1 0 0 1 0 0 0 1 i7 i6 i5 i4 i3 i2 i1 i0 | 2 | 1 |  | (A) CYi | 〇 |  | 〇 | 〇 | 5-12 |
| ADDC d9 | 1 0 0 1 0 0 1 d8 d7 d6 d5 d4 d3 d2 d1 d0 | 2 | 1 |  | (A) CYd9) | 〇 |  | 〇 | 〇 | 5-13 |
| ADDC @Rj | 1 0 0 1 0 1 j1 j0 | 1 | 1 |  | (A) CYRj)) j:0, 1, 2, 3 | 〇 |  | 〇 | 〇 | 5-14 |
| AND #i8 | 1 1 1 0 0 0 0 1 i7 i6 i5 i4 i3 i2 i1 i0 | 2 | 1 |  | (A) i |  |  |  |  | 5-27 |
| AND d9 | 1 1 1 0 0 0 1 d8 d7 d6 d5 d4 d3 d2 d1 d0 | 2 | 1 |  | (A) d9) |  |  |  |  | 5-28 |
| AND @Rj | 1 1 1 0 0 1 j1 j0 | 1 | 1 |  | (A) Rj)) j:0, 1, 2, 3 |  |  |  |  | 5-29 |
| BE #i8, r8 | 0 0 1 1 0 0 0 1 i7 i6 i5 i4 i3 i2 i1 i0 r7 r6 r5 r4 r3 r2 r1 r0 | 3 | 2 |  | (PC) (PC) 3 if (A) = #i8, then (PC) (PC)r8 if(A) < #i8, then CY 1 else CY  | 〇 |  |  |  | 5-62 |
| BE d9, r8 | 0 0 1 1 0 0 1 d8 d7 d6 d5 d4 d3 d2 d1 d0 r7 r6 r5 r4 r3 r2 r1 r0 | 3 | 2 |  | (PC) (PC)3 if (A) = d9), then (PC) (PC)r8 if(A) < d9), then CY 1 else CY  | 〇 |  |  |  | 5-63 |
| BE @Rj, #i8, r8 | 0 0 1 1 0 1 j1 j0 i7 i6 i5 i4 i3 i2 i1 i0 r7 r6 r5 r4 r3 r2 r1 r0 | 3 | 2 |  | (PC) (PC)3 if ((Rj)) = #i8, then (PC) (PC)r8 if ((Rj)) < #i8, then CY 1 else CY  j:0, 1, 2, 3 | 〇 |  |  |  | 5-64 |
| BN d9, b3, r8 | 1 0 0 d8 1 b2 b1 b0 d7 d6 d5 d4 d3 d2 d1 d0 r7 r6 r5 r4 r3 r2 r1 r0 | 3 | 2 |  | (PC) (PC)3 if (d9, b3) = 0 then (PC) (PC)r8 |  |  |  |  | 5-59 |
| BNE #i8, r8 | 0 1 0 0 0 0 0 1 d7 d6 d5 d4 d3 d2 d1 d0 r7 r6 r5 r4 r3 r2 r1 r0 | 3 | 2 |  | (PC) (PC)3 if (A)  i8, then (PC) (PC)r8 if(A) < i8, then CY 1 else CY  | 〇 |  |  |  | 5-65 |
| BNE d9, r8 | 0 1 0 0 0 0 1 d8 d7 d6 d5 d4 d3 d2 d1 d0 r7 r6 r5 r4 r3 r2 r1 r0 | 3 | 2 |  | (PC) (PC)3 if (A)  d9), then (PC) (PC)r8 if(A) < d9), then CY 1 else CY  | 〇 |  |  |  | 5-66 |
| BNE @Rj, #i8, r8 | 0 1 0 0 0 1 j1 j0 i7 i6 i5 i4 i3 i2 i1 i0 r7 r6 r5 r4 r3 r2 r1 r0 | 3 | 2 |  | (PC) (PC)3 if ((Rj)) = #i8, then (PC) (PC)r8 if ((Rj)) < #i8, then CY 1 else CY  j:0, 1, 2, 3 | 〇 |  |  |  | 5-67 |

Appendix 3. List of Instructions (continued)

| Mnemonic | Instruction code |  | Bytes | Cycles | Operation |  | PSW |  |  | Page |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | CY |  | AC | OV |  |
| BNZ r8 | 1 0 0 1 0 0 0 0 r7 r6 r5 r4 r3 r2 r1 r0 | 2 |  | 2 | (PC) (PC)2 if (A), then (PC) (PC)r8 |  |  |  |  | 5-56 |
| BP d9, b3, r8 | 0 1 1 d8 1 b2 b1 b0 d7 d6 d5 d4 d3 d2 d1 d0 r7 r6 r5 r4 r3 r2 r1 r0 | 3 |  | 2 | (PC) (PC)3 if (d9, b3), then (PC) (PC)r8 |  |  |  |  | 5-57 |
| BPC d9, b3, r8 | 0 1 0 d8 1 b2 b1 b0 d7 d6 d5 d4 d3 d2 d1 d0 r7 r6 r5 r4 r3 r2 r1 r0 | 3 |  | 2 | (PC) (PC)3 if (d9, b3), then (PC) (PC)r8 (d9, b3)  |  |  |  |  | 5-58 |
| BR r8 | 0 0 0 0 0 0 0 1 r7 r6 r5 r4 r3 r2 r1 r0 | 2 |  | 2 | (PC) (PC)2 (PC) (PC)r8 |  |  |  |  | 5-53 |
| BRF r16 | 0 0 0 1 0 0 0 1 r7 r6 r5 r4 r3 r2 r1 r0 r15 r14 r13 r12 r11 r10 r9 r8 | 3 |  | 4 | (PC) (PC)3 (PC) (PC)-1r16 |  |  |  |  | 5-54 |
| BZ r8 | 1 0 0 0 0 0 0 0 r7 r6 r5 r4 r3 r2 r1 r0 | 2 |  | 2 | (PC) (PC)2 if (A) = 0, then (PC) (PC)r8 |  |  |  |  | 5-55 |
| CALL a12 | 0 0 0 a11 1 a10 a9 a8 a7 a6 a5 a4 a3 a2 a1 a0 | 2 |  | 2 | (PC) (PC)2 (SP) (SP)1 ((SP)) (PC7-0) (SP) (SP)1 ((SP)) (PC15-8) (PC11-0) a12 |  |  |  |  | 5-68 |
| CALLF a16 | 0 0 1 0 0 0 0 0 a15 a14 a13 a12 a11 a10 a9 a8 a7 a6 a5 a4 a3 a2 a1 a0 | 3 |  | 2 | (PC) (PC) 3 (SP) (SP) 1 ((SP)) (PC7-0) (SP) (SP) 1 ((SP)) (PC15-8) (PC) a16 |  |  |  |  | 5-69 |
| CALLF r16 | 0 0 0 1 0 0 0 0 r7 r6 r5 r4 r3 r2 r1 r0 r15 r14 r13 r12 r11 r10 r9 r8 | 3 |  | 4 | (PC) (PC) 3 (SP) (SP) 1 ((SP)) (PC7-0) (SP) (SP) 1 ((SP)) (PC15-8) (PC) (PC)-1+r16 |  |  |  |  | 5-70 |
| CLR1 d9, b3 ★ | 1 1 0 d8 1 b2 b1 b0 d7 d6 d5 d4 d3 d2 d1 d0 | 2 |  | 1 | (d9, b3) 0 |  |  |  |  | 5-73 |
| DBNZ d9, r8 ★ | 0 1 0 1 0 0 1 d8 d7 d6 d5 d4 d3 d2 d1 d0 r7 r6 r5 r4 r3 r2 r1 r0 | 3 |  | 2 | (PC) (PC) 3 (d9) = (d9)-1 if(d9)  then (PC) (PC) +r18 |  |  |  |  | 5-60 |
| DBNZ @Rj, r8 ★ | 0 1 0 1 0 1 j1 j0 r7 r6 r5 r4 r3 r2 r1 r0 | 2 |  | 2 | (PC) (PC) 2 ((Rj)) = ((Rj))-1 j:0, 1, 2, 3 if (d9) 0, then (PC) (PC) +r8 |  |  |  |  | 5-61 |
| DEC d9 ★ | 0 1 1 1 0 0 1 d8 d7 d6 d5 d4 d3 d2 d1 d0 | 2 |  | 1 | (d9)  (d9) -1 |  |  |  |  | 5-23 |
| DEC @Rj ★ | 0 1 1 1 0 1 j1 j0 | 1 |  | 1 | ((Rj)) ((Rj)) -1 j:0, 1, 2, 3 |  |  |  |  | 5-24 |
| DIV | 0 1 0 0 0 0 0 0 | 1 |  | 7 | (A)(C), mod (B)  (A)(C) (B) | 〇 |  |  | 〇 | 5-26 |

Appendix 3. List of Instructions (continued)

| Mnemonic | Instruction code | Bytes | Cycles | Operation | PSW |  |  | Page |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | CY | AC | OV |  |
| INC d9 ★ | 0 1 1 0 0 0 1 d8 d7 d6 d5 d4 d3 d2 d1 d0 | 2 | 1 | (d9) (d9) 1 |  |  |  | 5-21 |
| INC @Rj ★ | 0 1 1 0 0 1 j1 j0 | 1 | 1 | ((Rj)) (Rj)) 1 j: 0, 1, 2, 3 |  |  |  | 5-22 |
| JMP a12 | 0 0 1 a11 1 a10 a9 a8 a7 a6 a5 a4 a3 a2 a1 a0 | 2 | 2 | (PC)(PC)2 (PC11-00) a12 |  |  |  | 5-51 |
| JMPF a16 | 0 0 1 0 0 0 0 1 a15 a14 a13 a12 a11 a10 a9 a8 a7 a6 a5 a4 a3 a2 a1 a0 | 3 | 2 | (PC) a16 |  |  |  | 5-52 |
| LD d9 | 0 0 0 0 0 0 1 d8 d7 d6 d5 d4 d3 d2 d1 d0 | 2 | 1 | (A)(d9) |  |  |  | 5-40 |
| LD @Rj | 0 0 0 0 0 0 1 j1 j0 | 1 | 1 | (A)(Rj)) j: 0, 1, 2, 3 |  |  |  | 5-41 |
| LDC | 1 1 0 0 0 0 0 1 | 1 | 2 | (A)(BNK)((TRR))+(A)) [ROM] |  |  |  | 5-46 |
| deleted | deleted | deleted | deleted | deleted | deleted | deleted | deleted | deleted |
| MOV #i8, d9 | 0 0 1 0 0 0 1 d8 d7 d6 d5 d4 d3 d2 d1 d0 i7 i6 i5 i4 I3 i2 i1 i0 | 3 | 2 | (d9)#i8 |  |  |  | 5-44 |
| MOV #i8, @Rj | 0 0 1 0 0 1 j1 j0 i7 i6 i5 i4 I3 i2 i1 i0 | 2 | 1 | ((Rj))#i8 j: 0, 1, 2, 3 |  |  |  | 5-45 |
| MUL | 0 0 1 1 0 0 0 0 | 1 | 7 | (B)(A)(C)(A)(C) (B) | 〇 |  | 〇 | 5-25 |
| NOP | 0 0 0 0 0 0 0 0 | 1 | 1 |  |  |  |  | 5-76 |
| NOT1 d9, b3 ★ | 1 0 1 d8 1 b2 b1 b0 d7 d6 d5 d4 d3 d2 d1 d0 | 2 | 1 | (d9, b3)(d9, b3) |  |  |  | 5-75 |
| OR #i8 | 1 1 0 1 0 0 0 1 i7 i6 i5 i4 i3 I2 i1 i0 | 2 | 1 | (A)(A) #i8 |  |  |  | 5-30 |
| OR d9 | 1 1 0 1 0 0 1 d8 d7 d6 d5 d4 d3 d2 d1 d0 | 2 | 1 | (A)(A) (d9) |  |  |  | 5-31 |
| OR @Rj | 1 1 0 1 0 1 j1 j0 | 1 | 1 | (A)(A) (Rj)) j: 0, 1, 2, 3 |  |  |  | 5-32 |
| POP d9 | 0 1 1 1 0 0 1 d8 d7 d6 d5 d4 d3 d2 d1 d0 | 2 | 2 | (d9)((SP)) (SP)(SP) - 1 |  |  |  | 5-48 |
| PUSH d9 | 0 1 1 0 0 0 0 d8 d7 d6 d5 d4 d3 d2 d1 d0 | 2 | 2 | (SP)(SP) +1 ((SP))(d9) |  |  |  | 5-47 |
| RET | 1 0 1 0 0 0 0 0 | 1 | 2 | (PC15-8)((SP)) (SP)(SP) - 1 (PC7-0)((SP)) (SP)(SP)-1 |  |  |  | 5-71 |
| RETI | 1 0 1 1 0 0 0 0 | 1 | 2 | (PC15-8)((SP)) (SP)(SP)-1 (PC7-0)((SP)) (SP)(SP)-1 |  |  |  | 5-72 |
| ROL | 1 1 1 0 0 0 0 0 | 1 | 1 |  A7 A6 A5 A4   A0 A1 A2A3 |  |  |  | 5-36 |
| ROLC | 1 1 1 1 0 0 0 0 | 1 | 1 |  CY A7 A6 A5 A4    A0 A1 A2A3 | 〇 |  |  | 5-37 |
| ROR | 1 1 0 0 0 0 0 0 | 1 | 1 |  A7 A6 A5 A4  A0A1A2A3 |  |  |  | 5-38 |

Appendix 3. List of Instructions (continued)

| Mnemonic | Instruction code | Bytes | Cycles |  | Operation |  | PSW |  |  | Page |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | CY |  | AC | OV |  |
| RORC | 1 1 0 1 0 0 0 0 | 1 | 1 |  CY A7 A6 A5 A4    A0  A1 A2 A3 |  | 〇 |  |  |  | 5-39 |
| SET d9,b3 ★ | 1 1 1 d8 1 b2 b1 b0 d7 d6 d5 d4 d3 d2 d1 d0 | 2 | 1 | (d9, b3) |  |  |  |  |  | 5-74 |
| ST d9 | 0 0 0 1 0 0 1 d8 d7 d6 d5 d4 d3 d2 d1 d0 | 2 | 1 | (d9)A) |  |  |  |  |  | 5-42 |
| ST @Rj | 0 0 0 1 0 1 j1 j0 | 1 | 1 | ((Rj))A) j:0, 1, 2, 3 |  |  |  |  |  | 5-43 |
| SUB #i8 | 1 0 1 0 0 0 0 1 i7 i6 i5 i4 i3 i2 i1 i0 | 2 | 1 | A)A)-  i |  | 〇 |  | 〇 | 〇 | 5-15 |
| SUB d9 | 1 0 1 0 0 0 1 d8 d7 d6 d5 d4 d3 d2 d1 d0 | 2 | 1 | A)A)- (d9) |  | 〇 |  | 〇 | 〇 | 5-16 |
| SUB @Rj | 1 0 1 0 0 1 j1 j0 | 1 | 1 | A)A) - ((Rj)) j:0, 1, 2, 3 |  | 〇 |  | 〇 | 〇 | 5-17 |
| SUBC #i8 | 1 0 1 1 0 0 0 1 i7 i6 i5 i4 i3 i2 i1 i0 | 2 | 1 | A)A) - CY- #i8 |  | 〇 |  | 〇 | 〇 | 5-18 |
| SUBC d9 | 1 0 1 1 0 0 1 d8 d7 d6 d5 d4 d3 d2 d1 d0 | 2 | 1 | A)A) - CY- (d9) |  | 〇 |  | 〇 | 〇 | 5-19 |
| SUBC @Rj | 1 0 1 1 0 1 j1 j0 | 1 | 1 | A)A) - CY- ((Rj)) j:0, 1, 2, 3 |  | 〇 |  | 〇 | 〇 | 5-20 |
| XCH d9 | 1 1 0 0 0 0 1 d8 d7 d6 d5 d4 d3 d2 d1 d0 | 2 | 1 | d9) ⇔ (A) |  |  |  |  |  | 5-49 |
| XCH @Rj | 1 1 0 0 0 1 j1 j0 | 1 | 1 | Rj)) ⇔ (A) j:0, 1, 2, 3 |  |  |  |  |  | 5-50 |
| XOR #i8 | 1 1 1 1 0 0 0 1 i7 i6 i5 i4 I3 i2 i1 i0 | 2 | 1 | A)A) #i8 |  |  |  |  |  | 5-33 |
| XOR d9 | 1 1 1 1 0 0 1 d8 d7 d6 d5 d4 d3 d2 d1 d0 | 2 | 1 | A)A) (d9) |  |  |  |  |  | 5-34 |
| XOR @Rj | 1 1 1 1 0 1 j1 j0 | 1 | 1 | A)A) ((Rj)) j:0, 1, 2, 3 |  |  |  |  |  | 5-35 |

When addressing bytes or individual bits in instruction ports marked with the "?" symbol, the port latch is selected. For other instructions, the external signal that is applied to the port is selected. 
However, when using port 5 as a bank address output port (when bit 3 in the EXT register is set to "1"), the port latch is selected, regardless of the instruction.

Appendix 4. List of Instruction Map

| LOWER 4BIT UPPER 4BIT | 0 | 1 |  | 2,3 |  | 4-7 | 8-F |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | NOP | BR r8 |  | LD d9 |  | LD @Rj | CALLa12 |
| 1 | CALLR r16 | BRF r16 |  | ST d9 |  | ST @Rj |  |
| 2 | CALLF a16 | JMPF a16 |  | MOV #i8, d9 |  | MOV #i8, @Rj | JMPa12 |
| 3 | MUL | BE #i8, r8 |  | BE d9,r8 |  | BE @Rj, #i8 |  |
| 4 | DIV | BNE #i8, r8 |  | BNE d9,r8 |  | BNE @Rj, #i8, r8 | BPC d9, b3, r8 |
| 5 |  |  |  | DBNZ d9,r8 |  | DBNZ @Rj, r8 |  |
| 6 | PUSH d9 |  | INC d9 |  | INC @Rj |  | BP d9, b3, r8 |
| 7 | POP d9 |  | DEC d9 |  | DEC @Rj |  |  |
| 8 | BZ r8 | ADD #i8 |  | ADD d9 |  | ADD @Rj | BN d9, b3, r8 |
| 9 | BNZ r8 | ADDC #i8 |  | ADDC d9 |  | ADDC @Rj |  |
| A | RET | SUB #i8 |  | SUB d9 |  | SUB @Rj | NOT1 d9, b3 |
| B | RETI | SUBC #i8 |  | SUBC d9 |  | SUBC @Rj |  |
| C | ROR | LDC |  | XCH d9 |  | XCH @Rj | CLR1 d9, b3 |
| D | RORC | OR #i8 |  | OR d9 |  | OR @Rj |  |
| E | ROL | AND #i8 |  | AND d9 |  | AND @Rj | SET1 d9, b3 |
| F | ROLC | XOR #i8 |  | XOR d9 |  | XOR @Rj |  |