# 2

Chapter 1.

Program Startup

  - File Name Specification

Upper-case and lower-case letters can be used in any combination in a file name that is specified in the EVA2HEX command line. For example, the following three file names are all equivalent:

sample.eva

SAmpLE.EVA

SAMPLE.EVA

In addition, when a file name is specified with no extension, EVA2HEX uses the following default file name extensions.

| File format | Default extension |
| --- | --- |
| EVA file | .EVA |
| HEX file | .HEX |

  - Parameter Specification Method

EVA2HEX[option] EVA_filename [HEX_filename]

(1) option field

This field specifies the options described in section 1.3. Specify the option field after the command name.

(2) EVA_filename field

This field specifies the name of the file after debugging is completed (the file with the ".EVA" extension).  (This file is known as the EVA file.)

(3) HEX_filename field

This field specifies the name of the Intellec HEX format file. (This file is known as the HEX file.) If the HEX_filename is omitted, it is identical to EVA_filename. If an external ROM data file is converted, the extension is .H00.

* EVA2HEX does not support prompts for parameter input.

Startup Example 1:	A>EVA2HEX  PROG012

EVA file	 >> HEX file, external ROM HEX file

PROG012.EVA	PROG012.HEX,  PROG012.H00

Startup Example 2:	A>EVA2HEX

The following message (simple help) is displayed.

SANYO LC86000 Series EVA-file to HEX-file generator  V1.00A

Copyright (C) SANYO Electric Co.,Ltd. 1992

Usage: eva2hex [optiona] EVA filename [HEX filename]

Optiona: /I ... information on/off (default: on )

  - Option Specification

The option begins with "/". "-" cannot be used.

Option

/I

The /I option disables the display (on the VDT) of information on the progress of the conversion process when converting an EVA file to a HEX file. If this option is not specified, information on the progress of the conversion process is displayed while conversion is in progress.

Monitor Display Example:

SANYO LC86000 Series EVA-file to HEX-file genetator V1.00A

Copyright (C) SANYO Electric Co.,Ltd. 1992

EVA fiLe name:   A.eva		       	EVA file

ROM data packed: FF(hex)			Default data

Chip name:        LC868016			Model name

ModuLe name: A      Internal CSEG(In)        0000 – 0063  records: 0007

ModuLe name: B      Internal CSEG(In)        0064 – 00DB  records: 0008

ModuLe name: C      Internal CSEG(In)        00DC – 02DD  records: 0033

CGROM data     bLock records:   00256

Option data    bLock records:   00016

Addresses and record length of specified programs

Program name linked to the above EVA file

Record lengths of each type of data

Conversion progress information

Chapter 2.

Errors

  - Fatal Errors

If a fatal error is detected while EVA2HEX is running, EVA2HEX displays a message on the VDT and interrupts processing. The error messages that are displayed by EVA2HEX are listed below.

 Fatal error : .....messages.....

‘filename’ File not cLose.

Cannot close file filename.

‘filename’ File not create.

Cannot create file filename.

‘filename’ File not open.

File filename not found.

‘filename’ not EVA file format.

File filename is not an EVA-format file.

‘filename’ user disk full.

Disk became full while writing filename.

Chipname undefined.

Model name in EVA file is incorrect.

ROM size over. (ROM size: XXXX)

The program size exceeded the ROM size.

Tablename allocation error.

Memory allocation for tablename failed due to insufficient memory.