  - Logical Operation Instructions

AND  #i8  (AND immediate data to accumulator)

| Instruction code | 1 1 1 0 0 0 0 1 i7i6i5i4i3i2i1i0 E1H |
| --- | --- |
| Number of bytes | 2 |
| Number of cycles | 1 |
| Function | (ACC)(ACC)#i8 |
| Affected flags |  |
| Interrupt acceptance | Permitted |

Description:

This instruction ANDs the immediate data (i7 to i0) with the contents of the Accumulator (ACC), and then sends the result to the ACC.

Example 1:

|  |  | ACC |
| --- | --- | --- |
| MOV | #0FFH,ACC | FFH |
| AND | #0FAH | FAH |
| AND | #0AFH | AAH |
| AND | #00FH | 0AH |
| AND | #0F0H | 00H |

Example 2:

|  |  | ACC |
| --- | --- | --- |
| MOV | #0FFH,ACC | FFH |
| AND | #0FEH | FEH |
| AND | #0FDH | FCH |
| AND | #0FBH | F8H |
| AND | #0F7H | F0H |
| AND | #0EFH | E0H |
| AND | #0DFH | C9H |
| AND | #0BFH | 80H |
| AND | #07FH | 00H |

AND  d9  (AND direct byte to accumulator)

| Instruction code | 1 1 1 0 0 0 1d8 d7d6d5d4d3d2d1d0 E2HE3H |
| --- | --- |
| Number of bytes | 2 |
| Number of cycles | 1 |
| Function | (ACC)(ACC)(d9) |
| Affected flags |  |
| Interrupt acceptance | Permitted |

Description:

This instruction ANDs the contents of data memory (RAM) or a Special Function Register (SFR), as specified by d8 to d0, with the contents of the Accumulator (ACC), and then sends the result to the ACC.

Example 1:

|  |  | ACC | RAM 23H |
| --- | --- | --- | --- |
| MOV | #0FFH,ACC | FFH | - |
| MOV | #055H,023H | FFH | 55H |
| AND | 023H | 55H | 55H |
| MOV | #0AAH,023H | 55H | AAH |
| AND | 023H | 00H | AAH |

Example 2:

|  |  | ACC | B |
| --- | --- | --- | --- |
| MOV | #0FFH,ACC | FFH | - |
| MOV | #0FEH,B | FFH | FEH |
| AND | B | FEH | FEH |
| MOV | #0FDH,B | FEH | FDH |
| AND | B | FCH | FDH |
| MOV | #0FBH,B | FCH | FBH |
| AND | B | F8H | FBH |
| MOV | #0F7H,B | F8H | F7H |
| AND | B | F0H | F7H |

AND  @Rj  (AND indirect byte to accumulator)

| Instruction code | 1 1 1 0 0 1j1j0 E4HE7H |
| --- | --- |
| Number of bytes | 1 |
| Number of cycles | 1 |
| Function | (ACC)(ACC)((Rj)) j=0,1,2,3 |
| Affected flags |  |
| Interrupt acceptance | Permitted |

Description:

This instruction ANDs the contents of data memory (RAM) or a Special Function Register (SFR), as specified by the indirect address register that is specified by j1 and j0, with the contents of the Accumulator (ACC), and then sends the result to the ACC.

Example 1:

|  |  | ACC | RAM 00H | RAM 68H |
| --- | --- | --- | --- | --- |
| MOV | #0FFH,ACC | FFH | - | - |
| MOV | #068H,000H | FFH | 68H | - |
| MOV | #0F0H,@R0 | FFH | 68H | F0H |
| AND | @R0 | F0H | 68H | F0H |
| MOV | #00FH,@R0 | F0H | 68H | 0FH |
| AND | @R0 | 00H | 68H | 0FH |

Example 2:

|  |  | ACC | RAM 02H | TRL |
| --- | --- | --- | --- | --- |
| MOV | #0FFH,ACC | FFH | - | - |
| MOV | #004H,002H | FFH | 04H | - |
| MOV | #0EFH,@R2 | FFH | 04H | EFH |
| AND | @R2 | EFH | 04H | EFH |
| MOV | #0DFH,@R2 | EFH | 04H | DFH |
| AND | @R2 | CFH | 04H | DFH |

OR  #i8  (OR immediate data to accumulator)

| Instruction code | 1 1 0 1 0 0 0 1 i7i6i5i4i3i2i1i0 D1H |
| --- | --- |
| Number of bytes | 2 |
| Number of cycles | 1 |
| Function | (ACC)(ACC)#i8 |
| Affected flags |  |
| Interrupt acceptance | Permitted |

Description:

This instruction ORs the immediate data (i7 to i0) with the contents of the Accumulator (ACC), and then sends the result to the ACC.

Example 1:

|  |  | ACC |
| --- | --- | --- |
| MOV | #000H,ACC | 00H |
| OR | #003H | 03H |
| OR | #00CH | 0FH |
| OR | #030H | 3FH |
| OR | #0C0H | FFH |

Example 2:

|  |  | ACC |
| --- | --- | --- |
| MOV | #000H,ACC | 00H |
| OR | #001H | 01H |
| OR | #002H | 03H |
| OR | #004H | 07H |
| OR | #008H | 0FH |
| OR | #010H | 1FH |
| OR | #020H | 3FH |
| OR | #040H | 7FH |
| OR | #080H | FFH |

OR  d9  (OR direct byte to accumulator)

| Instruction code | 1 1 0 1 0 0 1d8 d7d6d5d4d3d2d1d0 D2HD3H |
| --- | --- |
| Number of bytes | 2 |
| Number of cycles | 1 |
| Function | (ACC)(ACC)(d9) |
| Affected flags |  |
| Interrupt acceptance | Permitted |

Description:

This instruction ORs the contents of data memory (RAM) or a Special Function Register (SFR), as specified by d8 to d0, with the contents of the Accumulator (ACC), and then sends the result to the ACC.

Example 1:

|  |  | ACC | RAM 23H |
| --- | --- | --- | --- |
| MOV | #000H,ACC | 00H | - |
| MOV | #055H,023H | 00H | 55H |
| OR | 023H | 55H | 55H |
| MOV | #0AAH,023H | 55H | AAH |
| OR | 023H | FFH | AAH |

Example 2:

|  |  | ACC | B |
| --- | --- | --- | --- |
| MOV | #000H,ACC | 00H | - |
| MOV | #001H,B | 00H | 01H |
| OR | B | 01H | 01H |
| MOV | #002H,B | 01H | 02H |
| OR | B | 03H | 02H |
| MOV | #004H,B | 03H | 04H |
| OR | B | 07H | 04H |
| MOV | #008H,B | 07H | 08H |
| OR | B | 0FH | 08H |

OR  @Rj  (OR indirect byte to accumulator)

| Instruction code | 1 1 0 1 0 1j1j0 D4HD7H |
| --- | --- |
| Number of bytes | 1 |
| Number of cycles | 1 |
| Function | (ACC)(ACC)((Rj)) j=0,1,2,3 |
| Affected flags |  |
| Interrupt acceptance | Permitted |

Description:

This instruction ORs the contents of data memory (RAM) or a Special Function Register (SFR), as specified by the indirect address register that is specified by j1 and j0, with the contents of the Accumulator (ACC), and then sends the result to the ACC.

Example 1:

|  |  | ACC | RAM 00H | RAM 68H |
| --- | --- | --- | --- | --- |
| MOV | #000H,ACC | 00H | - | - |
| MOV | #068H,000H | 00H | 68H | - |
| MOV | #0F0H,@R0 | 00H | 68H | F0H |
| OR | @R0 | F0H | 68H | F0H |
| MOV | #000FH,@R0 | F0H | 68H | 0FH |
| OR | @R0 | FFH | 68H | 0FH |

Example 2:

|  |  | ACC 02H | RAM | TRL |
| --- | --- | --- | --- | --- |
| MOV | #0AAH,ACC | AAH | - | - |
| MOV | #004H,002H | AAH | 04H | - |
| MOV | #005H,@R2 | AAH | 04H | 05H |
| OR | @R2 | AFH | 04H | 05H |
| MOV | #050H,@R2 | AFH | 04H | 50H |
| OR | @R2 | FFH | 04H | 50H |

XOR  #i8  (XOR immediate data to accumulator)

| Instruction code | 1 1 0 0 0 1j1j0 C4HC7H |
| --- | --- |
| Number of bytes | 1 |
| Number of cycles | 1 |
| Function | (ACC)(ACC)((Rj)) j=0,1,2,3 |
| Affected flags |  |
| Interrupt acceptance | Permitted |

Description:

This instruction XORs the immediate data (i7 to i0) with the contents of the Accumulator (ACC), and then sends the result to the ACC.

Example 1:

|  |  | ACC |
| --- | --- | --- |
| MOV | #000H,ACC | 00H |
| XOR | #00FH | 0FH |
| XOR | #0F0H | FFH |
| XOR | #00FH | F0H |
| XOR | #0F0H | 00H |

Example 2:

|  |  | ACC |
| --- | --- | --- |
| MOV | #000H,ACC | 00H |
| XOR | #001H | 01H |
| XOR | #002H | 03H |
| XOR | #004H | 07H |
| XOR | #008H | 0FH |
| XOR | #008H | 07H |
| XOR | #004H | 03H |
| XOR | #002H | 01H |
| XOR | #001H | 00H |

XOR  d9  (XOR direct byte to accumulator)

| Instruction code | 1 1 1 1 0 0 1d8 d7d6d5d4d3d2d1d0 F2HF3H |
| --- | --- |
| Number of bytes | 2 |
| Number of cycles | 1 |
| Function | (ACC)(ACC)(d9) |
| Affected flags |  |
| Interrupt acceptance | Permitted |

Description:

This instruction XORs the contents of data memory (RAM) or a Special Function Register (SFR), as specified by d8 to d0, with the contents of the Accumulator (ACC), and then sends the result to the ACC.

Example 1:

|  |  | ACC | RAM 23H |
| --- | --- | --- | --- |
| MOV | #000H,ACC | 00H | - |
| MOV | #055H,023H | 00H | 55H |
| XOR | 023H | 55H | 55H |
| MOV | #0FFH,023H | 55H | FFH |
| XOR | 023H | AAH | FFH |

Example 2:

|  |  | ACC | B |
| --- | --- | --- | --- |
| MOV | #0FFH,ACC | FFH | - |
| MOV | #010H,B | FFH | 10H |
| XOR | B | EFH | 10H |
| MOV | #020H,B | EFH | 20H |
| XOR | B | CFH | 20H |
| MOV | #040H,B | CFH | 40H |
| XOR | B | 8FH | 40H |
| MOV | #080H,B | 8FH | 80H |
| XOR | B | 0FH | 80H |

XOR  @Rj  (XOR indirect byte to accumulator)

| Instruction code | 1 1 1 1 0 1j1j0 F4HF7H |
| --- | --- |
| Number of bytes | 1 |
| Number of cycles | 1 |
| Function | (ACC)(ACC)((Rj)) j=0,1,2,3 |
| Affected flags |  |
| Interrupt acceptance | Permitted |

Description:

This instruction XORs the contents of data memory (RAM) or a Special Function Register (SFR), as specified by the indirect address register that is specified by j1 and j0, with the contents of the Accumulator (ACC), and then sends the result to the ACC.

Example 1:

|  |  | ACC | RAM 01H | RAM 68H |
| --- | --- | --- | --- | --- |
| MOV | #000H,ACC | 00H | - | - |
| MOV | #068H,001H | 00H | 68H | - |
| MOV | #0F0H,@R1 | 00H | 68H | F0H |
| XOR | @R1 | F0H | 68H | F0H |
| MOV | #0FFH,@R1 | F0H | 68H | FFH |
| XOR | @R1 | 0FH | 68H | FFH |

Example 2:

|  |  | ACC | RAM 03H | TRL |
| --- | --- | --- | --- | --- |
| MOV | #0AAH,ACC | AAH | - | - |
| MOV | #004H,003H | AAH | 04H | - |
| MOV | #0FFH,@R3 | AAH | 04H | FFH |
| XOR | @R3 | 55H | 04H | FFH |
| XOR | @R3 | AAH | 04H | FFH |
| XOR | @R3 | 55H | 04H | FFH |
| XOR | @R3 | AAH | 04H | FFH |

ROL  (Rotate accumulator left)

| Instruction code | 1 1 1 0 0 0 0 0 E0H |
| --- | --- |
| Number of bytes | 1 |
| Number of cycles | 1 |
| Function | A7A6A5A4A3A2A1A0 |
| Affected flags |  |
| Interrupt acceptance | Permitted |

Description:

This instruction rotates the 8-bit data that is stored in the Accumulator (ACC) one bit to the left. Accordingly, the data in bit 7 of the ACC moves to bit 0.

Example 1:

|  |  | ACC |  |  |
| --- | --- | --- | --- | --- |
| MOV | #01H,ACC | 01H | 0000 | 0001B |
| ROL |  | 02H | 0000 | 0010B |
| ROL |  | 04H | 0000 | 0100B |
| ROL |  | 08H | 0000 | 1000B |
| ROL |  | 10H | 0001 | 0000B |
| ROL |  | 20H | 0010 | 0000B |
| ROL |  | 40H | 0100 | 0000B |
| ROL |  | 80H | 1000 | 0000B |
| ROL |  | 01H | 0000 | 0001B |
| MOV | #55H,ACC | 55H | 0101 | 0101B |
| ROL |  | AAH | 1010 | 1010B |
| ROL |  | 55H | 0101 | 0101B |
| ROL |  | AAH | 1010 | 1010B |
| ROL |  | 55H | 0101 | 0101B |

ROLC  (Rotate accumulator left through the carry flag)

| Instruction code | 1 1 1 1 0 0 0 0 F0H |
| --- | --- |
| Number of bytes | 1 |
| Number of cycles | 1 |
| Function | A7A6A5A4A3A2A1A0CY |
| Affected flags | CY |
| Interrupt acceptance | Permitted |

Description:

This instruction rotates the 8-bit data that is stored in the Accumulator (ACC), including the carry flag (CY), one bit to the left. Accordingly, the data in bit 7 of the ACC moves to CY, and the contents in CY move to bit 0.

Example 1:

|  |  | ACC |  |  | CY |
| --- | --- | --- | --- | --- | --- |
| MOV | #01H,ACC | 01H | 0000 | 0001B | - |
| SET1 | PSW,7 | 01H | 0000 | 0001B | 1 |
| ROLC |  | 03H | 0000 | 0011B | 0 |
| ROLC |  | 06H | 0000 | 0110B | 0 |
| ROLC |  | 0CH | 0000 | 1100B | 0 |
| ROLC |  | 11H | 0001 | 1000B | 0 |
| ROLC |  | 30H | 0011 | 0000B | 0 |
| ROLC |  | 60H | 0110 | 0000B | 0 |
| ROLC |  | C0H | 1100 | 0000B | 0 |
| ROLC |  | 80H | 1000 | 0000B | 1 |
| ROLC |  | 01H | 0000 | 0001B | 1 |
| MOV | #55H,ACC | 55H | 0101 | 0101B | 1 |
| ROLC |  | ABH | 1010 | 1011B | 0 |
| ROLC |  | 56H | 0101 | 0110B | 1 |
| ROLC |  | ADH | 1010 | 1101B | 0 |

ROR  (Rotate accumulator right)

| Instruction code | 1 1 0 0 0 0 0 0 C0H |
| --- | --- |
| Number of bytes | 1 |
| Number of cycles | 1 |
| Function | A7A6A5A4A3A2A1A0 |
| Affected flags |  |
| Interrupt acceptance | Permitted |

Description:

This instruction rotates the 8-bit data that is stored in the Accumulator (ACC) one bit to the right. Accordingly, the data in bit 0 of the ACC moves to bit 7.

Example 1:

|  |  | ACC |  |  |
| --- | --- | --- | --- | --- |
| MOV | #01H,ACC | 01H | 0000 | 0001B |
| ROR |  | 80H | 1000 | 0000B |
| ROR |  | 40H | 0100 | 0000B |
| ROR |  | 20H | 0010 | 0000B |
| ROR |  | 10H | 0001 | 0000B |
| ROR |  | 08H | 0000 | 1000B |
| ROR |  | 04H | 0000 | 0100B |
| ROR |  | 02H | 0000 | 0010B |
| ROR |  | 01H | 0000 | 0001B |
| MOV | #51H,ACC | 51H | 0101 | 0001B |
| ROR |  | A8H | 1010 | 1000B |
| ROR |  | 54H | 0101 | 0100B |
| ROR |  | 2AH | 0010 | 1010B |
| ROR |  | 15H | 0001 | 0101B |

RORC  (Rotate accumulator right through the carry flag)

| Instruction code | 1 1 0 1 0 0 0 0 D0H |
| --- | --- |
| Number of bytes | 1 |
| Number of cycles | 1 |
| Function | CYA7A6A5A4A3A2A1A0 |
| Affected flags | CY |
| Interrupt acceptance | Permitted |

Description:

This instruction rotates the 8-bit data that is stored in the Accumulator (ACC), including the carry flag (CY), one bit to the right. Accordingly, the data in bit 0 of the ACC moves to CY, and the contents in CY move to bit 7.

Example 1:

|  |  | ACC |  |  | CY |
| --- | --- | --- | --- | --- | --- |
| MOV | #01H,ACC | 01H | 0000 | 0001B | - |
| SET1 | PSW,7 | 01H | 0000 | 0001B | 1 |
| RORC |  | 80H | 1000 | 0000B | 1 |
| RORC |  | C0H | 1100 | 0000B | 0 |
| RORC |  | 60H | 0110 | 0000B | 0 |
| RORC |  | 30H | 0011 | 0000B | 0 |
| RORC |  | 18H | 0001 | 1000B | 0 |
| RORC |  | 0CH | 0000 | 1100B | 0 |
| RORC |  | 06H | 0000 | 0110B | 0 |
| RORC |  | 03H | 0000 | 0011B | 0 |
| RORC |  | 01H | 0000 | 0001B | 1 |
| MOV | #55H,ACC | 55H | 0101 | 0101B | 1 |
| RORC |  | AAH | 1010 | 1010B | 1 |
| RORC |  | D5H | 1101 | 0101B | 0 |
| RORC |  | 6AH | 0110 | 1010B | 1 |