Ｖｅｒ. 1.00

Revision :

1998/03/30	Rev (none)	Published preliminary release

1998/06/15	Rev 1.00	Published official release

- VMS Hardware Manual Rev.1.0

Chapter 1.	Overview

Chapter 2.	Internal System Configuration

Chapter 3.	Peripheral System Configuration

Chapter 4.	Control Functions

Chapter 5.	Instructions

- The following abbreviations and symbols are used in this manual

| Abbreviation | Symbol |
| --- | --- |
| ACC(A) | Accumulator |
| B | B register |
| C | C register |
| CY | Carry flag (bit 7 of PSW) |
| AC | Auxiliary carry flag (bit 6 of PSW) |
| OV | Overflow flag (bit 2 of PSW) |
| PC | Program Counter |
| ROM | Program memory |
| RAM | Data memory |
| SFR | Special Function Registers |
| PSW | Program Status Word |
| TRL | Table reference register - low byte |
| TRH | Table reference register - high byte |
| TRR | Table reference register (16 bits, expressed by TRH and TRL) |
| SP | Stack Pointer |
| Pn | Port n |
| Pmn | Port m, bit n |
| Rj | Indirect address register within current bank (part of internal RAM) |
| D9 | Direct addressing data |
| ＃i8 | Immediate data |
| B3 | Bit addressing data |
| R8 | Relative addressing data (signed 8 bits: -128 to +127) |
| R16 | Relative addressing data (unsigned 16 bits: 0 to 65535) |
| A12 | Absolute addressing data |
| A16 | Absolute addressing data |
| ( ) | Indicates contents |
|  | Transfer direction |
|  | Logical product |
|  | Logical sum |
|  | Exclusive logical sum |
| + | Addition |
| - | Subtraction |
|  | Multiplication |
|  | Division |
| Mod | Modulo |
| Set | Set a bit to "1" |
| Reset | Set a bit to "0" |
| Bit | Each individual digit of a binary value (either "0" or "1") |
| Clear | Set all bits to "0" |
| Carry | Transfer to the next highst digit position |
| Borrow | Borrowing from a higher-order digit |
| Cycle clock | Signal for one cycle that is generated during instruction execution |
| Number of bytes (BYTES) | Indicates the number of 8-bit units required for an instruction |
| Number of cycles (CYCLES) | Indicates the number of cycles required for completion of instruction execution |
| ??H | Hexadecimal value |
| ????????B | Binary value |
| MSB | Most Significant Bit (bit 7) |
| LSB | Least Significant Bit (bit 0) |
| Deleted | Deletion |
| R | Read permitted (used in tables) |
| W | Write permitted (used in tables) |
| X | Undefined (used in tables) |
| H | Unused ("1" is returned when read) (used in tables) |