VMS Specifications

Revision 0.90

2nd Development Department

CS Development and Production Division

Sega Enterprises

Contents subject to change until official version (Rev. 1.0) is distributed.

Revision：

0.75	1997/09/11	Preview Version

0.76	1997/09/16	Draft Version

Added figures and explanations

Added suspend function

0.77	1997/09/24	Changed file management numbers and memory map

Made mode buttons programmable

Added RAM save function

0.80	1998/01/31	Changed LCD screen (icons)

Changed names from "Execute" to "Game" and from "Write" to "Attention"

Changed buttons in external view

Changed alarm from two-tone to PWM

Other minor changes

0.87	1998/03/09	Added various supplemental explanations

Added explanation concerning batteries

Added font table as an appendix

0.89	1998/04/02	Deleted font table; fonts limited to use by system

Changed button configuration

Changed and added external views

Changed "buzzer" to "alarm" and "PWM" to "pulse generator"

Added Maple Bus Function Type Specification Compliance

Added section 4.6, "Miscellaneous," to section 4, "LCD"

Set the RAM area for the execution file

Added note concerning battery replacement

0.90	1998/05/10	Changed the name from suspend button to SLEEP button.

* Additions made to the latest revision before this one are marked by       and deletions by       .

Contents：

1	Overview	4

1.1	VMS Overview	4

1.2	VMS Configuration	5

1.3	VMS Functions	7

2	Mode Settings	9

3	File Management	10

3.1	Management Area	10

3.2	Data Area	10

3.3	Reserved Area	10

4	LCD Display	11

4.1	XRAM	11

4.2	Screen Mode	11

4.3	Icons	11

Screen Configuration	11

4.5	LCD Characteristics	12

4.6	Miscellaneous	12

5	Executable File Initiation	13

5.1	Downloading an Executable File	13

5.2	File Size	13

5.3	Subroutine	13

5.4	Interrupts	14

5.5	RAM	14

5.6	Save Processing During Executable File Operations	14

5.7	Auto Power Off	14

6	Communications Function	15

6.1	Maple Bus Protocol	15

6.2	Synchronous Serial Communications	15

7	Clock Function	15

7.1	Settings	15

8	Alarm Function	15

9	SLEEP Function	16

9.1	SLEEP Operation	16

10	Buttons	16

11	Batteries	17

11.1	Battery Life	17

11.2	Processing When Battery Power Is Exhausted	17

11.3	Battery Replacement	17

12	Postscript	18

# Overview

This document describes the VMS, a peripheral device for the next-generation game KATANA (preliminary name).

## VMS Overview

The VMS (Visual Memory System) is a memory cartridge that not only stores data, but also includes an LCD display that
visually expresses that data.

The VMS connects to KATANA's (preliminary name) special controller, called "SEED" (preliminary name), and can be used to display subscreens during a game and as a memory card that stores game data files.

The VMS can be connected or disconnected while the game machine is on.

When not connected to a controller, the data files stored in the VMS's memory can be displayed and deleted.  Files can
also be copied from one VMS to another by connecting two VMSs to each other.

Furthermore, by downloading special executable files (programs) from KATANA, the VMS becomes a compact portable game player; two-player games are also possible.

Fig 1.1  Conceptual Image of the VMS

In the top portion of Fig. 1.1, two VMSs are shown connected to each other as they exchange data.

The actual design will differ from the above conceptual image, in that the connectors will be located on the top of the VMS unit.

## VMS Configuration

This section describes the VMS configuration.

n	Potato Chip (custom IC for the VMS)

Core CPU:		8 bits:		Instruction cycle time:

When connected to game machine = 1[micro]s

When operating on standalone basis = 183[micro]s

(Note:) Operation on a standalone basis is extremely slow in order to minimize battery power consumption.

Memory	:		Mask-ROM:	16Kbyte	System-BIOS，IPL

:		Flash-EEPROM:	64K	Program code/data area

64K	Data area (of which 28K are reserved for the system)

:		RAM:	512 bytes	General purposes (of which 256 bytes are reserved for the system)

512bytes	I/O mapping (can also be used as a Maple buffer)

:		LCD表示RAM:	Bank 1	96 bytes

Bank 2	96 bytes

Bank 3	6 bytes (for icons; used by the system)

Serial I/F	:		Uses the following interfaces exclusively

¬ Maple:	LM-Bus

Synchronous SIO:	Two 8-bit serial interfaces

Timer	:		16bit:	For clock

:		16bit(or 8bit x2):	General purpose; of these, 8 bits are used exclusively for pulse generator output for alarms

I/O Port	:		Input/output:	16 pins (buttons, serial interfaces)

:		Input:	4 pins (control pins)

LCD-Driver／Controller	:			Common: 33 pins

Segment: 48 pins

n	LCD	:			LCD:	32 (V) x 48 (H) dots: Monochrome binary

Icons:	4 types (File, Game, Time, Attention: used by system)

n	Buzzer:			Voltage buzzer:	For alarms

n	Power supply:		Button batteries:	CR2032 x 2

External inputs: 	+5V +3.3V

External outputs:	+3.3V

n	Buttons:	6 buttons:	Four-direction key, A button, B button, Mode button, Suspend button, SLEEP button

n	Communications connector:	14 pins:	Serial interface, power supply, control

Connected to controller, another VMS, etc.

Fig 1.2  External View (preliminary)

Cap

Connectors (communications)

Suspend button

Mode button

B button

A button

Four-direction key

Keychain hole

Buzzer

Battery cover

Front view

Rear view

Fig 1.3  External Appearance and configuration (preliminary)

Fig 1.4  Block Diagram (preliminary)

## VMS Functions

When connected to a game machine, the VMS conforms with the Maple Bus 1.0 Standard Specifications, and supports the following function types.

1）FT1：Storage Function

2）FT2：B/W LCD Function

3）FT3：Timer Function

Accordingly, the Function Type (FT) is "00h-00h-00h-0Eh". (FD1 = FT3, FD2 = FT2, FD3 = FT1)

For details, refer to the specifications for each function.

An overview of the System-BIOS functions included in the VMS is provided below.

¬File management

This function manipulates and manages backup files and program files.

Files are managed in 1-block units (512 bytes), and reads and writes are also performed in block units.  FAT operations and file information processing use subroutines in the System-BIOS.  For details on file management methods, refer to section 3, "File Management."

LCD display

When the VMS is connected to a game machine, this function only draws graphics (transferring screen image data).

This function conforms with the data format that is stipulated in the Maple Bus Function Type specifications, and sends graphics images from the game machine to the VMS in accordance with the VMS screen configuration, and then BIOS transfers the resulting image to the LCD display RAM (XRAM).

The amount of data required for one screen is 32 dots (V) x 48 dots (H) = 1536 bits = 192 bytes.

When the VMS is operating on a standalone basis, this function handles the drawing of graphics.  The icons display the operation mode of the VMS.

File	：File management

Game	：Executable file initiation

Time	：Time display

Attention：Memory access in progress

®Executable file initiation

This function initiates execution of an executable file (program) that was downloaded from a game machine.

This function can only be executed while the VMS is operating on a standalone basis.  A program can not be initiated while the VMS is connected to a game machine.

A number of functions that can be provided for executable files are System-BIOS subroutines and can be used by the executable file simply by calling the subroutine.

¯Communications

When the VMS is connected to a game machine, communications are handled according to the Maple Bus protocol.

When the VMS is operating on a standalone basis, the VMS supports 8-bit synchronous serial communications for exchanging data with another VMS.

This function is also provided as a subroutine for executable files. (Not finalized)

°Clock

This function uses a timer to measure time.

This function is always operating, whether the VMS is connected to a game machine or is operating on a standalone basis.

±Alarm

This function sounds a buzzer by means of a pulse generator.  This function is also provided as a subroutine for executable files. (Not finalized)

This function conforms with the data format that is stipulated in the Maple Bus Function Type specifications, and when the VMS is connected to a game machine, this function allows the game machine to sound the buzzer.

²Mode switching

When the VMS is connected to a game machine, the VMS operation mode can be changed by pressing the mode button.

The mode status is displayed by means of icons.

When the VMS is operating on a standalone basis, the Auto Power Off function can also be used.

³Character font installation

8 dot (V) x 6 dot (H) alphabet, katakana, and symbol fonts can be installed in the VMS.  These fonts cannot be called and displayed from an executable file for the VMS that was downloaded from a game machine.

When the VMS is connected to a game machine and graphics are being displayed from the game machine side, fonts cannot be used.

Instead, transfer the screen image that is to be displayed as is.

Fonts can only be used by the System-BIOS.

# Mode Settings

The operating mode of the VMS is determined by the connection status and the mode button.

Fig 2.1  Modes

| Connection Status | Mode Button (Icon Display) | Operating Mode |
| --- | --- | --- |
| Connected to game machine | Off | System mode |
|  | Attention | Flash access in progress |
| Standalone operation | Game | Executable file initiation |
|  | File | File operations |
|  | Time | Clock display |
|  | Attention | Accessing flash memory |

¬System mode

This mode is controlled by the System-BIOS' external control program.
This mode handles communications according to the Maple Bus protocol, memory management, LCD display, and timer management.

Game mode

In this mode, the System-BIOS initiates executable files in flash memory.

All processing is controlled by the executable file, except for the Maple Bus protocol.

Transitions from this mode to another mode are also controlled by the executable file.

To execute a mode, transmission, the executable file calls a subroutine from the System-BIOS.

At that point, all of the contents of RAM and the registers are saved to flash memory.

This save operation requires approximately 8 seconds.

®File mode

This mode is controlled by the System-BIOS' file control program.

This mode can display, copy, and delete files in flash memory through button operations.

Refer to other documents for details on the configuration and operation of the file management screen.

¯Time mode

This mode is controlled by the System-BIOS' timer program. This mode can display a digital clock (showing the hours, minutes, and seconds), and can be used to set the time. When the VMS returns from system mode, it enters this mode.

Transitions among the modes occur in response to changes in the connection status and the Mode button + ??Enter button?? being pressed.

However, Game mode can suppress changes in the connection status and the Mode button + ??Enter button?? being pressed. The mode cannot be changed while data is being written to the flash memory.

“Attention” is a warning indicator that lights for “Read/Write” while flash memory is being accessed.

Fig 2.2  Mode Transitions

# File Management

File management in the VMS conforms with FT1: Storage Function in the Maple Bus 1.0 Function Type Specifications.

The size of the VMS flash memory is 128K.

The minimum read/write unit for a file is one block (512 bytes); the entire flash memory is divided into 256 blocks.

However, because 56 blocks are used as a system management area, the size of the area that can be used to store data is 200 blocks.

One executable file can exist in one partition, with a maximum size of 0080h blocks (64K: block numbers 0000h to 007Fh).

Fig 3.1  Memory Map

## Management Area

The 15 blocks at the top of memory (starting form block number 00FFh) are used for the management area.

The management area is divided into three areas: the system area, the FAT area, and the file information area.

The system area consists of one block, the FAT area consists of one block, and the file information consists of 13 blocks.

The system area is write-protected, except during formatting.

The FAT area has a chain structure in which every two bytes (16 bits) controls one block.

The file information area allocates 32 bytes to each file, and can therefore manage a maximum of 200 files.

There is only a root directory; no subdirectories are supported.

File names consist of 12 bytes (ASCII codes representing up to 12 normal-width characters).

## Data Area

The data area, where data files can be stored, consists of 200 blocks, from block number 0000h to 00C7h.

Data files are stored starting from 00C7h towards 0000h, while an executable file starts from 0000h.

The areas from 0000h to 007Fh and from 0080h to 00FFh are controlled through bank switching; switching is performed by the System-BIOS automatically.

reading and writing flash memory must always be done by calling the System-BIOS subroutines.

## Reserved Area

This area is used by the System-BIOS and in system mode.

# LCD Display

The LCD display in the VMS conforms with FT[2]: B/W LCD Function in the Maple Bus 1.0 Function Type Specifications.

The LCD that is built into the VMS consists of a 32-dot (V)  48-dot (H) dot matrix display, and four icons that indicate he operating mode f the VMS.

Drawing the LCD is accomplished by storing drawing data in the dedicated drawing RAM.

## XRAM

The LCD's dedicated drawing RAM is called "XRAM."

XRAM consists of three banks; the first and second banks are open to executable files, while the third bank is used by the System-BIOS.

The first bank of XRAM corresponds to the upper half of the LCD (16  48 dots), and the second bank of XRAM corresponds to the lower half of the LCD (16  48 dots).

One dot on the LCD corresponds to one bit in XRAM.  One byte of XRAM corresponds to 8 dots in a horizontal row on the LCD, and 6 bytes consist of one entire horizontal row on the LCD. For details, refer to "FT2 : B/W LCD Function, VMS User's Manual."

## Screen Mode

When the VMS is connected to a game machine, the System-BIOS sends drawing data from the game machine directly to the XRAM as a graphics screen.

Therefore, when using the VMS's display as a game subscreen, etc., transfer the screen image as is to the VMS.

During standalone operation, the character font in the System-BIOS cannot be used for text display on a graphics screen.

For a graphics screen, write the screen image data as is to XRAM.

## Icons

The System-BIOS uses the icons; use by an executable file is prohibited.

## Screen Configuration

Fig 4.1  LCD Screen

## LCD Characteristics

The screen refresh concept for the LCD display differs from that for a TV.

Once data is transferred to XRAM, it is displayed on the LCD, but only after a delay due to the response speed of the LCD. When the LCD response is delayed, ghosting or flickering may occur, resulting in a display that is difficult to see.  In addition, during standalone operation or when connected to a game machine, differences in the operating speeds result in different LCD display speeds.  During standalone operation, the display speed is slower.

The recommended refresh rate for the VMS' LCD is 1Hz for standalone operation and 4Hz when connected to a game machine.

## Miscellaneous

There is no contrast adjustment or brightness adjustment for the LCD.

There is no backlight for the LCD.

It is not possible to incorporate a design (such as a picture, etc.) in the polarized panel (the back sheet) with a reflective panel that reflects the light in the LCD.

# Executable File Initiation

This function initiates an executable file that was downloaded from a game machine.

The VMS can store and initiate only one executable file at a time.

The System-BIOS includes subroutines that form that VMS functions. Of these subroutines, several are provided for executable files, and an executable file can call these subroutines.

Program development of an executable file is performed using a VMS emulator (preliminary) that runs under Windows 95.

## Downloading an Executable File

Executable files are stored in flash memory in the area consisting of block numbers 0000h to 007Fh, starting from the 0000h block.  When an executable file is downloaded from a game machine application, confirm that there is contiguous free space starting from the 0000h block of the VMS.  Even if the free space has been confirmed, it still will not be possible to download an executable file if there is any other file in the area where the executable file is to be stored (the area from block 0000h to the end of the executable file).

Game machine application processing is as described below:

Get free space in VMS



Confirm that there is free space



Defragmentation processing (optimization of fragmented file storage)



Reconfirm that there is free space after defragmentation processing (Reconfirmation is necessary because it is possible that a block was damaged during defragmentation.)



Download

## File Size

The maximum size of an executable file is 0080h blocks (64K).

## Subroutine

A list of the available subroutines is shown below. (not finalized)

Each subroutine uses a RAM area (in the general-purpose RAM area) as a work area.

1. Data communications	:Performs synchronized serial communications.

2. Alarm	:Sounds the buzzer.

3. Flash memory write	:Writes flash memory.

4. Flash memory read	: Reads flash memory.

## Interrupts

A list of external and internal interrupts is provided below. (planned)

Except for the Mode Change interrupt, these interrupts cannot be masked.  (planned)

1. Low voltage interrupt

2. Timer interrupt

3. Mode Change interrupt (maskable)

4. SLEEP interrupt

## RAM

The RAM areas that executable files can use are shown below.

General-purpose RAM:	000h to 0FFh (bank 1)

I/O mapping RAM: 	000h to 1FFh (Set the address to the specified register and read/write one byte at a time.)

XRAM:	Bank 1, bank 2

## Save Processing During Executable File Operations

Data on the midpoint status of an executable file and parameters for an executable file (such as a game) are saved by writing the data to an area within the executable file. When creating an executable file (such as a game), set aside an area within the file for this purpose. Because FAT processing, etc., is not possible due to the hardware design, such data cannot be saved in a separate file.

In order to link the game machine with an application and then use the saved data from an executable file (such as a game), load the executable file from the VMS to the game machine, and then read that portion of the file that contains the saved data.

## Auto Power Off

The "Auto Power Off" function puts the VMS into the SLEEP state if no buttons are pressed or no communications are received for two minutes.

This function can be enabled/disabled by executable files.

# Communications Function

The VMS is capable of conducting serial communications with other equipment.

The VMS supports two serial communications protocols: the Maple Bus protocol and full-duplex synchronous serial communications.

The System-BIOS switches between the Maple Bus protocol in system mode and synchronous serial communications in standalone operation mode.

## Maple Bus Protocol

when the VMS is connected to a game machine, the communications connector switches to the Maple Bus protocol side.

The entire I/O mapping RAM becomes a transmission/receive buffer, and the synchronous serial side stops.

The physical connection with the game machine is made through an LM-Bus connection, and the VMS becomes an expansion device.

All processing is performed by the System-BIOS; this function is not accessible from an executable file.

The transfer speed is 2Mbps.

## Synchronous Serial Communications

When the VMS is operating on a standalone basis, the communications connector switches to synchronous serial side, and the Maple Bus protocol side stops.

There are two synchronous serial interfaces, allowing full duplex communications with other devices.

Data is transferred one byte at a time, with a maximum transfer speed of 2.4Kbps. (not finalized)

This function is available to executable files as a subroutine.

# Clock Function

The clock function in the VMS conforms with FT3 : Timer Function in the Maple Bus 1.0 Function Type Specifications.

This function can measure time in 500ms units, using a 32KHz crystal resonator and a dedicated counter.

The System-BIOS controls the clock function; an executable file can only read the clock function.

## Settings

On the setting screen, set the year, month, day, and time.

When the VMS is connected to a game machine, the date and time can be set by the game machine through the Maple Bus protocol.

# Alarm Function

The alarm function in the VMS conforms with FT3 : Timer Function in the Maple Bus 1.0 Function Type Specifications.

This function sounds the built-in voltage buzzer.

Only one alarm can be sounded at one time.

The sound is generated by the pulse generator method; the frequency can be set over a range from 300Hz to 4KHz, and the duty ratio can be set as desired. (planned)

The volume cannot be adjusted. The sound can be turned on and off.

This function is made available for executable programs as a subroutine. (planned)

When the VMS is connected to a game machine, the alarm function can be set by the game machine through the Maple Bus protocol.

# SLEEP Function

In order to reduce power consumption when operating on a standalone basis, the VMS is equipped with a SLEEP function.

The VMS enters the SLEEP state either because the SLEEP button is pressed or because the "Auto power Off" function was triggerred.  (Refer to section 5.7, "Auto Power Off.")  To return from the SLEEP state, press the SLEEP button.

## SLEEP Operation

When in Timer mode (clock display) or File mode (file management software), the LCD display shuts off and the VMS enters the idle state.

SLEEP processing in Game mode (after an executable file has been initiated is determined by the executable file.  (We plan to indicate a recommended processing method.)

The contents of RAM and the registers are retained, except in Time mode.  In SLEEP mode, all buttons are disabled except for the SLEEP button.

# Buttons

Four-direction key:	This key is used to move the cursor up, down, left, or right, and to scroll the screen.

A button:	This button is used primarily to finalize selections.

B button:	This button is used primarily to cancel selections.

Mode button:	This button changes the mode during standalone operation.  Each time this button is pressed, the mode changes according to the following cycle: File -> Game -> Time -> File -> Game ->...

SLEEP button:	This button changes the mode to the SLEEP state during standalone operation.

Reset button: 	This button initiates a "power on" reset, which initializes the entire VMS unit (including the clock, etc.), except for the contents of flash memory.

# Batteries

## Battery Life

The VMS is equipped with two CR2032 batteries for standalone operation.

Battery life depends on the status of executable file operations.

If an executable file is continuously executed, with the LCD display on (refresh rate: 1Hz), no alarm outputs, no use of the communications function, no executable file save processing, and no use of the SLEEP function, the batteries should last for about one week.

The relationship between operational status and battery life is described below.  Take battery life into consideration when creating executable files.

Flash memory reads:	This is the normal state of program execution.

LCD display updates:	Battery power consumption increases by a factor of 5 when overwriting XRAM as compared to when reading flash memory.

Frequent screen updates have an effect on battery life.

Alarm output:	Consumes an extremely small amount of power.

Flash memory writes:	Consumes 25 times more battery power than when reading flash memory. Saving the operation status and similar processing should be performed as infrequently and in as small amounts as possible.

Data exchanges after an executable file has been initiated:	Such operations consume a tremendous amount of battery power. Simple parameter exchange could be used to reflect the development of game characters, for example.

File exchanges between two VMSs:	Copying an entire file consumes a tremendous amount of battery power.  Because the receiving side in particular must write the data in flash memory, a large amount of battery power is consumed.  In addition, the larger a file is, the longer the operation will take and the greater that the power consumption will be.

## Processing When Battery Power Is Exhausted

The System-BIOS constantly monitors the battery voltage.

If the batteries are nearing the end of their life while in Game mode (while an executable file is being executed), the System-BIOS saves the contents of RAM and the registers. (planned to be implemented through the library, perhaps)

## Battery Replacement

The clock settings are initialized when the batteries are replaced.

Any file that is stored in flash memory is retained.

When replacing the batteries, always install two brand new CR2032 made by the same manufacturer.

Make sure that the polarity (+/-) of the batteries is correct when you install them.

# Postscript

The functions of the VMS are subject to change in whole or in part until the release of VMS Specifications Revision 1.0.