# 2

Chapter 4.

Source File Input Format

A source file consists of character strings of up to 511 characters per line (including a CR or LF code at the end of each line). In addition, no distinction is made between upper- and lower-case letters, except in symbols (such as labels and macro names) that are defined in the source program. For example, both "Nop" and "nop" are recognized as the mnemonic for the NOP instruction.  Furthermore, the distinction made between upper- and lower-case characters in symbols such as labels can also be disabled by specifying the assembler option "-I".

  - Statements

Statements consist of a combination of mnemonics (which define the object code that is to be produced during the assembly operation), operands and comments. One line of source code is equivalent to one mnemonic.  Multi-line statements are not supported.  Each statement consists of the four fields described below.

[label:]  [operation]  [operand]  [;comment]

| Field | Purpose |
| --- | --- |
| label | A label is applied to a statement so that other statements can use that label to access the statement. Labels must always be delimited with a colon (":"). |
| operation | Specifies the operation of the statement. |
| operand | Defines the data that is the target of the operation of the statement. |
| comment | Describes the statement; has no effect on the assembly operation. |

Note: Data that is enclosed in brackets may be omitted.

  - Label Names and Symbol Names

Label names and symbol names are character strings of any length (but of at least one character).  However, only the first 32 characters of a label name or symbol name are valid. The following characters may be used in a label or symbol name:

AZ, az, 09

$

?

-

@

.

In addition, the first character in a label or symbol name must be a letter, "_", ".", or "@". If the assembler option "-i" has not been specified, a distinction is made between upper- and lower-case letters. Furthermore, label names must be delimited with a colon (":").

  - Comments

Comments begin with a semi-colon (";") and end with a line feed.

  - Operators

The following chart lists the operators that can be used in M86K and their priority. No distinction is made between upper- and lower-case characters in operators that consist only of letters, such as NOT. NOT and not are recognized as the same operator.

| Operator | Description | Priority ranking |
| --- | --- | --- |
| NOT | 1's complement | 1 |
| HIGH | Upper byte |  |
| LOW | Lower byte |  |
| * | Multiplication | 2 |
| / | Division |  |
| MOD | Modulo |  |
| + | Addition | 3 |
| - | Subtraction |  |
| SHR | Right shift | 4 |
| SHL | Left shift |  |
| LAND | Logical product | 5 |
| LOR | Logical sum |  |
| LXOR | Exclusive logical sum |  |
| EQ | Equal to | 6 |
| NE | Not equal to |  |
| LT | Less than |  |
| LE | Less than or equal to |  |
| GT | Greater than |  |
| GE | Greater than or equal to |  |

  - Numeric constants

M86K permits description of numeric constants in four bases: binary, octal, decimal, and hexadecimal.  There are two formats for writing numeric constants: one in which the base is explicitly indicated ("123H"), and one in which the pseudo instruction RADIX is used to specify the default base beforehand. Values that are written with an explicitly specified base are converted to the default base. A numeric constant for which no base is explicitly specified, such as "123," is converted to the base that was specified by the pseudo instruction RADIX. If the pseudo instruction RADIX has not been used to specify a base, the default base is "decimal."

No matter which method is used to write constants, the assembler processes the values internally in 32-bit format. In addition, when a numeric constant or the value of an expression that is ultimately returned to a numeric constant is written as immediate data for an operand, only the bits that are necessary for that operand are stored; the remaining upper bits are discarded.

Formats for numeric constants with an explicitly specified base

| Base | Format | Examples |
| --- | --- | --- |
| 2 | Percent sign ("%"), followed by one or more digits 0 and 1 | %01111011%11111111%0000010000000000 |
|  | One or more digits 0 and 1, followed by "B"* | 01111011B11111111B0000010000000000B |
|  | One or more digits 0 and 1, followed by ".B" | 01111011.B11111111.B0000010000000000.B |
| 8 | One or more digits 0 through 7, followed by ".O" | 273.O377.O2000.O |
| 10 | One or more digits 0 through 9, followed by ".D" | 123.D255.D1024.D |
| 16 | Dollar sign ("$"), followed by one or more digits 0 through 9, "a" through "f", or "A" through "F" | $7B$FF$0400 |
|  | An initial digit 0 through 9, which may be followed by digits 0 through 9, "a" through "f", or "A" through "F", followed by "H" | 7BH0FFH0400H |
|  | An initial digit 0 through 9, which may be followed by digits 0 through 9, "a" through "f", or "A" through "F", followed by ".H" | 7B.H0FF.H0400.H |

*	No distinction is made between upper- and lower-case characters that are used to specify the base ("B", "O", "D", and "H"). This is not affected by the assembler option "-i".

*	This format is affected by the RADIX setting. For details, refer to the table below.

Interpretation of numeric constants in a format where the base is not explicitly specified

| Format | Example | Value in base specified by RADIX |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 2 | 8 | 10 | 16 |
| One or more digits 0 and 1 | 0101 | 510 | 6510 | 10110 | 25710 |
| One or more digits 0 through 7 | 123 | error | 8310 | 12310 | 29110 |
| One or more digits 0 through 9 | 789 | error | error | 789110 | 192910 |
| One or more digits 0 and 1, followed by "B" | 101B | 510 | 510 | 510 | 412310 |
| An initial digit 0 through 9, which may be followed by digits 0 through 9, "a" through "f", or "A" through "F" | 0FF | error | error | error | 25510 |

  - Character Constants

Characters enclosed in single quotes ( ' ) are handled as character constants. Character constants are a type of constant, the value of which is the ASCII code of the character that is specified. In addition to being able to write all printable ASCII characters, other codes may also be written in the following format. Note that when more than one character is enclosed in quotes, it is handled as a "character string constant" (see section 4.7), not a "character constant."

Format for inputting codes within character constants and character string constants

| Format | Code (hexadecimal) | Remarks |
| --- | --- | --- |
| \n | 0A | Line feed |
| \r | 0D | Return |
| \t | 09 | Horizontal tab |
| \b | 08 | Back space |
| \f | 0C | Paper feed |
| \" | 22 | Double quotes |
| \' | 27 | Single quotes |
| \\ | 5C | Yen mark or backslash |
| \ooo |  | "ooo" represents an octal number of up to three digits |
| \xhh |  | "hh" represents a hexadecimal number of up to two digits |

Example 1:  ADD#‘A’

Example 2:  DB ‘A’,’\012’,’C’

Example 3:  DB  Regarded as a character string constant, and generates an error as an operand for DB.

  - Character String Constant

A character string consisting of one or more characters that is enclosed in double quotes ( " ) or a character string consisting of two or more characters that is enclosed in single quotes ( ' ) is handled as a character string constant. A character string constant can be written as an operand for the pseudo instruction DC or .PRINTX. In addition to being able to write all printable ASCII codes in a character string, it is also possible to write other codes using the format described in section 4.6.

Example:  DC	“Thisisasamplestringwithspecialcodes\007\r\n”

  - Special Symbols

When an asterisk is used as an operand, it represents the value of the described location.

Example 1:  To indicate the value that was six bytes ahead of the location where the instruction is written:

BR      *-6

Example 2:  To indicate the value that was twelve bytes after the location where the instruction is written:

BR      * +12