# 2

  - IFDIF  (Assemble if two character strings are not identical)

IFDIF  <string1>, <string2>

The IFDIF pseudo instruction assembles the source program until either ELSE or ENDIF appears subsequently, but only if string1 and string2 are different. string1 and string2 must be enclosed in <>.  The comparison includes any space or tab characters within the <>.

Example:

page:    1 <ifdif.ASM>

ERR	SEQ.     S LOC. OBJ.	SOURCE STATEMENTS

0001		; a sample program for IFDIF

0002			chip	lc866032

0003			cseg

0004		tifidn	macro	arg1,arg2

0005			ifdif	<<arg1>>,<<arg2>>

0006 				inc 	a

Because one pair of <> is deleted during macro expansion, double symbols (<<>>) are required.

0007			else

0008				dec	a

0009			endif

0010			endm

0011

0012			tifidn	same, same

0012+1			ifdif	<same>,<same>

0012+2				inc	a

0012+3			else

0012+4	C 0000 7300			dec 	a

0012+5			endif

0013			tifidn	same, not_same

0013+1			ifdif	<same>,<not_same>

0013+2	C 0002 6300			inc	a

0013+3			else

0013+4				dec	a

0013+5			endif

0014			end

Because the two character strings are different, this portion is assembled.

Because the two character strings are identical, this portion is not assembled.

  - ELSE  (Assemble in the case of the condition that is the opposite of the above IF condition)

ELSE

The ELSE pseudo instruction assembles the source program if the opposite of the preceding IF condition is true, until ENDIF appears. For an example, refer to the description of the IFDEF pseudo instruction, etc.

  - ENDIF  (End conditional assembly)

ENDIF

The ENDIF pseudo instruction declares the end of conditional assembly.  For an example, refer to the description of the IFDEF pseudo instruction, etc.

  - .PRINTX  (Display on VDT during assembly)

.PRINTX  “string”

The .PRINTX pseudo instruction outputs the contents of string character string constants on the VDT during the assembly process. For details on character string constants, refer to section 4.7.

Example:

Source program

Display on screen

; a sample program for .PRINTX

chip	lc866000

switch	equ	1

SANYO (R) LC86K series Macro As

Copyright (c) SANYO Electric Co

Pass 1 .....

Start

..CSEG

Condition#1

..DSEG

End

Source file:	pprintx

Chip name:	LC866000

ROM  size:	64K bytes

RAM  size:	384 bytes

XRAM size:	128 bytes

Pass 2 .....

Start

..CSEG

Condition#1

..DSEG

End

.printx “Start”

cseg	inblock

.printx “..CSEG”

ld	count

add	b

st	data1

ifdef	switch

.printx “Condition#1”

inc	data1

else

.printx “Condition#2”

dec	data1

endif

dseg

.printx “..DSEG”

count:	ds	1

data1:	ds	1

.printx “End”

end

Because this portion is not assembled due to the IFDEF pseudo instruction, the corresponding output is also not displayed.

  - .LIST  (Output list)

.LIST

The .LIST pseudo instruction releases the status in which list file output was halted by the .XLIST pseudo instruction.

Example:

; a sample program for LIST

chip	lc866200

cseg	inblock

mov	#00, count

ld	count

add	#10h

st 	b

The lines that follow the line where .XLIST appears do not appear in the list file. Counting of the line numbers continues, however, so no disagreement in line numbering occurs.

After the .LIST pseudo instruction, output to the list file resumes.

.xlist

abc	equ	10h

dseg

count:	ds	4

.list

cseg	inblock

ld	b

sub	#abc

st	count

end

page:    1 <plist.ASM>

ERR	SEQ.     S LOC. OBJ.	SOURCE STATEMENTS

0001		; a sample program for LIST

0002			chip	lc866200

0003			cseg	inblock

0004	C 0000 220000’	mov	#00, count

0005	C 0003 0200’	ld	count

0006	C 0005 8110		add	#10h

0007	C 0007 1302		st	b

0008

0014			.list

0015			cseg	inblock

0016	C 0000 0302		ld	b

0017	C 0002 A110’	sub	#abc

0018	C 0004 1200’	st	count

0019			end

  - .XLIST  (Interrupt list output)

.XLIST

The .XLIST pseudo instruction interrupts output to the list file. For an example, refer to the description of the .LIST pseudo instruction.

  - .MACRO  (Output macro expansion)

.MACRO

The .MACRO pseudo instruction expands the main body of a macro and outputs it to the list file when the macro is called.

Example:

; a sample program for .MACRO

chip	lc866200

t.mac	macro

inc	a

inc	b

.XMACRO disables output of the macro expansion results to the list file. Note that the codes that are generated by the expanded statements are also not output.

endm

cseg	inblock

t.mac

.xmacro

t.mac

.macro

t.mac

.MACRO resumes output of the macro expansion results to the list file.

end

page:    1 <pmacro.ASM>

ERR  	SEQ.     S LOC. OBJ.   SOURCE STATEMENTS

0001		; a sample program for .MACRO

0002			chip	lc866200

0003		t.mac	macro

0004			inc	a

0005			inc 	b

0006			endm

0007

0008			cseg	inblock

0009			t.mac

0009+1	C 0000 6300		inc	a

0009+2	C 0002 6302		inc	b

0010			.xmacro

0011			t.mac

0012			.macro

0013			t.mac

0013+1	C 0008 6300		inc	a

0013+2	C 000A 6302		inc	b

0014			end

  - .XMACRO  (Interrupt macro expansion output)

.XMACRO

The .XMACRO pseudo instruction temporarily interrupts the output to the list file of the results of expansion of the macro main body when a macro is called. For an example, refer to the explanation of the .MACRO item.

  - .IF  (Output conditional skip)

.IF

The .IF pseudo instruction expands source program statements that were skipped during conditional instruction execution and outputs them to the list file.

Example:

; a sample program for .IF

chip    lc866200

t.if    macro   arg1

ifb     <<arg1>>

inc     a

.XIF disables the output to the list file of statements that were skipped by conditional assembly pseudo instructions. Lines which were not skipped because the specified condition was matched are output to the list file, regardless of the .XIF pseudo instruction.

.IF enables the output to the list file of statements that were skipped by conditional assembly pseudo instructions.

else

inc     b

endif

endm

cseg    inblock

t.if

.xif

t.if    abc

.if

t.if    def

end

page:    1 <pif.ASM>

ERR	SEQ.	S LOC. OBJ.	SOURCE STATEMENTS

0001		; a sample program for .IF

0002			chip	lc866200

0003

0004		t.if	macro 	arg1

0005			ifb	<<arg1>>

0006			inc 	a

0007			else

0008			inc	b

0009			endif

0010			endm

0011			cseg	inblock

0012			t.if

0012+1			ifb	<>

0012+2	C 0000 6300			inc	a

0012+3			else

0012+4				inc	b

0012+5			endif

0013			.xif

0014			t.if	abc

0014+1			ifb	<abc>

0014+3			else

0014+4	C 0002 6302			inc 	 b

0014+5			endif

0015			.if

0016			t.if	def

0016+1			ifb 	<def>

0016+2				inc	a

0016+3			else

0016+4	C 0004 6302			inc	b

0016+5			endif

0017			end

  - .XIF  (Interrupt conditional skip output)

.XIF

The .XIF pseudo instruction disables the expansion in the list file of source program statements that were skipped during conditional instruction execution. For an example, refer to the explanation of the .IF pseudo instruction.

  - INCLUDE  (Load file)

INCLUDE  filename

The INCLUDE pseudo instruction loads and assembles the source file specified by filename while assembling the source program. The filename specification must include the extension. The INCLUDE pseudo instruction can be nested up to nine levels. If the loaded file contains the END pseudo instruction, the assembly process stops there.

Example:

INCLUDE.ASM

; a sample program for INCLUDE

INCLUDE1.ASM

INCLUDE INCLUDE2.ASM

INCLUDE INCLUDE1.ASM

end

INCLUDE2.ASM

chip    lc866200

cseg

mov     #0, acc

M86K INCLUDE,,INCLUDE;

page:    1 <include.ASM>

ERR  SEQ.     S LOC. OBJ.	SOURCE STATEMENTS

0001			; a sample program for INCLUDE

0002			INCLUDE INCLUDE1.ASM

1/0001			INCLUDE INCLUDE2.ASM

2/0001			chip	lc866200

2/0002			cseg

2/0003	C 0000 230000		mov	#0, acc

0003			end

Indicates the Include nesting level

  - TITLE  (Specify list title)

TITLE  string

The parameter string of the TITLE pseudo instruction specifies a title for the list file. Unlike character string constants, string is not enclosed in quotes, etc. In addition, it is not possible to input a code that begins with the yen symbol.

Example:

string is displayed here on all pages.

page:    1 <title.ASM> sample program’s title for the listing

ERR	SEQ.	S LOC. OBJ.   SOURCE STATEMENTS

0001		; a sample program for TITLE

0002			TITLE	sample program’s title for the listing

0003			chip	lc864024

0004			cseg

0005	C 0000 00		nop

0006			end

  - PAGE  (End of page)

PAGE

The PAGE pseudo instruction forcibly ends a page during output to a list file. The end-of-page character is inserted directly before this pseudo instruction.

Example:

Source file

; a sample program for PAGE

chip    lc866032

page

cseg

page

nop

page

end

page:    2 <page.ASM>

ERR  SEQ.     S LOC. OBJ.   SOURCE STATEMENTS

0003                           page

0004                           cseg

page:    3 <page.ASM>

ERR  SEQ.     S LOC. OBJ.   SOURCE STATEMENTS

0005page

0006     C 0000 00nop

page:    4 <page.ASM>

ERR  SEQ.     S LOC. OBJ.   SOURCE STATEMENTS

0007                           page

0008                           end

List file

page:    1 <page.ASM>

ERR  SEQ.     S LOC. OBJ.	SOURCE STATEMENTS

0001	; a sample program for PAGE

0002                           chip    lc866032

  - CHIP  (Define chip that is target of assembly)

CHIP  chipname

The CHIP pseudo instruction notifies the assembler about which chip is the target of the assembly process.  The assembler switches the reserved word list and checks the memory size on the basis of chipname. This pseudo instruction is written at the top of a source file, before any other instructions or pseudo instructions.  If this pseudo instruction is not found, the assembler references the value of the environment variable CHIPNAME. If the name of the chip that is declared by this pseudo instruction does not match the name indicated by the environment variable CHIPNAME, a "warning-level" error is generated.

  - COMMENT  (Output comments to object file)

COMMENT  comment_string

The COMMENT pseudo instruction gives comments that are output to the assembled object file. Unlike character string constants, comment_string is not enclosed in quotes, etc. In addition, it is not possible to input a code that begins with the yen symbol. comment_string is stored in the object file starting from the 680th byte. A comment may consist of up to 255 characters.

Example:

Source file

; a sample program for COMMENT

chip    lc866024

comment This is a comment string embedded into OBJ file

cseg

nop

end

Object file dump (showing only the necessary portion)

Number of characters (one byte)

00000260  00 00 00 00 00 00 00 00-00 00 00 00 00 00 00 00   ................

00000270  00 00 00 00 00 60 00 00-80 01 00 00 80 00 00 00   .....’..........

00000280  C6 92 40 2B 4D 38 36 4B-20 20 20 20 63 6F 6D 6D   **+M86K    comm

00000290  65 6E 74 2E 41 53 4D 20-63 6F 6D 6D 65 6E 74 20   ent.ASM comment

000002A0  4C 43 38 36 36 30 32 34-30 54 68 69 73 20 69 73   LC8660240This is

000002B0  20 61 20 63 6F 6D 6D 65-6E 74 20 73 74 72 69 6E    a comment strin

000002C0  67 20 65 6D 62 65 64 64-65 64 20 69 6E 74 6F 20   g embedded into

000002D0  4F 42 4A 20 66 69 6C 65-00 00 01 01 00 01 00 05   OBJ file........

000002E0  00 01 00 00 00 00 00 00-00 00 E0 00 00 00 00 C4   ...............*

000002F0  00 00 00 00 C4 00 00 00-00 24 00 00 01 00 04 01   ....*....$......

00000300  00 00 00 24                                              ...$

  - WIDTH  (Specify number of columns in list file)

WIDTH  number

The WIDTH pseudo instruction specifies the number of columns in the list file (i.e., the number of characters per line). A number from 72 to 132 can be specified for number, but specifying the value equal to the number of columns in the source file plus at least 28 is recommended whenever possible. Furthermore, although this pseudo instruction can be described any number of times within one source file, normally it is only described once at the start of file. Note that if this pseudo instruction is not found, the default list file width is 132 columns.

Example:

Although WIDTH is evaluated in both pass 1 and pass 2, the list file is generated only in pass 2. Therefore, the last evaluated result for WIDTH in pass 1 is reflected here, so this line wraps around at this position.

1	2	3	4	5	6	7	8

12345678901234567890123456789012345678901234567890123456789012345678901234567890

page:    1 <width.ASM>

ERR  SEQ.     S LOC. OBJ.   SOURCE STATEMENTS

0001                          ; a sample program for WIDTH

0002                                  chip    lc866200

0003                                   cseg    ; this is a long line to indicat

0003                          e WIDTH’s effect

0004                                  WIDTH   72

0005     C 0000 00                    nop     ; this is also a long line

0005                           to indicate WIDTH’s effect

0006                                  WIDTH   78

0007                                  end

Because a line feed character is inserted at the 72nd character, the line wraps around at this position.

  - BANK  (Specify RAM area bank)

BANK  expression

The BANK pseudo instruction gives the bank number for symbols that were defined by the DS pseudo instruction in the RAM area described subsequent to the DSEG pseudo instruction.

Example:

page:    1 <bank.ASM>

ERR	SEQ.     S LOC. OBJ.   	SOURCE STATEMENTS

0001		; a sample program for BANK

0002			chip 	lc866032

0003			cseg	inblock

0004

0005	C 0000 220000’		mov	#0,data1

0006

0007	C 0003 6200’		inc	data1

0008	C 0005 0200’		ld	data1

0009	C 0007 1201’		st	data2

0010

0011	C 0009 6200’		inc	dataa

0012	C 000B 0200’		ld	dataa

0013	C 000D 1202’		st	datac

0014

0015			dseg

0016			bank	0

0017	D 0000	data1:	ds	1

0018	D 0001	data2:	ds	1

0019	D 0002	data3:	ds	1

0020

0021			bank	1

0022	D 0000	dataa:	ds	1

0023	D 0001	datab:	ds	1

0024	D 0002	datac:	ds	1

0025

0026			end

These symbols are assigned to bank 1.

These symbols are assigned to bank 0.

  - CHANGE  (Jump between external and internal ROM)

CHANGE  symbol

CHANGE is a special jump instruction that is used to switch between executing code stored in external ROM and code stored in internal ROM.  The operand symbol is limited to symbols that have been declared by the pseudo instruction OTHER_SIDE_SYMBOL. Note that this pseudo instruction is a special instruction for the LC868000 only, and will generate an error if executed by any other type of chip.

Example:

page:    1 <change.ASM>

ERR	SEQ.	S LOC. OBJ.	SOURCE STATEMENTS

0001		; a sample program for CHANGE

0002			chip	lc868032

0003			other_side_symbol       far_away

0004

0005			cseg

0006	C 0000 B80D21’	change  far_away

0006	C 0003 0000’

  - RADIX  (Specify default base)

RADIX  expression

The RADIX pseudo instruction specifies the base to which the value of a numeric constant is converted when the base of that constant is not explicitly indicated. Only certain values can be specified for expression: 2, 8, 10, and 16. Once this pseudo instruction is executed, the specified base remains valid until a different base is specified by another RADIX pseudo instruction. If this pseudo instruction is not specified, the default base is 10.

Example:

xxx	SET	10	Interpreted as 1010, since the default base is 10.

RADIX	16

xxx	SET	10	Interpreted as 1610, since the base was set to 16.

RADIX	2

xxx	SET	10	 Interpreted as 210, since the base was set to 2.

  - JMPO  (Generate optimal JMP instruction)

JMPO  expression

The JMPO pseudo instruction compares the current location with expression, and generates a JMP if the two locations are in the same block (i.e., the addresses are identical except for the lower 12 bits). If the two locations are not within the same block, or if the value of the destination address cannot be specifically determined because it is an external symbol, then JMP generates a JMPF.

Example:

Generates a JMP instruction when the current location and "expression" are within the same memory block

page:    1 <jmpo.ASM>

ERR	SEQ.	S LOC. OBJ.	SOURCE STATEMENTS

0001		; a sample program for JMPO

0002			chip	lc866032

0003			cseg

0004 	C 0000 2803’	jmpo	near

0005	C 0002 00		nop

0006	C 0003 00	near:	nop

0007	C 0004 211000’	jmpo	far

0008

0009			org	1000h

0010	C 1000 00	far:	nop

0011			end

Generates a JMPF instruction when the current location and "expression" are in different memory blocks

  - BRO  (Generate optimal BR instruction)

BRO  expression

The BRO pseudo instruction compares the current location with expression, and generates a BR if the branching destination is within a range of +127 and -128. If the branching destination is outside of a range of +127 and -128, then BRO generates a BRF.

Example:

Generates a BR instruction when the branching destination is within a range of +127 and -128

page:    1 <bro.ASM>

ERR  	SEQ.     S LOC. OBJ.	SOURCE STATEMENTS

0001				; a sample program for BRO

0002				chip	lc866032

0003				cseg

0004	C 0000 0101			bro	near

0005	C 0002 00			nop

0006	C 0003 00		near: 		nop

0007	C 0004 11FA00			bro	far

0008

0009				org	100h

0010	C 0100 00		far:	nop

0011				end

Generates a BRF instruction when the branching destination is outside of a range of +127 and -128

  - CALLO  (Generate optimal CAL instruction)

CALLO  expression

The CALLO pseudo instruction compares the current location with expression, and generates a CALL if the two locations are in the same block (i.e., the addresses are identical except for the lower 12 bits). If the two locations are not within the same block, or if the value of the destination address cannot be specifically determined because it is an external symbol, then CALLO generates a CALLF.

Example:

Generates a CALL instruction when the current location and "expression" are within the same memory block

page:    1 <callo.ASM>

ERR  	SEQ.     S LOC. OBJ. 	SOURCE STATEMENTS

0001			; a sample program for CALLO

0002			chip	lc866032

0003			cseg

0004	C 0000 0805’		callo	near

0005	C 0002 201000’		callo	far

0006

0007	C 0005 00	near:	nop

0008	C 0006 A0		ret

0009

0010			org	1000h

0011	C 1000 00	far:	nop

0012	C 1001 A0 		ret

0013			end

Generates a CALLF instruction when the current location and expression are in different memory blocks

  - BZO  (Generate BZ instruction that will not generate an address error)

BZO  expression

The BZO macro generates instruction codes that are equivalent to the BZ instruction, with no restriction on the difference between the location of the instruction and the branching destination in the same segment within the same source. The BZO macro uses the BNZ instruction, which is the logical opposite of the BZ instruction, and the BRO instruction. expression describes the branching destination.

Code generation macro:

; ***	Branch near relative address if accumulator is zero ***

bzo	macro	r8

local	_next_

bnz	_next_

bro	r8

_next_:

endm

  - BNZO  (Generate BNZ instruction that will not generate an address error)

BNZO  expression

The BNZO macro generates instruction codes that are equivalent to the BNZ instruction, with no restriction on the difference between the location of the instruction and the branching destination in the same segment within the same source. The BNZO macro uses the BZ instruction, which is the logical opposite of the BNZ instruction, and the BRO instruction. expression describes the branching destination.

Code generation macro:

; ***	Branch near relative address if accumulator is not zero ***

bnzo	macro	r8

local	_next_

bz	_next_

bro	r8

_next_:

endm

  - BPO  (Generate BP instruction that will not generate an address error)

BPO  expression

The BPO macro generates instruction codes that are equivalent to the BP instruction, with no restriction on the difference between the location of the instruction and the branching destination in the same segment within the same source. The BPO macro uses the BZ instruction, the BR instruction, and the BRO instruction.  expression describes the branching destination.

Code generation macro:

; ***	Branch near relative address if direct bit is positive ***

bpo	macro	d9,b3,r8

local	_next_

local	_true_

bp	d9,b3,_true_

br	_next_

_true_:	bro	r8

_next_:

endm

  - BPCO  (Generate BPC instruction that will not generate an address error)

BPCO  expression

The BPCO macro generates instruction codes that are equivalent to the BPC instruction, with no restriction on the difference between the location of the instruction and the branching destination in the same segment within the same source. The BPCO macro uses the BPC instruction, the BR instruction, and the BRO instruction. expression describes the branching destination.

Code generation macro:

; ***	Branch near relative address if direct bit is positive,

;	and clear ***

bpco	macro	d9,b3,r8

local	_next_

local	_true_

bpc	d9,b3,_true_

br	_next_

_true_:	bro	r8

_next_:

endm

  - BNO  (Generate BN instruction that will not generate an address error)

BNO  expression

The BNO macro generates instruction codes that are equivalent to the BN instruction, with no restriction on the difference between the location of the instruction and the branching destination in the same segment within the same source. The BNO macro uses the BN instruction, the BR instruction, and the BRO instruction. expression describes the same value as in the BN instruction.

Code generation macro:

; ***	Branch near relative address if direct bit is negative ***

bno	macro	d9,b3,r8

local	_next_

local	_true_

bn	d9,b3,_true_

br	_next_

_true_:	bro	r8

_next_:

endm

  - DBNZO  (Generate DBNZ instruction that will not generate an address error)

DBNZO  expression

The DBNZO macro generates instruction codes that are equivalent to the DBNZ instruction, with no restriction on the difference between the location of the instruction and the branching destination in the same segment within the same source. The DBNZO macro uses the DBNZ instruction, the BR instruction, and the BRO instruction. expression describes the same value as in the DBNZ instruction.

Code generation macro:

; ***	Decrement direct byte and branch near relative address

;	if direct byte is not zero ***

dbnzo	macro	d9,r8

local	_next_

local	_true_

dbnz	d9,_true_

br	_next_

_true_:	bro	r8

_next_:

endm

  - BEO  (Generate BE instruction that will not generate an address error)

BEO  expression

The BE macro generates instruction codes that are equivalent to the BE instruction, with no restriction on the difference between the location of the instruction and the branching destination in the same segment within the same source. The BE macro uses the BNE instruction and the BRO instruction. expression" describes the same value as in the BE instruction.

Code generation macro:

; ***	Compare immediate data or accumulator and branch

;	near relative address if equal ***

beo	macro	arg0,arg1,arg2

local	_next_

local	_txen_

ifb	<<arg2>>

bne	arg0,_next_

bro	arg1

_next_:

else

bne	arg0,arg1,_txen_

bro	arg2

_txen_:

endif

endm

  - BNEO  (Generate BNE instruction that will not generate an address error)

BNEO  expression

The BNEO macro generates instruction codes that are equivalent to the BNE instruction, with no restriction on the difference between the location of the instruction and the branching destination in the same segment within the same source. The BNEO macro uses the BE instruction and the BRO instruction. expression describes the same value as in the BNE instruction.

Code generation macro:

; ***	Compare immediate data or accumulator and branch

;	near relative address if equal ***

bneo	macro	arg0,arg1,arg2

local	_next_

local	_txen_

ifb	<<arg2>>

be	arg0,_next_

bro	arg1

_next_:

else

be	arg0,arg1,_txen_

bro	arg2

_txen_:

endif

endm