  - Serial Interface

    - Overview

POTATO has a built-in 2-channel synchronous serial interface with an 8-bit data length that uses port 1. Because the next-generation game machine interface also uses port 1, use of the next-generation game machine interface must be prohibited by software if the synchronous serial interface is being used. The main functions of this interface are listed below.

- 2-channel synchronous serial interface

- Transfer clock selection function

- Serial interface SIO0 transfer clock polarity switching function

- LSB-first/MSB-first switching function

- Operation mode switching function

- Overrun detection function

- Transfer bit length control function

    - function

- 2-channel synchronous serial interface

Two serial interfaces are provided, with SIO0 using P10 through 12 as I/O pins and SIO1 using P13 through 15 as I/O pins.

Normally, in the VMS SIO0 is uses ad the master and SIO1 is used as the slave.

- Transfer clock selection function

One of the following three clocks can be selected. In addition, the polarity of the transfer clock for SIO0 only can be selected.

- Internal clock

- External clock

- Software clock

- Serial interface SIO0 transfer clock polarity switching function (bus can be supported)

The polarity of the transfer clock SCK0 for serial interface SIO0 can be switched.

¬	When operation is stopped, SCK0 = 1 and data output is maintained

When operation is stopped, SCK0 = 0 and data output is bit 0 of SBUF0

- LSB-first/MSB-first switching function

It is possible to switch between starting transfers from the LSB or the MSB in data communications over the serial interface. This setting can be made separately for each channel.

- Overrun detection function

This function generates an error when a clock that exceeds 8 bits is received.

- Transfer bit length control function

This function selects whether to stop operation after 8 bits have been transferred, or to continue transfer operations after 8 bits have been transferred.

- Interrupt generation

When the interrupt request enable bit is set, the SIO0 and SIO1 interrupt requests are generated by overflows in an octal counter.

In order to control the serial interfaces, it is necessary to manipulate the following Special Function Registers:

	SCON0		SCON1		SBR

	SBUF0		SBUF1

	P1		P1DDR		P1FCR

    - Circuit Configuration

The configuration of the serial interfaces is shown in Fig. 3-5-1 and 2.

- Shift register ····· ¬

This consists of an 8-bit shift register (SBUF0 and 1), and operates according to the specified clock.

- Octal counter ·····

This counts the shift clock and detects the end of transfers.

- Baud rate generator ····· ®

This consists of an 8-bit register (SBR) for setting data and an 8-bit reload counter. If "internal clock" is selected for the transfer clock, data transfers are executed according to the clock that is generated here. This baud rate generator is used for both SIO0 and SIO1.

- Polarity switching circuit

This circuit controls the polarity of the transfer clock before and after serial transfer.

Fig. 3-5-1  Serial Interface (SIO0) Block Diagram

Fig. 3-5-2  Serial Interface (SIO1) Block Diagram

    - Related Registers

- SIO0 control register (SCON0)

| Symbol | Address | R/W | Bit 7 | Bit 6 | Bit 5 | Bit 4 | ビBit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCON0 | 130H | R/W | SCON07 | SCON06 | - | SCON04 | SCON03 | SCON02 | SCON01 | SCON00 |
| After reset |  |  | 0 | 0 | H | 0 | 0 | 0 | 0 | 0 |

| Bit name | Function |
| --- | --- |
| SCON07 (bit 7) | Polarity control |
|  | 0: When operation is stopped, SCK0 = 1 and data output is maintained 1: When operation is stopped, SCK0 = 0 and data output is bit 0 of SBUF0 |
| SCON06 (bit 6) | Overrun flag |
|  | 0: No overrun 1: Overrun |
| SCON04 (bit 4) | Transfer bit length control |
|  | 0: 8-bit transfer 1: Continuous transfer |
| SCON03 (bit 3) | Transfer control |
|  | 0: 0: LSB-first 1: MSB-first |
| SCON02 (bit 2) | LSB-/MSB-first select |
|  | 0: LSB-first 1: MSB-first |
| SCON01 (bit 1) | Serial transfer end flag |
|  | 0: Transfer in progress 1: Transfer completed |
| SCON00 (bit 0) | Interrupt request enable |
|  | 0: Interrupt request disabled 1: Interrupt request enabled |

SCON07 (bit 7):	SCK0 polarity control

This bit controls the polarity of the transfer clock SCK0 that is used by SIO0. When "1" is set, SCK0 becomes "0" when operation of SIO0 has stopped, and bit 0 of SBUF0 is output. This mode permits bus support. When "0" is set, SCK0 becomes "1" when operation of SIO0 has stopped, and the last data that was transferred is maintained on the output.

SCON06 (bit 6):	Overrun flag

This flag is used to detect a serial transfer error on SIO0. If a transfer clock is received (a falling edge is detected) after the transfer of 8 bits of data has been completed (SCON01 has been set to "1"), this flag is set. In addition, when executing a continuous transfer, the overrun flag is set after every eight bits. This bit is not reset automatically; it must be reset by software.

SCON04 (bit 4):	Transfer bit length control

This bit selects the SIO0 transfer data bit length: continuous (1) or 8 bits (0). When this bit is set to "1," two or more 8-bit bytes of data can be sent consecutively. This flag does not change after a transfer; it must be reset by software. When this bit is set to "0," eight bits of data can be transferred.

SCON03 (bit 3):	SIO0 operation control

This bit starts (1)/stops (0) SIO0 transfer. When this bit is set to "1," an 8-bit serial transfer on SIO0 starts; when the transfer is completed, this bit is reset. When this bit is set to "0," SIO0 operation stops.

SCON02 (bit 2):	LSB-/MSB-first select

This bit selects whether to start the transfer of data from the MSB (1) or the LSB (0). When this bit is set to "1," the MSB is transferred first; when this bit is set to "0," the LSB is transferred first. This setting applies to both transmitting and receiving.

SCON01 (bit 1):	SIO0 transfer end flag

This flag is used to detect the end of a serial transfer. This flag is set when an 8-bit serial transfer is completed. This bit is not reset automatically; it must be reset by software. If the falling edge of a transfer clock is detected while this bit is set to "1," the overrun flag is set.

SCON00 (bit 0):	SIO0 interrupt request enable control

This bit enables (1)/disables (0) interrupt requests due to the end of a transfer on SIO0. When this bit is set to "1," an interrupt request to vector address 0033H is generated; when this bit is set to "0," no interrupt request is generated.

Note:

- The transfer end flag is set to "1" when the transfer of 8 bits is completed, without regard for the transfer bit length setting. The overrun flag has no effect on the operation of the microcomputer.

- SIO1 control register (SCON1)

| Symbol | Address | R/W | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCON1 | 134H | R/W | - | SCON16 | - | SCON14 | SCON13 | SCON12 | SCON11 | SCON10 |
| After reset |  |  | H | 0 | H | 0 | 0 | 0 | 0 | 0 |

| Bit name | Function |
| --- | --- |
| SCON16 (bit 6) | Overrun flag |
|  | 0: No overrun 1: Overrun |
| SCON14 (bit 4) | Transfer bit length control |
|  | 0: 8-bit transfer 1: Continuous transfer |
| SCON13 (bit 3) | Transfer control |
|  | 0: Transfer stop 1: Transfer start |
| SCON12 (bit 2) | LSB-/MSB-first select |
|  | 0: LSB-first 1: MSB-first |
| SCON11 (bit 1) | Serial transfer end flag |
|  | 0: Transfer in progress 1: Transfer completed |
| SCON10 (bit 0) | Interrupt request enable |
|  | 0: Interrupt request disabled 1: Interrupt request enabled |

SCON16 (bit 6):	Overrun flag

This flag is used to detect a serial transfer error on SIO1. If a transfer clock is received (a falling edge is detected) after the transfer of 8 bits of data has been completed (SCON11 has been set to "1"), this flag is set. In addition, when executing a continuous transfer, the overrun flag is set after every eight bits. This bit is not reset automatically; it must be reset by software.

SCON14 (bit 4):	Transfer bit length control

This bit selects the SIO1 transfer data bit length: continuous (1) or 8 bits (0). When this bit is set to "1," two or more 8-bit bytes of data can be sent consecutively. This flag does not change after a transfer; it must be reset by software. When this bit is set to "0," eight bits of data can be transferred. In this case, the transfer end flag (SCON11) is set when the transfer of 8 bits is completed.

SCON13 (bit 3):	SIO1 operation control

This bit starts (1)/stops (0) SIO1 transfer. When this bit is set to "1," an 8-bit serial transfer on SIO1 starts; when the transfer is completed, this bit is reset. When this bit is set to "0," SIO1 operation stops.

SCON12 (bit 2):	LSB-/MSB-first select

This bit selects whether to start the transfer of data from the MSB (1) or the LSB (0). When this bit is set to "1," the MSB is transferred first; when this bit is set to "0," the LSB is transferred first. This setting applies to both transmitting and receiving.

SCON11 (bit 1):	SIO1 transfer end flag

This flag is used to detect the end of a serial transfer. This flag is set when an 8-bit serial transfer is completed. This bit is not reset automatically; it must be reset by software. If the falling edge of a transfer clock is detected while this bit is set to "1," the overrun flag is set.

SCON10 (bit 0):	SIO1 interrupt request enable control

This bit enables (1)/disables (0) interrupt requests due to the end of a transfer on SIO1. When this bit is set to "1," an interrupt request to vector address 003bH is generated; when this bit is set to "0," no interrupt request is generated.

Note:

- The transfer end flag is set to "1" when the transfer of 8 bits is completed, without regard for the transfer bit length setting. The overrun flag has no effect on the operation of the microcomputer.

- Baud Rate Generator Register (SBR)

| Symbol | Address | R/W | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SBR | 132H | R/W | SBR7 | SBR6 | SBR5 | SBR4 | SBR3 | SBR2 | SBR1 | SBR0 |
| After reset |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

This register sets the transfer rate when using the internal clock for the transfer clock. This value is shared by both SIO0 and SIO1. The transfer rate TSBR is derived according to the following equation:

T[SBR] = (256 - [SBR setting])  2  Tcyc  (Tcyc: Cycle clock cycle)

- Serial Buffer 0 (SBUF0)

This register stores each 8 bits of data handled in a serial transfer on SIO0.

| Symbol | Address | R/W | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SBUF0 | 131H | R/W | SBUF07 | SBUF06 | SBUF05 | SBUF04 | SBUF03 | SBUF02 | SBUF01 | SBUF00 |
| After reset |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

- Serial Buffer 1 (SBUF1)

This register stores each 8 bits of data handled in a serial transfer on SIO1.

| Symbol | Address | R/W | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SBUF1 | 135H | R/W | SBUF17 | SBUF16 | SBUF15 | SBUF14 | SBUF13 | SBUF12 | SBUF11 | SBUF10 |
| After reset |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

- Next-generation game machine dedicated interface circuit

The separate function described above uses port 1 as the I/O port for the next-generation game machine dedicated interface. The next-generation game machine dedicated interface and the synchronous serial interface cannot be used simultaneously.

Note:

Note the following points when conducting serial communications through the serial interface:
1	Do not make settings concerning serial communications while the unit is in the reset state.

2	Confirm the connection between two VMS units before making settings concerning serial communications.
The connection between two VMS units can be confirmed through the value of port 7. When two VMS units are connected, the values of certain bits in port 7 are as follows:

		PORT7  bit3 = ‘1’
		PORT7  bit2 = ‘0’

3.	When serial communications processing is completed, or if two VMS units are not connected, make the following settings:

		SCON0 = 00h
		SCON1 = 00h
		P1FCR = 0BFh
		P1DDR = 0A4h

The unit may not operate correctly if the settings for executing serial communications are made when two VMS units are not connected.

    - Serial Interface Operation

A serial transfer on a serial interface is initiated by setting the communications control bit (SCON03, SCON13) or the transfer bit length selection bit (SCON04, SCON14). There are two transfer modes:

- Normal mode

Two data lines and one clock line are used for data communications in this mode. SI is the data input line, and SO is the data output line. This mode is the general-purpose transfer method, and is suited for communications with a specific partner.

Use normal mode when connecting two VMS units.

- Bus mode (deleted)

SCK0

SI1

SO0

S01

SI0

SCK0

SCK1

SO0

SI1

SI0

SO1

VMS2

VMS1

SCK1

Transmitted

SIO0

Received

SIO1

Received

SIO1

Transmitted

SIO0

Fig. 3-5-3  Connection between VMS Units

The transfer mode is set by manipulating the Special Function Registers associated with port 1. (Refer to Table 3-5-1.) In addition, SIO0 and SIO1 can each be set to either normal mode or bus mode independently.

- Serial transfer timing

The data in the shift register is shifted in synchronization with the falling edge of the serial clock SCK0 or SCK1, and is output on the SO0 or SO1 pin. The data that is input on the SI0 or SI1 pin is loaded into the shift register at the rising edge of the serial clock.

    - Operation Mode Settings

- Normal mode

It is necessary to reset the port latch data corresponding to the output pin or the transfer clock when using the internal clock. The pins that are used in normal mode are shown in the table below.

Table 3-5-1.  Pins Used in Normal Mode

|  | SIO0 | SIO1 |
| --- | --- | --- |
| Input pin | P11/SI0/SB0 | P14/SI1/SB1 |
| Output pin | P10/SO0(P11/SI0/SB0) | P13/SO1(P14/SI1/SB1) |
| Transfer clock | P12/SCK0 | P15/SCK1 |

Note:

- Set SCKn high one Tcyc cycle before the start of transfer. If SCKn is set high less than one Tcyc cycle prior to the start of transfer, the correct data will not be output.

Table 3-5-2.  Settings for Port 1 for SIO0 (Special Function Registers)

| Pin | Function | Special Function Register Value |
| --- | --- | --- |
| P11/SI0/SB0 P10/SO0 | RX TX | P11DDR =0 P10 =0 P10DDR =1 P10FCR =1 |
| P11/SI0/SB0 P10/SO0 | RX General-purpose I/O | P11DDR =0 P10FCR =0 |
|  | Internal clock | P12 =0 P12DDR =1 P12FCR =1 |

* For the software clock, the program writes "0" and "1" in alternation to a port (P12), and that output is used as the transfer clock.

Table 3-5-3.  Settings for Port 1 for SIO1 (Special Function Registers)

| Pin | Function | Special Function Register Value |
| --- | --- | --- |
| P14/SI11/SB1 P13/SO1 | RX General-purpose I/O | P14DDR =0 P13FCR =0 |

- For the software clock, the program writes "0" and "1" in alternation to a port (P15), and that output is used as the transfer clock.

Fig. 3-5-4.  Signal Path in Normal Mode (Example for SIO1)

Note:

When setting Pn to output, set PnFCR to "1" before PnDDR. If PnDDR is set first, "0" might be output on Pn when PnDDR is set. This applies to both SIO0 and SIO1.

    - Serial Transfer Clock

The serial transfer clock uses the P12/SCK0 pin for SIO0, and the P15/SCK1 pin for SIO1. One of the following three types of serial transfer clock can be selected independently for SIO0 and one for SIO1 through the application circuit. In addition, in the case of SIO0 only, the polarity of the transfer clock can be switched.

- Internal clock

- External clock

- Software clock

- Internal clock

The transfer clock is generated by the serial transfer-dedicated baud rate generator (SBR) that is built into the LSI. This clock is shared by both SIO0 and SIO1. When running either or both of the serial interfaces according to the internal clock, it is necessary to operate the baud rate generator. In this case, the serial transfer clock is output from the clock pin (P12/SCK0, P15/SCK1) of the serial interface that is running according to the internal clock.

The relationship between the transfer rate and the baud rate generator setting is shown below. (The setting is made with a decimal value.)

TSBR=(256 - [SBR setting])  2  Tcyc  (Tcyc: Cycle clock cycle)

Fig. 3-5-6  Baud Rate Generator Configuration Diagram

Note:

When setting Pn to output, set PnFCR to "1" before PnDDR. If PnDDR is set first, "0" might be output on Pn when PnDDR is set. This applies to both SIO0 and SIO1.

- External clock

Serial transfers are performed according to a clock that is input from outside of the LSI.

- Software clock

The program writes "0" and "1" in alternation to the port P12/SCK0 or P15/SCK1 pin, and that output is used as the serial transfer clock.

Clock generation example

When using these transfer clocks, it is necessary to set the status of the P12/SCK0 or P15/SCK1 pin.

Table 3-5-7  Transfer Clock Settings

| Pin | Function | Special Function Register value |
| --- | --- | --- |
| P12/SCK0 | Internal clock | P12 =0 P12DDR =1 P12FCR =1 |
|  | External clock | P12DDR =0 |
|  | Software clock | P12 =0/1 P12DDR =1 P12FCR =0 |
| P15/SCK1 | Internal clock | P15 =0 P15DDR =1 P15FCR =1 |
|  | External clock | P15DDR =0 |
|  | Software clock | P15 =0/1 P15DDR =1 P15FCR =0 |

Notes:

- At least 1/2 of a cycle is needed for the serial data and serial clock pulse width. 
When using the sub-clock and external clock, caution is particularly essential. (When using a 32.768kHz crystal resonator for the sub-clock, the cycle clock cycle is 366[micro]s, so a pulse width of at least 183[micro]s is required.)

- When outputting the serial clock from port 1, observe the following sequence when setting the port 1 registers. If this sequence is not observed, serial transfers will not be performed correctly.

(1) Set P1FCR.

(2) Set P1DDR.

(3) Set SCONn. (Set the transfer control bits.)

    - Serial Transfer Timing

In a serial transfer, the transfer clock SCK0 is output at high level (SCK0 = 1) before and after an operation is performed on the SIO0 (when SCON07 = 0) or SIO1 interface. In addition, the last data to be transferred is maintained on the output pin. (Refer to Fig. 3-5-7.) However, the transfer clock SCK0 is output at low level (SCK0 = 0) before and after an operation is performed on the SIO0 interface when SCON07 = 1. In addition, bit 0 (SBUF00) of Serial Buffer 0 (SBUF0) is output (and maintained at that level) on the output pin. (Refer to Fig. 3-5-8.) Note that it is not possible to switch the polarity of the SIO1 interface.

SIO0

| SCON07=0 | When operation is stopped, SCK0 = 1 and data output is maintained |
| --- | --- |
| SCON07=1 | When operation is stopped, SCK0 = 0 and data output is bit 0 of SBUF0 |

SIO1

When operation is stopped, SCK1 = 1 and data output is maintained

Fig. 3-5-7.  Transfer Clock and Output Data (1)

Fig. 3-5-8.  Transfer Clock and Output Data (2)

    - LSB-/MSB-first Switching Function

When reading or writing the serial transfer buffer, it is possible to reverse the sequence from LSB to MSB. This function can be used to switch between LSB-first and MSB-first. The switch is made through the Serial Transfer Control Register (SCON0, SCON1), and is made before reading or writing the serial transfer buffer. In addition, if the switch is made after reading or writing the serial transfer buffer, the transfer is made in the sequence that was used when the buffer was read or written.

Fig. 3-5-9.  Correspondence Between the Serial Transfer Buffer and the Internal Bus When LSB-first Is Specified

Fig. 3-5-10.  Correspondence Between the Serial Transfer Buffer and the Internal Bus When MSB-first Is Specified

Figs. 3-5-11 and 3-5-12 show the timing charts for LSB-first and MSB-first serial transfer transmission and reception using SIO0.

Fig. 3-5-11.  Correspondence Between the Serial Transfer Buffer and the Internal Bus When LSB-first Is Specified

Fig. 3-5-12.  Correspondence Between the Serial Transfer Buffer and the Internal Bus When MSB-first Is Specified

    - Overrun Detection Function

The overrun detection function detects serial communication errors. When the interrupt source flag has been set (SCON01, SCON11), the overrun flag (SCON06, SCON16) is set at the falling edge of the transfer clock.

Fig. 3-5-13 shows the timing for normal communications and the timing when an overrun is generated. The interrupt source flag (SCON01, SCON11) is set at the rising edge of the transfer clock for the 8th bit of data. If, while in this state, the falling edge of the transfer clock is detected, the overrun detection flag is set. (Refer to the overrun generation timing chart.)

Note that the overrun flag has no effect on the operation of the microcomputer.

Notes:

- Wait at least 1/2 of a transfer clock cycle after the interrupt source flag has been set to "1" before checking the overrun flag.

- Even if the transfer mode that is set will exceed 8 bits, the overrun detection function operates according to the same timing as for an 8-bit transfer.

Fig. 3-5-13.  Correspondence Between the Serial Transfer Buffer and the Internal Bus When LSB-first Is Specified

    - Transfer Bit Length Control Function

When transferring more than 8-bits of serial data, set the transfer bit length control bit SCON04 or SCON14 (continuous transfer).

- Once SCON04 and SCON14 have been set, the serial transfer begins. These bits are not reset even after 8 bits have been transferred.

- The interrupt source flag is set according to the same timing as for an 8-bit transfer (after the completion of the transfer of 8 bits).

- The overrun detection bits SCON06 and SCON16 are set at the falling edge of the serial clock after the transfer of eight bits has been completed. (For the timing chart, refer to the section on the overrun detection function.)

- When the transfer bit length has been set to 8 bits, the transfer starts once the transfer control bit SCON03 or SCON13 is set. Once the transfer of 8 bits has been completed, the transfer control bit is reset. This causes the interrupt source flag SCON01 or SCON11 to be set. In addition, serial transfer stops automatically.

- When the transfer bit length has been set to "continuous transfer," the transfer starts once the transfer bit length control bit SCON04 or SCON14 is set, and continues until the bit is reset. The interrupt source flag is set after 8 bits have been transferred.

    - Program Examples

- SIO0 serial transfer (1) (transmission example)

Transfer conditions

- 8-bit transfer

- Transfer data:	038H (8 bits)

- MSB-first

- Falling edge output

- Normal mode

- Internal clock

- Baud rate:	25.6ms

- System clock:	32KHz crystal oscillating sub-clock

Working from the baud rate formula: T[SBR] = (256 - [SBR])  2  Tcyc

 [SBR] = 256 - T[SBR]/(2  Tcyc)

In this case, T[SBR] = 25.6ms, and Tcyc = 366[micro]s, so the value that is to be set in the Baud Rate Generator register (SBR) is determined as follows:

[SBR] = 256 - 25600/(2  366)

 221 (decimal) (approx.)

0DDH (hexadecimal)

Fig. 3-5-14.  Timing for Serial Transfer (1)

- SIO1 serial transfer (2) (reception example)

Transfer conditions

- 16-bit transfer

- LSB-first

- Bus mode

- External clock

- Same data is output from SO1 as from SB1.

- The upper 8 bits of the data that is loaded is stored in RAM at address #031H, and the lower 8 bits are stored in RAM at address #030H.

Fig. 3-5-15.  Timing for Serial Transfer (2)

Notes:

- In this example, misoperation will result if there is a rising edge (B) on the transfer clock during the interval from the execution of the instruction following SELF0 to the execution of the SELF1 instruction. The transfer rate should allow enough time for the cycle clock cycle.

- Set SCKn high one Tcyc cycle before the start of transfer. If SCKn is set high less than one Tcyc cycle prior to the start of transfer, the correct data will not be output.

- When setting Pn to output, set PnFCR to "1" before PnDDR. If PnDDR is set first, "0" might be output on Pn when PnDDR is set. This applies to both SIO0 and SIO1.