# 2

Chapter 5.

Errors

M86K detects three levels of errors: fatal errors, errors, and warnings. When a fatal error is detected, M86K immediately halts execution at that point. This level corresponds to problems such as "the work buffer is too small, etc." If an error is detected, M86K halts execution once the pass (pass 1 or pass 2) that was being executed at the moment when the error was detected is completed. This level corresponds to syntax errors, for example. If a warning is detected, the M86K does not halt execution, because a warning corresponds to minor problems such as an operand being out of range.

When a fatal error is detected, M86K does not generate all files that are indicated for output. If an error is detected in pass 1, M86K does not generate all files that are indicated for output; if an error is detected in pass 2, however, the list file only is generated (if the specification for generating the list file was made). The error display format is shown below.

filename(linenumber): source line

error message

Example:  sample.asm(54):   LD     xyz

xyz: undefine symbol

The symbol "xyz" is undefined.

  - Warnings

The meanings of the warning level messages that are generated by M86K are described below. Note that "???" in these messages indicates a variable portion of the message.

???: bit number exceeds limits

In a bit manipulation instruction, the specified bit was outside of the allowable range.

absolute expression expected

An expression in which the value is finalized at the point of assembly is required.

address beyond zero

A negative value was specified for the operand of an ORG instruction.

address exceeds limits

The value that was specified for the operand of an ORG instruction exceeded the size of ROM.

address exceeds ROM size

The address of an assembled instruction exceeded the size of ROM.

chip name is different from one specified by CHIPNAME (???).

The operand of the CHIP instruction differs from that which was specified in the environment variable.

END in included file

The END pseudo instruction was found within a source file that was specified by the INCLUDE pseudo instruction.

ENDF without FUNCTION

ENDF was found even though no function was being defined.

ENDM without MACRO

ENDM was found even though no macro was being defined.

EXITM outside MACRO

EXITM was found even though no macro was being defined.

function code buffer overflow

The contents of the function definition were too large for the buffer.

illegal combination of attributes:???

The attributes (bank and segment) of both elements of a two-element operator do not agree.

illegal style expression

SET or EQU operands had an invalid format.

JMP/CALL placed at the end of memory block (FREE)

A JMP or CALL instruction appeared in which the lower 12-bits of the address were 0FFEH or 0FFFH.  Because the segment placement mode is "FREE," there may be no problem, depending on the results of the link, but an error will be generated by the linker if the segment in question was placed at the start of a memory boundary.

Jump address is out of range (FREE)

The jump destination address is outside of the memory boundary. Because the segment placement mode is "FREE," there may be no problem, depending on the results of the link, but an error will be generated by the linker if the segment in question was placed at the start of a memory boundary.

LOCAL outside MACRO

LOCAL was found even though no macro was being defined.

macro name in expression

A symbol that has been registered as a macro was found in an expression.

macro name required

No macro name was found, even though a macro was being defined.

no character in string

No characters were found in a character string constant.

page width must be 72 ~ 132: ???

The operand of the WIDTH instruction must be between 72 and 132 (inclusive).

public ??? not defined

The value of a symbol declared in a PUBLIC pseudo instruction has not been defined.

SET conflicts with PUBLIC

An attempt was made to reset a value that was already set (by the SET instruction) for a symbol declared in a PUBLIC pseudo instruction.

symbol name required

No symbol was found in the operand for PUBLIC, EXTERN, or OTHER_SIDE_SYMBOL.

undefined symbol in expression

An undefined symbol was found in an expression. (This warning is detected in pass 2 only.)

value is out of range

The value is outside of the allowable range. (The "allowable range" varies for each operand.)

zero divide: ??? modulo 0

The right side of the MOD operator is "0."

zero divied: ??? / 0

The right side of the "/" operator is "0."

  - Errors

The meanings of the error level messages that are generated by M86K are described below. Note that ??? in these messages indicates a variable portion of the message.

???: 2, 8, 10 or 16 required

Only "2," "8," "10," or "16" can be specified as the operand for the pseudo instruction RADIX.

???: constant required

No numeric constant was found.

???: duplicated label

A duplicate label was found.

???: duplicated symbol

A duplicate symbol was found.

???: illegal character in numeric constant

An invalid character was found within a numeric constant.

???: no such chip in the table

The symbol that was specified by the CHIP instruction was not found in the reserved word file.

???: open error

An error was detected when a file was opened.

???: undefined symbol

An undefined symbol was referenced.

???: radix violation

Characters that are not valid for the specified base were found in a numeric constant.

???H,???:out of internal RAM area

The data segment address allocation exceeded the allowable range.

‘ not seen

No single quote (') was found on the right end of a character constant.

‘:’ not seen

In the case of a format that explicitly specifies the segment in the EXTERN operand, no colon (:) was found that delimits the segment from the symbol.

0x???: RAM address exceeds limits

The data segment address allocation was outside of the allowable range.

address duplicated

A duplicate address area in RAM was specified in the DS pseudo instruction.

address exceeds absolute limits

The address of an assembled instruction exceeded 65535.

bank number should be 0~15

The bank number must be within the range from 0 to 15.

Branch address beyond zero

A value smaller than address 0 (the start of the code segment in question) was specified as the branch destination address.

Branch address exceeds limits

The branch destination address exceeded the size of ROM.

CSEG conflicts with WORLD EXTERNAL_DATA

WORLD EXTERNAL_DATA and a pseudo instruction that specifies a segment cannot both be written within the same source file.

CSEG isn’t allowed in macro

A pseudo instruction that specifies a segment cannot be written within a macro definition.

DS must be in DSEG

The DS pseudo instruction can only specify a data segment.

DSEG conflicts with WORLD EXTERNAL_DATA

WORLD EXTERNAL_DATA and a pseudo instruction that specifies a segment cannot both be written within the same source file.

DSEG isn’t allowed in macro

A pseudo instruction that specifies a segment cannot be written within a macro definition.

ELSE without IFxxx

IFxxx corresponding to the pseudo instruction ELSE for conditional assembly was not found.

ENDF not seen

The ENDF pseudo instruction that declares the end of a function definition was not found.

ENDIF without IFxxx

IFxxx corresponding to the pseudo instruction ENDIF for conditional assembly was not found.

ENDM not seen

The ENDM pseudo instruction that declares the end of a macro definition was not found.

external symbol can’t be public

An external symbol was declared in a PUBLIC pseudo instruction.

Hardware configuration violation

The instruction (such as the CHANGE instruction) in question has not been implemented for the specified chip.

identifier expected

Something other than an identifier was found in a macro definition parameter list or in an EXTERN operand.

illegal character in ??? constant

illegal character in binary constant

An invalid character for the specified base was found within a numeric constant.

illegal symbol type

Declared of a symbol of an in valid type was attempted in a PUBLIC pseudo.

illegal word in external list

A syntax error was found in an EXTERN operand.

instructions can’t be in DSEG

An instruction other than DS was found in a data segment.

JMP/CALL placed at the end of memory block (INBLOCK)

A JMP or CALL instruction appeared in which the lower 12-bits of the address were 0FFEH or 0FFFH.  Because the segment placement mode is "INBLOCK," an error resulted.

Jump address beyond zero

A value smaller than address 0 (the start of the code segment in question) was specified as the jump destination address.

Jump address exceeds limits

The jump destination address exceeded the size of ROM.

Jump address is out of range (INBLOCK)

The jump destination address is outside of the memory boundary.  Because the segment placement mode is "INBLOCK," a linker error resulted.

local symbol can’t be public

A local symbol was declared in a PUBLIC pseudo instruction.

lost SET symbol

A symbol that was defined by the SET pseudo instruction was lost from in pass 2.  This is possibly due to an internal error in the assembler.

macro can’t be public

A macro was declared in a PUBLIC pseudo instruction.

maximum nesting of macro is 10

The maximum nesting level for macros is 10.

Multiple WORLD specified

Multiple WORLD pseudo instructions were written in the same source file.

name required for macro

No name was found in a macro definition.

no room for source line attribute object

There is insufficient memory to store source line attributes (information for debugging).

no value for EXT

Although the CHANGE instruction is being used, the register EXT was not found in the SFRs.

not the symbol defined by SET

An attempt was made to reset (with the SET command) a value for a symbol that was not the one that was defined by SET.

operand exceeds limits

The number of repetitions specified by the REPT macro pseudo instruction was not within the range from 1 to 65,535.

ORG isn’t allowed in macro

The ORG pseudo instruction cannot be written within a macro.

other-side symbol isn’t allowed

other-side symbol isn’t allowed here

A symbol declared with OTHER_SIDE_SYMBOL cannot be specified here.

other-side symbol or absolute constant is required

A symbol declared with OTHER_SIDE_SYMBOL or a constant is required.

positive value required

A negative value cannot be used.

public ??? not defined

There was no definition of a symbol declared in a PUBLIC pseudo instruction. (This problem generates a "warning" for a symbol that is only declared in a PUBLIC pseudo instruction and has no definition of value and is not referenced, and generates an "error" when there is no definition of value but the symbol is referenced.)

string is too long

The length of a character string constant exceeded the limit (255 characters).

symbol name required

No symbol was found on the left side of SET or EQU.

symbol not defined

No symbol was found specified in the operand for PUBLIC, EXTERN, or OTHER_SIDE_SYMBOL.  An internal assembler error is possible.

syntax error

A syntax error was found.

syntax error near ???

A syntax error was found in the vicinity of ???.

too complexed expression for an operand

An expression that was written for an operand was too complex and could not be interpreted.

too many CHIP pseudo operation

Multiple CHIP pseudo instructions were written in one source file.

too nested if-statements

Nesting of pseudo instructions for conditional assembly exceeded the limit (10 levels).

unbalanced conditional assembling controllers

unbalanced IF statement

The end of the source file was found while skipping due to conditional assembly.

unexpected end of file in string

The end of the source file was found within a character string constant.

unexpected end of line in string

The end of the line was found within a character string constant.

unexpected EOF in conditional assembling

The end of the source file was found while skipping due to conditional assembly.

unexpected terminator ??? in conditional assembling

The syntax analysis routine ended abnormally while skipping due to conditional assembly.  An internal assembler error is possible.

unmatched ELSE in skipping

unmatched ENDIF

The end of the source file was found while skipping due to conditional assembly.

WORLD conflicts xSEG

WORLD EXTERNAL_DATA and a pseudo instruction that specifies a segment cannot both be written within the same source file.

  - Fatal Errors

The meanings of the fatal error level messages that are generated by M86K are described below. Note that ??? in these messages indicates a variable portion of the message.

???(???): chip name not seen

???(???): chip name not seen.

???(???): decimal value required

???(???): hex-value and reserved-word are required

???(???): no chip name list

A syntax error was found in the reserved word file.

???(???): no reserved word seen

???(???): ROM size not seen

???(???): too many chip names

???(???):???: unknown chip name

???(???):???: unknown flag

A syntax error was found in the reserved word file.

???: illegal file name

An invalid character was found in the specified file name.

???: no such chip in the table

The chip name that was specified by the environment variable CHIPNAME was not found in the reserved word file.

???: no such user

The user name specified by "~user" was not found. (UNIX version only)

???: open error

An attempt to open the specified file failed.

???: unknown flag

An invalid assembler option was specified.

???: unreadable

The specified file cannot be loaded.

EMM v3.2 or later is required (v???.??? found)

The EMS driver version is old and is no longer supported.  Driver version 3.2 or later is required.

EMS allocation (??? pages) was failed

EMS memory allocation failed.

EMS deallocation was failed

An error was detected while opening EMS memory.

flushing error in workfile

An error was detected while flushing the work file. (There is no more free space on disk, etc.)

Getting EMM version was failed

Getting EMS status was failed

Getting free page count on EMS is failed

Getting physical page frame address was failed

During EMS memory initialization, an error was detected, such as during the EMS driver version check, etc.

making temp. name for ??? failed

An error was detected when a temporary name was given to an output file.

Neither CHIP pseudo operation nor CHIPNAME environment variable were defined. Further execution aborted.

No chip specification has been made through the CHIP pseudo instruction or the environment variable CHIPNAME; because no chip can be specified as the target chip for assembly, subsequent operation is halted.

no more MAIN memory (???) ???

Although there is an area remaining that must be allocated in main memory, there is no more space in main memory available for allocation.

no more memory (???)

There is no more memory that can be dynamically allocated (main memory, EMS memory, work files).

no more NODE buffer (???) ???

There is insufficient work area available for analyzing expressions.

no more PARAMETER buffer (???) ???

There is insufficient work area available for processing the parameter list for a macro definition or call.

no reserved word file available.

Reading of the reserved word file failed.

no room for file: ???

A file cannot be written to disk because the disk is full.

Pxxxx must be less than 65536

Specify a parameter buffer size of no more than 65535.

read error in workfile (???)

An error occurred while reading the work file.

removing ??? failed

Because an error was detected, an attempt was made to delete the output files created up to that point, but the attempt failed.

renaming ??? ==> ??? failed

An error was generated when an attempt was made to change the name of an output file that was created with a temporary name a regular name.

This error is generated when a file with the same name as the post-change name already exists, and that file is write-protected.

too many file names

Five or more file names are specified in the command line.

too many nested include files

The number of nested include files exceeded the limit (10 levels).

unlinking work file is failed

An error was detected while deleting a work file.

workfile ???: already exist

workfile ???: open error

An error was detected while creating a new work file.