VMS-BIOS

Specifications

Revision 0.90

Issued by

Sega Enterprises, Inc.

CS Development and Production Dep't, Second Division

Revision history

1998/03/30	Rev 0.10	Draft version

1998/04/01	Rev 0.20	Section 6, 	added OS subroutine operation diagram

Errata correction

1998/04/02	Rev 0.60	Initial release

1998/04/20	Rev 0.70	Section 4, 	added system initialization item

Section 5,	added system program description 
Changed subroutine call description and flow diagram

Section 6, 	added VMS initialization function description

Section 7, 	changed subroutine description numbering

Added Flash memory access precaution

Deleted fm0_rd item

Deleted fm1_rd item

fm_prd specification change and name change (fm_prd_ex)

fm_wrt specification change and name change (fm_wrt_ex)

fm_vrf specification change and name change (fm_vrf_ex)

1998/05/10	Rev 0.80	Section 5,	added description of return method from user program to file management mode

Section 7,	changed fm_wrt_ex specification

Appendix 2	added VMS mode selection flow description

1998/05/26	Rev 0.90	Section 8,	added description of low battery detection

Appendix 1	added low battery detection register

* Additions made to the latest revision before this one are marked by       and deletions by       .

Contents

1	Outline	4

2	VMS Outline	4

2.1	System-BIOS Outline	4

3	Memory Space	5

4	System BIOS Functions	7

5	System BIOS Data and Memory Allocation	8

5.1	Program Layout	8

5.2	Subroutine Call Flow	10

5.3	Returning From User Program to Mode Selection Screen	12

6	VMS Initialization	14

7	Subroutine Description	16

7.1	Flash Memory Access Functions	16

7.1.1	Precautions for Using Flash Memory Access Subroutines	16

7.1.2	Flash Memory Page Data Readout	18

7.1.3	Writing to Flash Memory	20

7.1.4	Flash Memory Verify	22

7.2	Clock Function	24

7.2.1	Clock Countup Timer	24

8	Automatic low battery detection function	26

8.1	Automatic low battery detection flag	26

Appendix 1	Table of Defined Variables	27

Appendix 2	VMS Mode Selection Operation Flow	28

# Outline

This document describes the System BIOS functions of the backup memory system "VMS" designed for the new-generation game machine (preliminary).

# VMS Outline

"VMS" stands for "Visual Memory System". The VMS is a backup memory cartridge equipped with a liquid-crystal display and operation buttons.

When connected to a dedicated controller in the new-generation game machine (preliminary), the VMS serves as a file backup memory and it can also display game sub screens on its LCD.

When not connected to the new-generation game machine, the VMS can function as a stand alone unit that allows displaying and deleting stored files. Two VMS units can be connected to allow file transfer.

Another application of the VMS is as a highly portable miniature game machine, using simple application programs downloaded from the new-generation game machine to the VMS. (Such application programs are called "user programs".)

## System-BIOS Outline

The functions described above are implemented by several programs that are contained in an internal ROM  on the VMS. These programs are called "OS programs". OS programs consist of subroutines which can be called by user programs. Two program types (system program and header) are used to call up subroutines. The entire system consisting of OS programs, system programs, and headers is called the "System-BIOS".

OS program subroutines are divided into subroutines that serve mainly for accessing the flash memory and subroutines for calculating time data. Application developers can use the System-BIOS to call these subroutines in user programs. This makes it easy for application developers to use VMS functions without having to deal with detailed VMS specifications.

# Memory Space

VMS uses two types of memory space: internal memory space and external memory space.

Internal memory space consists of the internal program area and internal RAM. The external memory space is made up of flash memory.

The internal program area is 64 kilobytes and contains the OS programs and system programs. User programs can reference this area as needed. The internal program area is allocated as shown in the memory map in Fig. 3.1. (For information on OS programs and system programs, please refer to section 5.)

The flash memory space is 128 kilobytes, divided into two banks of 64 kilobytes each. Bank-0 is the program area and bank-1 the data area. User programs are stored in the program area. A memory map of the flash memory is shown in Fig. 3.2. (For information on the internal and external program area and BIOS usage, please refer to section 4 and the following sections.)

The internal RAM has a memory space of 1222 bytes, divided into the following four sections: main RAM, special register area, LCD video RAM (XRAM), work RAM (VTRBF).

The main RAM is 512 bytes, divided into two banks of 256 bytes each. Because bank-0 is reserved for the System BIOS, user programs are generally prohibited from writing to bank-0 (except for certain cases listed in appendix 1).

The special register area is allocated to VMS specific registers (timer register, LCD control register, etc.). For information on registers and corresponding addresses, please refer to the VMS user's manual.

The LCD video RAM (XRAM) consists of three banks which serve for storing LCD image data.  (For information on RAM usage, please refer to the VMS user's manual.)

The work RAM is 512 bytes and serves as a buffer when VMS carries out data transfer according to the Maple Bus protocol. When the VMS is operating as a standalone unit and data transfer according to the Maple Bus protocol is therefore not being carried out, user programs can use this area.

A memory map of the internal RAM is shown in Fig. 3.3. (The access procedure for the work RAM differs from normal RAM access. For information, please refer to the VMS user's manual.)

Bank 0

Program area

Bank 1

１

Data area

System programs

16K byte

64K byte

64K byte

4K byte

Reserved area

OS programs

Reserved area

0000h

00000000h

3FFFh

E000h

FFFFh

EFFFh

FFFFh

Fig. 3.1  Internal program space	Fig. 3.2  Flash memory space

XRAM-2

XRAM-1

XRAM-0

1FFh

1FFh

000h

000h

Main RAM

Main RAM

Work RAM

VTRBF

Special register area

SFR

LCD video RAM

LCD video RAM

LCD video RAM

Bank 0

0FFh

100h

17Fh

180h

Bank 1

*	Bank 0 of the main RAM is reserved for system programs. Except for special cases, user programs cannot use this area.

Fig. 3.3  Internal RAM space

# System BIOS Functions

This section explains the System BIOS functions provided for VMS.

User programs running on the VMS can access System BIOS functions by calling special subroutines. However, there are certain limitations on which System BIOS functions (subroutines) can be called by user programs.

The following functions are provided by the System BIOS.

- System initialization

- VMS initialization function

- VMS execution mode selection

VMS comes with the following three execution modes:

- Game data and user program management and editing

- User program startup and return

- Time display and adjustment

For details on execution mode selection, please refer to appendix 2.

- Subroutines

- Flash Memory Access Functions

- Flash Memory Page Data Readout

- Writing to Flash Memory

- Flash Memory Verify

- Clock Function

- Clock Countup Timer

For details on VMS initialization, please refer to section 6. For details on subroutines, please refer to section 7.

# System BIOS Data and Memory Allocation

VMS comes with certain programs for using the System BIOS functions. These programs can be classified into the following three types:

- OS programs

- System programs

- Header

## Program Layout

The actual programs are arranged in memory as follows.

Internal memory space	External memory space

0000h

0000h

0100h

FFFFh

FFFFh

4000h

E000h

F000h

Bank 1

Bank 0

Program/Data area

Data area

System programs

Header

Reserved area

OS programs

Reserved area

(preliminary)

Fig. 5.1  VMS memory map

Details are explained below.

1.	System programs

System programs are required for using the VMS as a memory device. Major system programs are the file management system, clock functions, and programs for data transfer according to the Maple standard. A program for calling subroutines from the external memory space (user programs) is also located here. (A flow diagram showing the call-up process of specified subroutines is shown in section 5.2.)

The VMS initialization program is located in this area. For details on the initialization program, please refer to section 6.

2.	OS programs

The VMS program subroutines are located here. For information on subroutines that can be called by user programs, please refer to section 7.

The method of accessing to this area is also shown in section 5.2.

3.	Header

Contains information about internal memory space processing routines and return procedures from the internal memory space. Because this area also contains interrupt vectors for internal use by user programs, its source code is being made available to application developers. It also contains information about return from user programs to the mode selection screen. User programs must use this information to return to the file management system. (For details on mode selection screen, please refer to Appendix 2.) Within the given specifications, the area content may be modified by developers.

## Subroutine Call Flow

This section explains the operation flow that occurs when a user programs calls OS program subroutines and then returns to the user program. An actual flow diagram is shown in Fig. 5.2.

External memory space	Internal memory space

(header, user program)	 (system program, OS program)

LABEL   MNEMONIC

WORLD INTERNAL

OTHER SIDE SYSBOL os_ret

PUBLIC os_call

os_call:

callf os_main

change os_ret

os_main:

(actual OS program)

:

:

ret

LABEL   MNEMONIC

WORLD EXTERNAL

OTHER SIDE SYSBOL os_call

PUBLIC os_ret

jmp main

os_int:

change os_call

_head_ret:

RET        return to main

os_ret:

br _hrad_ret

main:

:

callf os_int　　　 start call flow

:

jmp main

±

°

¯

®

­

¬

Fig. 5.2  OS program call flow

Label processing description

	external memory space

main:	Main program in user program

os_int:	This subroutine shifts processing to internal memory space. 
In the example, processing passes to the internal memory space when the subroutine is called, and the main program resumes upon return from the internal memory space. This subroutine is included in the header.

os_ret:	Subroutine for returning to external memory space. 
The "change" command serves to return to this label from the internal memory space. After returning, processing moves to the interrupt return routine in the header.

	internal memory space

os_call:	Serves to call an OS program and return to the external memory space. 
After the OS program subroutine has executed, processing returns to the external memory space.

os_main:	Main OS program which executes the various subroutines.

The sample flow shown in Fig. 5.2 assumes that a user program is executing in the external memory space.

¬	When wishing to use an OS program during execution of an external program, call the "os_int" subroutine. Interrupt processing routines which need to jump to an OS program contain an "os_int" subroutine.

The "change" command in the os_int subroutine jumps to the OS program call routine (os_call) placed in the internal program area.

®	The OS program call routine calls the actual OS program subroutine (os_main). From this point on, the OS program starts to execute.

¯	When the OS program execution is finished, the RET command jumps to the next address of the call command in the OS program call routine. In the OS program call routine, the call command is always followed by a change command which moves processing to the external program area.

°	After returning from the OS program subroutine, the change command passes processing over to the external program. This program contains a subroutine (os_ret) that is called when returning from an internal program. The subroutine position is fixed. These programs are distributed to application developers as a library. Such programs are called headers. (The sample program contains the headers "os_int" and "os_ret".)

±	From the above described external program return routine, processing returns to the "os_int" subroutine and then by the RET command to the main program (main).

Note: 	Label names in the sample program are all preliminary. Label names will be different in the actual System-BIOS.

*	"change" command

The "change" command serves to move processing from the external memory space to the internal memory space, or from the internal memory space to external memory. By executing this command, a program that is currently executing in internal memory space (or external memory space) moves to external memory space (or  internal memory space). The program counter is reset to the specified label (or address).

## Returning From User Program to Mode Selection Screen

When a user program is executing, if the user presses the MODE button on VMS, the user program will terminate immediately and processing will return to the mode selection screen.

This section explains the operation flow from user program to the mode selection screen when the MODE button is pressed while a user program is executing.

Fig. 5.3 shows the operation flow where the OS program is called.

External memory space	Internal memory space

(header, user program)	(system program, OS program)

¬

LABEL   MNEMONIC

WORLD EXTERNAL

OTHER SIDE SYSBOL int_ret:

jmp main

user_end:

(Saving of data by user program)

change int_ret

main:

:

←Interrupt generated

when MODE button pressed

:

jmp main

LABEL   MNEMONIC

WORLD INTERNAL

PUBLIC int_ret

int_ret:

jmp mode_main

:

mode_main:

(Mode selection screen program)

:

:

ret

­

®

Fig. 5.3  Operation flow of returning to mode selection

Label processing description

	external memory space

main:	Main program in a user program.

A user program must contain description to allow for pressing of the MODE button to jump to the OS program return subroutine.

user_end:	Subroutine to terminate a user program in execution and move processing to the OS program. If data in the executing user program needs to be saved, then be sure to include this information in the user program so that the subroutine will save it before returning to the OS program. (The OS program does not keep data.)

	internal memory space

int_ret:	Return routine to serve as entry to returning to the internal memory space when a user program terminates. When processing returns to the internal memory area, the mode selection program will start.

mode_main:	Mode selection program.

For details on mode selection specification, please refer to Appendix 2.

The sample program flow in Fig. 5.3 assumes the user program is executing in the external memory space.

¬	While an external program is executing, pressing the MODE button will jump to the user_end subroutine. In the user_end subroutine, the "change" command will shift processing to the internal memory space. Therefore, if data in the executing user program needs to be saved, then be sure to save it before executing the "change" command.

When program control jumps from the user program to the user_end subroutine, the "change" command inside the user_end subroutine will shift processing to the mode_ret subroutine in the internal memory space.

®	When processing moves from the external memory space to the mode_ret subroutine, the mode selection program will start.

Note:	Label names used in this example are preliminary and are different from the labels used in the actual System-BIOS.

*	"change" command

The "change" command serves to move processing from the external memory space to the internal memory space, or from the internal memory space to external memory. By executing this command, a program that is currently executing in the internal memory space (or external memory space) moves to the external memory space (or internal memory space). The program counter is reset to the specified label (or address).

# VMS Initialization

This section explains the initialization that is performed at VMS startup.

The VMS is automatically initialized in the following cases.

- VMS is connected to new-generation game machine, and power to new-generation game machine is turned ON

- Reset switch on VMS is pressed

- Battery is inserted in VMS

Initialization comprises the following steps.

¬	Clear main RAM

	Write ‘00h’ to entire main RAM area (bank 0, bank 1).

	Initialization does not change XRAM values.

All registers are initialized by a hardware reset first, and then again by software. For information on the register values after a hardware reset, please refer to the VMS user's manual.

Set system clock and cycle time

	Switch system clock to sub-clock (crystal quartz oscillator).

	Set cycle time to 1/6 system clock.

(The cycle time is used as reference for command execution. For details, please refer to the VMS user's manual.)

®	Set base timer

	Select 14-bit base timer mode.

	Switch base timer clock to sub-clock (crystal quartz oscillator).

	Enable base timer 0 interrupt and start counting.

For details regarding base timer 0 operation, please refer to the VMS user's manual.

The base timer 0 is used by the clock function. For details regarding the clock function, please refer to section 7.3.

¯	Set master interrupt

	Enable master interrupt.
(The master interrupt flag controls enabling/disabling of all interrupts with "High level" and "Low level" priority.)

°	Set LCD driver

	Activate LCD controller.

	Set LCD clock to 1/2 of LCD driver input clock.

	Set LCD start address to ’000h’ of XRAM.

	Set character register.

	Set time allocation register.

	Set LCD to ON.

±	Set port 1

	Set port 1 to all-bit input.

	Set bit 7 of port 1 to audio output pin.

*	After initialization, bit 7 of port 1 is set to input mode. Therefore a user program will need to again select the output mode.

	Set bit 5 – bit 0 of port 1 (serial interface for VMS) to synchronous operation. For details regarding the synchronous serial interface, please refer to the VMS user's manual.

²	Set port 3

	Pull up all bits of port 3.

	Set port 3 to all-bit input.

	Enable interrupt triggering and HOLD mode cancel by port 3.

	Enable interrupt trigger request by port 3.

³	Initialize Maple Bus interface circuit

	Initialize Maple Bus interface circuit.

´	Set work RAM

	Enable use of work RAM.

# Subroutine Description

This section describes the subroutines available in the System BIOS.

## Flash Memory Access Functions

The following subroutines are available for flash memory access.

- Flash Memory Page Data Readout

- Read 128 bytes of data from the flash memory space.

- Write to Flash Memory

- Write 128 bytes of data to the flash memory space.

- Flash Memory Verify

- Verify data written to the flash memory.

*	When accessing the flash memory, the main clock in use must be switched to 600 kHz. For details, please refer to the next section.

### Precautions for Using Flash Memory Access Subroutines

When accessing the flash memory space, the following points must be observed.

VMS uses three types of system clock as reference for command execution (see Fig. 7.1).

When VMS is operating as a standalone unit, the quartz oscillator clock (32 kHz) will normally be used. However, for accessing the flash memory, the clock must be switched to the internal (RC) oscillator (600 kHz) before calling a flash memory access subroutine. After subroutine execution is completed, switch back to the previously used clock.

For information on the timing for clock switching, see Fig. 7.2.

| System clock source | Oscillation frequency | Command cycle time |
| --- | --- | --- |
| Ceramic (CF) oscillator | 6 MHz | 1.0 us |
| Internal (RC) oscillator | 600 kHz | 10.0 us |
| Quartz (X’TAL) oscillator | 32 kHz | 183.0 us |

Fig. 7.1  system clock table

600 kHz clock

Internal memory space

External memory space

User program

Header

OS program

Change clock to 600 kHz

Call OS program

Subroutine execution starts

Subroutine execution completed

Call OS Routine ends

Change clock to 32 kHz

Return from OS program

32 kHz clock

Call

OS call routine

Return to user program

Call OS

routine starts

Fig. 7.2  Flow diagram for clock switching during flash memory access

### Flash Memory Page Data Readout

Subroutine name:	fm_prd_ex (org 0120h)

Arguments:	High-order start address for flash memory read:	fmadd_h (RAM bank-1 07Eh)

Low-order start address for flash memory read:	fmadd_l (RAM bank-1 07Fh)

Bank address for flash memory read:	fmbank	(RAM bank-1 07Dh)

Return value:	Read data (128 bytes):	080h - 0FFh of RAM bank-1

Function:	Read one continuous page of data (128 bytes) from specified address

Description:	By calling this subroutine, a program can read one page of data (128 bytes) from flash memory.

Before using this subroutine, the following settings must be made.

- Select RAM bank to use

(1)	Select RAM bank-1 (Set bit 1 of PSW to "1")

For information on the PSW register, please refer to the VMS user's manual.

- Set start address for flash memory read

(2)	High-order address (8 bit): set to fmadd_h (07Eh of RAM bank-1)

(3)	Low-order address (8 bit): set to fmadd_l (07Fh of RAM bank-1)

- Select flash memory bank to read

(4)	Select flash memory bank-0

(Set 07Dh of RAM bank 1 to ’00h’)

*	If another value is set, normal operation is not assured.

The read data are written to 080h - 0FFh of RAM bank-1.

When making read settings, observe the following points.

- Data extending to 2 pages cannot be read. The read start address must therefore always be set to the beginning of each page.

- The start address of each page can be calculated according to the following equation:

start address value (2 byte) = 080h  x page number (0 - 511)

- (Because readout is performed in single-page units, bit 0 – bit 6 of the lower-level address must always be set to "0". If an address other than the start address of a page is set, normal operation is not assured.)

- The read-out data overwrite any previous content of the RAM.

* Register values after subroutine completion

Note that the following (memory) registers will have different values before the subroutine is called and after the subroutine has completed:

- ACC	 (accumulator)

- TRL	 (table lookup register lower byte)

- TRH	 (table lookup register higher byte)

- r0	 (RAM indirect address register)

*About pages

Beginning at the top, the flash memory space is subdivided into 128-byte units called pages. Flash memory is managed in page units. Because 1 bank of the flash memory space is 64 kilobytes, it has 512 pages.

"fm_prd_ex" execution is shown in Fig. 7.3.

* fmadd_h = A0h

When fmadd_l is set to 80h (page no. 321)

Flash memory space

Main RAM

000h

A100h

128 byte DATA

Bank 0

Bank 0

A080h

FFFh

000h

128 byte DATA

080h

FFFFh

0FFh

Bank 1

0000h

Bank 1

Fig. 7.3  Execution of fm_prd_ex

### Writing to Flash Memory

Subroutine name:	fm_wrt_ex (org 0100h)

Arguments:	High-order start address for flash memory write:	fmadd_h (RAM bank-1 07Eh)

Low-order start address for flash memory write:	fmadd_l (RAM bank-1 07Fh)

Bank address for flash memory write:	fmbank (RAM bank-1 07Dh)

Flash memory write data (128 bytes):	RAM bank-1 080h - 0FFh

Data write end detection algorithm:

Bit 0 of RAM bank-1 07Ch

(toggle bit method (0)/data polling method (1))

Return value:	result of write:	ACC (accumulator)

(Normal termination: 00h. Abnormal termination: FFh)

Function:	Write one continuous page of data (128 bytes) to the flash memory, starting at the specified address

Description:	By calling this subroutine, a program can write a page of data (128 bytes) to a continuous area in the flash memory, starting at the specified address.

Before using this subroutine, the following settings must be made.

- Select RAM bank to use

(1)	Select RAM bank 1 (Set bit 1 of PSW to "1")

- Prepare data to be written to flash memory

(2)	Store data to be written to flash memory in RAM bank 1, 080h - 0FFh

- Select flash memory bank to read

(3)	Select flash memory bank 0

(Set 07Dh of RAM bank 1 to ’00h’)

* If another value is set, normal operation is not assured.

- Set address for accessing flash memory

(4)	High-order address (8 bit): set to 07Eh of RAM bank-1

(5)	Low-order address (8 bit): set to 07Fh of RAM bank-1

- Specify data write end detection algorithm

(6)	Set data write end detection algorithm in 07Ch of RAM bank-1, as follows.

(6-1) Use toggle bit method: set 07Ch to 00h

(6-2) Use data polling method: set 07Ch to 01h

* If another value is set, normal operation is not assured.

When making write settings, observe the following points.

- fm_wrt_ex is a subroutine specifically for user programs. This subroutine can write only to the area where the user program is located. For this reason, be sure to secure an area within the user program before performing the data write.

- Data extending to 2 pages cannot be written. The write start address must therefore always be set to the beginning of each page.

The start address of each page can be calculated according to the following equation:

start address value (2 byte) = 080h  x page number (0 - 511)

(Because writing is performed in single-page units, bit 0 - 6 of the lower-level address must always be set to "0". If an address other than the start address of a page is set, normal operation is not assured.)

For information on pages, please refer to section 7.1.2.

* Register values after subroutine completion

Note that the following (memory) registers will have different values before the subroutine is called and after the subroutine has completed:

- ACC	 (accumulator)

- B	 (B register)

- C	 (C register)

- TRL	 (table lookup register lower byte)

- TRH	 (table lookup register higher byte)

- r0	 (RAM indirect access register)

fm_wrt_ex execution is shown in Fig. 7.4.

0000h

Flash memory space

Bank 1

* fmadd_h = A0h

When fmadd_l is set to 80h (page no. 321)

A100h

A080h

FFFFh

080h

0FFh

000h

FFFh

000h

128 byte DATA

Main RAM

Bank 0

Bank 0

Bank 1

128 byte DATA

Fig. 7.4  Execution of fm_wrt_ex

### Flash Memory Verify

Subroutine name:	fm_vrf_ex (org 0110h)

Arguments:	High-order address flash memory address for verify start:	fmadd_h (RAM bank 1 07Eh)

Low-order address flash memory address for verify start:	fmadd_l (RAM bank 1 07Fh)

Flash memory bank address for verify operation: 	fmbank (RAM bank 1 07Dh)

Data (128 bytes) for verify operation:  	RAM bank 1 080h - 0FFh

Return value:	Verify result:  accumulator (ACC) (normal end: 00h，error end: other than 00h)

Function:	Serves to verify whether data were written correctly to flash memory. For use after writing data to flash memory with fm_wrt_ex (see section 7.1.4).

Description:	This subroutine compares the 128 byte data specified when calling fm_wrt_ex with the data actually written to flash memory. Therefore the subroutine can only be used immediately after the fm_wrt_ex subroutine was called.

When calling this subroutine, the same arguments as used for the preceding fm_wrt_ex must be specified. If different arguments are specified, data verify will not be carried out properly.

After calling this subroutine, if all 128 bytes of data were found to match, 00h will be set in ACC, and the routine returns. If a data mismatch was detected, a value other then 00h will be set in ACC, and the routine returns.

* Register values after subroutine completion

Note that the following (memory) registers will have different values before the subroutine is called and after the subroutine has completed.

- TRL	 (table lookup register lower byte)

- TRH	 (table lookup register higher byte)

- r0	 (RAM indirect access register)

fm_vrf_ex execution is shown in Fig. 7.5.

A100h

A080h

FFFFh

080h

0FFh

000h

FFFh

000h

0000h

128 byte DATA

128 byte DATA

Main RAM

Flash memory space

Bank 0

Bank 0

Bank 1

Bank 1

* fmadd_h = A0

Fmbank = 01

fmadd_l = 80

00h set in ACC

Data match

Data do not match

Compare data contents

00h

ACC

Value other than 00h set in ACC

Not 00h

ACC

(page no. 321)

Fig. 7.5  Execution of fm_vrf_ex

## Clock Function

The clock functions implemented in VMS are as follows.

Time data automatic update

### Clock Countup Timer

Subroutine name:	timer_ex

Arguments:	None

Return value:	Year: 	year_h	 (RAM bank 0  017h, 18h)

Month:	mon_h 	 (RAM bank 0  019h)

Day:	day_h	 (RAM bank 0  01Ah)

Hour:	hour_h	 (RAM bank 0  01Bh)

Minute:	min_h	 (RAM bank 0  01Ch)

Second:	sec_h	 (RAM bank 0  01Dh)

* The year data are configured as 2 bytes, with the higher-level in byte in 17h and the lower level byte in 18h. Because "year_h" is assigned to RAM bank, 017h, address 018h can be accessed by specifying "year_h+1".

Function:	When the subroutine is called, it obtains the time data and places them in the specified location in RAM bank 0. (For information on the specified location, please refer to Appendix 1.)

Description:	This subroutine is a time counter using the base timer interrupt. To enable use of the subroutine, the following settings for the base timer interrupt must be made.

- Base timer interrupt settings

- This subroutine uses only the base timer 0 interrupt. The base timer interrupt is to be set as shown below.

(1) Base timer count stop	(BTCR 6 bit = '0’)

(2) 14 bit base timer mode selected	(BTCR 7 bit = ’0’)

(3) Sub clock used as base timer clock 	(ISL 4 bit = ’0’)

(4) Base timer interrupt 0 enabled	(BTCR 0 bit = ’1’)

- (5) Base timer count start	(BTCR 6 bit = ’1’)

Because the base timer 0 interrupt is used by the timer_ex subroutine, user programs may not access this interrupt. Otherwise, normal operation is not assured.

This subroutine should be called after jumping to the interrupt vector of the base timer interrupt 0 source. Also, be sure to clear the base timer 0 interrupt source (BTCR 1 bit = ’0’). 
(If this is not performed, the clock function will not operate properly.)

All time data obtained by this subroutine are in hex format. Conversion into decimal format must be performed by the user program.

# Automatic low battery detection function

Visual Memory comes with the ability to automatically detect low battery.

The following explains how this function works.

## Automatic low battery detection flag

Visual Memory can automatically check the battery's power consumption and when necessary display a low battery warning message on the screen. Gamedevelopers can use the automatic low battery detection flag to enable ordisable this function.

The following describes how to use this function.

Register to use:	06Eh (Bank-0)

Register values:	00h (enable the automatic low battery detection function)

FFh (disable the automatic low battery detection function)

(If any value other than the above ones is used, then normal operationcannot be guaranteed.)

How it work:		The automatic low battery detection function constantlymonitors the battery's voltage and if the voltage falls below a certainlevel it will stop the current program, wait for 3 seconds, then display thelow battery warning message on the screen.

Explanation:		The automatic low battery detection function consists of tasksfrom detecting low voltage to displaying the low battery warning message.

When the automatic low battery detection flag is set to 00h,the automaticlow battery detection function is enabled and when the battery is low it will display the low battery warning message, regardless of the current task of Visual Memory. If the flag is set to FFh, then the automatic low battery detection function is disabled.

Note:		The automatic low battery detection function cannot work if the user program performs saving data away when low battery is detected. The user program must monitor the low battery detection flag (bit 1 of PORT7) and handle it accordingly. For information on low battery detection flag, please refer to the VMS hardware manual.

When the user program is performing the following tasks, be sure to turn offthe automatic low battery detection function:

1. Communicating with another Visual Memory via the serial interface

2. Writing to the flash memory space

# Appendix 1	Table of Defined Variables

System BIOS requires the following variables.

Time data variables

year	010h (bank-0)	Year (BCD　4 digits)	* Not generated by timer_ex subroutine

mon	012h (bank-0)	Month (BCD　2 digits)	* Not generated by timer_ex subroutine

day	013h (bank-0)	Day (BCD　2 digits)	* Not generated by timer_ex subroutine

hour	014h (bank-0)	Hour (BCD　2 digits)	* Not generated by timer_ex subroutine

min	015h (bank-0)	Minute (BCD　2 digits)	* Not generated by timer_ex subroutine

sec	016h (bank-0)	Second (BCD　2 digits)	* Not generated by timer_ex subroutine

year_h	017h (bank-0)	Year (HEX　4 digits)

mon_h	019h (bank-0)	Month (HEX　2 digits)

day_h	01Ah (bank-0)	Day (HEX　2 digits)

hour_h	01Bh (bank-0)	Hour (HEX　2 digits)

min_h	01Ch (bank-0)	Minute (HEX　2 digits)

sec_h	01Dh (bank-0)	Second (HEX　2 digits)

sec_f	01Eh (bank-0)	Work RAM (used by timer, prohibited to user programs)

leaf_f	01Fh (bank-0)	Work RAM (used by timer, prohibited to user programs)

Low battery detection function

06Eh (bank-0)	Automatic low battery detection flag

(Register name is not set.)

Flash memory variables

fmbank	07Dh (bank-1)	Flash memory bank designation

fmadd_h	07Eh (bank-1)	Flash memory address (upper 8 bit)

fmadd_l	07Fh (bank-1)	Flash memory address (lower 8 bit)

Note:	For RAM bank-0, writing to addresses other than for time data is prohibited.

When using the System-BIOS, normal operation will not be assured if the user

writes to an area other than for time data.

動作が正常に行われない場合があります.

# Appendix 2	VMS Mode Selection Operation Flow

The following explains the various VMS modes and their selection.

The figure shows the VMS mode selection flow diagram.

RESET or power on

Time data

Adjustment mode

Press MODE button

Press A button

File mode selection

Game mode selection

Time mode selection

File mode

Game mode

Time mode

Mode selection screen

Fig. Appendix 2-1  Mode selection flow diagram

Each of the modes comes with the following functions.

	Mode selection screen	This screen allows selection and execution of one of the three VMS modes. Pressing the MODE button each time selects a new mode, and pressing the A button enters the selected mode. The icon indicates the current mode. Please refer to Fig. Appendix 2-1 for the modes and their corresponding icon.

	File mode	This mode handles game data and user program management and editing. Pressing the MODE button while in this mode will return to the mode selection screen.

	Game mode	This mode executes user programs stored on VMS. Pressing the MODE button while in this mode will return to the mode selection screen.

Because the BIOS does not support return processing from the game mode to the mode selection screen, be sure to handle the return processing in the user program. For information on the return processing to the mode selection screen, please refer to section 5.3.

	Time mode	This mode handles current time display and adjustment. Pressing the MODE button while in this mode will return to the mode selection screen. Also, while in this mode, pressing the left cursor key as well as the A button at the same time will enter the time setting mode.