# 2

Chapter 6.

Pseudo Instructions

Unlike normal instructions (instructions that indicate operations of the LC86K itself, such as ADD and MOV), pseudo instructions are used to issue instructions and make definitions to the assembler, and no machine language is generated for individual pseudo instructions (except for pseudo instructions that are used for optimization purposes, such as JMPO, and CHANGE pseudo instructions). In most cases, these pseudo instructions are used in combination with normal instructions.

| Category | Pseudo instruction | Function |
| --- | --- | --- |
| Link control | ORG WORLD CSEG DSEG END PUBLIC EXTERN OTHER_SIDE_SYMBOL | Specify origin Select ROM for code storage Specify code segment Specify data segment End program Specify external definition name Specify external reference name Declare CHANGE instruction jump label |
| Symbol definition | EQU SET | Assign value Assign temporary value |
| Data definition | DB DW DC DS | Define byte Define word Define character string Allocate data area (RAM) |
| Macro control | MACRO REPT IRP IRPC ENDM EXITM LOCAL | Define macro Repeat macro Continuous macro Character string macro End macro definition Interrupt macro expansion Define local label |
| Conditional assembly | IFDEF IFNDEF IFB IFNB IFE IFNE IFIDN IFDIF ELSE ENDIF .PRINTX .LIST .XLIST .MACRO .XMACRO .IF .XIF | Assemble if defined Assemble if undefined Assemble if operand is empty Assemble if operand is not empty Assemble if value of expression is "0" Assemble if value of expression is not "0" Assemble if two character strings are identical Assemble if two character strings are not identical Assemble in the case of the condition that is the opposite of the above IF condition End conditional assembly Display on VDT during assembly Output list Interrupt list output Output macro expansion Interrupt macro expansion output Output conditional skip Interrupt conditional skip output |

| Category | Pseudo instruction | Function |
| --- | --- | --- |
| Miscellaneous | INCLUDE TITLE PAGE CHIP COMMENT WIDTH BANK CHANGE RADIX | Load file Specify list title End of page Define chip that is target of assembly Output comments to object file Specify number of columns in list file Specify RAM area bank Jump between external and internal ROM Specify default base |
| Optimization | JMPO BRO CALLO BZO BNZO BPO BPCO BNO DBNZO BEO BNEO | Generate optimal JMP instruction Generate optimal BR instruction Generate optimal CALL instruction Generate BZ instruction that will not generate an address error Generate BNZ instruction that will not generate an address error Generate BP instruction that will not generate an address error Generate BPC instruction that will not generate an address error Generate BN instruction that will not generate an address error Generate DBNZ instruction that will not generate an address error Generate BE instruction that will not generate an address error Generate BNE instruction that will not generate an address error |

  - ORG  (Specify origin)

ORG  expression

The ORG pseudo instruction begins the specification of addresses in program memory (ROM) and data memory (RAM) from the value of expression. expression must be either a numeric constant or an expression that has a definite value at the time of assembly.

Example:

page:    1 <org.ASM>

ERR  SEQ.     S LOC. OBJ. 	SOURCE STATEMENTS

0001	;a sample program for ORG

0002			chip	lc866032

0003			extern	wait1s

0004			dseg

0005	D 0000	min1:	ds	1

0006	D 0001	min0:	ds	1

0007			cseg

0008			org	0h

0009	C 0000 6201’	label1:	inc	min0

0010	C 0002 0201’		ld	min0

0011	C 0004 A13C		sub	#60

0012	C 0006 900311		bzo	label2

0012	C 0009 F600

0013	C 000B 210200’		jmpf	label3

0014			org	100h

0015	C 0100 6200’	label2:	inc	min1

0016	C 0102 220100’		mov	#00,min0

0017	C 0105 210200’		jmpf	label3

0018			org	200h

0019	C 0200 100000’	label3:	callr	wait1s

0020	C 0203 210000’		jmpf	label1

0021			end

  - WORLD  (Select ROM for code storage)

WORLD  selection

This pseudo instruction specifies the ROM where the assembled code should be stored. This pseudo instruction has meaning only when the target chip is of the LC868000 Series. The following three values can be specified for selection:

INTERNAL	:	Store the code in the on-chip ROM.

EXTERNAL	:	Store the code in the ROM that is connected externally for code storage.

EXTERNAL_DATA	:	Store the code in the ROM that is connected externally for data storage.

If multiple WORLD pseudo instructions are specified in one file, an error results. If a chip other than one of the LC868000 Series is specified, and a value other than INTERNAL is selected in the WORLD pseudo instruction, an error results.

  - CSEG  (Declare start of code segment)

CSEG  mode

This pseudo instruction declares to the assembler the start of the segment where the program code is to be stored. If mode is not specified, or if mode is specified as INBLOCK, the start of the segment is located at a 4K boundary. If mode is specified as FREE, the start of the segment has no relation to a 4K boundary.

Example:

page:    1 <cseg.ASM>

ERR  SEQ.	S LOC. OBJ.   SOURCE STATEMENTS

0001		; a sample program for CSEG

0002			chip    lc864024

0003			extern  wait1s

0004			dseg

0005	D 0000	min1:	ds	1

0006	D 0001	min0:	ds	1

The local address is reset to "0" at the start of each segment.

Independent segments

0007			cseg	inblock

0008	C 0000 6201’	label1: inc	min0

0009     C 0002 0201’		ld	min0

0010     C 0004 A13C		sub	#60

0011     C 0006 900311 		bzo	label2

0011     C 0009 0000

0012     C 000B 210000’		jmpf	label3

0013			cseg	free

0014	c 0000 6200’	label2:	inc	min1

0015     c 0002 220100’		mov 	#00,min0

0016     c 0005 210000’		jmpf	label3

0017			cseg

0018	C 0000 100000’	label3:	callr	wait1s

0019	C 0003 210000’		jmpf	label1

0020			end

  - DSEG  (Declare start of data segment)

DESG

This pseudo instruction declares to the assembler the start of the area in data memory that is to be allocated.

Example:

page:    1 <dseg.ASM>

ERR  SEQ.     S LOC. OBJ.   SOURCE STATEMENTS

0001		; a sample program for CSEG

0002			chip	lc864024

0003			extern	wait1s

Data segment

Code segment

0004			cseg	inblock

0005	C 0000 6201’	label1:	inc	min0

0006	C 0002 0201’		ld	min0

0007	C 0004 A13C		sub	#60

0008	C 0006 900311		bzo	label2

0008	C 0009 0000

0009	C 000B 210000’		jmpf	label3

0010			cseg	free

0011	c 0000 6200’	label2:	inc	min1

0012	c 0002 220100’		mov	#00,min0

0013	c 0005 210000’		jmpf	label3

0014			cseg

0015	C 0000 100000’	label3:	callr	wait1s

0016	C 0003 210000’		jmpf	label1

0017

0018			dseg

0019	D 0000	min1:	ds	1

0020	D 0001	min0:	ds	1

0021			end

  - END  (End program)

END

This pseudo instruction declares the end of the source program. Because the assembler ends the assembly operation for the pass that is being executed at the moment that this instruction is detected, any statements that follow this instruction are ignored, even if those statements are valid.

Example:

; a sample program for END

chip    lc866032

cseg

mov	#20h, 01h

mov	#10h, 00h

ld	00h

add	0fh

end

Statements that come after the pseudo instruction END are not assembled.

inc	00h

inc	01h

ld	01h

page:    1 <end.ASM>

ERR	SEQ.	S LOC. OBJ.	SOURCE STATEMENTS

0001		; a sample program for END

0002			chip	lc866032

0003			cseg

0004	C 0000 220120		mov	#20h, 01h

0005	C 0003 220010		mov	#10h, 00h

0006	C 0006 0200		ld	00h

0007	C 0008 820F		add	0fh

0008			end

  - PUBLIC  (Specify external definition name)

PUBLIC  symbol {, symbol}

The PUBLIC pseudo instruction permits symbol, which is defined in this source program, to be referenced from other source files.

Example:

page:    1 <extern.ASM>

ERR	SEQ.	S LOC. OBJ.	SOURCE STATEMENTS

0001		; a sample program for EXTERN

0002			chip	lc866032

0003			extern	label1, label2

0004

0005			cseg inblock

0006 	C 0000 200000’		callf   label1

0007

0008	C 0003 200000’start:	callf	label2

0009	C 0006 0303		ld	c

0010	C 0008 90F9		bnz	start

0011

0012	C 000A A300		sub	a

0013

0014			end

When referencing a symbol for which the value was defined in another source file, it is necessary to declare that symbol beforehand with the EXTERN pseudo instruction.

A symbol that is to be referenced from other source files must be made visible to those source files by declaring them with the PUBLIC pseudo instruction.

page:    1 <public.ASM>

ERR	SEQ.	S LOC. OBJ.	SOURCE STATEMENTS

0001		; a sample program for PUBLIC

0002			chip	lc866032

0003			public 	label1, label2

0004

0005			cseg	inblock

0006	C 0000 220000’	label1:	mov 	#00, data1

0007	C 0003 23033C		mov	#60, c

0008	C 0006 A0		ret

0009

0010	C 0007 6200’	label2:	inc	data1

0011	C 0009 0200’		ld	data1

0012	C 000B 410A05		bne	#10, label3

0013	C 000E 220000’		mov	#00, data1

0014	C 0011 6201’		inc	data2

0015

0016	C 0013 7303	label3:	dec	c

0017	C 0015 A0		ret

0018

0019			dseg

0020	D 0000	data1:	ds	1

0021	D 0001	data2:	ds	1

0022

0023			end

The PUBLIC and EXTERN pseudo instructions can be used in combination to permit referencing of symbols that are defined in other source files.

  - EXTERN  (Specify external reference name)

EXTERN  [segmanet:]symbol {,[segment:]symbol}

The EXTERN pseudo instruction is used in order to allow the specified symbol to be referenced from other programs. The segment specification permits specification of a segment within either CSEG or DSEG. If nothing is specified, the code segment CSEG is assumed. For an example of the use of the EXTERN pseudo instruction, refer to the explanation of the PUBLIC pseudo instruction.

  - OTHER_SIDE_SYMBOL  (Declare CHANGE instruction jump label)

OTHER SIDE SYMBOL  label {,label}

This pseudo instruction declares an address label that is specified as an operand of the CHANGE instruction, which is used to switch between internal ROM and external ROM in the LC868000 Series. Although the label that is declared is a type of external symbol, one difference is that in a source file written with codes that are stored in internal ROM, the label is declared in external ROM (while in the case of a source file written with codes that are stored in external ROM, the label is declared in internal ROM). Note that this pseudo instruction is used only by the LC868000 Series, and generates an error in all other cases. For an example of the use of the OTHER_SIDE_SYMBOL pseudo instruction, refer to the description of the CHANGE pseudo instruction.