  - External Interrupt Function

    - Overview

POTATO has a function that detects external input signals on the P70/INT0, P71/INT1, P72/INT2/T0IN, P73/INT3/T0IN pins and then generates interrupt requests to four vector addresses. The types of signals that are detected are selected by the program. P70 is used to detect the LM-BUS connection to VMS, and P71 is used to detect low voltage.

- Pins on which signals are detected and their corresponding vector addresses

| Pin | Vector address | Pin | Vector address |
| --- | --- | --- | --- |
| P70/INT0 | 003H | P72/INT2/T0IN | 013H |
| P71/INT1 | 00BH | P73/INT3/T0IN | 01BH |

- Signals that can be detected

The priority ranking of the INT0 and INT1 pin interrupts can be set to either "highest level" or "low level" through the Master Interrupt Enable Control Register (IE). If "highest level" is set, that interrupt processing can be executed, regardless of the master interrupt enable setting. The priority ranking of interrupts other than the INT0 and INT1 interrupts can be set to either "high level" or "low level" through the Interrupt Priority Ranking Control Register (IP). IN addition, a noise elimination filter with a switchable time constant is connected to the P73/INT3/T0IN pin.

- Detection of another VMS unit

- The statuses of various ports when the unit is connected or not connected to another VMS unit are shown below.

|  | P70 | P72 | P73 |
| --- | --- | --- | --- |
| When connected to a VMS unit | Ｌ | Ｌ | Ｈ |
| When not connected to a VMS unit | Ｌ | Ｌ | Ｌ |

In order to use the external interrupt function, it is necessary to manipulate the following Special Function Registers:

- I01CR

- I23CR

- ISL

- IE

    - Circuit Configuration

Fig. 3-8-1.  External Interrupt Circuit Block Diagram

    - Related Registers

- External Interrupt 0, 1 Control Register (I01CR)

| Symbol | Address | R/W | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| I01CR | 15DH | R/W | I01CR7 | I01CR6 | I01CR5 | I01CR4 | I01CR3 | I01CR2 | I01CR1 | I01CR0 |
| After reset |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit name | Function |  |  |
| --- | --- | --- | --- |
| I01CR7 (bit 7) I01CR6 (bit 6) | INT1 detection level/edge selection |  |  |
|  | I01CR7 | I01CR6 | INT1 interrupt condition |
|  | 0 0 1 1 | 0 1 0 1 | Falling edge detection L level detection Rising edge detection H level detection |
| I01CR5 (bit 5) | INT1 interrupt source |  |  |
|  | 0: No interrupt source 1: Interrupt source |  |  |
| I01CR4 (bit 4) | INT1 interrupt enable control |  |  |
|  | 0: Interrupt disabled 1: Interrupt enabled |  |  |
| I01CR3 (bit 3) | INT0 detection level/edge selection |  |  |
| I01CR2 (bit 2) | I01CR3 | I01CR2 | INT0 interrupt condition |
|  | 0 0 1 1 | 0 1 0 1 | Falling edge detection L level detection Rising edge detection H level detection |
| I01CR1 (bit 1) | INT0 interrupt source |  |  |
|  | 0: No interrupt source 1: Interrupt source |  |  |
| I01CR0 (bit 0) | INT0 interrupt enable control |  |  |
|  | 0: Interrupt disabled 1: Interrupt enabled |  |  |

I01CR7 (bit 7):	INT1 detection level/edge selection

I01CR6 (bit 6):

These bits select the INT1 interrupt condition for signals input on the P71/INT1 pin.

| I01CR7 | I01CR6 | INT1 interrupt condition |
| --- | --- | --- |
| 0 | 0 | Falling edge detection |
| 0 | 1 | L level detection |
| 1 | 0 | Rising edge detection |
| 1 | 1 | H level detection |

I01CR5 (bit 5):	INT1 interrupt source

This bit is set if the condition specified by bits I01CR7 and 6 is met. If INT1 interrupts are enabled (I01CR4 = 1), then control jumps to vector address 000BH and interrupt processing begins. This bit is not reset, even when interrupt processing is completed. Therefore, it is necessary for this bit to be reset by software.

I01CR4 (bit 4):	INT1 interrupt enable control

This bit enables (1)/disables (0) the acceptance of external interrupt 1 (INT1). When this bit is set to "1," then when I01CR5 is set, INT1 interrupt processing is executed; when this bit is set to "0," interrupt processing is not executed.

I01CR3 (bit 3):	INT0 detection level/edge selection

I01CR2 (bit 2):

These bits select the INT0 interrupt condition for signals input on the P70/INT0 pin.

| I01CR3 | I01CR2 | INT0 interrupt condition |
| --- | --- | --- |
| 0 | 0 | Falling edge detection |
| 0 | 1 | L level detection |
| 1 | 0 | Rising edge detection |
| 1 | 1 | H level detection |

I01CR1 (bit 1):	INT0 interrupt source

This bit is set if the condition specified by bits I01CR3 and 2 is met. If INT0 interrupts are enabled (I01CR0 = 1), then control jumps to vector address 0003H and interrupt processing begins. This bit is not reset, even when interrupt processing is completed. Therefore, it is necessary for this bit to be reset by software.

I01CR0 (bit 0):	INT0 interrupt enable control

This bit enables (1)/disables (0) the acceptance of external interrupt 0 (INT0). When this bit is set to "1," then when I01CR1 is set, INT0 interrupt processing is executed; when this bit is set to "0," interrupt processing is not executed.

- External Interrupt 2, 3 Control Register (I23CR)

For details, refer to Chapter 3, section 3.2.4, "External Interrupt 2, 3 Control Register."

| Symbol | Address | R/W | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| I23CR | 15EH | R/W | I23CR7 | I23CR6 | I23CR5 | I23CR4 | I23CR3 | I23CR2 | I23CR1 | I23CR0 |
| After reset |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit name | Function |
| --- | --- |
| I23CR7 (bit 7) | INT3 rising edge detection control |
|  | 0: Do not detect 1: Detect |
| I23CR6 (bit 6) | INT3 falling edge detection control |
|  | 0: Do not detect 1: Detect |
| I23CR5 (bit 5) | INT3 interrupt source |
|  | 0: No interrupt source 1: Interrupt source |
| I23CR4 (bit 4) | INT3 interrupt enable control |
|  | 0: Interrupt disabled 1: Interrupt enabled |
| I23CR3 (bit 3)) | INT2 rising edge detection control |
|  | 0: Do not detect 1: Detect |
| I23CR2 (bit 2) | INT2 falling edge detection control |
|  | 0: Do not detect 1: Detect |
| I23CR1 (bit 1) | INT2 interrupt source |
|  | 0: No interrupt source 1: Interrupt source |
| I23CR0 (bit 0) | INT2 interrupt enable control |
|  | 0: Interrupt disabled 1: Interrupt enabled |

- Input Signal Select Register (ISL)

For details, refer to Chapter 3, section 3.2.4, "Input Signal Selection Register."

| Symbol | Address | R/W | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ISL | 15FH | R/W | - | - | ISL5 | ISL4 | ISL3 | ISL2 | ISL1 | ISL0 |
| After reset |  |  | H | H | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit name | Function |  |  |
| --- | --- | --- | --- |
| ISL5 (bit 5) | Base timer clock selection |  |  |
| ISL4 (bit 4) | ISL5 | ISL4 |  |
|  | 1 0 X | 1 1 0 | Timer/counter T0 prescaler Cycle clock Sub-clock (crystal oscillation) |
| ISL3 (bit 3) | Buzzer output frequency selection |  |  |
|  | 0: fBST/16 1: fBST/8 |  |  |
| ISL2 (bit 2) | Noise elimination filter time constant selection |  |  |
| ISL1 (bit 1) | ISL2 | ISL1 | Time constant |
|  | 1 0 X | 1 1 0 | 16Tcyc 64Tcyc 1Tcyc |
| ISL0 (bit 0) | T0 clock input pin selection |  |  |
|  | 0: P72/INT2/T0IN pin 1: P73/INT3/T0IN pin |  |  |

- Master Interrupt Enable Control Register (IE)

| Symbol | Address | R/W | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IE | 108H | R/W | IE7 | - | - | - | - | - | IE1 | IE0 |
| After reset |  |  | 0 | H | H | H | H | H | 0 | 0 |

| Bit name | Function |  |  |  |
| --- | --- | --- | --- | --- |
| IE7 (bit 7) | Master interrupt enable control (high level, low level) |  |  |  |
|  | 0: All interrupt requests disabled 1: All interrupt requests enabled |  |  |  |
| IE1 (bit 1) | INT0, INT1 interrupt priority control |  |  |  |
| IE0 (bit 0) | IE1 | IE0 | INT1 priority level | INT0 priority level |
|  | 0 1 X | 0 0 1 | Highest level Low level Low level | Highest level Highest level Low level |

IE7 (bit 7):	Master interrupt enable control

This bit enables (1)/disables (0) the acceptance of all "high level" and "low level" interrupts. When this bit is set to "1," all interrupts for which interrupt requests have been generated are enabled; when this bit is set to "0," "high level" and "low level" interrupts are disabled.

IE1 (bit 1):	INT0, 1 interrupt priority control

IE0 (bit 0):

These bits set the priority level for external interrupts INT0 and 1.

| IE1 | IE0 | INT1 priority level | INT0 priority level |
| --- | --- | --- | --- |
| 0 | 0 | Highest level | Highest level |
| 1 | 0 | Low level | Highest level |
| X | 1 | Low level | Low level |

Notes:

- Although "low level" priority for INT0 and 1 is controlled by IE7, "highest level" priority is not.

- It is not possible to set just external interrupt INT1 alone to "highest level.