  - Timer/Counter 0 (T0)

    - Overview

POTATO's built-in Timer/Counter 0 (T0) is a 16-bit timer/counter that has the four functions listed below. In addition, the Timer 0 prescaler is an 8-bit prescaler.

Mode 0: 8-bit reload timer  2 channels, with programmable prescaler

Mode 1: 8-bit reload timer with programmable prescaler + 8-bit reload counter

Mode 2: 16-bit reload timer with programmable prescaler

Mode 3: 16-bit reload counter

    - Functions

- 8-bit reload timer  2 channels, with programmable prescaler (mode 0)

T0 operates as two independent 8-bit reload timers (T0H, T0L) according to the clock from the 10-bit/8-bit prescaler.

- 8-bit reload timer with programmable prescaler + 8-bit reload counter (mode 1)

T0H operates as an 8-bit reload timer according to the clock from the 8-bit prescaler. TOL detects and counts external input signals from the P72/INT2/T0IN and P73/INT3/T0IN pins.

- 16-bit reload timer with programmable prescaler (mode 2)

T0 operates as a 16-bit reload timer (T0H + T0L) according to the clock from the 10-bit/8-bit prescaler.

- 16-bit reload counter (mode 3)

T0 operates as a 16-bit reload counter in which the overflow of T0L is used as the clock for T0H. TOL detects and counts external input signals from the P72/INT2/T0IN and P73/INT3/T0IN pins.

- Interrupt generation

When the interrupt request enable bit is set, T0H and T0L interrupt requests are generated when the T0H and T0L registers overflow.

The following Special Function Registers must be manipulated in order to control Timer/Counter 0 (T0).

- T0H		T0HR

- T0L		T0LR

- T0CNT		T0PRR

- ISL		I23CR

    - Circuit Configuration

The configuration of Timer/Counter 0 (T0) is shown in Fig. 3-2-1.

- Prescaler ····· ¬

The 8-bit prescaler consists of an 8-bit programmable counter, and does not have a cycle clock divide-by-4 circuit.

The prescaler counts up while the system is running; "00" is set in the divide-by-4 circuit (in the case of the 10-bit prescaler) if the program has changed the prescaler data, and counting begins from the changed data. The cycle clock is a signal that is generated once per cycle while executing instructions and in HALT mode.

- Timer/Counter 0 Low (T0L) ·····

This is an 8-bit reload timer/counter that uses the prescaler output or a signal on an external pin as a clock. The T0LR data is reloaded upon a T0L overflow in modes 0 or 1 and upon a T0H overflow in modes 2 and 3; the TOLR data is also reloaded when TOLRUN (bit 6 of T0CNT) is reset and system operation is stopped.

- Timer/Counter 0 High (T0H) ····· ®

This is an 8-bit reload timer that uses the prescaler output or T0L overflow as a clock. The Timer 0 High Reload (T0HR) register data is reloaded upon a T0H overflow; the TOHR data is also reloaded when TOHRUN (bit 7 of T0CNT) is reset and system operation is stopped.

- Timer/Counter 0 Control Register (T0CNT) ····· ¯

This register sets modes 0 through 3 for T0 and controls interrupts.

Fig. 3-2-1  Timer/Counter 0 Block Diagram

    - Related Registers

- Timer/Counter 0 Control Register (T0CNT)

| Symbol | Address | R/W |  | Bit 7 |  | Bit 6 |  | Bit 5 |  | Bit 4 |  | Bit 3 |  | Bit 2 |  | Bit 1 |  | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T0CNT | 14EH | R/W |  | T0HRUN |  | T0LRUN |  | T0LONG |  | T0LEXT |  | T0HOVF |  | T0HIE |  | T0LOVF |  | T0LIE |
| After reset |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |

| Bit name | Function |
| --- | --- |
| T0HRUN (bit 7) | T0H count control |
|  | 0: Count stop/data reload 1: Count start |
| T0LRUN (bit 6) | T0L count control |
|  | 0: Count stop/data reload 1: Count start |
| T0LONG (bit 5) | Timer/counter 0 bit length selection |
|  | 0: 8 bits 1: 16 bits |
| T0LEXT (bit 4) | T0L input clock selection |
|  | 0: Prescaler output 1: External pin input signal In the case of an external pin, specify the pin through the Input Signal Selection (ISL) register. |
| T0HOVF (bit 3) | T0H overflow flag |
|  | 0: No overflow flag 1: Overflow flag |
| T0HIE (bit 2) | T0H interrupt request enable |
|  | 0: Interrupt request disabled 1: Interrupt request enabled |
| T0LOVF (bit 1) | T0L overflow flag |
|  | 0: No overflow flag 1: Overflow flag |
| T0LIE (bit 0) | T0L interrupt request enable |
|  | 0: Interrupt request disabled 1: Interrupt request enabled |

T0HRUN (bit 7):	T0H count control

This bit starts (1)/stops (0) Timer/Counter 0 High (T0H). When this bit is set to "1," the clock is input to T0H, which begins counting; when this bit is set to "0," the clock supplied to T0H is stopped and the reload data (T0HR) is transferred to T0H simultaneously.

T0LRUN (bit 6):	T0L count control

This bit starts (1)/stops (0) Timer/Counter 0 Low (T0L). When this bit is set to "1," the clock is input to T0L, which begins counting; when this bit is set to "0," the clock supplied to T0L is stopped and the reload data (T0LR) is transferred to T0L simultaneously.

T0LONG (bit 6):	Timer/Counter 0 bit length selection

This bit selects the T0 bit length as either 16 bits (1) or 8 bits (0). When this bit is set to "1," the bit length of Timer/Counter 0 is 16 bits; when this bit is set to "0," the bit length is set to 8 bits. Set this bit to "0" when using modes 0 and 1, and to "1" when using modes 2 and 3.

| Mode | T0LONG | T0LEXT |
| --- | --- | --- |
| 0 | 0 | 0 |
| 1 | 0 | 1 |
| 2 | 1 | 0 |
| 3 | 1 | 1 |

T0LEXT (bit 4):	T0L input clock selection

This bit selects the T0L input clock as either external pin input signal (1) or as prescaler output (0). When this bit is set to "1," the input signal from the external input pin (either P72/INT2/T0IN or P73/INT3/T0IN) becomes the clock for T0L; when this bit is set to "0," the prescaler output becomes the clock. The Input Signal Selection (ISL) register selects either P72/INT2/T0IN or P73/INT3/T0IN as the external input pin.

T0HOVF (bit 3):	T0H overflow flag

This flag is set when an overflow occurs in T0H, and does not change when no overflow occurs. Therefore, this flag must be reset by software.

T0HIE (bit 2):	T0H interrupt request enable control

This bit enables (1)/disables (0) interrupt requests due to a T0H overflow. When this bit is set to "1," an interrupt request is generated to vector address 0023H in response to a T0H overflow; when this bit is set to "0," no interrupt request is generated.

T0LOVF (bit 1):	T0L overflow flag

This flag is set when an overflow occurs in T0L, and does not change when no overflow occurs. Therefore, this flag must be reset by software.

In 16-bit mode, this flag is not set even if an overflow occurs in T0L; when an overflow occurs in T0H, this flag is set at the same time as T0HOVF.

T0LIE (bit 0):	T0L interrupt request enable control

This bit enables (1)/disables (0) interrupt requests due to a T0L overflow. When this bit is set to "1," an interrupt request is generated to vector address 0013H in response to a T0L overflow; when this bit is set to "0," no interrupt request is generated.

Notes:

- T0HOVF and T0LOVF must be set to "0" by software.

- When operating in 16-bit mode, set T0HRUN and T0LRUN to "1" simultaneously.

- In 16-bit mode, T0HOVF and T0LOVF both go to "1" simultaneously.

- Input Signal Selection register (ISL)

This register is used to select the time constant for the noise elimination filter connected to the P73/INT3/T0IN pin, and to select the external signal input pin. This register cannot be accessed by game programs.

Use only bit manipulation instructions to access this register.

| Symbol | Address | R/W |  | Bit 7 |  | Bit 6 |  | Bit 5 |  | Bit 4 |  | Bit 3 |  | Bit 2 |  | Bit 1 |  | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ISL | 15FH | R/W |  | - |  | - |  | ISL5 |  | ISL4 |  | ISL3 |  | ISL2 |  | ISL1 |  | ISL0 |
| After reset |  |  | H |  | H |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |

| Bit name | Function |  |  |
| --- | --- | --- | --- |
| ISL5 (bit 5) ISL4 (bit 4) | Base timer clock selection |  |  |
|  | ISL5 | ISL4 |  |
|  | 1 0 X | 1 1 0 | Timer/Counter T0 prescaler Cycle clock Subclock (crystal oscillator) |
| ISL3 (bit 3) | Buzzer output frequency selection |  |  |
|  | 0: fBST/16 1: fBST/8 |  |  |
| ISL2 (bit 2) ISL1 (bit 1) | Noise elimination filter time constant selection |  |  |
|  | ISL2 | ISL1 | Time constant |
|  | 1 0 X | 1 1 0 | 16Tcyc 64Tcyc 1Tcyc |
| ISL0 (bit 0) | T0 clock input pin selection |  |  |
|  | 0: P72/INT2/T0IN pin 1: P73/INT3/T0IN pin |  |  |

ISL5 (bit 5):	Base timer clock selection

ISL4 (bit 4):

These bits select the base timer input clock.

| ISL5 | ISL4 | Base timer input clock |
| --- | --- | --- |
| 0 | 0 | Fixed to subclock (crystal oscillator) |

ISL3 (bit 3):	Buzzer output frequency selection

This bit selects the buzzer output frequency as either fBST/8 (1) or as fBST/16 (0). When "1" is set, the signal that is output from the buzzer output pin (BUZ) has a frequency that is 1/8 that of the base timer input frequency; when "0" is set, the signal that is output has a frequency that is 1/16 that of the base timer input frequency.

ISL2 (bit 2):	Noise elimination filter time constant selection

ISL1 (bit 1):

These bits select the noise elimination filter time constant.

| ISL2 | ISL1 | Time constant |
| --- | --- | --- |
| 0 | 0 | 1Tcyc |

The following table shows the signal and noise ranges for each time constant.

| Time constant | Noise *1 | Noise/signal *2 | Signal *3 |
| --- | --- | --- | --- |
| 1Tcyc | < 1Tcyc | 1Tcyc - 2Tcyc | 2Tcyc < |

*1）	A signal that does not meet the indicated time constant is deemed to be noise and is not accepted by the LSI.

*2）	A signal that falls within the indicated range for the time constant may be deemed to be noise and might not be accepted by the LSI.

*3）	A signal that exceeds the indicated time constant is deemed to be the correct signal and is accepted by the LSI.

ISL0 (bit 0):	T0 clock input pin selection

This bit selects the external signal input pin for the T0 as either P73/INT3/T0IN (1) or P72/INT2/T0IN (0). If "0" is set, P72/INT2/T0IN is selected as the external signal input pin.

- Timer 0 Prescaler Data Register (T0PRR)

The Timer 0 Prescaler Data Register sets the clock cycle for Timer/Counter 0; the clock cycle can be set to one of 256 levels through an 8-bit programmable counter.

For the 8-bit prescaler, the cycle clock signal is input directly. The clock cycle TPR for Timer/Counter 0 can be determined by setting the desired data in T0PRR (111H).

8-bit prescaler :TPR = 1 Tcyc (256-[T0PRR]) (decimal)

Tcyc: Cycle clock cycle

| Symbol | Address | R/W |  | Bit 7 |  | Bit 6 |  | Bit 5 |  | Bit 4 |  | Bit 3 |  | Bit 2 |  | Bit 1 |  | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T0PRR | 111H | R/W |  | T0PRR7 |  | T0PRR6 |  | T0PRR5 |  | TOPRR4 |  | T0PRR3 |  | T0PRR2 |  | T0PRR1 |  | T0PRR0 |
| After reset |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |

- Timer 0 Low register (T0L)

This is an 8-bit timer/counter. This timer/counter detects and counts either the clock from the prescaler or an external signal from P72/INT2/T0IN or P73/INT3/T0IN. When this register overflows, the T0L overflow flag is set.

| Symbol | Address | R/W |  | Bit 7 |  | Bit 6 |  | Bit 5 |  | Bit 4 |  | Bit 3 |  | Bit 2 |  | Bit 1 |  | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T0L | 112H | R |  | T0L7 |  | T0L6 |  | T0L5 |  | T0L4 |  | T0L3 |  | T0L2 |  | T0L1 |  | T0L0 |
| After reset |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |

- Timer 0 Low Reload register (T0LR)

This is the reload register for Timer/Counter 0 Low (T0L). In 8-bit mode, the reload data is reloaded into T0L each time that T0L overflows and when T0LRUN = 0; in 16-bit mode, the reload data is reloaded into T0L each time that T0H overflows and when T0HRUN = 0.

| Symbol | Address | R/W |  | Bit 7 |  | Bit 6 |  | Bit 5 |  | Bit 4 |  | Bit 3 |  | Bit 2 |  | Bit 1 |  | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T0LR | 113H | R/W |  | T0LR7 |  | T0LR6 |  | T0LR5 |  | T0LR4 |  | T0LR3 |  | T0LR2 |  | T0LR1 |  | T0LR0 |
| After reset |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |

- Timer 0 High register (T0H)）

This is an 8-bit timer/counter. This timer/counter detects and counts either the clock from the prescaler or T0L overflows. When this register overflows, the T0H overflow flag is set.

| Symbol | Address | R/W |  | Bit7 |  | Bit6 |  | Bit5 |  | Bit4 |  | Bit3 |  | Bit2 |  | Bit1 |  | Bit0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T0H | 114H | R |  | T0H7 |  | T0H6 |  | T0H5 |  | T0H4 |  | T0H3 |  | T0H2 |  | T0H1 |  | T0H0 |
| After reset |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |

- Timer 0 High Reload register (T0HR)

This is the reload register for Timer/Counter 0 High (T0H). The reload data is reloaded into T0H each time that T0H overflows and when T0HRUN = 0.

| Symbol | Address | R/W |  | Bit7 |  | Bit6 |  | Bit5 |  | Bit4 |  | Bit3 |  | Bit2 |  | Bit1 |  | Bit0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T0HR | 115H | R/W |  | T0HR 7 |  | T0HR6 |  | T0HR5 |  | T0HR4 |  | T0HR3 |  | T0HR2 |  | T0HR1 |  | T0HR0 |
| After reset |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |

- External Interrupt 2, 3 Control Register (I23CR)

This register sets external input signal detection and interrupts.

Regarding ISL0, refer to the "Input Signal Selection Register" (described later) for details.

| ISL0 | I23CR7 | I23CR6 | I23CR3 | I23CR2 | External Signal Counting Condition |
| --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | - | - | Count falling edge of P73/INT3/T0IN |
| 1 | 1 | 0 | - | - | Count rising edge of P73/INT3/T0IN |
| 1 | 1 | 1 | - | - | Count both edges of P73/INT3/T0IN |
| 0 | - | - | 0 | 1 | Count falling edge of P72/INT2/T0IN |
| 0 | - | - | 1 | 0 | Count rising edge of P72/INT2/T0IN |
| 0 | - | - | 1 | 1 | Count both edges of P72/INT2/T0IN |
| - | 0 | 0 | 0 | 0 | Do not count |

| Symbol | Address | R/W |  | Bit7 |  | Bit6 |  | Bit5 |  | Bit4 |  | Bit3 |  | Bit2 |  | Bit1 |  | Bit0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| I23CR | 15EH | R/W |  | I23CR7 |  | I23CR6 |  | I23CR5 |  | I23CR4 |  | I23CR3 |  | I23CR2 |  | I23CR1 |  | I23CR0 |
| After reset |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |

| Bit name | Function |
| --- | --- |
| I23CR7 (bit 7) | INT3 rising edge detection control |
|  | 0: Do not detect 1: Detect |
| I23CR6 (bit 6) | INT3 falling edge detection control |
|  | 0: Do not detect 1: Detect |
| I23CR5 (bit 5) | INT3 interrupt source |
|  | 0: No interrupt source 1: Interrupt source |
| I23CR4 (bit 4) | INT3 interrupt request enable control |
|  | 0: Interrupt request disabled 1: Interrupt request enabled |
| I23CR3 (bit 3) | INT2 rising edge detection control |
|  | 0: Do not detect 1: Detect |
| I23CR2 (bit 2) | INT2 falling edge detection control |
|  | 0: Do not detect 1: Detect |
| I23CR1 (bit 1) | INT2 interrupt source |
|  | 0: No interrupt source 1: Interrupt source |
| I23CR0 (bit 0) | INT2 interrupt request enable control |
|  | 0: Interrupt request disabled 1: Interrupt request enabled |

I23CR7 (bit 7):	INT3 rising edge detection control

This bit selects whether to detect (1) or not detect (0) the rising edge of the signal input to the P73/INT3/T0IN pin. When this bit is set to "1," I23CR5 is set (1) when the rising edge of the input signal is detected; if the INT3 interrupt request is enabled (I23CR4 = 1), then an interrupt is generated. When this bit is set to "0," the rising edge of the signal is not detected.

I23CR6 (bit 6):	INT3 falling edge detection control

This bit selects whether to detect (1) or not detect (0) the falling edge of the signal input to the P73/INT3/T0IN pin. When this bit is set to "1," I23CR5 is set (1) when the falling edge of the input signal is detected; if the INT3 interrupt request is enabled (I23CR4 = 1), then an interrupt is generated. When this bit is set to "0," the falling edge of the signal is not detected.

I23CR5 (bit 5):	INT3 interrupt source

This bit is set if the conditions specified by I23CR7 or I23CR6 are met; if the INT3 interrupt request is enabled, then control jumps to vector address 001BH and interrupt processing begins. The value of this bit does not change even when interrupt processing is completed. Therefore, this bit must be reset by software.

I23CR4 (bit 4):	INT3 interrupt request enable

This bit enables (1) or disables (0) external interrupt 3 (INT3). When this bit is set to "1," and I23CR5 has been set, then INT3 interrupt processing is executed. When this bit is set to "0," interrupt processing is not executed.

I23CR3 (bit 3):	INT2 rising edge detection control

This bit selects whether to detect (1) or not detect (0) the rising edge of the signal input to the P72/INT2/T0IN pin. When this bit is set to "1," I23CR1 is set (1) when the rising edge of the input signal is detected; if the INT2 interrupt request is enabled (I23CR0 = 1), then an interrupt is generated. When this bit is set to "0," the rising edge of the signal is not detected.

I23CR2 (bit 2):	INT2 falling edge detection control

This bit selects whether to detect (1) or not detect (0) the falling edge of the signal input to the P72/INT2/T0IN pin. When this bit is set to "1," I23CR1 is set (1) when the falling edge of the input signal is detected; if the INT2 interrupt request is enabled (I23CR0 = 1), then an interrupt is generated. When this bit is set to "0," the falling edge of the signal is not detected.

I23CR1 (bit 1):	INT2 interrupt source

This bit is set if the conditions specified by I23CR3 or I23CR2 are met; if the INT2 interrupt request is enabled, then control jumps to vector address 0013H and interrupt processing begins. The value of this bit does not change even when interrupt processing is completed. Therefore, this bit must be reset by software.

I23CR0 (bit 0):	INT2 interrupt request enable control

This bit enables (1) or disables (0) external interrupt 2 (INT2). When this bit is set to "1," and I23CR1 has been set, then INT2 interrupt processing is executed. When this bit is set to "0," interrupt processing is not executed.

Note:

- Edge detection is not performed if I23CR7 and 6 or I23CR3 and 2 are both "0." If both of either pair are "1," then both edges are detected.

- Input from the P73/INT3/T0IN pin is connected to the noise filter.

    - Circuit Configuration and Description of Operation

- Timer 0 mode settings

| Mode | T0LONG | T0LEXT |
| --- | --- | --- |
| 0 | 0 | 0 |
| 1 | 0 | 1 |
| 2 | 1 | 0 |
| 3 | 1 | 1 |

- Mode 0: 8-bit reload timer  2 channels, with programmable prescaler

In mode 0, Timer 0 functions as two 8-bit reload timers. The relationship between the timer value and the reload register (T0LR) setting is as shown below.

Time until T0HOVF is set (1) (decimal) = (256 - T0HR setting)  TPR

Time until T0LOVF is set (1) (decimal) = (256 - T0LR setting)  TPR

TPR: Clock cycle from prescaler

Once the count control bit (T0HRUN, T0LRUN) is set, the counting operation starts. If the bit is reset, the counting operation stops, and the contents of the reload register (T0HR, T0LR) are transferred to the counter (T0H, T0L).

If Timer/Counter 0 (T0H, T0L) overflows, the overflow flag (T0HOVF, T0LOVF) is set , and the contents of the reload register (T0HR, T0LR) are transferred to the counter (T0H, T0L).

If both the overflow flag (T0HOVF, T0LOVF) and the interrupt request enable flag (T0HIE, T0LIE) are set, an interrupt request is sent to the interrupt control circuit.

・T0H

・T0L

Fig. 3-2-2.  Circuit Configuration for Mode 0: 8-bit Reload Timer x 2 Channels

- Mode 0 Program Example

- Mode 1: 8-bit reload timer with programmable prescaler + 8-bit reload counter

- 8-bit reload timer

The upper 8 bits of Timer 0 (T0H) operate as an 8-bit reload timer. The relationship between the timer value and the reload register (T0HR) setting is as shown below.

Time until T0HOVF is set (1) (decimal) = (256 - T0HR setting)  TPR

TPR: Cycle of clock from prescaler

The data in the reload register is loaded into the counter T0H at each interval at which T0HOVF is set. In addition, the timer operation continues until the T0H count control bit (T0HRUN) is reset. The operation method is the same as for mode 0.

Fig. 3-2-3.  Block Diagram for Mode 1: 8-bit Reload Timer (T0H)

- 8-bit reload counter

The lower 8 bits of Timer 0 (T0L) are used to count the signals input through the external pin. The external signal is filtered through a noise filter. For details, refer to Chapter 3, section 3.2.4, "Related Registers and Input Signal Selection Register (ISL)."

The relationship between the counted value and the reload register (T0LR) setting is as follows:

Counted value until T0LOVF is set (1) (decimal) = 256 - (T0LR setting)

If the T0L overflow flag (T0LOVF) is set, the data in the reload register T0LR is transferred to the counter T0L. In addition, counting continues until the T0L count control bit (T0LRUN) is reset.

Fig. 3-2-4.  Block Diagram for Mode 1: 8-bit Reload Counter (T0L)

- Mode 1 Program Example

- Mode 2: 16-bit reload timer with programmable prescaler

Mode 2 connects T0H and T0L in series and uses them as a 16-bit timer.

To start the timer, set the counter control bits (T0HRUN and T0LRUN) for both T0H and T0L simultaneously.

The relationship between the timer value and the reload register (T0HR, T0LR) settings is as follows:

Time until T0HVOF is set (1) (decimal) = (65,536 - 256 (T0HR setting) - (T0LR setting))  TPR

TPR: Cycle of clock from prescaler

T0LOVF is set at the same time as T0HOVF, and each time that T0HOVF is generated, the reload data (T0LR, T0HR) is transferred to T0L and T0H, respectively. Timer operation continues until the count control bit is reset. The operation method is the same as for mode 0.

Read the Timer 0 (T0) data according to the following procedure:

T0L	LD	T0L	; Read T0L data (1).

	ST	020H

T0H	LD	T0H	; Read T0H data.

	ST	021H

T0L	LD	T0L	; Read T0L (2) data again.

	BP	T0L,7,DES	; When bit 7 of TOL (2) is "0" and

BN	020H,7,DES	; bit 7 of T0L (1) is "1"...

ST	020H

T0H	LD	T0H	; Read T0H (2).

ST	021H

DES:	－ next program

Fig. 3-2-5.  Block Diagram for Mode 2: 16-bit Reload Timer

- Mode 2 Program Example

- Mode 3: 16-bit reload counter

Mode 3 connects T0H and T0L in series and uses them as a 16-bit counter. The clock is an external signal that is input from either P72/INT2/T0IN or P73/INT3/T0IN. The external input pin is selected through the special function register ISL. A noise elimination filter is connected to the P73/INT3/T0IN pin.

To start the counter, set the counter control bits (T0HRUN and T0LRUN) for both T0H and T0L simultaneously.

The relationship between the counted value and the reload register (T0HR, T0LR) settings is as follows:

Count until T0HVOF is set (1) (decimal) = 65,536 - 256  (T0HR setting) - (T0LR setting)

T0LOVF is set at the same time as T0HOVF, and each time that T0HOVF is generated, the reload data (T0LR, T0HR) is transferred to T0L and T0H, respectively. Timer operation continues until the count control bit is reset. The operation method is the same as for mode 0.

Read the Timer 0 (T0) data according to the following procedure:

T0L	LD	T0L	; Read T0L data (1).

	ST	020H

T0H	LD	T0H	; Read T0H data.

	ST	021H

T0L	LD	T0L	; Read T0L (2) data again.

	BP	T0L,7,DES	; When bit 7 of TOL (2) is "0" and

BN	020H,7,DES	; bit 7 of T0L (1) is "1"...

ST	020H

T0H	LD	T0H	; Read T0H (2).

ST	021H

DES:	－ next program

Fig. 3-2-6.  Block Diagram for Mode 3: 16-bit Reload Counter

- Mode 3 Program Example