  - Standby function

    - Overview

POTATO has two standby modes (HALT and HOLD) that are designed to reduce current consumption during a loss of power or while a program is in a standby state.

The microcomputer ceases operations while in the standby state.

    - Related Registers

- Power Control Register (PCON)

* For details, refer to chapter 4, section 4.2.4, "Related Registers."

| Symbol | Address | R/W | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PCON | 107H | R/W | - | - | - | - | - | - | PCON1 | PCON0 |
| After reset |  |  | H | H | H | H | H | H | 0 | 0 |

| Bit name | Function |
| --- | --- |
| PCON1 bit 1) | HOLD mode control |
|  | 0: 1: et HOLD mode |
| PCON0 bit 0) | HALT mode control |
|  | 0: 1: et HALT mode |

    - Operating Statuses When in Standby

Table 4-3-1.  erating Status of Each Block When in Standby

| Item |  | HALT mode |  | HOLD mode |  |
| --- | --- | --- | --- | --- | --- |
| Setting method |  | PCON0 = 1 |  | PCON1 = 1 |  |
| Oscillation circuit | Main |  | Oscillation continues *1 |  | Oscillation stops |
|  | Internal RC |  |  |  |  |
|  | Sub-clock |  | Operation continues |  |  |
| Internal clock | S1 |  | Operation continues |  | Operation stops |
|  | S2 |  | Operation stops |  | Operation stops |
| CPU |  | Operation stops |  | Operation stops |  |
| I/O port |  | Retains data from directly prior to entering HALT mode |  | Retains data from directly prior to entering HALT mode |  |
| RAM |  | Retains data from directly prior to entering HALT mode |  | Retains data from directly prior to entering HALT mode |  |
| Base timer |  | Operation continues |  | Operation stops |  |
| Timer 0 |  | Operation continues |  | Operation stops |  |
| Timer 1 |  | Operation continues |  | Operation stops |  |
| Serial communications |  | Operation continues |  | Operation stops |  |
| Interrupt circuit |  | Operation continues |  | Operation stops |  |
| LCD display controller |  | Operation continues |  | Operation stops |  |
| Remote control communications circuit |  | Operation continues |  | Operation stops |  |
| Watchdog timer |  | Operation continues, or stops |  | Operation stops |  |
| Release sources |  | Reset Acceptance of interrupt request |  | Reset P70/INT0 pin, P71/INT1 pin Port 3 pin |  |

*1)	When the sub-clock is the system clock, oscillation can be stopped by a program.

(Internal RC: OCR1 = 1; Main: OCR0 = 1)

    - HALT Mode

HALT mode stops program execution while each of the oscillation circuits (main clock, sub-clock and internal RC) continue to operate.

Power consumption can be reduced through intermittent operation of the system by repeatedly setting HALT mode and then having it released in response to an interrupt.

- Setting HALT mode

HALT mode is set by setting bit 0 (PCON0) of the power control register.

- Releasing HALT mode

HALT mode is released in one of two ways: "release by reset" and "release upon acceptance of an interrupt request."

Release by reset

If a low-level signal is applied to the RES pin, HALT mode is released and the microcomputer enters the reset state. When the RES pin is returned to the high level, operation is identical to a start after a normal reset.

Release upon acceptance of an interrupt request

If an interrupt source is generated while the master interrupt enable flag (IE7) and the interrupt request enable flag are both set, an interrupt request is generated and HALT mode is released simultaneously. Subsequently, the microcomputer enters the interrupt processing routine.

However, if the system is in HALT mode and executing an interrupt service program, and the interrupt source that was generated has a priority level that is either the same level as the program that is being executed or lower, then that interrupt is not accepted.

Notes:

- If external interrupt INT0 or INT1 is set to "highest level," then that interrupt is not affected by the master interrupt enable flag.

- The interrupt level of a HALT release source should be higher than the interrupt level in effect when the system entered HALT mode.

Table 4-3-2.  HALT Release Source Interrupt Level

| Interrupt level when HALT was entered | Interrupt level of HALT release source |
| --- | --- |
| Normal level | Low level, high level or highest level |
| Low level | High level or highest level |
| High level | Highest level |
| Highest level | (Cannot be released by an interrupt.) |

Normal level: State when no interrupt is in effect

    - HOLD Mode

HOLD mode stops each of the oscillation circuits (main clock, sub-clock and internal RC). HOLD mode can be set in order to maintain data while minimizing current consumption.

- Setting HOLD mode

HOLD mode is set by setting bit 1 (PCON1) of the Power Control register (PCON).

- Releasing HOLD mode

HOLD mode is released in one of three ways: release by reset; release through P70/INT0 level detection or P71/INT1 level detection; or port 3 low level detection.

Release by reset

If a low-level signal is applied to the RES pin, HOLD mode is released and the microcomputer enters the reset state. When the RES pin is returned to the high level, operation is identical to a start after a normal reset.

Release through P70/INT0 level detection or P71/INT1 level detection

If the set level is detected on the P70/INT0 or P71/INT1 pin, HOLD mode is released, and the system enters HALT mode. In this case, if the interrupt request flag for either external interrupt INT0 or external interrupt INT1 is set, the microcomputer enters the corresponding interrupt processing routine; if the interrupt enable flag for the external interrupt has not been set, the system continues in HALT mode. HALT mode is released in the same fashion as described in Chapter 4, section 4.3.3, "Release upon acceptance of an interrupt request." In addition, before setting HOLD mode, it is necessary to set the External Interrupt 0 and 1 Control Register (IO1CR) so that the level (whether high level or low level) is set. That level is detected on the P70/INT0 and P71/INT1 pins.

It is not possible to release HOLD mode with the edge detection setting.

For details on the level detection conditions, refer to Chapter 3, section 3.8, "External Interrupt Function."

Release through port 3 low level detection

- P32INT = 1: Port 3 interrupt and HOLD mode release function

* P32INT = 1 must be set.

Release through port 3 low level detection

When the port 3 interrupt request enable flag is set, and a low-level signal is detected on port 3, the interrupt request flag is set, HOLD mode is released, and the system enters HALT mode. In this case, if the master interrupt enable flag has been set, HALT mode is released and the microcomputer enters the interrupt processing routine. If the master interrupt enable flag is reset (0), the system remains in HALT mode.

Note:

When releasing HOLD through port 3, disable any interrupts caused by a source other than port 3.

Fig. 4-3-1.  Standby Function State Transition Diagram