  - Data Transfer Instructions

LD  d9  (Load direct byte to accumulator)

| Instruction code | 0 0 0 0 0 0 1d8 d8d7d6d5d4d3d2d1d0 02H03H |
| --- | --- |
| Number of bytes | 2 |
| Number of cycles | 1 |
| Function | (ACC)(d9) |
| Affected flags |  |
| Interrupt acceptance | Permitted |

Description:

This instruction transfers the contents of data memory (RAM) or a Special Function Register (SFR), as specified by d8 to d0, to the Accumulator (ACC).

Example 1:

|  |  | ACC | RAM 70H | RAM 71H |
| --- | --- | --- | --- | --- |
| MOV | #0FF,ACC | FFH | - | - |
| MOV | #055H,070H | FFH | 55H | - |
| MOV | #0AAH,071H | FFH | 55H | AAH |
| LD | 070H | 55H | 55H | AAH |
| LD | 071H | AAH | 55H | AAH |

Example 2:

|  |  | ACC | B | SP |
| --- | --- | --- | --- | --- |
| MOV | #0FF,ACC | FFH | - | - |
| MOV | #0F0H,B | FFH | F0H | - |
| MOV | #00FH,SP | FFH | F0H | 0FH |
| LD | B | F0H | F0H | 0FH |
| LD | SP | 0FH | F0H | 0FH |
| LD | B | F0H | F0H | 0FH |

LD　@Rj　(Load indirect byte to accumulator)

| Instruction code | 0 0 0 0 0 1j1j0 04H07H |
| --- | --- |
| Number of bytes | 1 |
| Number of cycles | 1 |
| Function | (ACC)((Rj)) j=0,1,2,3 |
| Affected flags |  |
| Interrupt acceptance | Permitted |

Description:

This instruction transfers the contents of data memory (RAM) or a Special Function Register (SFR), as specified by the indirect address register that is specified by j1 and j0, to the Accumulator (ACC).

Example 1:

|  |  | ACC | RAM 00H | RAM 01H | RAM 70H | RAM 7FH |
| --- | --- | --- | --- | --- | --- | --- |
| MOV | #0FFH,ACC | FFH | - | - | - | - |
| MOV | #070H,000H | FFH | 70H | - | - | - |
| MOV | #07FH,001H | FFH | 70H | 7FH | - | - |
| MOV | #0F0H,@R0 | FFH | 70H | 7FH | F0H | - |
| MOV | #00FH,@R1 | FFH | 70H | 7FH | F0H | 0FH |
| LD | @R0 | F0H | 70H | 7FH | F0H | 0FH |
| LD | @R1 | 0FH | 70H | 7FH | F0H | 0FH |

Example 2:

|  |  | ACC | RAM 02H | RAM 03H | B 102H | C 103H |
| --- | --- | --- | --- | --- | --- | --- |
| MOV | #0FF,ACC | FFH | - | - | - | - |
| MOV | #004H,002H | FFH | 04H | - | - | - |
| MOV | #005H,003H | FFH | 04H | 05H | - | - |
| MOV | #0AAH,@R2 | FFH | 04H | 05H | AAH | - |
| MOV | #055H,@R3 | FFH | 04H | 05H | AAH | 55H |
| LD | @R2 | AAH | 04H | 05H | AAH | 55H |
| LD | @R3 | 55H | 04H | 05H | AAH | 55H |

ST  d9  (Store direct byte to accumulator)

| Instruction code | 0 0 0 1 0 0 1d8 d7d6d5d4d3d2d1d0 12H13H |
| --- | --- |
| Number of bytes | 2 |
| Number of cycles | 1 |
| Function | (d9)(ACC) |
| Affected flags |  |
| Interrupt acceptance | Permitted |

Description:

This instruction transfers the contents of the Accumulator (ACC) to data memory (RAM) or a Special Function Register (SFR), as specified by d8 to d0.

Example 1:

|  |  | ACC | RAM 70H | RAM 71H |
| --- | --- | --- | --- | --- |
| MOV | #0FFH,ACC | FFH | - | - |
| MOV | #055H,070H | FFH | 55H | - |
| MOV | #0AAH,071H | FFH | 55H | AAH |
| ST | 070H | FFH | FFH | AAH |
| MOV | #000H,ACC | 00H | FFH | AAH |
| ST | 071H | 00H | FFH | 00H |

Example 2:

|  |  | ACC | B | SP |
| --- | --- | --- | --- | --- |
| MOV | #012H,ACC | 12H | - | - |
| MOV | #0F0H,B | 12H | F0H | - |
| MOV | #00FH,SP | 12H | F0H | 0FH |
| ST | B | 12H | 12H | 0FH |
| MOV | #034H,ACC | 34H | 12H | 0FH |
| ST | SP | 34H | 12H | 34H |
| ST | B | 34H | 34H | 34H |

ST  @Rj  (Store indirect byte to accumulator)

| Instruction code | 0 0 0 1 0 1j1j0 14H17H |
| --- | --- |
| Number of bytes | 1 |
| Number of cycles | 1 |
| Function | ((Rj))(ACC) j=0,1,2,3 |
| Affected flags |  |
| Interrupt acceptance | Permitted |

Description:

This instruction transfers the contents of the Accumulator (ACC) to data memory (RAM) or a Special Function Register (SFR), as specified by the indirect address register that is specified by j1 and j0.

Example 1:

|  |  | ACC | RAM 00H | RAM 01H | RAM 70H | RAM 7FH |
| --- | --- | --- | --- | --- | --- | --- |
| MOV | #0FFH,ACC | FFH | - | - | - | - |
| MOV | #070H,000H | FFH | 70H | - | - | - |
| MOV | #07FH,001H | FFH | 70H | 7FH | - | - |
| MOV | #0F0H,@R0 | FFH | 70H | 7FH | F0H | - |
| MOV | #00FH,@R1 | FFH | 70H | 7FH | F0H | 0FH |
| ST | @R0 | FFH | 70H | 7FH | FFH | 0FH |
| ST | @R1 | FFH | 70H | 7FH | FFH | FFH |

Example 2:

|  |  | ACC | RAM 02H | RAM 03H | TRL 104H | TRH 105H |
| --- | --- | --- | --- | --- | --- | --- |
| MOV | #000H,ACC | 00H | - | - | - | - |
| MOV | #004H,002H | 00H | 04H | - | - | - |
| MOV | #005H,003H | 00H | 04H | 05H | - | - |
| MOV | #0AAH,@R2 | 00H | 04H | 05H | AAH | - |
| MOV | #055H,@R3 | 00H | 04H | 05H | AAH | 55H |
| ST | @R2 | 00H | 04H | 05H | 00H | 55H |
| ST | @R3 | 00H | 04H | 05H | 00H | 00H |

MOV  #i8,d9  (Move immediate data to direct byte)

| Instruction code | 0 0 1 0 0 0 1d8 d7d6d5d4d3d2d1d0 i7i6i5i4i3i2i1i0 22H23H |
| --- | --- |
| Number of bytes | 3 |
| Number of cycles | 2 |
| Function | (d9)#i8 |
| Affected flags |  |
| Interrupt acceptance | Permitted at 2nd cycle |

Description:

This instruction transfers immediate data (i7 to i0) to data memory (RAM) or a Special Function Register (SFR), as specified by d8 to d0.

Example 1:

|  |  | RAM 00H | RAM 01H | RAM 02H | RAM 03H |
| --- | --- | --- | --- | --- | --- |
| MOV | #0FFH,000H | FFH | - | - | - |
| MOV | #0FEH,001H | FFH | FEH | - | - |
| MOV | #0FDH,002H | FFH | FEH | FDH | - |
| MOV | #0FCH,003H | FFH | FEH | FDH | FCH |
| MOV | #0FBH,003H | FFH | FEH | FDH | FBH |
| MOV | #0FAH,002H | FFH | FEH | FAH | FBH |
| MOV | #0F9H,001H | FFH | F9H | FAH | FBH |
| MOV | #0F8H,000H | F8H | F9H | FAH | FBH |

Example 2:

|  |  | ACC | B | TRL |
| --- | --- | --- | --- | --- |
| MOV | #0FFH,100H | FFH | - | - |
| MOV | #0FEH,102H | FFH | FEH | - |
| MOV | #0FDH,104H | FFH | FEH | FDH |
| MOV | #0FAH,104H | FFH | FEH | FAH |
| MOV | #0F9H,102H | FFH | F9H | FAH |
| MOV | #0F8H,100H | F8H | F9H | FAH |

MOV  #i8,@Rj (Move immediate data to indirect byte)

| Instruction code | 0 0 1 0 0 1j1j0 i7i6i5i4i3i2i1i0 24H27H |
| --- | --- |
| Number of bytes | 2 |
| Number of cycles | 1 |
| Function | ((Rj))#i8 j=0,1,2,3 |
| Affected flags |  |
| Interrupt acceptance | Permitted |

Description:

This instruction transfers immediate data (i7 to i0) to data memory (RAM) or a Special Function Register (SFR), as specified by the indirect address register that is specified by j1 and j0.

Example 1:

|  |  | RAM 00H | RAM 01H | RAM 7EH | RAM 7FH |
| --- | --- | --- | --- | --- | --- |
| MOV | #07FH,000H | 7FH | - | - | - |
| MOV | #07EH,001H | 7FH | 7EH | - | - |
| MOV | #0FDH,@R0 | 7FH | 7EH | - | FDH |
| MOV | #0FCH,@R1 | 7FH | 7EH | FCH | FDH |
| MOV | #0FBH,@R0 | 7FH | 7EH | FCH | FBH |
| MOV | #0FAH,@R1 | 7FH | 7EH | FAH | FBH |
| MOV | #0F9H,@R0 | 7FH | 7EH | FAH | F9H |
| MOV | #0F8H,@R1 | 7FH | 7EH | F8H | F9H |

Example 2:

|  |  | RAM 02H | RAM 03H | ACC 100H | B 102H |
| --- | --- | --- | --- | --- | --- |
| MOV | #000H,002H | 00H | - | - | - |
| MOV | #002H,003H | 00H | 02H | - | - |
| MOV | #0FDH,@R2 | 00H | 02H | FDH | - |
| MOV | #0FCH,@R3 | 00H | 02H | FDH | FCH |
| MOV | #0FBH,@R2 | 00H | 02H | FBH | FCH |
| MOV | #0FAH,@R3 | 00H | 02H | FBH | FAH |

LDC  (Load code byte relative to TRR to accumulator)

| Instruction code | 1 1 0 0 0 0 0 1 C1H |
| --- | --- |
| Number of bytes | 1 |
| Number of cycles | 2 |
| Function | (ACC)(BNK)((TRR)+(ACC)) [ROM] |
| Affected flags |  |
| Interrupt acceptance | Permitted at 2nd cycle |

Description:

This instruction transfers contents of the address in program memory (ROM) that is specified by the sum of the contents of the Table Reference Register (TRR) and the contents of the Accumulator (ACC), to the ACC. The ROM data that is referenced during internal program operation and external program operation differs. During internal program operation, internal ROM is referenced; during external program operation, BANK0 of external ROM is referenced.

The LDC instruction cannot reference BANK1 of external ROM.

Example:

|  |  | ACC | TRR |  | TRR +ACC |
| --- | --- | --- | --- | --- | --- |
|  |  |  | TRH | TRL |  |
| MOV | #001H,TRH | - | 01H | - | - |
| MOV | #023H,TRL | - | 01H | 23H | - |
| MOV | #000H,ACC | 00H | 01H | 23H | 0123H |
| LDC |  | 30H | 01H | 23H | 0153H |
| MOV | #001H,ACC | 01H | 01H | 23H | 0124H |
| LDC |  | FFH | 01H | 23H | 0222H |
| MOV | #002H,ACC | 02H | 01H | 23H | 0125H |
| LDC |  | 57H | 01H | 23H | 017AH |
| MOV | #003H,ACC | 03H | 01H | 23H | 0126H |
| LDC |  | EAH | 01H | 23H | 020DH |
|  |  | PC | ROM |  |  |
|  |  | 0123H | 30H |  |  |
|  |  | 0124H | FFH |  |  |
|  |  | 0125H | 57H |  |  |
|  |  | 0126H | EAH |  |  |

PUSH  d9  (Push direct byte to stack)

| Instruction code | 0 1 1 0 0 0 0d8 d7d6d5d4d3d2d1d0 60H61H |
| --- | --- |
| Number of bytes | 2 |
| Number of cycles | 2 |
| Function | (SP)(SP)+1,((SP))(d9) |
| Affected flags |  |
| Interrupt acceptance | Permitted at 2nd cycle |

Description:

This instruction increments the Stack Pointer (SP), and then transfers the contents of data memory (RAM) or a Special Function Register (SFR), as specified by d8 to d0, to the address in RAM specified by the SP.

Example:

|  |  | ACC | B | RAM 00H | SP | RAM 20H | RAM 21H | RAM 22H |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MOV | #0AAH,ACC | AAH | - | - | - | - | - | - |
| MOV | #055H,B | AAH | 55H | - | - | - | - | - |
| MOV | #012H,000H | AAH | 55H | 12H | - | - | - | - |
| MOV | #01FH,SP | AAH | 55H | 12H | 1FH | - | - | - |
| PUSH | ACC | AAH | 55H | 12H | 20H | AAH | - | - |
| PUSH | B | AAH | 55H | 12H | 21H | AAH | 55H | - |
| PUSH | 000H | AAH | 55H | 12H | 22H | AAH | 55H | 12H |
| POP | B | AAH | 12H | 12H | 21H | AAH | 55H | 12H |
| POP | ACC | 55H | 12H | 12H | 20H | AAH | 55H | 12H |
| POP | 000H | 55H | 12H | AAH | 1FH | AAH | 55H | 12H |

POP  d9  (Pop direct byte from stack)

| Instruction code | 0 1 1 1 0 0 0d8 d7d6d5d4d3d2d1d0 70H71H |
| --- | --- |
| Number of bytes | 2 |
| Number of cycles | 2 |
| Function | (d9)((SP)),(SP)(SP) - 1 |
| Affected flags |  |
| Interrupt acceptance | Permitted at 2nd cycle |

Description:

This instruction transfers the contents of the address in RAM specified by the Stack Pointer (SP) to data memory (RAM) or a Special Function Register (SFR), as specified by d8 to d0, and then decrements the SP.

Example:

|  |  | ACC | B | TRL | SP | RAM 20H | RAM 21H | RAM 22H |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MOV | #0AAH,ACC | AAH | - | - | - | - | - | - |
| MOV | #055H,B | AAH | 55H | - | - | - | - | - |
| MOV | #012H,TRL | AAH | 55H | 12H | - | - | - | - |
| MOV | #01FH,SP | AAH | 55H | 12H | 1FH | - | - | - |
| PUSH | ACC | AAH | 55H | 12H | 20H | AAH | - | - |
| PUSH | B | AAH | 55H | 12H | 21H | AAH | 55H | - |
| PUSH | TRL | AAH | 55H | 12H | 22H | AAH | 55H | 12H |
| POP | B | AAH | 12H | 12H | 21H | AAH | 55H | 12H |
| POP | ACC | 55H | 12H | 12H | 20H | AAH | 55H | 12H |
| POP | TRL | 55H | 12H | AAH | 1FH | AAH | 55H | 12H |

XCH  d9  (Exchange direct byte with accumulator)

| Instruction code | 1 1 0 0 0 0 1d8 d7d6d5d4d3d2d1d0 C2HC3H |
| --- | --- |
| Number of bytes | 2 |
| Number of cycles | 1 |
| Function | (ACC)(d9) |
| Affected flags |  |
| Interrupt acceptance | Permitted |

Description:

This instruction exchanges the contents of the Accumulator (ACC) with the contents of data memory (RAM) or a Special Function Register (SFR), as specified by d8 to d0.

Example 1:

|  |  | ACC | RAM 23H |
| --- | --- | --- | --- |
| MOV | #0FFH,ACC | FFH | - |
| MOV | #055H,023H | FFH | 55H |
| XCH | 023H | 55H | FFH |
| XCH | 023H | FFH | 55H |
| XCH | 023H | 55H | FFH |
| XCH | 023H | FFH | 55H |

Example 2:

|  |  | ACC | B |
| --- | --- | --- | --- |
| MOV | #0FFH,ACC | FFH | - |
| MOV | #0FEH,B | FFH | FEH |
| XCH | B | FEH | FFH |
| XCH | B | FFH | FEH |
| XCH | B | FEH | FFH |
| XCH | B | FFH | FEH |

XCH  @Rj  (Exchange indirect byte with accumulator)

| Instruction code | 1 1 0 0 0 1j1j0 C4HC7H |
| --- | --- |
| Number of bytes | 1 |
| Number of cycles | 1 |
| Function | (ACC)((Rj)) j=0,1,2,3 |
| Affected flags |  |
| Interrupt acceptance | Permitted |

Description:

This instruction exchanges the contents of the Accumulator (ACC) with the contents of data memory (RAM) or a Special Function Register (SFR), as specified by the indirect address register that is specified by j1 and j0.

Example 1:

|  |  | ACC | RAM 01H | RAM 68H |
| --- | --- | --- | --- | --- |
| MOV | #0FFH,ACC | FFH | - | - |
| MOV | #068H,001H | FFH | 68H | - |
| MOV | #0F0H,@R1 | FFH | 68H | F0H |
| XCH | @R1 | F0H | 68H | FFH |
| XCH | @R1 | FFH | 68H | F0H |
| XCH | @R1 | F0H | 68H | FFH |
| XCH | @R1 | FFH | 68H | F0H |

Example 2:

|  |  | ACC | RAM 03H | TRL |
| --- | --- | --- | --- | --- |
| MOV | #0AAH,ACC | AAH | - | - |
| MOV | #004H,003H | AAH | 04H | - |
| MOV | #055H,@R3 | AAH | 04H | 55H |
| XCH | @R3 | 55H | 04H | AAH |
| XCH | @R3 | AAH | 04H | 55H |
| XCH | @R3 | 55H | 04H | AAH |