Chapter 5.
Instructions

- Instruction Overview

The POTATO instruction set includes 70 instructions.

These encompass 45 opcodes, which are grouped into the following eight types of functions:

- Arithmetic operation instructions	ADD,ADDC,SUB,SUBC,INC,DEC,MUL,DIV

- Boolean operation instructions	AND,OR,XOR,ROL,ROLC,ROR,RORC

- Data transfer instructions	LD,ST,MOV,LDC,PUSH,POP,XCH

- Jump instructions	JMP,JMPF,BR,BRF

- Conditional branching instructions	BZ,BNZ,BP,BPC,BN,DBNZ,BE,BNE

- Subroutine instructions	CALL,CALLF,CALLR,RET,RETI

- Bit manipulation instructions	CLR1,SET1,NOT1

- Miscellaneous instruction	NOP

- Macro instruction	CHANGE

    - Arithmetic Operation Instructions

The arithmetic operation instructions primarily use the accumulator, and include the four basic arithmetic operations as well as increment and decrement. The results of one of the four basic arithmetic operations are set in CY, AC, and OV.

- CY (Carry Flag)

| Operation instruction | Operation result | CY |
| --- | --- | --- |
| When an addition instruction was executed | When a carry is generated from bit 7 (MSB) | 1 |
|  | When no carry is generated from bit 7 (MSB) | 0 |
| When a subtraction or compare instruction was executed | When a borrow is required for bit 7 (MSB) | 1 |
|  | When a borrow is not required for bit 7 (MSB) | 0 |
| When a multiplication instruction was executed | － | 0 |

- AC (auxiliary carry flag)

| Operation instruction | Operation result | AC |
| --- | --- | --- |
| When an addition instruction was executed | When a carry is generated from bit 3 | 1 |
|  | When no carry is generated from bit 3 | 0 |
| When a subtraction instruction was executed | When a borrow is required for bit 3 | 1 |
|  | When a borrow is not required for bit 3 | 0 |

- OV (Overflow flag)

| Operation instruction | Operation result | OV |
| --- | --- | --- |
| When an addition or multiplication instruction was executed | When a carry is generated from bit 7, and no carry is generated from bit 6 | 1 |
|  | When a carry is generated from bit 6, and no carry is generated from bit 7 | 1 |
|  | When an overflow error was generated while executing an addition or subtraction instruction involving signed variables | 1 |
|  | All other cases | 0 |
| When a multiplication instruction was executed | When the product is 256 or higher | 1 |
|  | When the product is 255 or lower | 0 |
| When a division instruction was executed | When an attempt was made to divide by zero | 1 |
|  | When dividing by any other number | 0 |

    - Logical Operation Instructions

The Boolean operation instructions are used to perform Boolean operations and to rotate bits. CY is also affected after executing the RORC or ROLC instruction.

    - Data Transfer Instructions

The data transfer instructions are used to write, read, save and replace data in data memory (RAM), the Special Function Registers (SFR), external data ROM, and external RAM.

    - Jump Instructions

Jump instructions unconditionally transfer control to the target instruction.

    - Conditional Branching Instructions

A conditional branching instruction determines whether a condition that is specified by the instruction is met (true) or not (false), and then, if the evaluation is "true," branches to the target address. If the evaluation is "false," the instruction does not branch; instead, execution continues with the next instruction.

The BE and BNE instructions branch on the basis of a comparison of two 8-bit data bytes. CY is set or reset by these instructions, according to the results of the comparison.

|  | Operand |  |  | Carry flag (CY) |
| --- | --- | --- | --- | --- |
|  | #i8,r8 | d9,r8 | @Rj,#i8,r8 |  |
| Relationship | #i8>(ACC) | (d9)>(ACC) | #i8>((Rj)) | 1 |
|  | #i8=(ACC) | (d9)=(ACC) | #i8=((Rj)) | 0 |
|  | #i8<(ACC) | (d9)<(ACC) | #i8<((Rj)) | 0 |

    - Subroutine Instructions

Subroutine instructions branch unconditionally and are used to transfer control to the target instruction. The address of the instruction is stored in the stack so that, after branching a return instruction (RET, RETI) can be used to return control to the instruction that follows the CALL instruction. The stack is located in data memory (RAM), and is pointed at by the Stack Pointer (SP). it is necessary to allocate an area in RAM for use by the stack according to the nesting level of the subroutine.

    - Bit Manipulation Instructions

The bit manipulation instructions are used to manipulate individual bits in specified contents of data memory (RAM) or Special Function Registers (SFR).

    - Miscellaneous Instruction

- The NOP instruction consumes one machine cycle without doing anything.

    - Macro Instruction

This is POTATO's own standard macro instruction. This macro instruction switches between internal program and external program execution.

    - Addressing

There are several addressing methods that are used for addressing program memory (ROM), data memory (RAM), and the Special Function Registers (SFR).

    - Program Memory (ROM) Addressing

Jump instructions, branching instructions, and subroutine instructions specify the destination address in program ROM as part of the instruction code. In this case, the address is specified by one of the following addressing methods:

- r8 (8-bit relative addressing)

This form of addressing permits jumps (branching) to an address within -128 to +127 addresses of the starting address of the next instruction that follows the instruction that is currently being executed. The jump is expressed through signed 8-bit data.

[80H to 7FH: -128 to +127]

- r16 (16-bit relative addressing)

This form of addressing permits jumps anywhere within the 64K program ROM space.

The address is expressed through unsigned 16-bit data.

[0000H to FFFFH: +0 to +65535]

- a12 (12-bit relative addressing)

This form of addressing leaves as is the bits PC15 through PC12 (which represent the current page) of the starting address (represented by PC15 through PC00) of the next instruction that follows the instruction that is currently being executed, and replaces the bits PC11 through PC00 with 12-bit addressing data [000H to FFFH].

This form of addressing permits jumps within a page (PC15 to PC12).

Note:

- Note that, in the above instance, the "current page" will be different from the page where the JMP instruction or CALL instruction is located if the JMP instruction or CALL instruction is located at the end of a page.

- a16 (16-bit absolute addressing)

This form of addressing permits jumps anywhere within the 64K program ROM space.

The address is expressed through 16-bit data.

[0000H to FFFFH: 0 to 65535]

- Table jumps

It is possible to execute a jump by setting the destination address in the stack, and then forcibly loading that value into the Program Counter (PC) by means of the RET instruction.

Refer to example 1. In line 1, the Stack Pointer (SP) is set to 09H.

If the RET instruction is executed at this point, a jump will be executed to the address where address 08H in RAM is the upper byte and address 07H in RAM is the lower byte. Therefore, the jump destination address is set in lines 2 and 3.

Because the jump destination in this example is PC = 0C13H, line 2 sets the lower byte, 13H, and line 3 sets the upper byte, 0CH. When the RET instruction in line 4 is executed, the SP is set to 07H and control jumps to 0C13H. However, because Example 1 requires the SP value to be known ahead of time, normally a PUSH instruction is used as shown in Example 2.

Example 1:

MOV	#09H,SP

MOV	#13H,07H

MOV	#0CH,08H

RET

Example 2:

MOV	#13H,ACC

PUSH	ACC

MOV	#0CH,ACC

PUSH	ACC

RET

In Example 3, the program branches to one of 128 addresses, ranging from 00H to 7FH, depending on the data in address 70H in RAM

Lines 1 and 2 set the lower byte of the address, while line 4 sets the upper byte of the address. When the RET instruction in the line 6 is executed, the program branches to the jump table in lines 7 and 8, and then jumps to the branching destination indicated in those lines.

This technique is called a "table jump," and is an effective tool for branching to multiple addresses conditionally.

Example 3:

A0:	LD	070H

ROL

ADD	#LOW(A1)

PUSH	ACC

MOV	#HIGH(A1),ACC

PUSH	ACC

RET

;

ORG	0C00H

A1:	JMP	B00	     Jump table



JMP	B7F

;

B00:	XXXXXX

    - Data Memory (RAM) and Special Function Register (SFR) Addressing

- d9 (direct addressing)

This form of addressing directly specifies an address in RAM or an SFR with 9 bits (d8 through d0).

Addresses 000H through 0FFH	············	Specifies RAM.

Addresses 100H through 1FFH	············	Specifies SFR.

- b3 (bit addressing)

This form of addressing uses 3-bit bit addressing data in combination with d9 (direct addressing) to specify a specific bit in RAM or SFR.

|  | MSB |  |  |  |  |  |  | LSB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 |
| Bit addressing data ------- | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
|  | (111) | (110) | (101) | (100) | (011) | (010) | (001) | (000) |

- @Rj(indirect addressing)

Indirect addressing specifies an address in RAM or SFR by setting in a specific address in RAM (called the "indirect address register") the address in RAM or SFR that is actually to be accessed, and then accessing the indirect address register. The indirect address registers are labelled @R0, @R1, @R2, and @R3. A specific indirect address register (one of the four from @R0 to @R3) is specified through the 2-bit indirect addressing data (j1 and j0). As shown in the table below, four indirect address banks are allocated in the first 16 bytes (00H through 0FH) of each RAM bank. The RAM bank is set by RAMBK0 (bit 1 of the PSW). The indirect address register bank is set by IRBK1, 0 (bits 4 and 3 of the PSW). Note that when executing an indirect addressing instruction, RAM in the RAM bank that is set by IRBK1 and 0 and by RAMBK0 is used for the indirect address register and the RAM that is specified by the indirect address register. During a reset, both IRBK0 and IRBK1 are set to "0" and RAMBK0 is set to "0" as well, so the respective absolute addresses of @R0, @R1, @R2, and @R3 are 00H, 01H, 02H, and 03H in RAM bank 0.

Indirect address registers ······················ @R3, @R2, @R1, and @R0

Indirect addressing data (j1, j0) ············ (11) (10) (01) (00)

Indirect Addressing Register Map

| Indirect address register name | Function | Bank 0 (IRBK1=0) (IRBK0=0) | Bank 1 (IRBK1=0) (IRBK0=1) | Bank 2 (IRBK1=1) (IRBK0=0) | Bank 3 (IRBK1=1) (IRBK0=1) |
| --- | --- | --- | --- | --- | --- |
| @R0 | RAM access | RAM 00H | RAM 04H | RAM 08H | RAM 0CH |
| @R1 | RAM access | RAM 01H | RAM 05H | RAM 09H | RAM 0DH |
| @R2 | SFR access | RAM 02H | RAM 06H | RAM 0AH | RAM 0EH |
| @R3 | SFR access | RAM 03H | RAM 07H | RAM 0BH | RAM 0FH |

- Example of using indirect addressing

This example illustrates an operation using the indirect address register.

Refer to Example 1. Line 2 sets the immediate data 68H in RAM 9at address 00H). If RAM (address 00H) is used as an indirect address register, address 68H in RAM will be accessed. For example, line 3 sets the immediate data 10H in the address that is specified by the indirect address register (@R0), which is address 68H in RAM.

Line 5 adds the contents of the address that is specified by the indirect address register (@R0), which is address 68H in RAM, and the Accumulator (ACC).

Example 1:

MOV	#055H,ACC

MOV	#068H,00H

MOV	#010H,@R0

ADD	#015H

ADD	@R0

In the following example, an SFR is specified through indirect addressing.

Refer to Example 2. Lines 1 and 2 clear bits 4 and 3 of the PSW and set up RAM addresses 00H through 03H as the indirect address registers. Line 4 sets the immediate data 02H in address 02H in RAM. If address 02H in RAM is used as the indirect address register, address 02H in RAM is accessed. For example, in line 5 the immediate data 12H is set in the SFR (address 02H: B register) that is specified by the indirect address register @R2.

Line 6 increments the B register, which is accessed through indirect addressing again.

Example 2:

CLR1	PSW,4

CLR1	PSW,3

MOV	#0ACH,ACC

MOV	#002H,02H

MOV	#012H,@R2

INC	@R2

The following example changes the bank through the PSW and then specifies an SFR through indirect addressing.

Refer to Example 3. Lines 1 and 2 set the bank to "2" in the PSW and set up RAM addresses 08H through 0BH as the indirect address registers. Line 4 sets the immediate data 02H in address 0BH in RAM. If address 0BH in RAM is used as the indirect address register, address 02H in RAM is accessed. For example, in line 5 the immediate data 12H is set in the SFR (address 02H: B register) that is specified by the indirect address register @R2.

Line 6 increments the B register, which is accessed through indirect addressing again.

Example 3:

SET1	PSW,4

CLR1	PSW,3

MOV	#0ACH,ACC

MOV	#002H,0BH

MOV	#012H,@R2

INC	@R2