  - Base Timer

    - Overview

POTATO's built-in base timer is a 14-bit binary up-counter that has the four functions listed below

- Clock timer

- 14-bit binary up counter

- Fast forward mode (when using a 6-bit base timer)

    - Function

- Clock timer

When the 32.768Khz sub-clock is used for the count clock for the base timer, time can be measured in 0.5-second intervals. The input signal is used to specify "sub clock" as the count clock for the base timer.

- 14-bit binary up counter

The 8-bit binary up counter and the 6-bit binary up counter can be used as a 14-bit binary up counter. These counters can be cleared by software.

- Fast forward mode (when using the 6-bit base timer)

If the 6-bit base timer is used, time can be measured in approximately 2ms intervals when the 32.768KHz sub-clock is used for the count clock. The bit length can be selected through the Base Timer Control Register (BTCR).

- Interrupt generation

When the interrupt request enable bit is set and an interrupt request is generated from the base timer, an interrupt request to vector address 001BH is generated. There are two types of interrupt requests from the base timer: "base timer interrupt 0" and "base timer interrupt 1."

In order to control the base timer, it is necessary to manipulate the following Special Function Registers:

	BTCR		ISL

	P1DDR		P1

	P1FCR
	Timer 0-related		Interrupt related

    - Circuit Configuration

The base timer configuration is shown in Fig. 3-4-1.

- 8-bit binary up counter ····· (1)

The input for this up counter is the signal selected by the Input Signal Select register (ISL).

This counter creates the 4KHz/2KHz buzzer output signal, and generates the base timer interrupt 1 source. The overflow event of this timer serves as a clock for the 6-bit binary counter.

- 6-bit binary up counter ·····(2)

The input for this 6-bit up counter is the signal selected by the ISL Special Function Register, or the overflow signal from the 8-bit counter. This counter generates the base timer interrupt 0 and 1 sources. Switching the input clock is handled through the Base Timer Control Register (BTCR).

- Base timer input clock source ·····(3)

The base timer input clock is selected through the Input Signal Select register (ISL) from among " cycle clock," "timer 0 prescaler," or "sub-clock."

Fig. 3-4-1.  Base Timer Block Diagram

    - Related Registers

- Base Timer Control register (BTCR)

| Symbol | Address | R/W | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BTCR | 17FH | R/W | BTCR7 | BTCR6 | BTCR5 | BTCR4 | BTCR3 | BTCR2 | BTCR1 | BTCR0 |
| After reset |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

*	In the VMS, game software is prohibited from accessing BTVR7, BTCR6, and BTCR0. Bit manipulation instructions must always be used when accessing this register.

| Bit name | Function |  |  |  |
| --- | --- | --- | --- | --- |
| BTCR7 (bit 7) | Base timer interrupt 0 cycle control |  |  |  |
|  | 0: 16384/fBST |  |  |  |
| BTCR6 (bit 6) | Base timer operation control |  |  |  |
|  | 1: Base timer operation start |  |  |  |
| BTCR5 (bit 5) | Base timer interrupt 1 cycle control |  |  |  |
| BTCR4 (bit 4) | BTCR7 | BTCR5 | BTCR4 |  |
|  | x x 0 0 | 0 0 1 1 | 0 1 0 1 | 32/fBST 128/fBST 512/fBST 2048/fBST |
| BTCR3 (bit 3) | Base timer interrupt 1 source |  |  |  |
|  | 0: No interrupt source 1: Interrupt source |  |  |  |
| BTCR2 (bit 2) | Base timer interrupt 1 request enable control |  |  |  |
|  | 0: Interrupt request disable 1: Interrupt request enable |  |  |  |
| BTCR1 (bit 1) | Base timer interrupt 0 source |  |  |  |
|  | 0: No interrupt source 1: Interrupt source |  |  |  |
| BTCR0 (bit 0) | Base timer interrupt 0 request enable control |  |  |  |
|  | 1: Interrupt source |  |  |  |

BTCR7 (bit 7):	Base timer interrupt 0 cycle control	Fixed at "0"

This bit selects either 64/fBST (1) or 16384f/BST (0) as the base timer interrupt 0 source generation cycle. When this bit is set to "0," the interrupt 0 source is generated by an overflow in the 14-bit counter, and the interrupt source generation interval is 16384/fBST. When using fast forward mode, set "1."

BTVR6 (bit 6):	Base timer operation control	Fixed at "1"

This bit starts (1)/stops (0) the base timer counting operation. When this bit is set to "1," the 14-bit counter counts up; when this bit is set to "0," the 14-bit counter is cleared and then stopped.

BTCR5 (bit 5):	Base timer interrupt 1 cycle control

BTCR4 (bit 4):	These bits select the base timer interrupt 1 source generation cycle.

| BTCR7 | BTCR5 | BTCR4 | Base timer interrupt 1 cycle |
| --- | --- | --- | --- |
| x | 0 | 0 | 32/fBST |
| x | 0 | 1 | 128/fBST |
| 0 | 1 | 0 | 512/fBST |
| 0 | 1 | 1 | 2048/fBST |

fBST: Input clock frequency

BTCR3 (bit 3):	Base timer interrupt 1 source flag

This bit is set at each interval when the base timer 1 source is generated according to the settings of BTCR7, 5, and 4, and does not change when the source is not generated. Therefore, this bit must be reset by software.

BTCR2 (bit 2):	Base timer interrupt 1 request enable control

This bit enables (1)/disables (0) interrupt requests through base timer interrupt 1. If this bit is set to "1," then when the base timer 1 interrupt source is generated an interrupt request to vector address 001BH is generated; if this bit is set to "0," then no interrupt requests are generated.

BTCR1 (bit 1):	Base timer interrupt 0 source flag

This bit is set at each interval when the base timer 0 source is generated according to the setting of BTCR7, and does not change when the source is not generated. Therefore, this bit must be reset by software.

BTCR0 (bit 0):	Base timer interrupt 0 request enable control	Fixed at "0"

This bit enables (1)/disables (0) interrupt requests through base timer interrupt 0. If this bit is set to "1," then when the base timer 0 interrupt source is generated an interrupt request to vector address 001BH is generated; if this bit is set to "0," then no interrupt requests are generated.

Notes:

- When BTCR7 and 5 = 1 (fast forward mode), do not select both the system clock and the base time clock simultaneously.

- When overwriting BTCR5 and 4, note that BTCR3 may be set to "1" as a result.

- If either the cycle clock or the sub-clock was selected as the base timer clock source, and then HOLD mode is set while the base timer is still running, then the base timer might miscount due to the effects of unstable oscillation that initially occurs when the main clock and sub-clock start to oscillate after HOLD mode is cancelled. When entering HOLD mode, therefore, stopping the base timer is recommended.

- Input Signal Select register (ISL)

For details, refer to Chapter 3, section 3.2.4, "Input Signal Select Register."

| Symbol | Address | R/W | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ISL | 15FH | R/W | - | - | ISL5 | ISL4 | ISL3 | ISL2 | ISL1 | ISL0 |
| After reset |  |  | H | H | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit name | Function |  |  |
| --- | --- | --- | --- |
| ISL5 (bit 5) | Base timer clock selection |  |  |
| ISL4 (bit 4) | ISL5 | ISL4 |  |
|  | X | 0 | Sub-clock (quartz oscillation) fixed |
| ISL3 (bit 3) | Buzzer output frequency selection |  |  |
|  | 0: fBST/16 fixed |  |  |
| ISL2 (bit 2) | Noise elimination filter time constant selection |  |  |
| ISL1 (bit 1) | ISL2 | ISL1 | Time constant |
|  | 1 0 X | 1 1 0 | 16Tcyc 64Tcyc 1Tcyc |
| ISL0 (bit 0) | T0 clock input pin selection |  |  |
|  | 0: P72/INT2/T0IN pin 1: P73/INT3/T0IN pin |  |  |

    - Using the Base Timer

- Clock timer