# 2

Chapter 7.

List File Format

The list file generated by M86K has the format shown below. Basically, this format shows the contents of the source file as is, with line numbers and machine language codes listed on the left. The placement of the lines and columns was designed with the format of the printout in mind. In short, one page consists of 60 lines (including the header), and one line consists of 132 columns. (If the source line is longer than 132 characters, it wraps around.) On the left side, the column positions are fixed so that various types of information can be displayed. Horizontal tabs that are included in the source lines are converted into spaces so that there is no visible change in positioning.

Page number           Source file name                                   Statement

Line number

Include level

Segment type

Header

page:    1 <sample.ASM>

ERR  SEQ.     S LOC. OBJ.   SOURCE STATEMENTS

0001                     ; a sample for source listing

0002

0003	chip	LC866032

0004

0005	include	macros.asm

1/0001                     ; a header file that contains shared macro definitions

1/0002

1/0003                     clr_reg	macro	xxx

1/0004	mov	#0, xxx

1/0005	endm

Symbol value

0006

0007	cseg

0008          0000007B PARAM1	equ	123

Expanded line number

0009

0010	clr_reg acc

0010+1   C 0000 230000	mov 	#0, acc

0011     C 0003 23407B	mov 	#PARAM1, p0

0012

Address

0013		rept		10

0015		endm

0015+1   C 0006 00		nop

0015+2   C 0007 00		nop

0015+3   C 0008 00		nop

0015+4   C 0009 00		nop

0015+5   C 000A 00		nop

0015+6   C 000B 00		nop

0015+7   C 000C 00		nop

0015+8   C 000D 00		nop

0015+9   C 000E 00		nop

Code

0015+10  C 000F 00		nop

0016

0017     C 0010 73616D mess0:	dc		“sample message #00ｫn”

0017     C 0013 706C65

0017     C 0016 206D65

0017     C 0019 737361

0017     C 001C 676520

0017     C 001F 233030

0017     C 0022 0A

0018

0019	end

Header:	The header appears at the start of each page, and consists of a blank line to allow space for binding, the page number, the source file name, and headings for each column position in the main body of the list.

Page number:	Pages are numbered sequentially, starting from "1."

Source file name:	This indicates the name of the source file for which assembly was specified. If the drive name and path name were also specified when the file was specified, they are shown as well.

Statement:	Indicates the contents of the source file, the results of expansion of a macro call when list output was not suppressed, and the results of expansion when include files are loaded.

Line number:	These are the line numbers in the source file (in decimal format).  When a single line in the source file becomes two or more lines in the list file (for example, when the code portion spans several lines), the same line number is repeated.

Include level:	This indicates the nesting level of nested include files. This does not appear on lines that show the contents of the source file. This level is "1" for lines that show the contents of a file that is included directly from the source file. This digit becomes "2" for a file that is included from a level 1 include file. This number is separated from the line number by a slash.

Symbol value:	When a value is set in a symbol, if the value is definite at the time of assembly, that value is shown as an 8-digit hexadecimal number.  If the value is not definite, nothing is shown.

Expanded line number:	This number indicates that a given line was not in the original source file, but was generated as a result of a macro call (including repeated macros). A sequential number, starting from "1," is added to the line number in the sequence of expansion. When one macro call is completed and a different one appears, the sequential numbers that are assigned start again from "1."

Segment type:	When the corresponding line generates code in CSEG or DSEG, the segment type is indicated by a single character. An upper-case "C" indicates "CSEG INBLOCK," a lower-case "c" indicates "CSEG FREE," and "D" indicates DSEG.

Address:	When the corresponding line generates code in CSEG or DSEG, the address where the first byte of the code is located is indicated with a four-digit hexadecimal number. Note that "address" as used here refers to the offset from the start of the segment in question.

Code:	When the code that is generated as a result of assembling the source line should be written in ROM, that code is depicted as two-digit hexadecimal codes, sufficient for the required number of bytes. A maximum of three bytes of codes are displayed for one line, with the two digits the farthest to the left representing the bytes corresponding to the smallest address. If the length of the code exceeds three bytes, the code is displayed on a newly generated line that has the same line number.