# 2

  - EQU  (Assign value)

symbolname  EQU  expression

The EQU pseudo instruction assigns the value expression to symbolname. A symbol that has been defined by using the EQU pseudo instruction cannot be defined again. Using the EQU pseudo instruction effectively makes it possible to add visual meaning to constant data, which improves the efficiency with which maintenance work can be performed.

Example:

":" is not described between the symbol for which the value is being defined and "EQU".

When the defined value can be calculated,

that value is shown (in hexadecimal).

Any expression can be described.

page:    1 <equ.ASM>

ERR	SEQ.	S LOC. OBJ.	SOURCE STATEMENTS

0001			; a sample program for EQU

0002				chip	lc866032

0003

0004		00000064 	loop_max	equ	100

0005		00000001 	mode_a			equ	1

0006		00000002 	mode_b			equ	2

0007		00000003 	mode_c			equ	3

0008

0009				cseg	inblock

0010	C 0000 220000’		mov	#00, loop_ctr

0011

0012	C 0003 230201	label1:	mov	#mode_a, b

0013	C 0006 0818’		call	sub1

0014	C 0008 230202		mov	#mode_b, b

0015	C 000B 0818’		call	sub1

0016	C 000D 230303		mov	#mode_c, c

0017	C 0010 6200’		inc	loop_ctr

0018	C 0012 0200’		ld	loop_ctr

0019	C 0014 4164EC		bne	#loop_max, label1

0020	C 0017 A0		ret

0021

0022	C 0018 0302	sub1:	ld	b

0023	C 001A 310107		be	#mode_a, suj10

0024	C 001D 310208		be	#mode_b, suj11

0025	C 0020 310309		be	#mode_c, suj12

0026	C 0023 A0 	suj0:	ret

0027

0028	C 0024 1201’	suj10:	st	data_a

0029	C 0026 01FB		br	suj0

0030	C 0028 1202’	suj11:	st	data_b

0031	C 002A 01F7		br	suj0

0032	C 002C 1203’	suj12:	st	data_c

0033	C 002E 01F3		br	suj0

0034

0035				dseg

0036	D 0000	loop_ctr:	ds	1

0037	D 0001	data_a: 	ds	1

0038	D 0002	data_b: 	ds	1

0039	D 0003	data_c:	ds	1

0040

0041				end

  - SET  (Assign temporary value)

symbolname  SET  expression

The SET pseudo instruction assigns the value expression to symbolname. A symbol that has been defined by using the SET pseudo instruction can be defined again with the SET instruction. A symbol that has been defined by this pseudo instruction cannot be declared in a PUBLIC pseudo instruction or defined again by using the EQU instruction.

Example:

When the defined value can be calculated,

that value is shown (in hexadecimal).

":" is not described between the symbol for which the value is being defined and "SET".

page:    1 <set.ASM>

ERR  SEQ.     S LOC. OBJ.     SOURCE STATEMENTS

0001		; a sample program for SEY

0002			chip	lc866032

0003			cseg	inblock

0004

0005              00000000 dd      set     0

0006

0007	C 0000 220000’		mov	#dd,zz+dd

0008

0009	C 0003 6300		inc	a

0010	C 0005 6302		inc	b

0011

0012              00000001 dd      set     dd+1

0013

0014	C 0007 220101’		mov	#dd,zz+dd

0015

0016	C 000A 7300		dec	a

0017	C 000C 7302		dec	b

0018

0019			dseg

0020	D 0000	zz:	ds	2

0021

0022			end

Any expression can be described, including the symbol for which the value is being defined.

  - DB  (Define byte data)

labelname  DB  expression {,expression}

The DB pseudo instruction stores 8-bit data that corresponds to the operand expression in program memory (ROM). More than one operand can be described; delimit each operand with a comma (","). When there are two or more operands, they are evaluated in order from left to right, and are stored in sequence in ascending addresses. If no operand is described between two commas, that operand is regarded as being "0".

Example:

page:    1 <db.ASM>

ERR  SEQ.     S LOC. OBJ.   SOURCE STATEMENTS

0001                     ;a sample program for DB

0002                                  chip    lc864032

0003          00001234 area        equ      1234h

0004                                  cseg

0005     C 0000 414243             db      ‘A’,’B’,’C’,0

0005     C 0003 00

V    0006     C 0004 34                  db      area

** Warning, value is out of range

0007     C 0005 12                  db      high(area)

0008     C 0006 34                  db      low(area)

0009                                  end

0x41

0x42

0x43

0x00

0x34

0x12

0x34

Regarding “db area” in the above example, because the value of the symbol area has a 16-bit width, a value is out of range error (warning level) will be generated during the assembly process. However, the value of the lower 8 bits will be output in the object code.

  - DW  (Define word data)

labelname  DW  expression {,expression}

The DW pseudo instruction is used to store 16-bit data that corresponds to the operand expression in program memory (ROM). The upper byte is stored first, and the lower byte is stored in the next address (the higher address). More than one operand can be described; delimit each operand with a comma (","). When there are two or more operands, they are stored in a continuous area.  If no operand is described between two commas, that operand is regarded as being "0".

Example:

page:    1 <dw.ASM>

ERR	SEQ.	S LOC. OBJ.	SOURCE STATEMENTS

0001			;a sample program for DW

0002				chip	lc864032

0003		00001234 area	equ	1234h

0004				cseg

0005	C 0000 004100		dw	‘A’,’B’,0

0005	C 0003 420000

0006	C 0006 1234		dw	area

0007	C 0008 0012		dw	high(area)

0008	C 000A 0034		dw	low(area)

0009				end

0x00

0x41

0x00

0x42

0x00

0x00

0x12

0x34

0x00

0x12

0x00

0x34

If an 8-bit value is allocated using the DW pseudo instruction, the upper 8 bits of the 16 bits are always comprised of zeroes.

  - DC  (Define character string data)

labelname  DC  “string”

The DC pseudo instruction stores the contents of string (a character string constant) as the ASCII code values of each character in sequence in program memory (ROM). For details on character string constants, refer to section 4.7.

Example:

73

61

6d

70

6c

65

20

6d

65

73

73

61

67

65

20

23

30

30

0a

cseg inblock 12

47

12

34

org	1234h

mess0: dc “sample message #00\n” 73

61

6d

70

6c

65

20

6d

65

73

73

61

67

65

20

23

30

30

00

mess1:	dc	“sample message #01\0”

table:	dw	mess0

dw	mess1

when this segment is allocated starting from address 0

  - DS  (Define byte area)

labelname  DS  absolute_expression

The DS pseudo instruction allocates an area consisting of the number of bytes specified by absolute_expression in data memory (RAM). The description of absolute_expression must be in absolute format (in which all of the values are determined). This pseudo instruction cannot be used unless it comes after the DSEG pseudo instruction.

Example:

page:    1 <ds.ASM>

ERR  	SEQ.     S LOC. OBJ.   	SOURCE STATEMENTS

0001		; a sample program for DS

0002			chip	lc864032

0003			dseg

0004	D 0000	area0:	ds	1

0005	D 0001	area1:	ds	2

0006			cseg	inblock

0007	C 0000 0200’	start:	ld	area0

0008 	C 0002 1201’		st	area1

0009	C 0004 1202’		st	area1+1

0010 			end

dseg

area0:

area1:

In the above example, a one-byte area is allocated under the name area0, and another two-byte area under the name area1 immediately after area0.

  - MACRO  (Define macro)

name  MACRO  parameter {, parameter}

The MACRO pseudo instruction defines macros. The main body of the macro definition is formed by all of the statements that follow the MACRO pseudo instruction until the ENDM pseudo instruction is reached. name calls the macro that was defined. Because name is required so that it can be replaced with the main body of the macro definition, it must be specified.  because parameters is a list of parameters, they should be specified in accordance with the contents of the macro that was defined.

Note that when calling other macros from within a macro, or when using a pseudo instruction, such as IFB, that requires "<" or ">", sufficient "<" and ">" are needed for the nesting level.

Example:

_push	macro

push	acc

Stores the registers acc, c, and b in the stack.

push	c

push	b

endm

_pop	macro

pop	b

Pops the values stored in the stack in the order b, c, and acc.

pop	c

pop	acc

endm

_shl    macro   count

ifne    count

rept    count

rolc

endm

else

.printx    “logical shift count is zero !!\007”

endif

endm

Generates the code for performing the number of left shifts specified by the parameters.

However, if the parameter is "0," no code for performing shifts is generated.

cseg

start:	_push

_shl	0

This is the description in the source program.

_shl	2

_shl	1

_pop

The result of assembly of the above source code is shown on the following page.

  - REPT  (Repeat macro)

REPT  count

The REPT pseudo instruction is used to repeat a series of statements from the REPT pseudo instruction up to the ENDM pseudo instruction the number of times specified by count. An integer from 1 to 65,535 (inclusive) can be specified for count.

Example:	Filing locations where there is no program code with NOP (when the boundary is at 256)

page:    1 <rept.ASM>

ERR	SEQ. 	S LOC. OBJ.	SOURCE STATEMENTS

0001		; a sample program for REPT

0002			chip	lc864024

0003			cseg	inblock

0004	C 0000 230000 	start:	mov	#0,acc

0005	C 0003 1200		st	00h

0006	C 0005 6300		inc	acc

The main body of the macro definition is not displayed.

0007	C 0007 1201		st	01h

0008	C 0009 6300		inc	acc

0009	C 000B 1202		st	02h

0010	C 000D A0	last:	ret

0011			rept	255-(last-start)

0013			endm

0013+1	C 000E 00		nop

0013+2	C 000F 00		nop

Expanded statement

0013+3	C 0010 00		nop

0013+240	C 00FD 00		nop

0013+241	C 00FE 00		nop

0013+242	C 00FF 00		nop

0014			end

  - IRP  (Continuous macro)

IRP  parameter, argument  {,argument }...

The IRP pseudo instruction repeats a series of statements from the IRP pseudo instruction up to the ENDM pseudo instruction the number of times specified by argument. During the repetition, each time that parameter appears in one of the statements, an item in the argument field is substituted for parameter, in order.

Example:

_push	macro

irp     reg_name,acc,b,psw,c

push    reg_name

endm

endm

_pop	macro

irp     reg_name,c,psw,b,acc

push    reg_name

endm

endm

0016

0017                           _push

0017+1                             irp     reg_name,acc,b,psw,c

0017+3                             endm

0017+1   C 0000 6100                   push    acc

0017+2   C 0002 6102                   push    b

0017+3   C 0004 6101                   push    psw

0017+4   C 0006 6103                   push    c

0018                           _pop

0018+1                             irp     reg_name,c,psw,b,acc

0018+3                             endm

0018+1   C 0008 6103                   push    c

0018+2   C 000A 6101                   push    psw

0018+3   C 000C 6102                   push    b

0018+4   C 000E 6100                   push    acc

  - IRPC  (Character string macro)

IRPC  parameter, string

The IRPC pseudo instruction repeats a series of statements from the IRPC pseudo instruction up to the ENDM pseudo instruction a number of times equal to the number of characters in string. Unlike character string constants, string is not enclosed in quotes, etc. In addition, it is not possible to input a code that begins with the yen symbol. During the repetition, each occurrence of parameter in one of the statements is replaced with a character in string; this replacement is repeated until all of the characters in string have been used.

Example:

; a sample program for IRPC      parameter

chip	lc866032

dseg

irpc	x,01234567          string

buf&x:	ds	2

endm

end

Each occurrence of parameter is replaced with a character from string

Delimiter when parameter occurs as part of an identifier

Expansion results

page:    1 <irpc.ASM>

ERR	SEQ.		S LOC. OBJ.	SOURCE STATEMENTS

0001			; a sample program for IRPC

0002				chip	lc866032

0003				dseg

0004				irpc	x,01234567

0006				endm

0006+1	D 0000	buf0:	ds	2

0006+2	D 0002 	buf1:	ds	2

0006+3	D 0004	buf2:	ds	2

0006+4	D 0006	buf3:	ds	2

0006+5	D 0008	buf4:	ds	2

0006+6	D 000A	buf5:	ds	2

0006+7	D 000C	buf6:	ds	2

0006+8	D 000E	buf7:	ds	2

0007			end

  - ENDM  (End macro definition)

ENDM

The ENDM pseudo instruction declares the end of a macro definition statement.

Example:

"macro" and "irp" are each paired with an "endm" pseudo instruction

Macro definition start

Macro definition start

_push	macro

irp     reg_name,<<acc,b,psw,c>>

push     reg_name

endm

endm

Macro definition end

Macro definition end

  - EXITM  (Interrupt macro expansion)

EXITM

The EXITM pseudo instruction interrupts macro expansion. This pseudo instruction is used in combination with conditional assembly pseudo instructions in order to obtain expansion results that differ according to the arguments that are given to identical macros.

Example:

page:    1 <exitm.ASM>

ERR  SEQ.     S LOC. OBJ.   SOURCE STATEMENTS

0001		; a sample program for EXITM

0002			chip	LC866032

0003		rpush	macro	a1,a2,a3,a4

0004			ifb	<<a1>>

Because one pair of "<>" is deleted during macro expansion, double symbols ("<<>>") are required.

0005 				.printx “not enough argument”

0006				exitm

0007			endif

0008			ifnb	<<a2>>

0009				push	a1

0010				push	a2

0011				push	a3

0012				push	a4

0013			endif

0014			endm

0015			cseg	inblock

0016			rpush	acc,b,psw,c

0016+1			ifb	<acc>

0016+2				.printx “not enough argument”

0016+3				exitm

0016+4			endif

0016+5			ifnb	<b>

0016+6	C 0000 6100			push	acc

0016+7	C 0002 6102			push	b

0016+8	C 0004 6101			push	psw

0016+9	C 0006 6103			push	c

0016+10			endif

0017 			rpush

0017+1			ifb	<>

0017+2				.printx “not enough argument”

0017+3				exitm

0018			end

Because the first argument is given,

this portion is assembled.

Because there is no second argument, this portion is expanded; when EXITM is recognized, expansion is halted.

  - LOCAL  (Define local label)

LOCAL  name {, name}

The LOCAL pseudo instruction is used to declare a label that can be used within a macro definition. If the name declared by the LOCAL pseudo instruction appears within a macro expansion, the macro assembler substitutes a new name for name that will not conflict with any other names.

Example:

; a sample program for LOCAL

chip	lc864008

b_ne	macro	val,dst

local	skip

be	val,skip

bro	dst

skip:

endm

cseg

b_ne	#0, over

org	200h

over:	b_ne	#0, under

nop

under:	nop

end

In the above example, the BRO pseudo instruction is used to define the BNEO macro instruction that automatically generates an instruction word according to the branching destination. The results of assembly are shown on the next page.

page:    1 <local.ASM>

ERR	SEQ.     S LOC. OBJ.	SOURCE STATEMENTS

0001		; a sample program for LOCAL

0002			chip	lc864008

0003		b_ne	macro	val,dst

0004 			local 	skip

0005			be 	val,skip

0006			bro	dst

0007		skip:

0008			endm

0009

0010			cseg

0011			b_ne	#0, over

0011+1			local	_L0000000L_

0011+2	C 0000 310003		be	#0,_L0000000L_

0011+3	C 0003 11FB01		bro	over

0011+4		_L0000000L_:

0012

0013			org	200h

0014		over:	b_ne	#0, under

0014+1			local	_L0000001L_

0014+2	C 0200 310002		be	#0,_L0000001L_

0014+3	C 0203 0101		bro	under

0014+4		_L0000001L_:

0015	C 0205 00		nop

0016	C 0206 00 	under:	nop

0017			end

The identifier declared by LOCAL is replaced by a unique name.

The name generated has the format_L######L_(where###### is a serisl number, starting with 000000）

  - IFDEF  (Assemble if defined)

IFDEF  symbol

The IFDEF pseudo instruction assembles the source program until either ELSE or ENDIF appears subsequently, but only if symbol has already been defined.

Example:

page:    1 <ifdef.ASM>

ERR	SEQ.     S LOC. OBJ.	SOURCE STATEMENTS

0001		; a sample program for IFDEF

0002			chip	lc864024

0003		00000001	abc	equ	1

0004			dseg

0005	D 0000	count:	ds	1

0006

0007			cseg	inblock

0008	C 0000 230010		mov 	#10h, acc

0009			ifdef	abc

0010	C 0003 8302			add	b

0011	C 0005 1200’			st	count

0012			else

0013				inc	acc

0014			endif

0015	C 0007 A303		sub	c

0016			ifdef	efg

0017				add 	count

0018			endif

0019 			end

Because efg is an undefined symbol, this portion is not assembled.

Because abc is a defined symbol, this portion is assembled.

  - IFNDEF  (Assemble if undefined)

IFNDEF  symbol

The IFNDEF pseudo instruction assembles the source program until either ELSE or ENDIF appears subsequently, but only if symbol has not been defined.

Example:

page:    1 <ifndef.ASM>

ERR  SEQ.     S LOC. OBJ.   	SOURCE STATEMENTS

0001		; a sample program for IFDEF

0002			chip	lc864024

0003		00000001	abc	equ	1

0004			dseg

0005	D 0000	count:	ds	1

0006

0007			cseg	inblock

0008	C 0000 230010		mov	#10h, acc

0009			ifndef	abc

0010				add	b

0011				st	count

0012			else

0013	C 0003 6300		inc	acc

0014			endif

0015	C 0005 A303		sub	c

0016			ifndef	efg

0017	C 0007 8200’			add	count

0018			endif

0019			end

Because efg is an undefined symbol, this portion is assembled.

Because abc is a defined symbol, this portion is not assembled.

  - IFB  (Assemble if operand is empty)

IFB  <argument>

The IFB pseudo instruction assembles the source program until either ELSE or ENDIF appears subsequently, but only if argument is empty. If the argument contains any space or tab characters, it is regarded as not being empty. argument must be enclosed in <>.

Example:

page:    1 <ifb.ASM>

ERR  SEQ.     S LOC. OBJ.	SOURCE STATEMENTS

0001		; a sample program for IFB

0002			chip	lc864016

0003		tifb	macro	arg

0004			ifb	<<arg>>

Because one pair of "<>" is deleted during macro expansion, double symbols ("<<>>") are required.

0005				inc	a

0006			else

0007				inc	b

0008			endif

0009			endm

0010

0011			tifb	xxx

0011+1			ifb	<xxx>

0011+2				inc	a

0011+3			else

0011+4	C 0000 6302			inc	b

0011+5			endif

0012			tifb

0012+1			ifb	<>

0012+2	C 0002 6300			inc	a

0012+3			else

0012+4				inc	b

0012+5			endif

0013			end

Because the argument for IFB is empty, this portion is assembled.

Because the argument for IFB is not empty, this portion is not assembled.

  - IFNB  (Assemble if operand is not empty)

IFNB  <argument>

The IFNB pseudo instruction assembles the source program until either ELSE or ENDIF appears subsequently, but only if argument is not empty. If the argument contains any space or tab characters, it is regarded as not being empty. argument must be enclosed in <>.

Example:

page:    1 <ifnb.ASM>

ERR  	SEQ.     S LOC. OBJ.   	SOURCE STATEMENTS

0001		; a sample program for IFNB

0002			chip	lc864016

0003		tifb	macro	arg

Because one pair of "<>" is deleted during macro expansion, double symbols ("<<>>") are required.

0004			ifnb	<<arg>>

0005				inc	a

0006			else

0007				inc	b

0008			endif

0009			endm

0010

0011			tifb	xxx

0011+1			ifnb	<xxx>

0011+2	C 0000 6300			inc	a

0011+3			else

0011+4				inc	b

0011+5			endif

0012			tifb

0012+1			ifnb	<>

0012+2				inc 	a

0012+3			else

0012+4	C 0002 6302			inc	b

0012+5			endif

0013			end

Because the argument for IFNB is empty, this portion is not assembled.

Because the argument for IFNB is not empty, this portion is assembled.

  - IFE  (Assemble if value of expression is "0")

IFE  expression

The IFE pseudo instruction assembles the source program until either ELSE or ENDIF appears subsequently, but only if the value of expression is "0."

Example:

page:    1 <ife.ASM>

ERR  	SEQ.	S LOC. OBJ.	SOURCE STATEMENTS

0001		; a sample program for IFE

0002			chip	lc866032

0003			cseg

0004		00000003 aa		set	3

0005			ife	aa-2

0006				inc	70h

0007			else

0008	C 0000 7270			dec	70h

0009			endif

0010		00000002 aa		set	aa-1

0011			ife	aa-2

0012	C 0002 6270			inc	70h

0013			else

0014				dec	70h

0015			endif

0016			end

Because the value of the expression is "0,", this portion is assembled.

Because the value of the expression is not "0,", this portion is not assembled.

  - IFNE  (Assemble if value of expression is not "0")

IFNE  expression

The IFNE pseudo instruction assembles the source program until either ELSE or ENDIF appears subsequently, but only if the value of expression is not 0.

Example:

page:    1 <ifne.ASM>

ERR	SEQ.     S LOC. OBJ.	SOURCE STATEMENTS

0001		; a sample program for IFNE

0002			chip	lc866032

0003			cseg

0004		00000003	aa	set	3

0005			ifne	aa-2

0006	C 0000 6270			inc	70h

0007			else

0008				dec	70h

0009			endif

0010		00000002	aa 	set	aa-1

0011			ifne	aa-2

0012				inc	70h

0013			else

0014	C 0002 7270			dec	70h

0015			endif

0016			end

Because the value of the expression is "0,", this portion is not assembled.

Because the value of the expression is not "0,", this portion is assembled.

  - IFIDN  (Assemble if two character strings are identical)

IFIDN  <string1>, <string2>

The IFIDN pseudo instruction assembles the source program until either ELSE or ENDIF appears subsequently, but only if string1 and string2 are identical. string1 and string2 must be enclosed in <>. The comparison includes any space or tab characters within the <>.

Example:

page:    1 <ifidn.ASM>

ERR	SEQ.     S LOC. OBJ. 	SOURCE STATEMENTS

0001		; a sample program for IFIDN

0002			chip	lc866032

0003			cseg

0004		tifidn	macro	arg1,arg2

Because one pair of "<>" is deleted during macro expansion, double symbols (<<>>) are required.

0005			ifidn	<<arg1>>,<<arg2>>

0006				inc	a

0007			else

0008				dec	a

0009			endif

0010			endm

0011

0012			tifidn	same, same

0012+1			ifidn	<same>,<same>

0012+2	C 0000 6300			inc	a

0012+3			else

0012+4				dec	a

0012+5			endif

0013			tifidn	same, not_same

0013+1			ifidn 	<same>,<not_same>

0013+2				inc	a

0013+3			else

0013+4	C 0002 7300			dec	a

0013+5			endif

0014			end

Because the two character strings are different, this portion is not assembled.

Because the two character strings are identical, this portion is assembled.