  - Dot Matrix LCD Controller/Driver

    - Overview

The LCD controller/driver automatically reads data that is stored in display RAM and generates the signals to drive the dot matrix LCD. The display mode is a graphics mode in which one bit of data in display RAM corresponds to one dot on the LCD.

The dot matrix LCD controller/driver consists of the following circuit blocks:

- Display RAM

- Display controller register

- LCD power supply circuit

    - Functions

- Display duty:	1/33 duty

- Display bias:	1/5 bias

- Graphics display

- Liquid crystal instruction:

Display:	ON/OFF

- Graphics display

1584 dots can be displayed (1 chip)

In order to control the liquid crystal display, it is necessary to manipulate the following Special Function Control Registers.

- MCR	: LCD on/off control

- STAD	: LCD start address control

- CNR	: Horizontal byte count control

- TDR	: Display duty control

- VCCR	: LCD contrast control

- XBNK	: Display RAM bank address control

    - Display RAM

Display RAM consists of three banks of 96  8 bits of static RAM. The LCD controller/driver reads data that is stored in this display RAM and generates the signals to drive the dot matrix LCD. Before writing or reading data in display RAM, set the system clock to RC oscillation.

| Symbol | Address | R/W | Name | Initial value | Bank |
| --- | --- | --- | --- | --- | --- |
| XRAM | 180H - 1FBH | R/W | Display RAM | Undefined | Bank０ |
|  | 180H - 1FBH |  |  |  | Bank１ |
|  | 180H - 185H |  |  |  | Bank２ |

    - Display Control Registers

- Mode Control Register (MCR)

This register controls the start/stop of LCD controller operation, cursor display, and the LCD clock division ratio. The Mode Control Register is a write-only register. It is important to note that if a bit manipulation instruction, an INC instruction, a DEC instruction, or a DBNZ instruction is used on a write-only register, bits other than the specified bits will be set. The following instructions are used with the MCR:

	MOV		MOV @

	ST		ST @

	POP

In addition, when accessing this register, bits 7 through 5 and bit 0 must be set to their fixed values.

| Symbol | Address | R/W | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCR | 120H | W | MCR7 | MCR6 | MCR5 | MCR4 | MCR3 | - | - | MCR0 |
| After reset |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit name | Function |  |  |  |
| --- | --- | --- | --- | --- |
| MCR7 (bit 7) | LCD clock division selection |  |  |  |
| MCR6 (bit 6) | MCR7 | MCR6 | MCR5 | Division ratio |
| MCR5 (bit 5) | 0 | 0 | 0 | 1/1 * Always set MCR7 through MCR5 to "0." |
| MCR4 (bit 4) | LCD clock divide-by-2 circuit selection |  |  |  |
|  | 0: Selects the signal selected by MCR7 through MCR5, divided by 2, as the LCD clock. |  |  |  |
|  | 1: Selects the signal selected by MCR7 through MCR5, as is, as the LCD clock. (Direct mode) |  |  |  |
| MCR3 (bit 3) | LCD controller control |  |  |  |
|  | 0: LCD controller stop 1: LCD controller start/continue |  |  |  |
| MCR0 (bit 0) | Display mode selection |  |  |  |
|  | 1: Graphics mode * Always set MCR4 = 1. |  |  |  |

MCR7 (bit 7):	LCD clock division selection

MCR6 (bit 6):

MCR5 (bit 5):

Always set MCR7 through MCR5 to "0."

MCR4 (bit 4): 	This bit controls whether or not to divide by 2 the LCD clock that was selected by MCR7 through MCR5.

The frame frequency is:

1/2 cycle (MCR4 = 0): 82.7Hz

1/1 cycle (MCR4 = 1): 165.5Hz

MCR3 (bit 3): 	LCD controller control

This bit controls LCD controller operation start (1)/stop (0).

When this bit is set to "1," the LCD controller begins to operate. When this bit is set to "0," the LCD controller stops operating.

MCR0 (bit 0): 	Display mode selection

Select graphics mode (1) for the display mode.

Graphics display:  MCR0 = 1

- LCD display start address control register (STAD)

This register controls the LCD start address.

| Symbol | Address | R/W | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| STAD | 122H | R/W | STAD7 | STAD6 | STAD5 | STAD4 | STAD3 | STAD2 | STAD1 | STAD0 |
| After reset |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit name | Function |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| STAD7 (bit 7) to STAD0 (bit 0) | LCD RAM display start address setting |  |  |  |  |  |  |  |  |
|  | STAD7 | STAD6 | STAD5 | STAD4 | STAD3 | STAD2 | STAD1 | STAD0 | Start address |
|  | 0 0  1 | 0 0  1 | 0 0 1 | 0 0 1 | 0 0 1 | 0 0 1 | 0 0 1 | 0 0 1 | 0 1  255 |

STAD7 (bit 7): 	LCD RAM display start address setting

｜

STAD0 (bit 0):

These bits set the starting address of the display data for the LCD. (XRAM 180H is assumed as STAD = 00H.)

The data changes in two-byte units.

| Start address | XRAM address | STAD7 | STAD6 | STAD5 | STAD4 | STAD3 | STAD2 | STAD1 | STAD0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0H | 180H(Bank 0) | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1H | 182H(Bank 0) | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| 2H | 184H(Bank 0) | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| 3H | 186H(Bank 0) | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |
| 4H | 188H(Bank 0) | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |
| 5H | 18AH(Bank 0) | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 1 |
| 6H | Cannot be set | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 |
| 7H | Cannot be set | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 |
| 8H | 190H(Bank 0) | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 |
| 9H | 192H(Bank 0) | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 |
| 0AH | 194H(Bank 0) | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 0 |
| 0BH | 196H(Bank 0) | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 1 |
| 0CH | 198H(Bank 0) | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 |
| 0DH | 19AH(Bank 0) | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 1 |
| 0EH | Cannot be set | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 0 |
| 0FH | Cannot be set | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| 10H | 1A0H(Bank 0) | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
| 11H | 1A2H(Bank 0) | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 |
| 3DH | 1FAH(Bank 0) | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 1 |
| 3EH | Cannot be set | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 0 |
| 3FH | Cannot be set | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 |
| 40H | 180H(Bank 1) | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 41H | 182H(Bank 1) | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |
| 7DH | 1FAH(Bank 1) | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 1 |
| 7EH | Cannot be set | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |
| 7FH | Cannot be set | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 80H | 180H(Bank 2) | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 81H | 182H(Bank 2) | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| 82H | 184H(Bank 2) | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| 83H - FFH | Cannot be set |  |  |  |  |  |  |  |  |

As indicated above, some settings result in misoperation if they are set as the start address. xx6H, xx7H, xxEH, and xxFH cannot be set.

- Character Number Register (CNR) 123H

This register is set by an internal system program.

Game programs are prohibited from accessing this register.

- Time Division Register (TDR) 124H

This register is set by an internal system program.

Game programs are prohibited from accessing this register.

- Bank Address Register (XBNK)

This register controls the display RAM bank addresses.

| Symbol | Address | R/W | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| XBNK | 125H | R/W | - | - | - | - | - | - | XBNK1 | XBNK0 |
| After reset |  |  | H | H | H | H | H | H | 0 | 0 |

| Bit name | Function |  |  |
| --- | --- | --- | --- |
| XRBK1 (bit 1) To XRBK0 (bit 0) | LCD display RAM start address setting |  |  |
|  | XRBK1 | XRBK0 | Bank address |
|  | 0 0 1 | 0 1 0 | 0 1 2 |
|  | 1 | 1 | Setting prohibited |

XRBK1 (bit 1):

to

to XRBK0 (bit 0):	Display RAM bank address control

Data can be written to display RAM at the address specified by the Bank Address Register. Banks 0 and 1 of RAM data are 96 bytes each; game programs are allowed to access bank 0 and bank 1 RAM. (Only bit 0 can be set.)

Bank 2 contains only 6 bytes, and is used for icon display. The system program displays icons.

- LCD Contrast Control Register (VCCR)

This register turns the LCD display on and off. Note that there is no built-in contrast control circuit. The LCD Contrast Control Register is a write-only register. It is important to note that if a bit manipulation instruction, an INC instruction, a DEC instruction, or a DBNZ instruction is used on a write-only register, bits other than the specified bits will be set. The following instructions are used with the VCCR:

	MOV		MOV @

	ST		ST @

	POP

In addition, when accessing this register, bits 5 through 0 must be set to their fixed values.

| Symbol | Address | R/W | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VCCR | 127H | W | VCCR7 | VCCR6 | VCCR5 | VCCR4 | VCCR3 | VCCR2 | VCCR1 | VCCR0 |
| After reset |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit name | Function |
| --- | --- |
| VCCR7 (bit 7) | LCD display control |
|  | 0: LCD display off 1: LCD display on |
| VCCR6 (bit 6) | LCD display RAM access control |
|  | 0: Access from CPU to display RAM enabled 1: Access from CPU to display RAM disabled |
| VCCR (bit 5) to VCCR0 (bit 0) | * Be certain to set VCCR5 through VCCR0 to "0." |

VCCR7 (bit 7): 	LCD display control

This bit controls the LCD display: ON (1)/OFF (0).

- LCD display ON (VCCR7 = 1)

(2) LCD display OFF (VCCR7 = 0)

Set the LCD display to ON (VCCR7 = 1) after initiating LCD controller operation (MCR3 = 1).

- Setting sequence

1. MCR3=1

2. VCCR7=1

Be certain to always follows this sequence.

Conversely, when turning the LCD display OFF, make the settings in the following sequence:

1. VCCR7=0

2. MCR3=0

VCCR6 (bit 6):	LCD display RAM access control

When the sub-clock (crystal oscillation) has been set for the system clock and the LCD display is ON, be certain to disable access from the CPU to the LCD display RAM (VCCR6 = 1) after changing the system clock. In addition, enable access from the CPU to the LCD display RAM (VCCR6 = 0) when reading or writing the LCD display RAM, or when setting the system clock to CF oscillation or RC oscillation while the LCD is on.

Setting sequence

When changing the system clock from CF oscillation or RC oscillation to crystal oscillation while the LCD display is on:

VCCR6=1

OCR5=1,OCR4=0

When changing the system clock from crystal oscillation to CF oscillation or RC oscillation while the LCD display is on:

OCR5 = 0/1, OCR4 = 1 (CF oscillation), OCR5 = 0, OCR4 = 0 (RC oscillation)

VCCR6 = 0

VCCR5 (bit 5) through VCCR0 (bit 0):

|

Always set these bits to "0."

Note:

- When the LCD is on, set the VCCR last.