VMS

Sound

Development

Specifications

Revision 0.60

Issued by

Sega Enterprises, Inc.

CS Development and Production Dep't, Second Division

Revision：

- 1998/5/26			Initial release

Contents

1	VMS Sound Output Hardware Outline	4

2	Sound Output Principle	4

2.1	Timer 1 Outline	4

2.1.1	Timer 1 Block Configuration	4

2.1.2	Related Registers	6

2.1.3	Mode Setting	6

2.2	8-Bit Counter Mode	7

2.2.1	Output Waveform and Parameter Settings	7

2.2.2	8-Bit Counter Mode Setting	8

2.2.3	Frequency Response Characteristics	9

2.3	Table of Available Output Frequencies	10

3	Sample Program	11

4	Remarks	12

# VMS Sound Output Hardware Outline

VMS can use an internal timer (timer 1) to produce sound output.

The following two output methods are possible.

	8-bit pulse generator output

	Variable bit length pulse generator output (9 - 16bits)

Both methods use the timer 1 circuit. Normally, the 8-bit pulse generator output method is used.

# Sound Output Principle

This section describes the VMS sound output method. 
VMS sound output uses timer 1.

## Timer 1 Outline

This section describes timer 1 that is used for VMS sound output.

Timer 1 incorporated in the VMS is a 16-bit timer with the following four functions.

Mode 0:  8-bit reload timer  2 channels

Mode 1:  8-bit reload timer + 8-bit pulse generator

Mode 2:  16-bit reload timer

Mode 3:  Variable bit length pulse generator (9 - 16bits)

Among these modes, VMS uses mode 1 for sound output.

For information on using the other modes, please refer to the VMS Hardware manual.

### Timer 1 Block Configuration

This section describes the block configuration of timer 1.

A configuration diagram of timer 1 is shown in Fig. 2.1.

- Timer 1 lower level (T1L)  ······  

This is an 8-bit reload timer using the cycle clock or cycle clock 1/2 signal as clock signal.

At the overflow of T1L, the T1lR data are reloaded. When T1LRUN (T1CNT, bit6) is set to "0", the T1LR data are transferred to T1L.

- Timer 1 lower level comparator (T1LC)  ······  

This circuit consists of the 8-bit timer 1 lower level comparison data register (T1LC) and an 8-bit data comparator circuit. The circuit compares the T1L and T1LC data.

- Timer 1 higher level (T1H)  ······  

This is an 8-bit reload timer using the cycle clock or the T1L overflow as clock signal.

At the overflow of T1H, the 1HR data are reloaded. Reload also occurs when T1HRUN (T1CNT, bit7) is reset.

- Timer 1 higher level comparator (T1HC)  ······  

This circuit consists of the 8-bit timer 1 higher level comparison data register (T1HC) and an 8-bit data comparator circuit. The circuit compares the T1H and T1HC data.

- Timer 1 control register (T1CNT)  ······  

Serves for T1 mode setting and interrupt control.

Fig 2.1  VMS Timer 1 Block Diagram

### Related Registers

The following registers are required for controlling timer 1.

- T1L (11Bh)	Timer 1 lower level counter register

- T1LR (11Bh)	Timer 1 lower level reload register

- T1LC (11Ah)	Timer 1 lower level comparison data register

- T1CNT (118h)	Timer 1 control register

- P1 (114h)	Port 1 latch register

- P1DDR (145h)	Port 1 data direction register

- P1FCR (146h)	Port 1 control register

- OCR (10Eh)	Resonance control register

For details on the above timer, please refer to section 3.3 of the VMS Hardware manual.

### Mode Setting

This section describes how to set timer 1 to the mode for VMS sound output (mode 1).

The following four registers are required for setting the mode.

T1CNT	(bit5: T1LONG)

P1	(bit7: P17)

P1DDR	(bit7: P17DDR)

P1FCR	(bit7: P17DDR)

The register values for the modes are listed in the table below, along with the cycle clock used for each mode.

| Mode | Clock cycle | T1LONG | P17FCR | P17DDR | P17 |
| --- | --- | --- | --- | --- | --- |
| 1 | Tcyc | 0 | 1 | 1 | 0 |

Fig 2.2  Time 1 Mode Setting

Tcyc in the table is the cycle clock.

To use the sound output capability of VMS, be sure to set the system clock to the sub-clock (32 KHz).

At other system clock settings, correct sound output may not be obtained.

The cycle clock is defined as follows.

System clock 32 KHz (Tcyc = 183.0 us)

For information on setting the system clock, please refer to the VMS Hardware manual.

*	Problems when using other system clock settings

Besides the 32 KHz clock, the VMS can use a 600 KHz and 6 MHz system clock, but when the latter two are selected, the following problems occur.

- 600KHz	When the 600 KHz clock is selected, the output frequency tolerance will be -50%, +100%, which will cause a wide fluctuation in the actual output sound.

- 6MHz	When the 6 MHz clock is selected, power consumption will increase considerably, resulting in a shorter battery life.

## 8-Bit Counter Mode

This section describes VMS sound output when using 8-bit counter mode. For information on basic operation, please refer to the VMS Hardware manual.

### Output Waveform and Parameter Settings

This section describes the signal waveform that can be output in 8-bit counter mode, and lists the parameters that determine the waveform.

The output waveform is shown in Fig. 2.3.

8-bit counter value (T1L)

255

[T1LC] setting value

[T1LR] setting value

Time (t)

Output sound signal

T1LC - T1LR

Beeper output

256 - T1LR

- Fig 2.3  Output waveform

Sound output signal L level pulse width (decimal)

= (T1LC setting value - T1LR setting value)  Tcyc

Sound output signal cycle (decimal)

= (256 - T1LR setting value)  Tcyc

Tcyc: cycle clock

*	Make sure that T1LC >= T1LR

- Set the T1LC and T1LR value in hexadecimal notation.

- When applying the above equation, convert both setting values to decimal notation.

### 8-Bit Counter Mode Setting

This section describes the sound signal output procedure in 8-bit counter mode.

To output a sound signal in 8-bit counter mode, make the settings as described below.

- Set the parameters (T1LR, T1LC) according to the desired output waveform. 
Use equations (1) and (2) given below to define the waveform.

- Sound output signal L level pulse width (decimal)

= (T1LC setting value - T1LR setting value)  Tcyc······Equation (1)

- Sound output signal cycle (decimal)

= (256 - T1LR setting value)  Tcyc······························Equation (2)

- Tcyc: cycle clock

- For details on output waveform and parameters, please refer to section 2.2.1.

- Select the mode for timer 1. 
The following four registers are required for setting the mode.

T1CNT	(bit5: T1LONG)

P1	(bit7: P17)

P1DDR	(bit7: P17DDR)

P1FCR	(bit7: P17FCR)

The register values for the modes are listed in the table below, along with the cycle clock used for each mode.

| Mode | T1LONG | P17FCR | P17DDR | P17 |
| --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 1 | 0 |

Fig 2.4  Time 1 Mode Setting

- Start the count for timer 1 (lower 8bits)

- To start/stop the timer, make the following settings.

- Waveform parameter update	Set T1CNT bit4 (ELDT1C) to "1". Note that the waveform parameters set in step 1 do not become effective until this setting is made.

- If the parameters were changed while T1CNT bit4 was "1", the parameter setting value becomes effective immediately after the change.

- Timer 1 count start	Set T1CNT bit6 (T1LRUN) to "1".

- To stop audio output in the 8-bit counter made, make the Following setting.

- Set the timer1(T1L) count stop flag (T1CNT bit6)to "0".

- While timer 1 (lower 8bits) is operating, the waveform parameters can be changed. To output sound of a different frequency without interruption, change the waveform output parameters without stopping timer 1. (Leave T1CNT bit4 [ELDT 1C]) set to "1".)

### Frequency Response Characteristics

The frequency response of the beeper in the VMS is shown below.

The T1LR value indicates the frequency range that can be output by the VMS.

For details, please refer to the explanation of the relationship between T1LR value and output frequency in section 2.3.

## Table of Available Output Frequencies

The output frequencies (theoretical values) available with a system clock of 32 KHz are listed below.

Due to limitations of the beeper, not all frequencies can actually be output. You should use the recommended frequencies indicated in the table.

The L level pulse width of the output signal is set to 1/2 of the output signal cycle (duty factor = 50%).

: Recommended frequencies

Fig. 2.6  Waveform Parameters and Output Frequencies

# Sample Program

Start

mov	#A3h,	ocr	; set system clock to 32 kHz

; set clock division ratio to 1/6

Timer 1 initial setting

mov	#000h,	T1LR	; T1LR=0	 256 - 0 = 256

mov	#080h,	T1LC	; T1LC=128	  256 - 128 = 128

;    L level pulse width 128 Tcyc

;    sound signal cycle 256 Tcyc

mov	#80h,	P1FCR	; set P17 to sound output mode

clr1	P1,	7	; set sound output port

mov	#80h,	P1DDR	; set output of P17

mov	#0D4h,	T1CNT	; update waveform parameter

; start counter (sound output start)

Sound output signal setting

- Signal frequency setting

- L level setting

Timer 1 setting

Timer 1 start

mov	#040h,	T1LC	;T1LC=64	 256 - 64 = 192

;    L level pulse width 192 Tcyc

;    sound signal cycle 256 Tcyc

mov	#000h,	T1CNT	; stop updating waveform parameter

; stop counter (sound output stop)

Sound output signal setting change

(L level pulse width change)

Sound output

Sound output stop

Output waveform

256 x Tcyc

128 x Tcyc

Output waveform

256 x Tcyc

192 x Tcyc

# Remarks