  - Jump Instructions

JMP  a12  (Jump near absolute address)

| Instruction code | 0 0 1a11 1a10a9a8 a7a6a5a4a3a2a1a0 28H~2FH,38H~3FH |
| --- | --- |
| Number of bytes | 2 |
| Number of cycles | 2 |
| Function | (PC)(PC) + 2, (PC11~00)a12 |
| Affected flags |  |
| Interrupt acceptance | Permitted at 2nd cycle |

Description:

This instruction increments the Program Counter (PC) twice, and then transfers the data a11 through a0 to bits 11 through 00 of the PC.

Example 1:

The value of label LA is 0F0EH.

|  |  |  | PC | Instruction code |
| --- | --- | --- | --- | --- |
|  | NOP |  | 0FFBH | 00H |
|  | NOP |  | 0FFCH | 00H |
|  | JMP | LA | 0FFDH | 3F0EH |
| LA: | INC | ACC | 0F0EH | 6300H |
|  | ROR |  | 0F10H | C0H |

Example 2:

The value of label LA is 1F0EH.

|  |  |  | PC | Instruction code |
| --- | --- | --- | --- | --- |
|  | NOP |  | 0FFCH | 00H |
|  | NOP |  | 0FFDH | 00H |
|  | JMP | LA | 0FFEH | 3F0EH |
| LA: | INC | ACC | 1F0EH | 6300H |
|  | ROR |  | 1F10H | C0H |

JMPF  a16  (Jump far absolute address)

| Instruction code | 0 0 1 0 0 0 0 1 a15a14a13a12a11a10a9a8 a7a6a5a4a3a2a1a0 21H |
| --- | --- |
| Number of bytes | 3 |
| Number of cycles | 2 |
| Function | (PC)a16 |
| Affected flags |  |
| Interrupt acceptance | Permitted at 2nd cycle |

Description:

This instruction transfers the data a15 through a0 to the Program Counter (PC).

Example 1:

The value of label LA is 0F0EH.

|  |  |  | PC | Instruction code |
| --- | --- | --- | --- | --- |
|  | NOP |  | 0FFAH | 00H |
|  | NOP |  | 0FFBH | 00H |
|  | JMPF | LA | 0FFCH | 210F0EH |
| LA: | INC | ACC | 0F0EH | 6300H |
|  | ROR |  | 0F10H | C0H |

Example 2:

The value of label LA is 0F0EH.

|  |  |  | PC | Instruction code |
| --- | --- | --- | --- | --- |
|  | NOP |  | 0FFCH | 00H |
|  | NOP |  | 0FFDH | 00H |
|  | JMPF | LA | 0FFEH | 210F0EH |
| LA: | INC | ACC | 0F0EH | 6300H |
|  | ROR |  | 0F10H | C0H |

BR  r8  (Branch near relative address)

| Instruction code | 0 0 0 0 0 0 0 1 r7r6r5r4r3r2r1r0 01H |
| --- | --- |
| Number of bytes | 2 |
| Number of cycles | 2 |
| Function | (PC)  (PC) + 2,(PC)  (PC) + r8 |
| Affected flags |  |
| Interrupt acceptance | Permitted at 2nd cycle |

Description:

This instruction increments the Program Counter (PC) twice, adds the data r7 through r0 to the PC, and then transfers that result to the PC.

Example 1:

The value of label LA is 0F5FH.

|  |  |  | PC | Instruction code |
| --- | --- | --- | --- | --- |
|  | NOP |  | 0F1CH | 00H |
|  | NOP |  | 0F1DH | 00H |
|  | BR | LA | 0F1EH | 013FH |
| LA: | INC | ACC | 0F5FH | 6300H |
|  | ROR |  | 0F61H | C0H |

Example 2:

The value of label LA is 1F0EH.

|  |  |  | PC | Instruction code |
| --- | --- | --- | --- | --- |
|  | NOP |  | 1F0CH | 00H |
|  | NOP |  | 1F0DH | 00H |
| LA: | INC | ACC | 1F0EH | 6300H |
|  | ROR |  | 1F10H | C0H |
|  | NOP |  | 1F11H | 00H |
|  | NOP |  | 1F12H | 00H |
|  | BR | LA | 1F13H | 01F9H |

BRF  r16  (Branch far relative address)

| Instruction code | 0 0 0 1 0 0 0 1 r7r6r5r4r3r2r1r0 r15r14r13r12r11r10r9r8 11H |
| --- | --- |
| Number of bytes | 3 |
| Number of cycles | 4 |
| Function | (PC)(PC) + 3,(PC)(PC)  1 + r16 |
| Affected flags |  |
| Interrupt acceptance | Permitted at 4th cycle |

Description:

This instruction increments the Program Counter (PC) three times, then decrements the PC, adds the data r15 through r0 to the PC, and then transfers that result to the PC.

Example 1:

The value of label LA is 105FH.

|  |  |  | PC | Instruction code |
| --- | --- | --- | --- | --- |
|  | NOP |  | 0F1CH | 00H |
|  | NOP |  | 0F1DH | 00H |
|  | BRF | LA | 0F1EH | 113F01H |
| LA: | INC | ACC | 105FH | 6300H |
|  | ROR |  | 1061H | C0H |

Example 2:

The value of label LA is 1F0EH.

|  |  |  | PC | Instruction code |
| --- | --- | --- | --- | --- |
|  | NOP |  | 1FFCH | 00H |
|  | NOP |  | 1FFDH | 00H |
| LA: | INC | ACC | 1F0EH | 6300H |
|  | ROR |  | 1F10H | C0H |
|  | NOP |  | 1F11H | 00H |
|  | NOP |  | 1F12H | 00H |
|  | BRF | LA | 1F13H | 11F8FFH |

  - Conditional Branching Instructions

BZ  r8  (Branch near relative address if accumulator is zero)

| Instruction code | 1 0 0 0 0 0 0 0 r7r6r5r4r3r2r1r0 80H |
| --- | --- |
| Number of bytes | 2 |
| Number of cycles | 2 |
| Function | (PC)(PC) + 2, if (ACC) = 0 then (PC)(PC) + r8 |
| Affected flags |  |
| Interrupt acceptance | Permitted at 2nd cycle |

Description:

This instruction increments the Program Counter (PC) twice, and then, if the Accumulator (ACC) is zero, adds the data r7 through r0 to the PC, and transfers that result to the PC. If the ACC is not zero, the next instruction is executed.

Example 1:

|  |  |  | PC | Instruction code | ACC |
| --- | --- | --- | --- | --- | --- |
|  | MOV | #000H,ACC | 0F1BH | 230000H | 00H |
|  | BZ | LA | 0F1EH | 803FH | 00H |
| LA: | INC | ACC | 0F5FH | 6300H | 01H |
|  | ROR |  | 0F61H | C0H | 80H |

	Because ACC = "0" when the BZ instruction is executed, the program branches to the label LA.

Example 2:

|  |  |  | PC | Instruction code | ACC |
| --- | --- | --- | --- | --- | --- |
|  | MOV | #001H,ACC | 0F1BH | 230001H | 01H |
|  | BZ | LA | 0F1EH | 803FH | 01H |
|  | DEC | ACC | 0F20H | 7300H | 00H |
|  | ROR |  | 0F22H | C0H | 00H |
| LA: | INC | ACC |  |  |  |

	Because ACC  "0" when the BZ instruction is executed, the program simply executes the next instruction.

BNZ  r8  (Branch near relative address if accumulator is not zero)

| Instruction code | 1 0 0 1 0 0 0 0 r7r6r5r4r3r2r1r0 90H |
| --- | --- |
| Number of bytes | 2 |
| Number of cycles | 2 |
| Function | (PC)(PC) + 2, if (ACC) 0 then (PC)(PC) + r8 |
| Affected flags |  |
| Interrupt acceptance | Permitted at 2nd cycle |

Description:

This instruction increments the Program Counter (PC) twice, and then, if the Accumulator (ACC) is not zero, adds the data r7 through r0 to the PC, and transfers that result to the PC. If the ACC is zero, the next instruction is executed.

Example 1:

|  |  |  | PC | Instruction code | ACC |
| --- | --- | --- | --- | --- | --- |
|  | MOV | #001H,ACC | 0F1BH | 230001H | 01H |
| BNZ | LA | 0F1EH | 903FH | 01H |  |
| LA: | INC | ACC | 0F5FH | 6300H | 02H |
|  | ROR |  | 0F61H | C0H | 01H |

	Because ACC  "0" when the BNZ instruction is executed, the program branches to the label LA.

Example 2:

|  |  |  | PC | Instruction code | ACC |
| --- | --- | --- | --- | --- | --- |
|  | MOV | #000H,ACC | 0F1BH | 230000H | 00H |
|  | BNZ | LA | 0F1EH | 903FH | 00H |
|  | DEC | ACC | 0F20H | 7300H | FFH |
|  | ROR |  | 0F22H | C0H | FFH |
| LA: | INC | ACC |  |  |  |

	Because ACC = "0" when the BNZ instruction is executed, the program simply executes the next instruction.

BP  d9,b3,r8  (Branch near relative address if direct bit is positive)

| Instruction code | 0 1 1d8 1b2b1b0 d7d6d5d4d3d2d1d0 r7r6r5r4r3r2r1r0 68H6FH,78H7FH |
| --- | --- |
| Number of bytes | 3 |
| Number of cycles | 2 |
| Function | (PC)(PC) + 3, if (d9,b3) = 1 then (PC)(PC) + r8 |
| Affected flags |  |
| Interrupt acceptance | Permitted at 2nd cycle |

Description:

This instruction increments the Program Counter (PC) three times, and then, if the bit specified by b2 to b0 of the address in data memory (RAM) or the Special Function Register (SFR) that is specified by d8 to d0 is set (1), this instruction adds the data r7 through r0 to the PC, and transfers that result to the PC. If the bit specified by b2 to b0 of the address in data memory (RAM) or the Special Function Register that is specified by d8 to d0 is reset (0), the next instruction is executed.

Example 1:

|  |  |  | PC | Instruction code | B |
| --- | --- | --- | --- | --- | --- |
|  | MOV | #001H,B | 0F1AH | 230201H | 01H |
|  | BP | B,0,LA | 0F1DH | 78023FH | 01H |
| LA: | INC | B | 0F5FH | 6302H | 02H |
|  | NOP |  | 0F61H | 00H | 02H |

	Because bit 0 of B is "1" when the BP instruction is executed, the program branches to the label LA.

Example 2:

|  |  |  | PC | Instruction code | ACC |
| --- | --- | --- | --- | --- | --- |
|  | MOV | #080H,ACC | 0F1AH | 230080H | 80H |
|  | BP | ACC,0,LA | 0F1DH | 78003FH | 80H |
|  | DEC | ACC | 0F20H | 7300H | 7FH |
|  | ROR |  | 0F22H | C0H | BFH |
| LA: | INC | ACC |  |  |  |

	Because bit 0 of the ACC is "0" when the BP instruction is executed, the program simply executes the next instruction.

BPC  d9,b3,r8  (Branch near relative address if direct bit is positive, and clear)

| Instruction code | 0 1 0d8 1b2b1b0 d7d6d5d4d3d2d1d0 r7r6r5r4r3r2r1r0 48H~4FH,58H~5FH |
| --- | --- |
| Number of bytes | 3 |
| Number of cycles | 2 |
| Function | (PC)(PC) + 3, if (d9,b3) = 1 then (PC)(PC) + r8 , (d9,b3) = 0 |
| Affected flags |  |
| Interrupt acceptance | Permitted at 2nd cycle |

Description:

This instruction increments the Program Counter (PC) three times, and then, if the bit specified by b2 to b0 of the address in data memory (RAM) or the Special Function Register (SFR) that is specified by d8 to d0 is set (1), this instruction resets that bit, then adds the data r7 through r0 to the PC, and transfers that result to the PC. If the bit specified by b2 to b0 of the address in data memory (RAM) or the Special Function Register that is specified by d8 to d0 is reset (0), the next instruction is executed.

Example 1:

|  |  |  | PC | Instruction code | B |
| --- | --- | --- | --- | --- | --- |
|  | MOV | #003H,B | 0F1AH | 230203H | 03H |
|  | BPC | B,0,LA | 0F1DH | 58023FH | 02H |
| LA: | INC | B | 0F5FH | 6302H | 03H |
|  | NOP |  | 0F61H | 00H | 03H |

	Because bit 0 of B is "1" when the BPC instruction is executed, the program branches to the label LA.

Example 2:

|  |  |  | PC | Instruction code | ACC |
| --- | --- | --- | --- | --- | --- |
|  | MOV | #080H,ACC | 0F1AH | 230080H | 80H |
|  | BPC | ACC,0,LA | 0F1DH | 58003FH | 80H |
|  | DEC | ACC | 0F20H | 7300H | 7FH |
|  | ROR |  | 0F22H | C0H | BFH |
| LA: | INC | ACC |  |  |  |

	Because bit 0 of the ACC is "0" when the BPC instruction is executed, the program simply executes the next instruction.

Notes:

- When this instruction is applied to one of the ports P0, P1, P2, P3, P4, or P5, the port latch of that port is selected; the external signal that is applied to that port is not selected. Furthermore, applying this instruction to port P7 does not change its status.

BN  d9,b3,r8    (Branch near relative address if direct bit is negative)

| Instruction code | 1 0 0 d8 1 b2b1b0 d7d6d5d4d3d2d1d0 r7r6r5r4r3r2r1r0 88H~8FH,98H~9FH |
| --- | --- |
| Number of bytes | 3 |
| Number of cycles | 2 |
| Function | (PC)(PC) + 3, if (d9,b3) = 0 then (PC)(PC) + r8 |
| Affected flags |  |
| Interrupt acceptance | Permitted at 2nd cycle |

Description:

This instruction increments the Program Counter (PC) three times, and then, if the bit specified by b2 to b0 of the address in data memory (RAM) or the Special Function Register (SFR) that is specified by d8 to d0 is reset (0), this instruction adds the data r7 through r0 to the PC, and transfers that result to the PC. If the bit specified by b2 to b0 of the address in data memory (RAM) or the Special Function Register that is specified by d8 to d0 is set (1), the next instruction is executed.

Example 1:

|  |  |  | PC | Instruction code | B |
| --- | --- | --- | --- | --- | --- |
|  | MOV | #0FEH,B | 0F1AH | 2302FEH | FEH |
|  | BN | B,0,LA | 0F1DH | 98023FH | FEH |
| LA: | INC | B | 0F5FH | 6302H | FFH |
|  | NOP |  | 0F61H | 00H | FFH |

	Because bit 0 of B is "0" when the BN instruction is executed, the program branches to the label LA.

Example 2:

|  |  |  | PC | Instruction code | ACC |
| --- | --- | --- | --- | --- | --- |
|  | MOV | #001H,ACC | 0F1AH | 230001H | 01H |
|  | BN | ACC,0,LA | 0F1DH | 98003FH | 01H |
|  | DEC | ACC | 0F20H | 7300H | 00H |
|  | ROR |  | 0F22H | C0H | 00H |
| LA: | INC | ACC |  |  |  |

	Because bit 0 of the ACC is "1" when the BN instruction is executed, the program simply executes the next instruction.

DBNZ  d9,r8  (Decrement direct byte and branch near relative address if direct byte is not zero)

| Instruction code | 0 1 0 1 0 0 1d8 d7d6d5d4d3d2d1d0 r7r6r5r4r3r2r1r0 52H~53H |
| --- | --- |
| Number of bytes | 3 |
| Number of cycles | 2 |
| Function | (PC)(PC) + 3, (d9) = (d9)  1,if (d9)  0 then (PC)(PC) + r8 |
| Affected flags |  |
| Interrupt acceptance | Permitted at 2nd cycle |

Description:

This instruction increments the Program Counter (PC) three times, and then decrements the address in data memory (RAM) or the Special Function Register (SFR) that is specified by d8 to d0. If the contents of the decremented RAM address or SFR are not zero, this instruction adds the data r7 through r0 to the PC, and transfers that result to the PC. If the contents of the decremented RAM address or Special Function Register are zero, the next instruction is executed.

Example 1:

|  |  |  | PC | Instruction code | B |
| --- | --- | --- | --- | --- | --- |
|  | MOV | #002H,B | 0F1AH | 230202H | 02H |
|  | DBNZ | B,LA | 0F1DH | 53023FH | 01H |
| LA: | INC | B | 0F5FH | 6302H | 02H |
|  | NOP |  | 0F61H | 00H | 02H |

	Because B  "0" after being decremented when the DBNZ instruction is executed, the program branches to the label LA.

Example 2:

|  |  |  | PC | Instruction code | ACC |
| --- | --- | --- | --- | --- | --- |
|  | MOV | #001H,ACC | 0F1AH | 230001H | 01H |
|  | DBNZ | ACC,LA | 0F1DH | 53003FH | 00H |
|  | DEC | ACC | 0F20H | 7300H | FFH |
|  | ROR |  | 0F22H | C0H | FFH |
| LA: | INC | ACC |  |  |  |

	Because ACC = "0" after being decremented when the DBNZ instruction is executed, the program simply executes the next instruction.

Notes:

- When this instruction is applied to one of the ports P0, P1, P2, P3, P4, or P5, the port latch of that port is selected; the external signal that is applied to that port is not selected. Furthermore, applying this instruction to port P7 does not change its status.

DBNZ  @Rj,r8  (Decrement indirect byte and branch near relative address if indirect byte is not zero)

| Instruction code | 0 1 0 1 0 1j1j0 r7r6r5r4r3r2r1r0 54H~57H |
| --- | --- |
| Number of bytes | 2 |
| Number of cycles | 2 |
| Function | (PC)  (PC) + 2, ((Rj)) = ((Rj))  1, if ((Rj)) 0 then (PC)(PC) + r8 j = 0,1,2,3 |
| Affected flags |  |
| Interrupt acceptance | Permitted at 2nd cycle |

Description:

This instruction increments the Program Counter (PC) twice, and then decrements the address in data memory (RAM) or the Special Function Register (SFR) that is specified by the indirect address register that is specified by j1 and j0. If the contents of the decremented RAM address or SFR are not zero, this instruction adds the data r7 through r0 to the PC, and transfers that result to the PC. If the contents of the decremented RAM address or Special Function Register are zero, the next instruction is executed.

Example 1:

|  |  |  | PC | Instruction code | B | RAM 03H |
| --- | --- | --- | --- | --- | --- | --- |
|  | MOV | #002H,B | 0F18H | 230202H | 02H | - |
|  | MOV | #002H,003H | 0F1BH | 220302H | 02H | 02H |
|  | DBNZ | @R3,LA | 0F1EH | 573FH | 01H | 02H |
| LA: | INC | B | 0F5FH | 6302H | 02H | 02H |

	Because B  "0" after being decremented when the DBNZ instruction is executed, the program branches to the label LA.

Example 2:

|  |  |  | PC | Instruction code | ACC | RAM 03H |
| --- | --- | --- | --- | --- | --- | --- |
|  | MOV | #001H,ACC | 0F18H | 230001H | 01H | - |
|  | MOV | #000H,003H | 0F1BH | 220300H | 01H | 00H |
|  | DBNZ | @R3,LA | 0F1EH | 573FH | 00H | 00H |
|  | DEC | ACC | 0F20H | 7300H | FFH | 00H |
| LA: | INC | ACC |  |  |  |  |

	Because ACC = "0" after being decremented when the DBNZ instruction is executed, the program simply executes the next instruction.

Notes:

- When this instruction is applied to one of the ports P0, P1, P2, P3, P4, or P5, the port latch of that port is selected; the external signal that is applied to that port is not selected. Furthermore, applying this instruction to port P7 does not change its status.

BE  #i8,r8  (Compare immediate data to accumulator and branch near relative address if equal)

| Instruction code | 0 0 1 1 0 0 0 1 i7i6i5i4i3i2i1i0 r7r6r5r4r3r2r1r0 31H |
| --- | --- |
| Number of bytes | 3 |
| Number of cycles | 2 |
| Function | (PC)  (PC) + 3, if (ACC) = #i8 then (PC)  (PC) + r8 |
| Affected flags | CY |
| Interrupt acceptance | Permitted at 2nd cycle |

Description:

This instruction increments the Program Counter (PC) three times, and then compares the immediate data (i7 to i0) with the contents of the Accumulator (ACC). If the compared data are the same, this instruction adds the data r7 through r0 to the PC, and then transfers that result to the PC. If the data are not the same, the next instruction is executed.

Furthermore, if the ACC is less than the immediate data, the carry flag (CY) is set; if the ACC is greater than or equal to the immediate data, the carry flag (CY) is reset.

ACC < #i8    CY=1

ACC > #i8    CY=0

Example 1:

|  |  |  | PC | Instruction code | ACC | CY |
| --- | --- | --- | --- | --- | --- | --- |
|  | MOV | #002H,ACC | 0F1AH | 230002H | 02H | - |
|  | BE | #002H,LA | 0F1DH | 31023FH | 02H | 0 |
| LA: | INC | ACC | 0F5FH | 6300H | 03H | 0 |

	Because ACC = 02H when the BE instruction is executed, CY is reset and the program branches to the label LA.

Example 2:

|  |  |  | PC | Instruction code | ACC | CY |
| --- | --- | --- | --- | --- | --- | --- |
|  | MOV | #003H,ACC | 0F1AH | 230003H | 03H | - |
|  | BE | #004H,LA | 0F1DH | 31043FH | 03H | 1 |
|  | DEC | ACC | 0F20H | 7300H | 02H | 1 |
| LA: | INC | ACC |  |  |  |  |

	Because ACC < 04H when the BE instruction is executed, CY is set and the program executes the next instruction.

BE  d9,r8  (Compare direct byte to accumulator and branch near relative address if equal)

| Instruction code | 0 0 1 1 0 0 1d8 d7d6d5d4d3d2d1d0 r7r6r5r4r3r2r1r0 32H~33H |
| --- | --- |
| Number of bytes | 3 |
| Number of cycles | 2 |
| Function | (PC) (PC) + 3, if (ACC) = (d9) then (PC)(PC) + r8 |
| Affected flags | CY |
| Interrupt acceptance | Permitted at 2nd cycle |

Description:

This instruction increments the Program Counter (PC) three times, and then compares the contents of data memory (RAM) or a Special Function Register (SFR), as specified by d8 to d0, with the contents of the Accumulator (ACC). If the compared data are the same, this instruction adds the data r7 through r0 to the PC, and then transfers that result to the PC. If the data are not the same, the next instruction is executed.

Furthermore, if the ACC is less than the contents of data memory (RAM) or the Special Function Register (SFR), the carry flag (CY) is set; if the ACC is greater than or equal to the contents of data memory (RAM) or the Special Function Register (SFR), the carry flag (CY) is reset.

ACC < d9 (RAM or SFR)    CY=1

ACC > d9 (RAM or SFR)    CY=0

Example 1:

|  |  |  | PC | Instruction code | ACC | B | CY |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | MOV | #002H,ACC | 0F17H | 230002H | 02H | - | - |
|  | MOV | #002H,B | 0F1AH | 230202H | 02H | 02H | - |
|  | BE | B,LA | 0F1DH | 33023FH | 02H | 02H | 0 |
| LA: | INC | ACC | 0F5FH | 6300H | 03H | 02H | 0 |

	Because ACC = B when the BE instruction is executed, CY is reset and the program branches to the label LA.

Example 2:

|  |  |  | PC | Instruction code | ACC | B | CY |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | MOV | #003H,ACC | 0F17H | 230003H | 03H | - | - |
|  | MOV | #0F2H,B | 0F1AH | 2302F2H | 03H | F2H | - |
|  | BE | B,LA | 0F1DH | 33023FH | 03H | F2H | 1 |
|  | DEC | ACC | 0F20H | 7300H | 02H | F2H | 1 |
| LA: | INC | ACC |  |  |  |  |  |

	Because ACC < B when the BE instruction is executed, CY is set and the program executes the next instruction.

BE  @Rj,#i8,r8  (Compare immediate data to indirect byte and branch near relative address if equal)

| Instruction code | 0 0 1 1 0 1j1j0 i7i6i5i4i3i2i1i0 r7r6r5r4r3r2r1r0 34H~37H |
| --- | --- |
| Number of bytes | 3 |
| Number of cycles | 2 |
| Function | (PC) (PC) + 3, if ((Rj)) = # i8 then (PC)(PC) + r8 j = 0,1,2,3 |
| Affected flags | CY |
| Interrupt acceptance | Permitted at 2nd cycle |

Description:

This instruction increments the Program Counter (PC) three times, and then compares the contents of data memory (RAM) or a Special Function Register (SFR), as specified by the indirect address register that is specified by j1 and j0, with the immediate data (i7 to i0). If the compared data are the same, this instruction adds the data r7 through r0 to the PC, and then transfers that result to the PC. If the data are not the same, the next instruction is executed.

Furthermore, if the contents of data memory (RAM) or the Special Function Register (SFR), as specified by the indirect address register that is specified by j1 and j0, is less than the immediate data (i7 to i0), the carry flag (CY) is set; if the contents of data memory (RAM) or the Special Function Register (SFR) are greater than or equal to the immediate data (i7 to i0), the carry flag (CY) is reset.

@Rj < #i8    CY=1

@Rj > #i8    CY=０

Example 1:

|  |  |  | PC | Instruction code | B | RAM 03H | CY |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | MOV | #005H,B | 0F17H | 230205H | 05H | - | - |
|  | MOV | #002H,003H | 0F1AH | 220302H | 05H | 02H | - |
|  | BE | @R3,#5H,LA | 0F1DH | 37053FH | 05H | 02H | 0 |
| LA: | INC | B | 0F5FH | 6302H | 06H | 02H | 0 |

	Because B = 05H when the BE instruction is executed, CY is reset and the program branches to the label LA.

Example 2:

|  |  |  | PC | Instruction code | ACC | RAM 02H | CY |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | MOV | #003H,ACC | 0F17H | 230003H | 03H | - | - |
|  | MOV | #000H,002H | 0F1AH | 220200H | 03H | 00H | - |
|  | BE | @R2,#9H,LA | 0F1DH | 36093FH | 03H | 00H | 1 |
|  | DEC | ACC | 0F20H | 7300H | 02H | 00H | 1 |
| LA: | INC | ACC |  |  |  |  |  |

	Because ACC < 09H when the BE instruction is executed, CY is set and the program executes the next instruction.

BNE  #i8,r8  (Compare immediate data to accumulator and branch near relative address if not equal)

| Instruction code | 0 1 0 0 0 0 0 1 i7i6i5i4i3i2i1i0 r7r6r5r4r3r2r1r0 41H |
| --- | --- |
| Number of bytes | 3 |
| Number of cycles | 2 |
| Function | (PC)  (PC) + 3, if (ACC) # i8 then (PC)(PC) + r8 |
| Affected flags | CY |
| Interrupt acceptance | Permitted at 2nd cycle |

Description:

This instruction increments the Program Counter (PC) three times, and then compares the immediate data (i7 to i0) with the contents of the Accumulator (ACC). If the compared data are not the same, this instruction adds the data r7 through r0 to the PC, and then transfers that result to the PC. If the data are the same, the next instruction is executed. Furthermore, if the ACC is less than the immediate data, the carry flag (CY) is set; if the ACC is greater than or equal to the immediate data, the carry flag (CY) is reset.

ACC < #i8    CY=1

ACC > #i8    CY=0

Example 1:

|  |  |  | PC | Instruction code | ACC | CY |
| --- | --- | --- | --- | --- | --- | --- |
|  | MOV | #002H,ACC | 0F1AH | 230002H | 02H | - |
|  | BNE | #000H,LA | 0F1DH | 41003FH | 02H | 0 |
| LA: | INC | ACC | 0F5FH | 6300H | 03H | 0 |

	Because ACC > 00H when the BNE instruction is executed, CY is reset and the program branches to the label LA.

Example 2:

|  |  |  | PC | Instruction code | ACC | CY |
| --- | --- | --- | --- | --- | --- | --- |
|  | MOV | #003H,ACC | 0F1AH | 230003H | 03H | - |
|  | BNE | #003H,LA | 0F1DH | 41033FH | 03H | 0 |
|  | DEC | ACC | 0F20H | 7300H | 02H | 0 |
| LA: | INC | ACC |  |  |  |  |

	Because ACC = 03H when the BNE instruction is executed, CY is set and the program executes the next instruction.

BNE  d9,r8  (Compare direct byte to accumulator and branch near relative address if not equal)

| Instruction code | 0 1 0 0 0 0 1d8 d7d6d5d4d3d2d1d0 r7r6r5r4r3r2r1r0 42H~43H |
| --- | --- |
| Number of bytes | 3 |
| Number of cycles | 2 |
| Function | (PC)  (PC) + 3, if (ACC) (d9) then (PC) (PC) + r8 |
| Affected flags | CY |
| Interrupt acceptance | Permitted at 2nd cycle |

Description:

This instruction increments the Program Counter (PC) three times, and then compares the contents of data memory (RAM) or a Special Function Register (SFR), as specified by d8 to d0, with the contents of the Accumulator (ACC). If the compared data are not the same, this instruction adds the data r7 through r0 to the PC, and then transfers that result to the PC. If the data are the same, the next instruction is executed.

Furthermore, if the ACC is less than the contents of data memory (RAM) or the Special Function Register (SFR), the carry flag (CY) is set; if the ACC is greater than or equal to the contents of data memory (RAM) or the Special Function Register (SFR), the carry flag (CY) is reset.

ACC < d9(RAM or SFR)    CY=1

ACC > d9(RAM or SFR)    CY=0

Example 1:

|  |  |  | PC | Instruction code | ACC | B | CY |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | MOV | #002H,ACC | 0F17H | 230002H | 02H | - | - |
|  | MON | #003H,B | 0F1AH | 230203H | 02H | 03H | - |
|  | BNE | B,LA | 0F1DH | 43023FH | 02H | 03H | 1 |
| LA: | INC | ACC | 0F5FH | 6300H | 03H | 03H | 1 |

	Because ACC < B when the BNE instruction is executed, CY is set and the program branches to the label LA.

Example 2:

|  |  |  | PC | Instruction code | ACC | B | CY |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | MOV | #0F2H,ACC | 0F17H | 2300F2H | F2H | - | - |
|  | MOV | #0F2H,B | 0F1AH | 2302F2H | F2H | F2H | - |
|  | BNE | B,LA | 0F1DH | 43023FH | F2H | F2H | 0 |
|  | DEC | ACC | 0F20H | 7300H | F1H | F2H | 0 |
| LA: | INC | ACC |  |  |  |  |  |

	Because ACC = B when the BNE instruction is executed, CY is reset and the program executes the next instruction.

BNE  @Rj,#i8,r8  (Compare immediate data to indirect byte and branch near relative address if not equal)

| Instruction code | 0 1 0 0 0 1j1j0 i7i6i5i4i3i2i1i0 r7r6r5r4r3r2r1r0 44H~47H |
| --- | --- |
| Number of bytes | 3 |
| Number of cycles | 2 |
| Function | (PC)  (PC) + 3, if ((Rj)) # i8 then (PC) (PC) + r8 j = 0,1,2,3 |
| Affected flags | CY |
| Interrupt acceptance | Permitted at 2nd cycle |

Description:

This instruction increments the Program Counter (PC) three times, and then compares the contents of data memory (RAM) or a Special Function Register (SFR), as specified by the indirect address register that is specified by j1 and j0, with the immediate data (i7 to i0). If the compared data are not the same, this instruction adds the data r7 through r0 to the PC, and then transfers that result to the PC. If the data are the same, the next instruction is executed.

Furthermore, if the contents of data memory (RAM) or the Special Function Register (SFR), as specified by the indirect address register that is specified by j1 and j0, is less than the immediate data (i7 to i0), the carry flag (CY) is set; if the contents of data memory (RAM) or the Special Function Register (SFR) are greater than or equal to the immediate data (i7 to i0), the carry flag (CY) is reset.

@Rj < #i8    CY=1

@Rj > #i8    CY=０

Example 1:

|  |  |  | PC | Instruction code | ACC | B | CY |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | MOV | #002H,ACC | 0F17H | 230002H | 02H | - | - |
|  | MON | #003H,B | 0F1AH | 230203H | 02H | 03H | - |
|  | BNE | B,LA | 0F1DH | 43023FH | 02H | 03H | 1 |
| LA: | INC | ACC | 0F5FH | 6300H | 03H | 03H | 1 |

	Because B < 08H when the BNE instruction is executed, CY is set and the program branches to the label LA.

Example 2:

|  |  |  | PC | Instruction code | ACC | RAM 02H | CY |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | MOV | #003H,ACC | 0F17H | 230003H | 03H | - | - |
|  | MOV | #000H,002H | 0F1AH | 220200H | 03H | 00H | - |
|  | BNE | @R2,#3H,LA | 0F1DH | 46033FH | 03H | 00H | 0 |
|  | DEC | ACC | 0F20H | 7300H | 02H | 00H | 0 |
| LA: | INC | ACC |  |  |  |  |  |

	Because ACC = 03H when the BNE instruction is executed, CY is reset and the program executes the next instruction.