# 2

Chapter 2.

Specifying Linkage Loader Options

This section explains how to use the linkage loader options in order to specify and control the tasks performed by L86K. In the MS-DOS version, all options begin with the linkage loader option character, either "/" or "-". In the UNIX version, all options begin with the linkage loader option character "-".

  - Creating a HEX File for LC868000 Series External ROM

Option

-B=bank number

The -B option specifies the bank number of the external ROM data file in the LC868000 Series (WORLD EXTERNAL_DATA). Specify a hexadecimal value (from 1 to FF) for bank number. The bank number that is specified here becomes the data file extension.

Example:

A> L86K /B=1 SAMPLE;

The data file that is created here is SAMPLE.H01.

  - CSEG Loading Address Specification Method

Option

-C=address

The -C option, which is valid for the object module that is described immediately after this option, specifies the code segment loading address. Specify a hexadecimal value for address.

If this option is omitted, L86K loads the code segment of the object module at any suitable position.

  - DSEG Loading Address Specification Method

Option

-D=address

The -D option, which is valid for the object module that is described immediately after this option, specifies the data segment loading address. Specify a hexadecimal value for address.

If this option is omitted, L86K loads the data segment of the object module at any suitable position.

  - Enabling Duplicate Definition of DSEG Addresses

Option

-E

When the -E option is specified, no error is generated if multiple symbols are defined in the same address in DSEG.

  - No Distinction Between Upper-Case and Lower-Case

Option

-I

Under the default setting, L86K makes a distinction between upper- and lower-case, but if the -I option is specified, no distinction is made between upper- and lower-case.

  - Creating the Loading Map

Option

-P

The -P option creates a file in which the link results map (a list of the link status of each segment and the placement of public symbols) is written. The file name of this map file is the name specified in the EVA file field in the command line or prompt, with the extension changed to .MAP. Note that no map file is created if a fatal error that makes it impossible for the linking process to continue occurs.

  - Creating a Local Symbol List

Option

-L

The -L option is valid only when combined with the -P option. The -L option adds a list of local symbols to each module in the map file.

Segment attributes

In the case of the INBLOCK attribute, CSEG-I; in the case of the FREE attribute, CSEG-F

Segment block start address

In the case of a DSEG symbol, a bank number and address are output

Module name

LC86K series Linkage map list                                          PAGE    1

Linkage date: Wed Dec 09 13:28:02 1992

sample1.EVA loading map

module		segment   no.	start addreaa  size  next address

Segment block size (unit: byte)

* CSEG

sample1		CSEG-I	0000	0004  0004

sample2		CSEG-F      1	0004	0005  0009

sample2		CSEG-F      2	0009	0003  000C

Segment block end address + 1

public symbol list

symbol 		segment	address

ACC		DSEG	0:0100  [  EQU  ]

If multiple segment blocks exist within one module, a serial number is output from the start within the module

ADCR		DSEG	0:0160  [  EQU  ]

ADRR		DSEG	0:0161  [  EQU  ]

local symbol list

symbol		segment   address

** sample1  **

l1		CSEG        0000

Local symbols are output for each module

** sample2  **

l2		CSEG        0004

  - Specifying the Display of Warning Messages Concerning Operand Data

Option

-W

The -W option displays a warning message when the value of an operand described for JMP, etc., is outside of the valid range. (In the case of JMP, the operand is 12 bits, so the valid range of values for the operand is 0 to 4095.) (The value that is stored in the instruction code consists only of valid bits; overflow bits are discarded.)

  - CSEG FREE Block Optimized Loading

L86K normally links (allocates) according to the CSEG section described in the object module and the sequence specified in the command module, but when it does so L86K aligns the executable file segment data (code segments) with 4096-byte boundaries. This results in a number of empty areas within the code segment area.

L86K has four placement functions that allocate each segment block in the optimal position in order to minimize the occurrence of such empty areas and therefore use memory efficiently.

Each of these loading methods is described below.

Option:

-A

The -A option loads all code segment blocks that are the target of the linking process in order, based on size.  While doing so, if a segment block that crosses a 4096-byte boundary has the INBLOCK attribute, that block is aligned with the boundary; if the segment block has the FREE attribute, that block is placed normally without being aligned with the boundary.

Option:

-F

The -F option is valid only when used in combination with the -A option. After linking code segment blocks that have the INBLOCK attribute in the sequence specified in the command line, this option allocates code segments that have the FREE attribute in those empty areas that appear due to the reason described above.  (If there is no more space to allocate code segments that have the FREE attribute, those segments are allocated after the final address in order, based on size.)

Option:

-O

The -O option is valid only when used in combination with the -A option. After linking code segment blocks that have the INBLOCK attribute in order, based on their size, this option allocates code segments that have the FREE attribute in empty areas. (If there is no more space to allocate to code segments that have the FREE attribute, those segments are allocated after the final address in order, based on size.)

Option:

-R

The -R option is valid only when used in combination with the -A option. After linking code segment blocks that have the INBLOCK attribute in order, based on their size, this option allocates code segments that have the FREE attribute in empty areas. If any two consecutive 4096-byte areas each contain empty area, the two empty areas are brought together by repositioning the second INBLOCK code segment; a code segment with the FREE attribute is then assigned to the newly formed area. (If there is no more space to allocate to code segments that have the FREE attribute, those segments are allocated after the final address in order, based on size.)

  - Specifying Symbol Sort Processing

Option:

-s

If the total number of public symbol definitions in all of the link target object files (including the relevant SFR definitions in LC86K.LIB) exceeds 8192 definitions, or if the total number of local symbol definitions in all of the link target object files exceeds 8192 definitions, symbol sort processing will become slower.  A message that indicates the progress of the sort processing is displayed while processing is in progress:

Public(Local) symbol table: Sorting ..  nn / nn blocks

Public(Local) symbol table: Sorting(merge) .. nn %

If the -S option is specified, however, symbol sort processing is not performed in these circumstances; instead, the following message is displayed and link processing is interrupted:

** Link Error, Public symbol table overflow: nn symbols

The number of symbols display4ed here is the number of symbols in excess of 8192.