Chapter 4.

Control Functions

  - Interrupt Function

The interrupt function is used to temporarily interrupt the program that the microcomputer is currently executing and then execute another program in order to address an urgent need. POTATO includes circuits for generating 12 types of interrupt requests. In the VMS, some types of interrupt processing cannot be set as desired from within a game program. The interrupts are shown in Table 4-1-1.

    - Types of Interrupts

Table. 4-1-1.  List of interrupts

| Priority ranking | Interrupt type | Internal/external | Vector address | Interrupt request | Source flag | Enable flag | Register address | Priority ranking setting |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | External interrupt INT0 | external | 0003H | P70/INT0 pin event detection | I01CR1 | I01CR0 | 15DH | Highest/low |
| 2 | External interrupt INT1 | external | 000BH | P71/INT1 pin event detection | I01CR5 | I01CR4 | 15DH |  |
| 3 | External interrupt INT2 | external | 0013H | P72/INT2 pin event detection | I23CR1 | I23CR0 | 15EH | High/low |
|  | Timer/counter T0L (lower 8 bits) | Internal |  | Timer/counter T0L lower 8 bits overflow | T0CNT1 | T0CNT0 | 110H |  |
| 4 | External interrupt INT3 | external | 001BH | P73/INT3 pin event detection | I23CR5 | I23CR4 | 15EH | High/low |
|  | Base timer | Internal |  | Base timer overflow | BTCR1 | BTCR0 | 17FH |  |
|  |  |  |  |  | BTCR3 | BTCR2 |  |  |
| 5 | Timer/counter T0H (upper 8 bits) | Internal | 0023H | Timer/counter T0H upper 8 bits overflow | T0CNT3 | T0CNT2 | 110H | High/low |
| 6 | Timer T1 | Internal | 002BH | Timer T1L overflow | T1CNT1 | T1CNT0 | 118H | High/low |
|  |  |  |  | Timer T1H overflow | T1CNT3 | T1CNT2 |  |  |
| 7 | SIO0 | Internal | 0033H | SIO0 end detection | SCON01 | SCON00 | 130H | High/low |
| 8 | SIO1 | Internal | 003BH | SIO1 end detection | SCON11 | SCON10 | 134H | High/low |
| 9 | VMS interrupt | Internal | 0043H | VMS communications reception end detection | RFB | RFBENA | 160H/161H | High/low |
| 10 | Port 3 interrupt (P32INT = 1) | external | 004BH | Port 3 low level detection | P31INT | P30INT | 14EH | High/low |

Note:

- The "priority ranking" indicates the order of priority given to interrupts when multiple interrupts are generated simultaneously. However, the priority ranking changes when specified in the Interrupt Priority Control Register (IP).

    - Interrupt Function Operation

- If an interrupt is generated from an interrupt request source that is shown in Table 4-1-1, the corresponding interrupt request flag is set.
If the interrupt request enable flag that corresponds to the interrupt request source is set, the microcomputer's interrupt control circuit is notified of the interrupt request.

- The interrupt control circuit accepts the interrupt according to the priority ranking rules.

Interrupts can have a priority of either "highest level," "high level," or "low level;" in order to enable a high level or low level interrupt, it is necessary to set the master interrupt enable flag (IE7) in addition to the individual interrupt enable flags. IE7 controls high level and low level interrupts. In addition, if "highest level" has been set for INT0 or INT1 by the interrupt priority control flags (IE1, 0), interrupt processing is executed regardless of the master interrupt enable flag.

- The interrupt sources with an interrupt priority ranking from 3 to 9 can be specified as having either "high level" or "low level" interrupt priority according to the Interrupt Priority Control Register (IP).

- If an interrupt is generated, then after execution of the instruction that is currently being executed is completed, the interrupt control circuit automatically stores the contents of the program counter (PC) in the stack (in RAM), and then the microcomputer executes the interrupt service program. Because the program counter data uses two bytes of the stack, the stack pointer (SP) is incremented by 2.  After control returns from the interrupt service program, the SP is decremented by 2.

- After executing the interrupt service program, the microcomputer executes the RETI instruction in order to resume execution of the original program.

- Up to three interrupts can be nested.

- Interrupt request flag acceptance processing is not performed while executing the RETI instruction, while executing any instructions (such as MOV or ST) that write to the Special Function Registers listed below, or while writing to the data area EEPROM:

- IE

- IP

- PCON

- EXT

In order to use the interrupt function, it is necessary to manipulate the following Special Function Registers:

- IE

- IP

- SP (because it is undefined after a reset)

- Special Function Registers in the function block that accepts interrupts

    - Circuit Configuration

Fig. 4-1-1.  Interrupt Function Block Diagram

    - Related Registers

- Master Interrupt Enable Control Register (IE)

For details, refer to Chapter 3, section 3.7.3, "Master Interrupt Enable Control Register."

| Symbol | Address | R/W | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IE | 108H | R/W | IE7 | - | - | - | - | - | IE1 | IE0 |
| After reset |  |  | 0 | H | H | H | H | H | 0 | 0 |

| Bit name | Function |  |  |  |
| --- | --- | --- | --- | --- |
| IE7 (bit 7) | Master interrupt enable control (high level, low level) |  |  |  |
|  | 0: All interrupt requests disabled 1: All interrupt requests enabled |  |  |  |
| IE1 (bit 1) IE0 (bit 0) | INT0, INT1 interrupt priority control |  |  |  |
|  | IE1 | IE0 | INT1 priority level | INT0 priority level |
|  | 0 1 X | 0 0 1 | Highest level Low level Low level | Highest level Highest level Low level |

- Interrupt Priority Ranking Control Register (IP)

| Symbol | Address | R/W | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IP | 109H | R/W | IP7 | IP6 | IP5 | IP4 | IP3 | IP2 | IP1 | IP0 |
| After reset |  |  | 0 | - | 0 | 0 | 0 | 0 | 0 | 0 |

| Bit name | Function |
| --- | --- |
| IP7 (bit 7) | Port 3 interrupt priority level setting |
|  | 0: Low level 1: High level |
| IP5 (bit 5) | SIO1 interrupt priority level setting |
|  | 0: Low level 1: High level |
| IP4 (bit 4) | SIO0 interrupt priority level setting |
|  | 0: Low level 1: High level |
| IP3 (bit 3) | T1 priority level setting |
|  | 0: Low level 1: High level |
| IP2 (bit 2) | T0H priority level setting |
|  | 0: Low level 1: High level |
| IP1 (bit 1) | INT3 and base timer interrupt priority level setting |
|  | 0: Low level 1: High level |
| IP0 (bit 0) | INT2 and T0L interrupt priority level setting |
|  | 0: Low level 1: High level |

IP7 (bit 7):	Port 3 interrupt priority level setting

This bit selects either "high" (1) or "low" (0) for the port 3 interrupt priority level.  When this bit is set to "1," the priority level for this interrupt is set to "high level," giving this interrupt higher priority than low level INT0 and INT1 interrupts (IE0 = 1). When this bit is set to "0," the priority level for this interrupt is set to "low level."

IP5 (bit 5):	SIO1 interrupt priority level setting

This bit selects either "high" (1) or "low" (0) for the SIO1 interrupt priority level.  When this bit is set to "1," the priority level for this interrupt is set to "high level," giving this interrupt higher priority than low level INT0 and INT1 interrupts (IE0 = 1).  When this bit is set to "0," the priority level for this interrupt is set to "low level."

IP4 (bit 4):	SIO0 interrupt priority level setting

This bit selects either "high" (1) or "low" (0) for the SIO0 interrupt priority level.  When this bit is set to "1," the priority level for this interrupt is set to "high level," giving this interrupt higher priority than low level INT0 and INT1 interrupts (IE0 = 1). When this bit is set to "0," the priority level for this interrupt is set to "low level."

IP3 (bit 3):	T1 priority level setting

This bit selects either "high" (1) or "low" (0) for the T1 interrupt priority level.  When this bit is set to "1," the priority level for this interrupt is set to "high level," giving this interrupt higher priority than low level INT0 and INT1 interrupts (IE0 = 1). When this bit is set to "0," the priority level for this interrupt is set to "low level."

IP2 (bit 2):	T0H priority level setting

This bit selects either "high" (1) or "low" (0) for the T0H interrupt priority level.  When this bit is set to "1," the priority level for this interrupt is set to "high level," giving this interrupt higher priority than low level INT0 and INT1 interrupts (IE0 = 1). When this bit is set to "0," the priority level for this interrupt is set to "low level."

IP1 (bit 1):	INT3 and base timer interrupt priority level setting

This bit selects either "high" (1) or "low" (0) for the INT3/base timer interrupt priority level. When this bit is set to "1," the priority level for this interrupt is set to "high level," giving this interrupt higher priority than low level INT0 and INT1 interrupts (IE0 = 1). When this bit is set to "0," the priority level for this interrupt is set to "low level."

IP0 (bit 0):	INT2 and T0L interrupt priority level setting

This bit selects either "high" (1) or "low" (0) for the INT2/T0L interrupt priority level. When this bit is set to "1," the priority level for this interrupt is set to "high level," giving this interrupt higher priority than low level INT0 and INT1 interrupts (IE0 = 1). When this bit is set to "0," the priority level for this interrupt is set to "low level."

    - Interrupt Priority Ranking

The priority ranking of the interrupt levels is as follows:

Highest level > high level > low level

The priority ranking of multiple interrupt sources of the same priority ranking level that are generated simultaneously is as listed in Table 4-1-1. In addition, the overlapping interrupt control circuit controls overlapping interrupts, permitting nesting of "low level"  "high level"  "highest level" interrupt routines.

Highest level:	External interrupts INT0 and INT1 (when set to "highest level")

This level is not controlled by the mask interrupt enable flag (IE7).

High level:	Those interrupt sources other than INT0 and INT1 that correspond to the bits that are set in the interrupt priority control register (IP).

This level is controlled by the mask interrupt enable flag (IE7).

Low level:	Those interrupt sources INT0 and INT1 for which "low level" is set in IE0 or IE1, and those interrupt sources other than INT0 and INT1 that correspond to the bits that are reset (0) in the interrupt priority control register (IP).

This level is controlled by the mask interrupt enable flag (IE7).

- To give the SIO1 end interrupt higher priority than the INT0 interrupt, set IE0 to "1" and IP5 to "1." (IE0 = 1, IP = 00100000B)

- To give the SIO1 end interrupt priority between the INT2 interrupt and the INT0 interrupt, set IE0 to "1" and IP5 and IP0 to "1." (IE0 = 1, IP = 00100001B)

- Notes concerning overlapping interrupts

- When a low-level interrupt request is generated while executing the service program for a high-level interrupt, the low-level interrupt is accepted after one instruction is executed after the end of the service program for the high-level interrupt.

- When an interrupt request of the same level as an interrupt request for which a service program is already being executed is generated, that second interrupt request is not accepted.