- Chapter 2.

Internal System Configuration

  - Memory Space

POTATO has an internal memory space and a flash memory space.

The internal memory space includes a program memory space (64K) and a data memory space (512 bytes), while all 64K of program memory space (internal program ROM) can be accessed straight through by incrementing the pointer sequentially each time that a normal instruction is executed. Addresses 000 to 0FFH in the data memory space are allocated for 256 bytes of data memory (internal RAM). In addition, the 256 bytes from 100 to 1FFH are allocated for the Special Function Registers (SFR). Internal RAM consists of two banks, with the bank being specified by bit 1 (RAMBK0) of the Program Status Word (PSW) in the Special Function Registers (SFR). BANK 0 is also used as the stack area. The accumulator (ACC), PSW, timer, and input/output ports are allocated by the SFR, for a complete memory-mapped I/O configuration.

The flash memory space consists of a 128K space. This space consists of two 64K banks. Furthermore, only BANK 0 can be handled as a 64K external program memory, and a special macro instruction (CHANGE) is used to switch between internal and external programs. Writing data to the flash memory space must always be performed by calling a BIOS program subroutine.

The BIOS program memory space contains (as subroutines) a program for writing data to flash memory and checking the data that was written, and a preparation program for writing the data. This area is a BIOS-dedicated program area.

VMS game programs must be stored in the external program area (flash memory BANK 0).

Fig. 2-1-1.  Memory Space

  - Program Counter (PC)

The Program Counter (PC) consists of 16 bits, and stores the program memory (ROM) address that contains the instruction that should be executed next. The processor executes the instructions in a program on the basis of the value in the PC. Normally, the PC is incremented for each instruction that is executed, but when a branch instruction or a subroutine instruction is executed, an interrupt is received, or a reset is executed, a value that is appropriate for that operation is set in the PC.

The data that is set in the PC for each operation is shown in Table 2-2-1.

Table 2-2-1.  Program Counter Settings

| Type of operation |  | Program counter value |
| --- | --- | --- |
| Reset |  | 0000H (internal program space) |
| External interrupt 0 |  | 0003H |
| External interrupt 1 |  | 000BH |
| External interrupt 2, timer counter T0L interrupt |  | 0013H |
| External interrupt 3, base timer interrupt |  | 001BH |
| Timer/counter T0H interrupt |  | 0023H |
| Timer T1L, timer T1H interrupt |  | 002BH |
| SIO0 interrupt |  | 0033H |
| SIO1 interrupt |  | 003BH |
| VMS SI0 interrupt |  | 0043H |
| Port 3 interrupt |  | 004BH |
| Unconditional branching instructions | JMP a12 | PC15 to 12 = Current page PC11 to 00 = a12 |
|  | JMPF a16 | PC15 to 00 = a16 |
|  | BR r8 | (PC + 2) + r8[-128～ + 127] |
|  | BRF r16 | (PC + 2) + r16[0~ + 65535] |
| Conditional branching instructions | BZ, BNZ, BP, BNE BPC, BN, DBNZ, BE | (PC + 2 or + 3) + r8[-128～ + 127] |
| CALL instructions | CALL a12 | PC15 to 12 = Current page PC11 to 00 = a12 |
|  | CALLF a16 | PC15 to 00 = a16 |
|  | CALLR r16 | (PC + 2) + r16[0~ + 65535] |
| Macro instructions | CHANGE label name (or address) | Value specified by label or address from a different program mode |

- NOTES:

- For the sake of convenience, each 4K of ROM space is called a "page."

- The "current page" is the page in the ROM space that includes the instruction that follows the instruction that is currently being executed.

- If an interrupt is generated while an internal program is running, a subroutine is called using the setting indicated in the above Table 2-2-1 in the internal program space. If an interrupt is generated while an external program is running, a subroutine is called in bank 0 of the external program space, with the lower 16 bits being the value in the above table. In a game program, some interrupt vectors can not be set as desired. it is always necessary to incorporate the specified program. Refer to Chapter 4, section 4.1, "Interrupt Function."

  - Internal Program ROM

The 64K program memory space includes 16K of user program memory and 4K of BIOS program memory (ROM). In addition, the 256 bytes from FF00H to FFFFH in the same space as the ROM are used as an option specification area for creating the mask version.

Fig. 2-3-1  ROM Space

  - Internal Data Memory

POTATO has 1222 bytes of built-in data memory (RAM), which includes 198 bytes of XRAM and 512 bytes of VTRBF. In addition, the Special Function Registers (SFR) reside at 100H to 1FFH.

Table 2-4-1.  Data Memory by Type (RAM)

| Type | POTATO |
| --- | --- |
| RAM size | 1222 bytes |
| XRAM | Bank 0 180H to 1FBH (96 bytes) |
|  | Bank 1 180H to 1FBH (96 bytes)） |
|  | Bank 2 180H to 185H (6 bytes) |
| Main RAM | Bank 0 000H to 0FFH (256 bytes) |
|  | Bank 1 000H to 0FFH (256 bytes) |
| VTRBF | 166H (256 bytes x 2 banks) |

The 16 byte area from 00H to 0FH in RAM consists of four banks of indirect address registers: @R0, @R1 (for RAM), @R2 and @R3 (for SFR), starting from the low address. The indirect address register bank that is used for addressing is specified by bits 3 and 4 (indirect address register bank flags: IRBK0, 1) of the Program Status Word (PSW). In addition, this 16-byte area can also be used as normal RAM.

The relationship between the indirect address registers and data memory is shown in Table 2-4-2.

Fig. 2-4-1.  Arrangement of Indirect Address Registers

Table 2-4-2.  Indirect Address Register Map

| Indirect address register name | Function | Bank 0 (IRBK1=0) (IRBK0=0) | Bank 1 (IRBK1=0) (IRBK0=1) | Bank 2 (IRBK1=1) (IRBK0=0) | Bank 3 (IRBK1=1) (IRBK0=1) |
| --- | --- | --- | --- | --- | --- |
| @R0 | RAM access | RAM 00H | RAM 04H | RAM 08H | RAM 0CH |
| @R1 | RAM access | RAM 01H | RAM 05H | RAM 09H | RAM 0DH |
| @R2 | SFR access | RAM 02H | RAM 06H | RAM 0AH | RAM 0EH |
| @R3 | SFR access | RAM 03H | RAM 07H | RAM 0BH | RAM 0FH |

- Table 2-4-3 shows a data memory list. Refer to the respective items for details on the contents of each register.

Table 2-4-3.  Data Memory Map

R=READ	X=Undefined

W=WRITE	H=Does not exist

| Symbol | Address | R/W | Name | Initial value |
| --- | --- | --- | --- | --- |
| RAM(BANK0) | 000H-0FFH | R/W | Data memory | XXXXXXXX (retained after reset) |
| RAM(BANK1) | 000H-0FFH | R/W | Data memory | XXXXXXXX (retained after reset) |
| ACC | 100H | R/W | Accumulator | 00000000 |
| PSW | 101H | R/W | Program Status Word | 00000000 |
| B | 102H | R/W | B register | 00000000 |
| C | 103H | R/W | C register | 00000000 |
| TRL | 104H | R/W | Table reference register - low byte | 00000000 |
| TRH | 105H | R/W | Table reference register - high byte | 00000000 |
| SP | 106H | R/W | Stack pointer | XXXXXXXX |
| PCON | 107H | R/W | Power control register | HHHHHH00 |
| IE | 108H | R/W | Master interrupt enable control register | 0HHHHH00 |
| IP | 109H | R/W | Interrupt priority control register | 00000000 |
| EXT | 10DH | R/W | External memory control register | HHHH0000 |
| OCR | 10EH | R/W | Oscillation control register | 0H00HH00 |
| T0CNT | 110H | R/W | Timer 0 control register | 00000000 |
| T0PRR | 111H | R/W | Timer 0 prescaler data | 00000000 |
| T0L | 112H | R | Timer 0 low | 00000000 |
| T0LR | 113H | R/W | Timer 0 low reload data | 00000000 |
| T0H | 114H | R | Timer 0 high | 00000000 |
| T0HR | 115H | R/W | Timer 0 high reload data | 00000000 |
| T1CNT | 118H | R/W | Timer 1 control register | 00000000 |
| T1LC | 11AH | R/W | Timer 1 low compare data | 00000000 |
| T1L | 11BH | R | Timer 1 low | 00000000 |
| T1LR | 11CH | W | Timer 1 low reload data | 00000000 |
| T1HC |  | R/W | Timer 1 high compare data | 00000000 |
| T1H | 11DH | R | Timer 1 high | 00000000 |
| T1HR |  | W | Timer 1 high reload data | 00000000 |
| MCR | 120H | W | Mode control register | 00000000 |
| STAD | 122H | R/W | Start address register | 00000000 |
| CNR | 123H | W | Character count register | H0000000 |
| TDR | 124H | W | Time interrupt register | HH000000 |
| XBNK | 125H | R/W | Bank address register | H0000000 |
| VCCR | 127H | W | LCD contrast control register | 00000000 |
| SCON0 | 130H | R/W | SIO0 control register | 00H00000 |
| SBUF0 | 131H | R/W | SIO0 buffer | 00000000 |
| SBR | 132H | R/W | SIO baud rate generator | 00000000 |
| SCON1 | 134H | R/W | SIO1 control register | H0H00000 |
| SBUF1 | 135H | R/W | SIO1 buffer | 00000000 |
| Delete |  |  |  |  |
| Delete |  |  |  |  |

- Continued on next page

Table 2-4-3.  Data Memory Map (Continued)

| Symbol |  | Address |  | R/W |  | Name |  | Initial value |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P1 | 144H |  | R/W |  | Port 1 latch |  | 00000000 |  |
| P1DDR | 145H |  | W |  | Port 1 data direction register |  | 00000000 |  |
| P1FCR | 146H |  | W |  | Port 1 function control register |  | 00000000 |  |
| P3 | 14CH |  | R/W |  | Port 3 latch |  | 00000000 |  |
| P3DDR | 14D |  | W |  | Port 3 data direction register |  | 00000000 |  |
| P3INT | 14EH |  | R/W |  | Port 3 interrupt control register |  | HHHHH000 |  |
| P7 | 15CH |  | R |  | Port 7 latch |  | HHHHXXXX |  |
| I01CR | 15DH |  | R/W |  | External interrupt 0, 1 control |  | 00000000 |  |
| I23CR | 15EH |  | R/W |  | External interrupt 2, 3 control |  | 00000000 |  |
| ISL | 15FH |  | R/W |  | Input signal selection |  | HH000000 |  |
| VSEL | 163H |  | R/W |  | Control register |  | HHH0HH00 |  |
| VRMAD1 | 164H |  | R/W |  | System address register 1 |  | 00000000 |  |
| VRMAD2 | 165H |  | R/W |  | System address register 2 |  | HHHHHHH0 |  |
| VTRBF | 166H |  | R/W |  | TX/RX buffer |  | XXXXXXXX |  |
| BTCR | 17FH |  | R/W |  | Base timer control |  | 00000000 |  |
| RAM(XRAM) (BANK0) | 180H-1FBH |  | R/W |  | LCD memory |  | XXXXXXXX (retained after reset) |  |
| RAM(XRAM) (BANK1) | 180H-1FBH |  | R/W |  |  |  |  |  |
| RAM(XRAM) (BANK2) | 180H-185H |  | R/W |  |  |  |  |  |

(1) In direct addressing mode

When executing an instruction such as:  MOV #i8, d9

¬ In RAM Bank 0 (PSW 21 = 0)                             In RAM Bank 1 (PSW 21 = 1)

(2) Indirect addressing mode

When executing an instruction such as:  MOV #i8, @Rj

¬ RAM BANK0 (PSW 21=0)                                   RAM BANK1 (PSW 21=1)

  - Flash Memory

POTATO has a 128K flash memory space.

The flash memory space consists of two 64K banks, and data can be written to and read from flash memory by referencing the BIOS program. In addition, the data in the ROM space in each bank can be referenced by using the ROM table reference instruction (LDC). Caution is required because the LDC instruction operates differently with an internal program as compared to with an external program. In addition, Bank 0 (only) can be used as a 64K external program memory space. The dedicated macro instruction CHANGE is used to switch between internal and external programs.

Flash memory size:	64K 2 banks

Banks:	Bank 0 and Bank 1

Addresses in each bank:	0000H to FFFFH

Writing and reading the flash memory space 9including external program memory) is accomplished through the BIOS program. For details, refer to Chapter 3, section 3.10, "Flash EEPROM."

  - Accumulator

The accumulator is an 8-bit register that is used when performing arithmetic operations on data, transfers, input/output, and other processing. The accumulator is allocated in address 100H in the data memory space, and is initialized to 00H when a reset is executed.

- Accumulator (ACC)

| Symbol | Address | R/W | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACC | 100H | R/W | ACC7 | ACC6 | ACC5 | ACC4 | ACC3 | ACC2 | ACC1 | ACC0 |
| After reset |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

  - B Register, C Register

The B register and C register are 8-bit registers that are used in combination with the ACC to set data for arithmetic operations and to store the results of arithmetic operations when executing multiplication/division instructions.

These registers are allocated to addresses 102H and 103H in the data memory space, and are initialized to 00H when a reset is executed.

- B register (B)

| Symbol | Address | R/W | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B | 102H | R/W | B7 | B6 | B5 | B4 | B3 | B2 | B1 | B0 |
| After reset |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

- C register (C)

| Symbol | Address | R/W | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C | 103H | R/W | C7 | C6 | C5 | C4 | C3 | C2 | C1 | C0 |
| After reset |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

When performing multiplication, the multiplicand consists of 16 bits with the upper 8 bits stored in the ACC and the lower 8 bits stored in the C register, and the multiplier consists of 8 bits, stored in the B register.

The result of the operation (the product) consists of 24 bits, with the highest 8 bits stored in the B register the middle 8 bits stored in the ACC, and the lower 8 bits stored in the C register. In other words, (ACC)(C)  (B) = (B)(ACC)(C).

When performing division, the dividend consists of 16 bits with the upper 8 bits stored in the ACC and the lower 8 bits stored in the C register, and the divisor consists of 8 bits, stored in the B register.

The result of the operation (the quotient) consists of 16 bits, with the highest 8 bits stored in the ACC, the lower 8 bits stored in the C register and the remainder stored in the B register. In other words, (ACC)(C)  (B) = (ACC)(C) mod (B).

  - Program Status Word

The Program Status Word (PSW) consists of flags that indicate the results of arithmetic operations, and flags that specify banks for data memory (RAM) and indirect address registers. The PSW is allocated to address 101H in the data memory space, and each bit is initialized to "0" when a reset is executed.

- Program Status Word (PSW)

| Symbol | Address | R/W | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSW | 101H | R/W | CY | AC | - | IRBK1 | IRBK0 | OV | RAMBK0 | P |
| After reset |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

CY (bit 7):	Carry flag

CY is set (1) when there is a carry in the result of an arithmetic operation, or when a borrow is generated in a subtraction or compare operation; CY is reset (0) when neither of these events occurs. This bit is affected if a rotate instruction that includes CY is executed, and is reset (0) when a multiplication or division instruction is executed.

AC (bit 6):	Auxiliary carry flag

AC is set (1) when there is a carry from bit 3 of the ACC in the result of an arithmetic operation, or when a borrow is generated in a subtraction operation; CY is reset (0) when neither of these events occurs.

IRBK1 (bit 4):	Indirect address register bank flag 1

IRBK0 (bit 3):	Indirect address register bank flag 0

These bits specify one of the four banks that comprise the register group that is to be used as the indirect address register for indirect addressing instructions within each bank of data memory (RAM).

| Bank | IRBK1 | IRBK0 |
| --- | --- | --- |
| 0 | 0 | 0 |
| 1 | 0 | 1 |
| 2 | 1 | 0 |
| 3 | 1 | 1 |

OV (bit 2):	Overflow flag

OV is set (1) if an overflow occurs and is reset (0) if it does not.

This bit is set (1) if the act of adding a negative number to a negative number or of subtracting a positive number from a negative number generates a positive number, or if the act of adding a positive number to a positive number or subtracting a negative number from a positive number results in a negative number. In all other cases, this bit is reset (0). This bit is also set (1) when the contents of the B register are not "0" after a multiplication or division operation, and is reset if the contents of the B register are "0."

RAMBK0 (bit 1):	Data memory (RAM) bank flag

This flag specifies the data memory (RAM) bank. When an instruction that accesses RAM is executed, the RAM address in the specified bank is accessed.

| Bank | RAMBK0 |
| --- | --- |
| 0 | 0 |
| 1 | 1 |

P (bit 0):	This bit indicates the accumulator parity (ACC).

This bit is set (1) if the total number of bits that are set in the accumulator is odd, and reset (0) if the total number of bits that are set in the accumulator is even. This bit is a read-only bit. This bit cannot be written.

  - Stack Pointer

Bank 0 of data RAM is used as stack memory. The Stack Pointer (SP) is an 8-bit register that specifies an address in this stack area.

The SP is allocated in address 106H in the data memory space. The SP is incremented right before a save to the stack memory, and is decremented after data is returned from the stack memory.

When a reset is executed, the value of the SP is undefined, and must be initialized at the beginning of the BIOS program.

- Stack Pointer (SP)

| Symbol | Address | R/W | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SP | 106H | R/W | SP7 | SP6 | SP5 | SP4 | SP3 | SP2 | SP1 | SP0 |
| After reset |  |  | X | X | X | X | X | X | X | X |

- When executing a PUSH instruction, the SP is incremented and the data in data memory specified by the operand is pushed onto the stack. When executing a POP instruction, after the data is stored in data memory specified by the operand, the SP is decremented.

- Note that if a PUSH or POP instruction is executed after bank 1 is specified, the RAM in bank 0 specified by the Stack Pointer (SP) is still used as the stack memory. In addition, if the operand is a RAM address, bank 0 RAM, not bank 1 RAM, is accessed.

- When executing a CALL instruction, the SP is incremented, the lower 8 bits of the PC are saved in the stack, the SP is incremented again, and the upper 8 bits of the PC are saved. When the RET instruction is executed, the data specified by the SP is stored in the upper 8 bits of the PC, the SP is decremented, the data specified by the new value of the SP is stored in the lower 8 bits of the PC, and then the SP is decremented again.

- When an interrupt is accepted, the SP is incremented, the lower 8 bits of the PC are saved in the stack, the SP is incremented again, and the upper 8 bits of the PC are saved. When the RETI instruction is executed in order to return from the interrupt processing, the data specified by the SP is stored in the upper 8 bits of the PC, the SP is decremented, the data specified by the new value of the SP is stored in the lower 8 bits of the PC, and then the SP is decremented again.

  - The Table Reference Register (TRR)

The Table Reference Register (TRR) is a 16-bit register that specifies addresses for program memory (ROM) and flash memory (EEPROM). The low byte (TRL) is allocated to address 104H in the data memory space, and the high byte (TRH) is allocated to address 105H in the data memory space. This register is initialized to 00H when a reset is executed.

When executing the table reference instruction (LDC), the data stored in the TRR is added to the data stored in the ACC, and then the data that is read from the address indicated by the combined value is transferred to the ACC.

When a flash memory write/read is executed (through the BIOS program), the address indicated by the data stored in the TRR is referenced in the specified bank.

- Table Reference Register (Low) TRL

| Symbol | Address | R/W | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TRL | 104H | R/W | TRL7 | TRL6 | TRL5 | TRL4 | TRL3 | TRL2 | TRL1 | TRL0 |
| After reset |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

- Table Reference Register (High) TRH

| Symbol | Address | R/W | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TRH | 105H | R/W | TRH7 | TRH6 | TRH5 | TRH4 | TRH3 | TRH2 | TRH1 | TRH0 |
| After reset |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

  - CHANGE Instruction

Switching between the internal and external program is accomplished by executing the CHANGE instruction. When executed while currently running an internal program (game program), the mode changes to game program (internal program), and the program counter is set to the address that is specified by the label or address.

- Format:

- Operation:

(1) Execution while in internal program mode

- The mode changes from internal program mode to game program mode.

- The program counter is set to the address in the game program specified by the label or address.

(2) Execution while in game program mode

- The mode changes from game program mode to internal program mode.

However, if bit 1 (LDCEXT) of the external memory control register is set, executing the CHANGE instruction has no effect; the system remains in game program mode, and does not change to the internal program.

- The program counter is set to the address in the internal program specified by the label or address.

(3) The change in program mode occurs after a dedicated macro instruction is executed.

- Example: