# 4

Chapter 1.

Overview

  - General

The POTATO custom chip that is the core of the VMS (Visual Memory System), the memory system for our next-generation game machine, consists of a CPU core that operates with a minimum bus cycle time of 0.5[micro]s, 128K of flash EEPROM, 20K of ROM, 710 bytes of RAM, a dot-matrix LCD automatic display controller/driver, a 16-bit timer/counter/pulse generator (or a two-channel 8-bit timer), a 16-bit timer (or a two-channel 8-bit timer), a two-channel 8-bit synchronous serial interface, a dedicated interface for the next-generation game machine, and an interrupt function with 13 sources and 10 vectors.

  - Features

- Flash EEPROM

65,536 8 bits:	Program/data area

65,536 8 bits:	Data area

- ROM

16,384 8 bits:	Program area

4096 8 bits:	BIOS program area

- RAM

256 8 bits×2 banks:	Calculation area

198 8 bits:	Display area

256 8 bits x 2 banks*:	Work area

*	When connected with the next-generation game machine, this area is used as a TX/RX buffer for the dedicated interface.

- Bus cycle time/instruction cycle time

The bus cycle time indicates the ROM read time.

| Bus cycle time | Instruction cycle time | System clock source | Oscillating frequency | Supply voltage | Miscellaneous |
| --- | --- | --- | --- | --- | --- |
| 0.5μs | 1.0μs | CF oscillation | 6MHz | 3.15～3.8Ｖ | OCR7=1 *1 |
| 3.8μs | 7.5μs | Internal RC oscillation | 800KHz | 3.15～3.8Ｖ | OCR7=1 *1 |
| 91.5μs | 183.0μs | Crystal (X'tal) oscillation | 32KHz | 3.15～3.8Ｖ | OCR7=1 *1 |

*1）	OCR7: This is bit 7 of the Oscillation Control Register (OCR); this bit controls the system clock generation circuit and the cycle time. Refer to Chapter 4, section 4.2.4, "Related Registers," for further details.

OCR7 = 1: The cycle time is the system clock divided by 6.

- Ports

- Input/output ports:	2 ports (P1, P3)

- Input port:	1 port (P7)

- Segment output port for driving the LCD:	48 lines

- Common output port for driving the LCD:	33 lines

- LCD controller

Display duty:	1/33 duty

Display bias:	1/5 bias

Liquid crystal instruction display:	On/Off

Clock for external voltage step-up (external step-up circuit)

Graphic display:	1584-dot maximum display

- Serial interfaces

8-bit serial interface 2 channels (synchronous)

Built-in 8-bit baud rate generator (The baud rate generator is shared with a two-channel serial interface.)

Dedicated next-generation game interface (start pattern/end pattern auto discrimination)

Note:	The synchronous serial interface and the dedicated next-generation game interface cannot be used simultaneously.

- Timer

Timer 0:	16-bit timer/counter

Built-in 2-bit prescaler + 8-bit programmable prescaler

Timer 1:	16-bit timer/pulse generator

Base timer:	Clock selection function

32.768kHz crystal oscillation, system clock, or timer 0 is selected through the programmable prescaler output.

500ms overflow signal generation function for clock (when 32.768kHz crystal oscillation is selected)

Function that generates an overflow signal on every cycle of either 976[micro]s, 3.9ms, 15.6ms, or 62.5ms (when 32.768kHz crystal oscillation is selected)

- Interrupts

13 sources, 10 vectors

(1)	External interrupt INT0

(2)	External interrupt INT1

(3)	External interrupt INT2, timer counter T0L (timer 0, lower 8 bits)

(4)	External interrupt INT3, base timer

(5)	Timer/counter T0H (timer 0, upper 8 bits)

(6)	Timer T1L (timer 1, lower 8 bits), timer T1H (timer 1, upper 8 bits)

(7)	Serial interface 0 (SIO0)

(8)	Serial interface 1 (SIO1)

(9)	Dedicated next-generation game machine interface

(10)	Port 3

Built-in interrupt priority register

The microcontroller interrupts can be weighted as one of three levels: low level, high level, and highest level. The 11 interrupts sources from "external interrupt INT2, timer counter T0L (timer 0, lower 8 bits)" to "Port 3" can be specified as having an interrupt priority level of either "low level" or "high level." In addition, external interrupts INT0 and INT1 can be specified as having an interrupt priority level of either "low level" or "highest level.

- Subroutine stack level

A maximum of 128 levels (The stack is set in RAM.)

- Built-in fast multiplication and division instructions

16 bits 8 bits (execution time: 7 instruction cycles)

16 bits 8 bits (execution time: 7 instruction cycles)

- Three types of oscillation circuits

RC oscillation circuit (built in):	System clock (resistor R and capacitor C built in, no external circuits required)

CF oscillation circuit:	System clock

Crystal oscillation circuit:	Clock, system clock, LCD clock

- Standby functions

HALT mode

This mode halts instruction execution, and can be cancelled by either a reset or by the generation of an interrupt.

HOLD mode

This mode halts CF oscillation, RC oscillation, and crystal oscillation. There are three methods for canceling HOLD mode:

(1)	Input a low level signal to the reset pin.

(2)	Input the specified level to either the P70/INT0 pin or the P71/INT1 pin.

(3)	Input the port 3 interrupt condition.

- Flash EEPROM

Data memory space:	128K bytes

Data memory space overwriting:	Used for BIOS program

Overwriting block size:	128K bytes

Erase/program voltage:	3.15 to 3.8V

Number of times overwriting is possible:	50,000 times (writing FF and 00 once each) (Ta = 25°C, memory management by program)

Program memory space:	64K bytes

Switching between mask ROM/flash EEPROM program space

Use CHANGE instruction; initially:	Mask ROM

Switching from flash EEPROM program space to mask ROM program space can be permitted/prohibited (EXT register)

  - System Block Diagram

Fig. 1-3-1.  System Block Diagram