  - Reset Function

    - Overview

The reset function initializes the microcomputer when the power is turned on or while the microcomputer is running.

    - Function

The microcomputer is equipped with the following two functions.

・External reset function through RES pin

A reset can definitely be initiated by applying a low-level signal to the RES pin for at least 200[micro]s. However, it is important to note that applying a low-level signal for a shorter duration may also initiate a reset.

If a suitable time constant is connected to the RES pin externally, the RES pin can also be used to initiate the power-on reset.

The reset circuit configuration is shown in Fig. 4-4-1.

Fig. 4-4-1.  Reset Circuit Block Diagram

    - Hardware Status During a Reset

If a reset is generated through the RES pin, all of the hardware is initialized according to the reset signal, which is synchronized with the system clock.

Once a reset is initiated, the hardware is initialized immediately, even in the case of a power-on reset, because the system clock switches to internal RC oscillation. After waiting in order to allow the main clock oscillation to stabilize, the system clock switches to the main clock.

During a reset, the Program Counter is initialized to 0000H. For the initial values of the Special Function Registers, refer to Table 4-4-1, "Data Memory/Register Map."

The contents of data RAM, the stack pointer, and LCD RAM are maintained. Caution is required after a power-on reset, however, because these contents are undefined.

Table 4-4-1.  Initial Values of Each Special Function Register

| Symbol | Address | R/W | Name | Initial value |
| --- | --- | --- | --- | --- |
| RAM(BANK0) | 000H-0FFH | R/W | Data memory | XXXXXXXX (retained after reset) |
| RAM(BANK1) | 000H-0FFH | R/W | Data memory | XXXXXXXX (retained after reset) |
| ACC | 100H | R/W | Accumulator | 00000000 |
| PSW | 101H | R/W | Program Status Word | 00H00000 |
| B | 102H | R/W | B register | 00000000 |
| C | 103H | R/W | C register | 00000000 |
| TRL | 104H | R/W | Table Reference Register lower byte | 00000000 |
| TRH | 105H | R/W | Table Reference Register upper byte | 00000000 |
| SP | 106H | R/W | Stack Pointer | XXXXXXXX |
| PCON | 107H | R/W | Power Control Register | HHHHHH00 |
| IE | 108H | R/W | Master Interrupt Enable Control Register | 0HHHHH00 |
| IP | 109H | R/W | Interrupt Priority Ranking Control Register | 00000000 |
| EXT | 10DH | R/W | External Memory Control Register | HHHH0000 |
| OCR | 10EH | R/W | Oscillation Control Register | 0H00HH00 |
| T0CNT | 110H | R/W | Timer 0 Control Register | 00000000 |
| T0PRR | 111H | R/W | Timer 0 Prescaler Data | 00000000 |
| T0L | 112H | R | Timer 0 Lower | 00000000 |
| T0LR | 113H | R/W | Timer 0 Lower Reload Data | 00000000 |
| T0H | 114H | R | Timer 0 Upper | 00000000 |
| T0HR | 115H | R/W | Timer 0 Upper Reload Data | 00000000 |
| T1CNT | 118H | R/W | Timer 1 Control Register | 00000000 |
| T1LC | 11AH | R/W | Timer 1 Lower Compare Data | 00000000 |
| T1L | 11BH | R | Timer 1 Lower | 00000000 |
| T1LR |  | W | Timer 1 Lower Reload Data | 00000000 |
| T1HC | 11CH | R/W | Timer 1 Upper Compare Data | 00000000 |
| T1H | 11DH | R | Timer 1 Upper | 00000000 |
| T1HR |  | W | Timer 1 Upper Reload Data | 00000000 |
| MCR | 120H | W | Mode Control Register | 00000000 |
| STAD | 122H | R/W | Start Address Register | 00000000 |
| CNR | 123H | W | Character Count Register | H0000000 |
| TDR | 124H | W | Time Division Register | HH000000 |
| XBNK | 125H | R/W | Bank Address Register | HHHHHH00 |
| VCCR | 127H | W | LCD Contrast Control Register | 00000000 |
| SCON0 | 130H | R/W | SIO0 Control Register | 00H00000 |
| SBUF0 | 131H | R/W | SIO0 Buffer | 00000000 |
| SBR | 132H | R/W | SIO Baud Rate Generator | 00000000 |
| SCON1 | 134H | R/W | SIO1 Control Register | H0H00000 |
| SBUF1 | 135H | R/W | SIO1 Buffer | 00000000 |
| Deleted | Deleted | Deleted | Deleted | Deleted |
| Deleted | Deleted | Deleted | Deleted | Deleted |
| P1 | 144H | R/W | Port 1 latch | 00000000 |
| P1DDR | 145H | W | Port 1 Data Direction Register | 00000000 |
| P1FCR | 146H | W | Port 1 Function Control Register | 00000000 |
| P3 | 14CH | R/W | Port 3 latch | 00000000 |

Continued on next page

Initial Values of Each Special Function Register (continued)

| Symbol | Address | R/W | Name | Initial value |
| --- | --- | --- | --- | --- |
| P3DDR | 14DH | W | Port 3 Data Direction Register | 00000000 |
| P3INT | 14EH | R/W | Port 3 Interrupt Control Register | HHHHH000 |
| P7 | 15CH | R | Port 7 latch | HHHHXXXX |
| I01CR | 15DH | R/W | External Interrupt 0, 1 Control | 00000000 |
| I23CR | 15EH | R/W | External Interrupt 2, 3 Control | 00000000 |
| ISL | 15FH | R/W | Input Signal Select | HH000000 |
| VSEL | 163H | R/W | Control Register | HHH0HH00 |
| VRMAD1 | 164H | R/W | System Address Register 1 | 00000000 |
| VRMAD2 | 165H | R/W | System Address Register 2 | HHHHHHH0 |
| VTRBF | 166H | R/W | TX/RX Buffer | XXXXXXXX |
| BTCR | 17FH | R/W | Base Timer Control | 00000000 |
| RAM(XRAM) (BANK0) | 180H-1FBH | R/W | LCD display memory | XXXXXXXX (retained after reset) |
| RAM(XRAM) (BANK1) | 180H-1FBH | R/W |  |  |
| RAM(XRAM) (BANK2) | 180H-185H | R/W |  |  |