  - Arithmetic Operation Instructions

ADD  #i8  (ADD immediate data to accumulator)

| Instruction code | 1 0 0 0 0 0 0 1 i7i6i5i4i3i2i1i0 81H |
| --- | --- |
| Number of bytes | 2 |
| Number of cycles | 1 |
| Function | (ACC)(ACC)+#i8 |
| Affected flags | CY，AC，OV |
| Interrupt acceptance | Permitted |

Description:

This instruction adds the immediate data (i7 to i0) to the contents of the Accumulator (ACC), and then sends the result to the ACC.

Example:

|  |  | ACC | CY | AC | OV |
| --- | --- | --- | --- | --- | --- |
| MOV | #055H,ACC | 55H | - | - | - |
| ADD | #013H | 68H | 0 | 0 | 0 |
| ADD | #00AH | 72H | 0 | 1 | 0 |
| ADD | #00FH | 81H | 0 | 1 | 1 |
| ADD | #080H | 01H | 1 | 0 | 1 |

ADD  d9  (ADD direct byte to accumulator)

| Instruction code | 1 0 0 0 0 0 1d8 d7d6d5d4d3d2d1d0 82H83H |
| --- | --- |
| Number of bytes | 2 |
| Number of cycles | 1 |
| Function | (ACC)←(ACC)+(d9) |
| Affected flags | CY，AC，OV |
| Interrupt acceptance | Permitted |

Description:

This instruction adds the contents of data memory (RAM) or a Special Function Register (SFR), as specified by d8 to d0, to the contents of the Accumulator (ACC), and then sends the result to the ACC.

Example 1:

|  |  | ACC | RAM 23H | CY | AC | OV |
| --- | --- | --- | --- | --- | --- | --- |
| MOV | #055H,ACC | 55H | - | - | - | - |
| MOV | #068H,023H | 55H | 68H | - | - | - |
| ADD | #00CH | 61H | 68H | 0 | 1 | 0 |
| ADD | 023H | C9H | 68H | 0 | 0 | 1 |

Example 2:

|  |  | ACC | B | CY | AC | OV |
| --- | --- | --- | --- | --- | --- | --- |
| MOV | #070H,ACC | 70H | - | - | - | - |
| MOV | #095H,B | 70H | 95H | - | - | - |
| ADD | #002H | 72H | 95H | 0 | 0 | 0 |
| ADD | B | 07H | 95H | 1 | 0 | 0 |

ADD  @Rj  (ADD indirect byte to accumulator)

| Instruction code | 1 0 0 0 0 1j1j0 84H87H |
| --- | --- |
| Number of bytes | 1 |
| Number of cycles | 1 |
| Function | (ACC)(ACC)+((Rj)) j=0,1,2,3 |
| Affected flags | CY，AC，OV |
| Interrupt acceptance | Permitted |

Description:

This instruction adds the contents of data memory (RAM) or a Special Function Register (SFR), as specified by the indirect address register that is specified by j1 and j0, to the contents of the Accumulator (ACC), and then sends the result to the ACC.

Example 1:

|  |  | ACC | RAM 00H | RAM 68H | CY | AC | OV |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MOV | #055H,ACC | 55H | - | - | - | - | - |
| MOV | #068H,000H | 55H | 68H | - | - | - | - |
| MOV | #010H,@R0 | 55H | 68H | 10H | - | - | - |
| ADD | #015H | 6AH | 68H | 10H | 0 | 0 | 0 |
| ADD | @R0 | 7AH | 68H | 10H | 0 | 0 | 0 |

Example 2:

|  |  | ACC | RAM 02H | TRL | CY | AC | OV |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MOV | #0AAH,ACC | AAH | - | - | - | - | - |
| MOV | #004H,002H | AAH | 04H | - | - | - | - |
| MOV | #055H,@R2 | AAH | 04H | 55H | - | - | - |
| ADD | #001H | ABH | 04H | 55H | 0 | 0 | 0 |
| ADD | @R2 | 00H | 04H | 55H | 1 | 1 | 0 |

ADDC  #i8  (ADD immediate data and carry flag to accumulator)

| Instruction code | 1 0 0 1 0 0 0 1 i7i6i5i4i3i2i1i0 91H |
| --- | --- |
| Number of bytes | 2 |
| Number of cycles | 1 |
| Function | (ACC)(ACC)+(CY)+#i8 |
| Affected flags | CY，AC，OV |
| Interrupt acceptance | Permitted |

Description:

This instruction adds the carry flag (CY) and the immediate data (i7 to i0) to the contents of the Accumulator (ACC), and then sends the result to the ACC.

Example:

|  |  | ACC | CY | AC | OV |
| --- | --- | --- | --- | --- | --- |
| MOV | #055H,ACC | 55H | - | - | - |
| ADD | #013H | 68H | 0 | 0 | 0 |
| ADDC | #00AH | 72H | 0 | 1 | 0 |
| ADDC | #00FH | 81H | 0 | 1 | 1 |
| ADDC | #080H | 01H | 1 | 0 | 1 |
| ADDC | #001H | 03H | 0 | 0 | 0 |

ADDC  d9  (ADD direct byte and carry flag to accumulator)

| Instruction code | 1 0 0 1 0 0 1d8 d7d6d5d4d3d2d1d0 92H93H |
| --- | --- |
| Number of bytes | 2 |
| Number of cycles | 1 |
| Function | (ACC)←(ACC)+(CY)+(d9) |
| Affected flags | CY，AC，OV |
| Interrupt acceptance | Permitted |

Description:

This instruction adds the carry flag (CY) and the contents of data memory (RAM) or a Special Function Register (SFR), as specified by d8 to d0, to the contents of the Accumulator (ACC), and then sends the result to the ACC.

Example 1:

|  |  | ACC | RAM 23H | CY | AC | OV |
| --- | --- | --- | --- | --- | --- | --- |
| MOV | #055H,ACC | 55H | - | - | - | - |
| MOV | #068H,023H | 55H | 68H | - | - | - |
| ADD | #00CH | 61H | 68H | 0 | 1 | 0 |
| ADDC | 023H | C9H | 68H | 0 | 0 | 1 |
| SET1 | PSW,7 | C9H | 68H | 1 | 0 | 1 |
| ADDC | 023H | 32H | 68H | 1 | 1 | 0 |

Example 2:

|  |  | ACC | B | CY | AC | OV |
| --- | --- | --- | --- | --- | --- | --- |
| MOV | #070H,ACC | 70H | - | - | - | - |
| MOV | #095H,B | 70H | 95H | - | - | - |
| ADD | #002H | 72H | 95H | 0 | 0 | 0 |
| ADDC | B | 07H | 95H | 1 | 0 | 0 |
| ADDC | B | 9DH | 95H | 0 | 0 | 0 |

ADDC  @Rj  (ADD indirect byte and carry flag to accumulator)

| Instruction code | 1 0 0 1 0 1j1j0 94H97H |
| --- | --- |
| Number of bytes | 1 |
| Number of cycles | 1 |
| Function | (ACC)(ACC)+(CY)+((Rj)) j=0,1,2,3 |
| Affected flags | CY，AC，OV |
| Interrupt acceptance | Permitted |

Description:

This instruction adds the carry flag (CY) and the contents of data memory (RAM) or a Special Function Register (SFR), as specified by the indirect address register that is specified by j1 and j0, to the contents of the Accumulator (ACC), and then sends the result to the ACC.

Example 1:

|  |  | ACC | RAM 00H | RAM 68H | CY | AC | OV |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MOV | #055H,ACC | 55H | - | - | - | - | - |
| MOV | #068H,000H | 55H | 68H | - | - | - | - |
| MOV | #010H,@R0 | 55H | 68H | 10H | - | - | - |
| ADD | #015H | 6AH | 68H | 10H | 0 | 0 | 0 |
| ADDC | @R0 | 7AH | 68H | 10H | 0 | 0 | 0 |
| SET1 | PSW,7 | 7AH | 68H | 10H | 1 | 0 | 0 |
| ADDC | @R0 | 8BH | 68H | 10H | 0 | 0 | 1 |

Example 2:

|  |  | ACC | RAM 02H | TRL | CY | AC | OV |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MOV | #0AAH,ACC | AAH | - | - | - | - | - |
| MOV | #004H,002H | AAH | 04H | - | - | - | - |
| MOV | #055H,@R2 | AAH | 04H | 55H | - | - | - |
| ADD | #001H | ABH | 04H | 55H | 0 | 0 | 0 |
| ADDC | @R2 | 00H | 04H | 55H | 1 | 1 | 0 |
| ADDC | @R2 | 56H | 04H | 55H | 0 | 0 | 0 |

SUB  #i8  (Subtract immediate data from accumulator)

| Instruction code | 1 0 1 0 0 0 0 1 i7i6i5i4i3i2i1i0 A1H |
| --- | --- |
| Number of bytes | 2 |
| Number of cycles | 1 |
| Function | (ACC)(ACC) - #i8 |
| Affected flags | CY，AC，OV |
| Interrupt acceptance | Permitted |

Description:

This instruction subtracts the immediate data (i7 to i0) from the contents of the Accumulator (ACC), and then sends the result to the ACC.

Example:

|  |  | ACC | CY | AC | OV |
| --- | --- | --- | --- | --- | --- |
| MOV | #055H,ACC | 55H | - | - | - |
| SUB | #013H | 42H | 0 | 0 | 0 |
| SUB | #003H | 3FH | 0 | 1 | 0 |
| SUB | #03FH | 00H | 0 | 0 | 0 |
| SUB | #002H | FEH | 1 | 1 | 0 |

SUB  d9  (Subtract direct byte from accumulator)

| Instruction code | 1 0 1 0 0 0 1d8 d7d6d5d4d3d2d1d0 A2HA3H |
| --- | --- |
| Number of bytes | 2 |
| Number of cycles | 1 |
| Function | (ACC)(ACC) - (d9) |
| Affected flags | CY，AC，OV |
| Interrupt acceptance | Permitted |

Description:

This instruction subtracts the contents of data memory (RAM) or a Special Function Register (SFR), as specified by d8 to d0, from the contents of the Accumulator (ACC), and then sends the result to the ACC.

Example 1

|  |  | ACC | RAM 23H | CY | AC | OV |
| --- | --- | --- | --- | --- | --- | --- |
| MOV | #055H,ACC | 55H | - | - | - | - |
| MOV | #068H,023H | 55H | 68H | - | - | - |
| SUB | #00CH | 49H | 68H | 0 | 1 | 0 |
| SUB | 023H | E1H | 68H | 1 | 0 | 0 |

Example 2:

|  |  | ACC | RAM | CY | AC | OV |
| --- | --- | --- | --- | --- | --- | --- |
| MOV | #080H,ACC | 80H | - | - | - | - |
| MOV | #095H,B | 80H | 95H | - | - | - |
| SUB | #002H | 7EH | 95H | 0 | 1 | 1 |
| SUB | B | E9H | 95H | 1 | 0 | 1 |

SUB  @Rj  (Subtract indirect byte from accumulator)

| Instruction code | 1 0 1 0 0 1j1j0 A4HA7H |
| --- | --- |
| Number of bytes | 1 |
| Number of cycles | 1 |
| Function | (ACC)(ACC) - ((Rj)) j=0,1,2,3 |
| Affected flags | CY，AC，OV |
| Interrupt acceptance | Permitted |

Description:

This instruction subtracts the contents of data memory (RAM) or a Special Function Register (SFR), as specified by the indirect address register that is specified by j1 and j0, from the contents of the Accumulator (ACC), and then sends the result to the ACC.

Example 1:

|  |  | ACC | RAM 00H | RAM 68H | CY | AC | OV |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MOV | #055H,ACC | 55H | - | - | - | - | - |
| MOV | #068H,00H | 55H | 68H | - | - | - | - |
| MOV | #010H,@R0 | 55H | 68H | 10H | - | - | - |
| SUB | #016H | 3FH | 68H | 10H | 0 | 1 | 0 |
| SUB | @R0 | 2FH | 68H | 10H | 0 | 0 | 0 |

Example 2:

|  |  | ACC | RAM 02H | TRL | CY | AC | OV |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MOV | #0AAH,ACC | AAH | - | - | - | - | - |
| MOV | #004H,002H | AAH | 04H | - | - | - | - |
| MOV | #0AAH,@R2 | AAH | 04H | AAH | - | - | - |
| SUB | #001H | A9H | 04H | AAH | 0 | 0 | 0 |
| SUB | @R2 | FFH | 04H | AAH | 1 | 1 | 0 |

SUBC  #i8  (Subtract immediate data and carry flag from accumulator)

| Instruction code | 1 0 1 1 0 0 0 1 i7i6i5i4i3i2i1i0 B1H |
| --- | --- |
| Number of bytes | 2 |
| Number of cycles | 1 |
| Function | (ACC)(ACC) - (CY) - #i8 |
| Affected flags | CY，AC，OV |
| Interrupt acceptance | Permitted |

Description:

This instruction subtracts the carry flag (CY) and the immediate data (i7 to i0) from the contents of the Accumulator (ACC), and then sends the result to the ACC.

Example:

|  |  | ACC | CY | AC | OV |
| --- | --- | --- | --- | --- | --- |
| MOV | #055H,ACC | 55H | - | - | - |
| SUB | #013H | 42H | 0 | 0 | 0 |
| SUBC | #003H | 3FH | 0 | 1 | 0 |
| SUBC | #03FH | 00H | 0 | 0 | 0 |
| SUBC | #002H | FEH | 1 | 1 | 0 |
| SUBC | #03EH | BFH | 0 | 1 | 0 |

SUBC  d9  (Subtract direct byte and carry flag from accumulator)

| Instruction code | 1 0 1 1 0 0 1d8 d7d6d5d4d3d2d1d0 B2HB3H |
| --- | --- |
| Number of bytes | 2 |
| Number of cycles | 1 |
| Function | (ACC)(ACC) - (CY) - (d9) |
| Affected flags | CY，AC，OV |
| Interrupt acceptance | Permitted |

Description:

This instruction subtracts the carry flag (CY) and the contents of data memory (RAM) or a Special Function Register (SFR), as specified by d8 to d0, from the contents of the Accumulator (ACC), and then sends the result to the ACC.

Example 1:

|  |  | ACC | RAM 23H | CY | AC | OV |
| --- | --- | --- | --- | --- | --- | --- |
| MOV | #055H,ACC | 55H | _- | - | - | - |
| MOV | #068H,023H | 55H | 68H | - | - | - |
| SUB | #00CH | 49H | 68H | 0 | 1 | 0 |
| SUBC | 023H | E1H | 68H | 1 | 0 | 0 |
| SUBC | 023H | 78H | 68H | 0 | 1 | 1 |

Example 2:

|  |  | ACC | B | CY | AC | OV |
| --- | --- | --- | --- | --- | --- | --- |
| MOV | #080H,ACC | 80H | - | - | - | - |
| MOV | #095H,B | 80H | 95H | - | - | - |
| SUB | #002H | 7EH | 95H | 0 | 1 | 1 |
| SUBC | B | E9H | 95H | 1 | 0 | 1 |
| SUBC | B | 53H | 95H | 0 | 0 | 1 |

SUBC  @Rj  (Subtract indirect byte and carry flag from accumulator)

| Instruction code | 1 0 1 1 0 1j1j0 B4HB7H |
| --- | --- |
| Number of bytes | 1 |
| Number of cycles | 1 |
| Function | (ACC)(ACC) - (CY) - ((Rj)) j=0,1,2,3 |
| Affected flags | CY，AC，OV |
| Interrupt acceptance | Permitted |

Description:

This instruction subtracts the carry flag (CY) and the contents of data memory (RAM) or a Special Function Register (SFR), as specified by the indirect address register that is specified by j1 and j0, from the contents of the Accumulator (ACC), and then sends the result to the ACC.

Example 1:

|  |  | ACC | RAM 00H | RAM 68H | CY | AC | OV |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MOV | #055H,ACC | 55H | - | - | - | - | - |
| MOV | #068H,00H | 55H | 68H | - | - | - | - |
| MOV | #040H,@R0 | 55H | 68H | 40H | - | - | - |
| SUB | #016H | 3FH | 68H | 40H | 0 | 1 | 0 |
| SUBC | @R0 | FFH | 68H | 40H | 1 | 0 | 0 |
| SUBC | @R0 | BEH | 68H | 40H | 0 | 0 | 0 |

Example 2:

|  |  | ACC | RAM 02H | TRL | CY | AC | OV |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MOV | #0AAH,ACC | AAH | - | - | - | - | - |
| MOV | #004H,002H | AAH | 04H | - | - | - | - |
| MOV | #0AAH,@R2 | AAH | 04H | AAH | - | - | - |
| SUB | #001H | A9H | 04H | AAH | 0 | 0 | 0 |
| SUBC | @R2 | FFH | 04H | AAH | 1 | 1 | 0 |
| SUBC | @R2 | 54H | 04H | AAH | 0 | 0 | 0 |

INC  d9  (Increment direct byte)

| Instruction code | 0 1 1 0 0 0 1d8 d7d6d5d4d3d2d1d0 62H63H |
| --- | --- |
| Number of bytes | 2 |
| Number of cycles | 1 |
| Function | (d9)(d9)+1 |
| Affected flags |  |
| Interrupt acceptance | Permitted |

Description:

This instruction increments the contents of data memory (RAM) or a Special Function Register (SFR), as specified by d8 to d0.

Example 1:

|  |  | ACC |
| --- | --- | --- |
| MOV | #0FDH,ACC | FDH |
| INC | ACC | FEH |
| INC | ACC | FFH |
| INC | ACC | 00H |
| INC | ACC | 01H |

Example 2:

|  |  | RAM 7FH |
| --- | --- | --- |
| MOV | #0FDH,07FH | FDH |
| INC | 07FH | FEH |
| INC | 07FH | FFH |
| INC | 07FH | 00H |
| INC | 07FH | 01H |

Notes:

- CY, AC, and OV do not change.

- When this instruction is applied to one of the ports P0 through P5, the port latch of that port is selected; the external signal that is applied to that port is not selected.

- Furthermore, applying this instruction to port P7 does not change its status.。

INC  @Rj  (Increment indirect byte)

| Instruction code | 0 1 1 0 0 1j1j0 64H67H |
| --- | --- |
| Number of bytes | 1 |
| Number of cycles | 1 |
| Function | ((Rj))((Rj))+1 j=0,1,2,3 |
| Affected flags |  |
| Interrupt acceptance | Permitted |

Description:

This instruction increments the contents of data memory (RAM) or a Special Function Register (SFR), as specified by the indirect address register that is specified by j1 and j0.

Example 1:

|  |  | ACC | RAM 03H |
| --- | --- | --- | --- |
| MOV | #000H,003H | - | 00H |
| MOV | #0FDH,@R3 | FDH | 00H |
| INC | @R3 | FEH | 00H |
| INC | @R3 | FFH | 00H |
| INC | @R3 | 00H | 00H |

Example 2:

|  |  | RAM 7FH | RAM 01H |
| --- | --- | --- | --- |
| MOV | #07FH,001H | - | 7FH |
| MOV | #0FDH,@R1 | FDH | 7FH |
| INC | @R1 | FEH | 7FH |
| INC | @R1 | FFH | 7FH |
| INC | @R1 | 00H | 7FH |

Notes:

- CY, AC, and OV do not change.

- When this instruction is applied to one of the ports P0 through P5, the port latch of that port is selected; the external signal that is applied to that port is not selected.

- Furthermore, applying this instruction to port P7 does not change its status.

DEC  d9  (Decrement direct byte)

| Instruction code | 0 1 1 1 0 0 1d8 d7d6d5d4d3d2d1d0 72H73H |
| --- | --- |
| Number of bytes | 2 |
| Number of cycles | 1 |
| Function | (d9)(d9)－1 |
| Affected flags |  |
| Interrupt acceptance | Permitted |

Description:

This instruction decrements the contents of data memory (RAM) or a Special Function Register (SFR), as specified by d8 through d0.

Example 1:

|  |  | ACC |
| --- | --- | --- |
| MOV | #002H,ACC | 02H |
| DEC | ACC | 01H |
| DEC | ACC | 00H |
| DEC | ACC | FFH |
| DEC | ACC | FEH |

Example 2:

|  |  | RAM 7FH |
| --- | --- | --- |
| MOV | #002H,07FH | 02H |
| DEC | 07FH | 01H |
| DEC | 07FH | 00H |
| DEC | 07FH | FFH |
| DEC | 07FH | FEH |

Notes:

- CY, AC, and OV do not change.

- When this instruction is applied to one of the ports P0 through P5, the port latch of that port is selected; the external signal that is applied to that port is not selected.

- Furthermore, applying this instruction to port P7 does not change its status.

DEC  @Rj  (Decrement indirect byte)

| Instruction code | 0 1 1 1 0 1j1j0 74H77H |
| --- | --- |
| Number of bytes | 1 |
| Number of cycles | 1 |
| Function | ((Rj))((Rj)) - 1 j=0,1,2,3 |
| Affected flags |  |
| Interrupt acceptance | Permitted |

Description:

This instruction decrements the contents of data memory (RAM) or a Special Function Register (SFR), as specified by the indirect address register that is specified by j1 and j0.

Example 1:

|  |  | ACC | RAM 02H |
| --- | --- | --- | --- |
| MOV | #000H,002H | - | 00H |
| MOV | #002H,@R2 | 02H | 00H |
| DEC | @R2 | 01H | 00H |
| DEC | @R2 | 00H | 00H |
| DEC | @R2 | FFH | 00H |

Example 2:

|  |  | RAM 7FH | RAM 00H |
| --- | --- | --- | --- |
| MOV | #07FH,000H | - | 7FH |
| MOV | #002H,@R0 | 02H | 7FH |
| DEC | @R0 | 01H | 7FH |
| DEC | @R0 | 00H | 7FH |
| DEC | @R0 | FFH | 7FH |

Notes:

- CY, AC, and OV do not change.

- When this instruction is applied to one of the ports P0 through P5, the port latch of that port is selected; the external signal that is applied to that port is not selected.

- Furthermore, applying this instruction to port P7 does not change its status.

MUL  (Multiply accumulator and c register times b register)

| Instruction code | 0 0 1 1 0 0 0 0 30H |
| --- | --- |
| Number of bytes | 1 |
| Number of cycles | 7 |
| Function | (B)(ACC)(C(ACC)(C) (B) |
| Affected flags | CY，OV |
| Interrupt acceptance | Permitted after 7th cycle |

Description:

This instruction multiplies the unsigned 16-bit data that is represented by the Accumulator (ACC) and the C register (C) by the unsigned 8-bit data that is represented by the B register (B). Of the 24-bit result of the operation, the lower 8 bits are sent to C, the middle 8 bits are sent to the ACC, and the upper 8 bits are sent to B.

After the operation, if B is "0" then the overflow flag (OV) is reset, and if B is not "0" then OV is set; the carry flag (CY) is always reset.

Example 1:

|  |  | ACC | C | B | CY | AC | OV |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MOV | #0C4H,PSW | - | - | - | 1 | 1 | 1 |
| MOV | #011H,ACC | 11H | - | - | 1 | 1 | 1 |
| MOV | #023H,C | 11H | 23H | - | 1 | 1 | 1 |
| MOV | #052H,B | 11H | 23H | 52H | 1 | 1 | 1 |
| MUL |  | 7DH | 36H | 05H | 0 | 1 | 1 |

Example 2:

|  |  | ACC | C | B | CY | AC | OV |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MOV | #0C4H,PSW | - | - | - | 1 | 1 | 1 |
| MOV | #007H,ACC | 07H | - | - | 1 | 1 | 1 |
| MOV | #005H,C | 07H | 05H | - | 1 | 1 | 1 |
| MOV | #010H,B | 07H | 05H | 10H | 1 | 1 | 1 |
| MUL |  | 70H | 50H | 00H | 0 | 1 | 0 |

DIV  (Divide accumulator and c register by b register)

| Instruction code | 0 1 0 0 0 0 0 0 40H |
| --- | --- |
| Number of bytes | 1 |
| Number of cycles | 7 |
| Function | (ACC)(C),mod(B)(ACC)(C) (B) |
| Affected flags | CY，OV |
| Interrupt acceptance | Permitted after 7th cycle |

Description:

This instruction divides the 16-bit data that is represented by the Accumulator (ACC) (the upper byte) and the C register (C) (the lower byte) by the contents of the B register (B) (unsigned 8-bit data). The upper byte of the resulting quotient is sent to the ACC while the lower byte is sent to C; the remainder is sent to B. After the operation, if B is "0" then the overflow flag (OV) is set, and if B is not "0" then OV is reset; the carry flag (CY) is always reset.

Example 1:

|  |  | ACC | C | B | CY | AC | OV |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MOV | #0C4H,PSW | - | - | - | 1 | 1 | 1 |
| MOV | #078H,ACC | 79H | - | - | 1 | 1 | 1 |
| MOV | #005H,C | 79H | 05H | - | 1 | 1 | 1 |
| MOV | #007H,B | 79H | 05H | 07H | 1 | 1 | 1 |
| DIV |  | 11H | 49H | 06H | 0 | 1 | 0 |

Example 2:

|  |  | ACC | C | B | CY | AC | OV |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MOV | #0C0H,PSW | - | - | - | 1 | 1 | 0 |  |
| MOV | #007H,ACC | 07H | - | - | 1 | 1 | 0 |  |
| MOV | #010H,C | 07H | 10H | - | 1 | 1 | 0 |  |
| MOV | #000H,B | 07H | 10H | 00H | 1 | 1 | 0 |  |
| DIV |  | FFH | 10H | 00H | 0 | 1 | 1 | Error |