Vibration Function Library

""(Preliminary)

Version 0.10

Reference and Tutorial

1998/10/8 Rev.0.10

(C) SEGA Enterprises,LTD.

SYSTEM R&D Dept. S.Uchida

Table of Contents

Chapter 1	Overview of the vibration function library	3

1.1	Characteristic	3

1.2	Specification	3

1.3	How to read this document	3

Chapter 2	Enabling the vibration of "maxi"	4

2.1	Specification of "maxi"	4

2.2	Checking the connection	4

2.3	Starting the vibration	5

2.4	Parameters required for vibration, and setup example	5

2.5	Stopping the vibration	6

2.6	Setting automatic vibration stop time	6

2.7	Description of functions for vibration of "maxi"	7

Chapter 3	Supporting vibration devices other than "maxi"	10

3.1	Checking connection	10

3.2	Getting device information	10

3.2.1	Vibration device's connection direction	10

3.2.2	Vibration unit	11

3.2.3	Number of vibration units and number of vibration units that can be set at the same time	11

3.2.4	Getting the vibration unit information	12

3.2.5	Details on the vibration unit information	13

3.3	Description of functions to get information	13

3.4	Starting vibration	16

3.5	Parameters required for vibration, and setup example	17

3.6	Stopping the vibration	18

3.7	Setting automatic vibration stop time	18

3.8	Description of the vibration functions	19

Chapter 4	Optional vibration waveform	20

# Overview of the vibration function library

## Characteristic

This library is a Shinobi extension library to enable access to "maxi", as well as other vibration devices equipped with Maple vibration functions, connected to the control pad, etc.

## Specification

The following table lists the specification of this function library.  Note that the specification may be changed in the future.

| Item | Description |
| --- | --- |
| Hardware supported | Maple Bus specification-based vibration function devices |
| Number of devices supported | 8 |

Table 1-1. Specifications

## How to read this document

1.	For applications that support only "maxi"
"maxi" refers to the name of the vibration device planned for the first development and launch.  When developing an application which requires support for the "maxi" only, start reading from chapter 2.

2.	If you want to try to support all vibration devices which will be released
Before starting the vibration of a device, you need to do some basic preparation and get the information about the vibration device.  Proceed to chapters 3 and 4.

In either case, a very good knowledge of the device the application is going to handle is required.  Therefore, be sure also to refer to the specification of the device.

※The name "maxi" may be changed in the future.  This document will continue to use the name "maxi" even if the name will be changed.  Also, the "maxi" specification may be changed as well.

# Enabling the vibration of "maxi"

By using this function library in an appropriate way, you can support a variety of vibration devices connected to the controller.  The following briefly describes how to start the vibration of "maxi" (the first vibration device planned for release).

## Specification of "maxi"

The following table lists the specification required for use of the library functions to access the device.  For more detailed specification, refer to the hardware specification of "maxi" and the Maple vibration function specification.

| Item | Description |
| --- | --- |
| Number of vibration units | 1 |
| Position of the vibration unit | front |
| Vibration axis of the vibration unit | no |
| Continuous vibration | allowed |
| Set vibration direction (positive, reverse) | allowed |
| Vibration strength that can be set | -7～0～+7 |
| Set up optional vibration waveform | not allowed |
| Vibration frequently that can be set | 0FH(15Hz)～4FH(79Hz)(not decided yet) |

Table 2-2. "maxi" specification

## Checking the connection

Use the pdVibMxIsReady() function to check to see whether "maxi" is connected.

if (pdVibMxIsReady(PDD_PORT_A2)) {

/* "maxi" is connected */

}

## Starting the vibration

To start the vibration, use the pdVibMxStart() function.  However, the parameters to start the vibration must be set to the structure.

Sint32 ret;

PDS_VIBPARAM param;

param.unit = 1;	/* Unit number (fixed) */

param.flag = PDD_VIB_FLAG_CONTINUOUS;	/* Has continuous vibration */

param.power = 7;	/* Set power to positive direction at 7 (maximum) */

param.freq = 20;	/* Frequency at 20Hz */

param.inc = 0;	/* No convergence and exhalation (0Hz) */

ret = pdVibMxStart(PDD_PORT_A2, &param);	/* Start vibration */

if (ret != PDD_VIBERR_OK) {

/* Fail to get it to vibrate */

}

In this example, the vibration is done with continuos vibration, at the maximum power, and with the vibration frequency at 20 Hz.  In the initial state, the continuous vibration automatically stops in about five seconds.  This automatic stop time of the vibration can be changed by using the pdVibMxSetStopTime() function.

## Parameters required for vibration, and setup example

This section describes members of the PDS_VIBPARAM structure used to pass to the pdVibMxStart() function.  Some examples are also included.

| Member | Description of member | Value | Description of each value |
| --- | --- | --- | --- |
| unit | vibration unit number | 1 | This member specifies the vibration unit to pass the vibration parameters to. For "maxi", because it has only one vibration unit, this member is fixed at 1. |
| flag | vibration flag | PDD_VIB_FLAG_CONTINUOUS | This value specifies to use the continuous vibration. The vibration continues until the automatic vibration stop time has come. If the flag is not specified, vibration will be single vibration (one-shot). |
|  |  | PDD_VIB_FLAG_EXHALATION | This value specifies to gradually increase the vibration (exhalation vibration). This value cannot be used together with PDD_VIB_FLAG_CONVERGENCE. When the continuous vibration reaches its maximum power, the exhalation vibration will start at the strength specified by the "power" member. |
|  |  | PDD_VIB_FLAG_CONVERGENCE | This value specifies to gradually decrease the vibration (convergence vibration). This value cannot be used together with PDD_VIB_FLAG_EXHALATION When the continuous vibration reaches its minimum power, the convergence vibration will start at the strength specified by the "power". |
| power | vibration power | -7～0～+7 | A positive value indicates a vibration in the positive direction, and a negative value indicates a vibration in the reverse direction. The larger the value, the stronger is the vibration. |
| freq | vibration number (vibration frequency) | 15≦n≦79 (planned) | This member specifies the vibration number (vibration frequency). |
| inc | vibration slope cycle | 1≦n≦255 | This member specifies the cycle for convergence and exhalation vibration. If the vibration is not convergence or exhalation, this member is ignored. |

Table 2-3. Parameter list

Example 1: Single vibration at 50 Hz vibration frequency

param.unit = 1;

param.flag = 0;

param.power = 7;

param.freq = 50;

param.inc = 0;

Example 2: Convergence single vibration at 20 Hz vibration frequency

param.unit = 1;

param.flag = PDD_VIB_FLAG_CONVERGENCE;

param.power = -7;

param.freq = 20;

param.inc = 1;

Example 3: Exhalation, continuous vibration at 40 Hz vibration frequency

param.unit = 1;

param.flag = PDD_VIB_FLAG_CONTINUOUS | PDD_VIB_FLAG_EXHALATION;

param.power = 7;

param.freq = 40;

param.inc = 1;

Regarding the vibration parameter, experiment with its different values and make sure to use the actual machine to check the vibration.

## Stopping the vibration

Use the pdVibMxStop() function to stop the vibration.

Sint32 ret;

ret = pdVibMxStop(PDD_PORT_A2);	/* Stop vibration */

if (ret != PDD_VIBERR_OK) {

/* Fail to stop vibration */

}

The following method can also stop the vibration.

Sint32 ret;

PDS_VIBPARAM param;

param.unit = 1;	/* Unit number (fixed) */

param.flag = 0;	/* No flag */

param.power = 0;	/* Set power to 0 to stop vibration*/

param.freq = 0;	/* At 0Hz */

param.inc = 0;	/* No convergence and exhalation */

ret = pdVibMxStart(PDD_PORT_A2, &param);	/* Stop vibration */

if (ret != PDD_VIBERR_OK) {

/* Fail to stop */

}

## Setting automatic vibration stop time

The vibration device comes with a feature to automatically stop the continuous vibration after a certain length of time has elapsed.  By default, the automatic vibration stop time is set to approximately 5 seconds.  If necessary, this value can be changed, using the pdVibMxSetStopTime() function.

Sint32 ret;

Uint32 time;

time = 10;		/* Use 10( about 2.5 seconds) */

ret = pdVibMxSetStopTime(PDD_PORT_A2, time);	/* Set automatic stop time */

if (ret != PDD_VIBERR_OK) {

/* Fail to set */

}

The automatic vibration stop time can be set between 0x00(approximately 0.25 second) and 0xff(approximately 64 seconds), at 0.25 second increments.  In the example above, the automatic vibration stop time is set to approximately 2.5 seconds.

When the power of the device is turned on, its automatic vibration stop time is initialized to 5 seconds.  In all other cases the automatic vibration stop time is not initialized.  Therefore, when using the device in an application, be sure to set a value to the automatic vibration stop time when the application is initialized.  On the other hand, there is no way to disable automatic vibration stop time .

## Description of functions for vibration of "maxi"

| Function | Description |
| --- | --- |
| pdVibMxIsReady | This function checks connection to "maxi". |
| pdVibMxStart | This function starts the vibration. |
| pdVibMxStop | This function stops the vibration. |
| pdVibMxSetStopTime | This function sets the automatic vibration stop time. |

Table 2-4. Function list

	API

| pdVibMxIsReady |  |  | "maxi" function |
| --- | --- | --- | --- |
| Syntax | Sint32 pdVibMxIsReady(Uint32 port) |  |  |
| Parameter | port | Port number PDD_PORT_A1/A2/B1/B2/C1/C2/D1/D2 |  |
| Returned Value | TRUE "maxi" is connected. FALSE "maxi" is not connected. |  |  |
| Description |  This function checks to see whether "maxi" is connected. |  |  |
| See also |  |  |  |
| Remarks |  In this version, if the vibration device is connected TRUE is returned. |  |  |
| Example | If (pdVibMxIsReady(PDD_PORT_A2)) return TRUE; |  |  |

| pdVibMxStart |  |  | "maxi" function |
| --- | --- | --- | --- |
| Syntax | Sint32 pdVibMxStart(Uint32 port, const PDS_VIBPARAM* param) |  |  |
| Parameter | port param | Port number PDD_PORT_A1/A2/B1/B2/C1/C2/D1/D2 Address of vibration parameter structure |  |
| Returned Value | PDD_VIBERR_OK Normal termination PDD_VIBERR_NO_VIBRATOR "maxi" is not connected. PDD_VIBERR_BUSY "maxi" is in the BUSY state. |  |  |
| Description |  This function starts the vibration of "maxi". |  |  |
| See also | PDS_VIBPARAM |  |  |
| Remarks |  |  |  |
| Example | PDS_VIBPARAM param; param.unit = 1; param.flag = PDD_VIB_FLAG_CONTINUOUS; param.power = -7; param.freq = 15; param.inc = 0; if (pdVibMxStart(PDD_PORT_A2, &param) == PDD_VIBERR_OK) { /* Succeed */ } |  |  |

| pdVibMxStop |  |  | "maxi" function |
| --- | --- | --- | --- |
| Syntax | Sint32 pdVibMxStop(Uint32 port) |  |  |
| Parameter | port | Port number PDD_PORT_A1/A2/B1/B2/C1/C2/D1/D2 |  |
| Returned Value | PDD_VIBERR_OK Normal termination PDD_VIBERR_NO_VIBRATOR "maxi" is not connected. PDD_VIBERR_BUSY "maxi" is in the BUSY state. |  |  |
| Description |  This function stops the vibration of "maxi". |  |  |
| See also | pdVibMxStart() |  |  |
| Remarks |  |  |  |
| Example | if (pdVibMxStop(PDD_PORT_A2) == PDD_VIBERR_OK) { /* Succeed */ } |  |  |

| pdVibMxSetStopTime |  |  | "maxi" function |
| --- | --- | --- | --- |
| Syntax | Sint32 pdVibMxSetStopTime(Uint32 port, Uint32 time) |  |  |
| Parameter | port time | Port number PDD_PORT_A1/A2/B1/B2/C1/C2/D1/D2 Automatic vibration stop time 00H - FFH(1H is approximately 0.25 second) |  |
| Returned Value | PDD_VIBERR_OK Normal termination PDD_VIBERR_NO_VIBRATOR "maxi" is not connected. PDD_VIBERR_BUSY "maxi" is in the BUSY state. |  |  |
| Description |  This function sets the automatic vibration stop time to stop the vibration of "maxi". |  |  |
| See also |  |  |  |
| Remarks |  |  |  |
| Example | if (pdVibMxSetStopTime(PDD_PORT_A2, 0x10) == PDD_VIBERR_OK) { /* Succeed */ } |  |  |

	Structure

| PDS_VIBPARAM |  |  | structure |
| --- | --- | --- | --- |
| Definition | typedef struct { Uint8 unit; Uint8 flag; Sint8 power; Uint8 freq; Uint8 inc; Uint8 reserved[3]; } PDS_VIBPARAM; |  |  |
| Description | This structure stores the vibration parameters of the vibration device. |  |  |
| Member | unit flag power freq inc reserved | Unit number to send vibration parameters to Vibration flag Vibration power Vibration frequency Vibration slope cycle (Reserved) |  |
| See also | pdVibMxStart() |  |  |

# Supporting vibration devices other than "maxi"

The previous chapter explains how to use functions limited to "maxi" only.  However, various types of new vibration devices may come to the market in the future.  To support these devices, the number of vibration units as well as parameters that can be used must be obtained before vibration can be started.

## Checking connection

To find out whether a vibration device is connected, use the pdVibIsReady() function, as follows:

if (pdVibIsReady(PDD_PORT_A2)) {

/* Vibration device is connected */

}

## Getting device information

There are two types of information: vibration device information (PDS_VIBINFO), and information on individual vibration unit within the device (PDS_VIBUNITINFO).

### Vibration device's connection direction

Depending on the direction of the extension connector on the controller, the user may see a different vibration direction.  For example, if the vibration pack is connected to the standard controller, the vibration direction is reversed.

To check the connection direction of the vibration device, use the pdVibGetDirection() function.

Sint32 dir;

dir = pdVibGetDirection(PDD_PORT_A2)

switch (dir) {

case PDD_VIB_DIRECTION_NORMAL:		/* Normal */

break;

case PDD_VIB_DIRECTION_FLIP:		/* Flipped */

break;

case PDD_VIB_DIRECTION_LEFT:		/* Left */

break;

case PDD_VIB_DIRECTION_RIGHT:	/* Right */

break;

}

### Vibration unit

In this library, the hardware which exists in a vibration device and serves as a vibration source is called a "vibration unit".  Multiple vibration units can exists in a vibration device.  This library supports from 1 to 15 vibration units.  A vibration unit has fixed parameters as well as other parameters, as shown in the following table.

| Parameter of vibration unit | Description |
| --- | --- |
| Vibration position | This parameter indicates the position of a vibration unit within the vibration device. There are four types: front, back, left, and right. |
| Vibration axis | This parameter indicates the vibration axis (direction) in a vibration unit. The possible values are: no axis, X axis, Y axis, and Z axis. |
| Vibration power | Some vibration units use only a fixed power level, but other may allow up to 8 levels of vibration power. |
| Continuous vibration | Some vibration units support the use of continuous vibration until the next vibration setting. |
| Vibration direction | In some vibration units, the vibration direction can be specified: positive or negative. |
| Optional vibration waveform | In some vibration units, optional vibration waveform can be created and set to vibrate accordingly. |
| Vibration number (vibration frequency) |  |

Table 3-5. Parameters of vibration unit

### Number of vibration units and number of vibration units that can be set at the same time

To get the number of vibration units that can be set at the same time for vibration, use the pdVibGetInfo() function

PDS_VIBINFO info;

if (pdVibGetInfo(PDD_PORT_A2, &info)) == PDD_VIBERR_OK) {

/* Succeed in getting information */

printf("Number of vibration units builit in: %d.\n", info.units);

printf("Number of vibration units that can be set for vibration

at the same time:%d.\n",info.se_units);

} else {2

/* Vibration device is not connected. */

}

The number of vibration units that can be set for vibration at the same time refers to the number of units that can be set to vibrate when the command is issued once (the pdVibStart() function is called once).

### Getting the vibration unit information

Use the pdVibEnumerateUnit() function to get the information of all of the vibration units in a vibration device.  To get this information, communication with the device is required, which means for each unit 1INT of time is required.  See the following code as a reference.

Uint32 flag;

Sint32 user_enum_unit_func(Uint32 unit, Sint32 stat, const PDS_VIBUNITINFO* info,

Uint32 param)

{

switch (stat) {

case PDD_VIBERR_NO_VIBRATOR:

printf("No vibration device\n");

flag = TRUE;						/* No callback after this line */

break;

case PDD_VIBERR_OK:

printf("Information of unit %d\n", unit);

printf("Position: %d\n", info->position);

:

:

if (unit == param) flag = TRUE;	/* Callback of all units completed */

break;

}

return PDD_VIBRET_OK;

}

-------------------

Sint32 ret;

flag = FALSE;

if (pdVibGetInfo(PDD_PORT_A2, &info) != PDD_VIBERR_OK) return NG;

if (pdVibEnumerateUnit(PDD_PORT_A2, user_enum_unit_func, info.units) !=

PDD_VIBERR_OK) return NG;

while (!flag);	/* Wait for callback of all units */

In this way, the user_enum_unit_func() function is called the number of times equal to the number of the vibration units, to get information of each of the vibration units.  However, be aware that the pdVibEnumerateUnit() function returns immediately, while the callback triggers once for every 1INT.

### Details on the vibration unit information

The following table describes in details the PDS_VIBUNITINFO structure storing the vibration unit information thus obtained.

| Member | Desription of member | Value | Description of each value |
| --- | --- | --- | --- |
| position | This member indicates the position of the vibration unit. | PDD_VIB_POS_FRONT | This value indicates the position of the vibration is at the front of the vibration device. |
|  |  | PDD_VIB_POS_BACK | This value indicates the position of the vibration is at the back of the vibration device. |
|  |  | PDD_VIB_POS_LEFT | This value indicates the position of the vibration is at the left side of the vibration device. |
|  |  | PDD_VIB_POS_RIGHT | This value indicates the position of the vibration is at the right side of the vibration device. |
| axis | This member indicates the vibration axis. | PDD_VIB_AXIS_NONE | This value indicates no vibration axis is used (rotation movement etc.). |
|  |  | PDD_VIB_AXIS_X | This value indicates the vibration axis is the X axis (left and right directions relative to the player). |
|  |  | PDD_VIB_AXIS_Y | This value indicates the vibration axis is the Y axis (up and down directions relative to the player). |
|  |  | PDD_VIB_AXIS_Z | This value indicates the vibration axis is the Z axis (front and back directions relative to the player). |
| pow_enable | This member indicates whether the vibration power can be changed. | 0 | This value indicates the vibration power is fixed. |
|  |  | 1 | This value indicates the vibration power can be adjusted, in eight levels. |
| dir_enable | This member indicates whether the vibration direction can be specified. | 0 | This value indicates the vibration direction cannot be specified (vibration direction is fixed). |
|  |  | 1 | This value indicates the vibration direction cant be specified (+ or - to indicate positive or reverse direction). |
| wave_enable | This member indicates whether optional vibration waveform is allowed. | 0 | This value indicates no optional vibration waveform is allowed. |
|  |  | 1 | This value indicates optional vibration waveform is allowed. |
| min_freq | This member indicates the minimum vibration frequency that can be set. | - | 0 indicates frequency cannot be set. |
| max_freq | This member indicates the maximum vibration frequency that can be set. | - | 0 indicates frequency cannot be set. |

Table 3-6. Vibration unit information

## Description of functions to get information

| Function | Description |
| --- | --- |
| pdVibIsReady | This function checks to see whether the vibration device is connected. |
| pdVibGetInfo | This function gets the vibration device information. |
| pdVibEnumerateUnit | This function gets information of all vibration units in a vibration device. |
| pdVibGetDirection | This function gets the direction of the vibration device that is connected. |
| PD_VIBENUMUNITCALLBACK | Callback function type |

Table 3-7. Function list

| Structure | Description |
| --- | --- |
| PDS_VIBINFO | This structure stores information of a vibration device. |
| PDS_VIBUNITINFO | This structure stores information of the vibration units. |

Table 3-8. Structure list

	API

| pdVibIsReady |  |  | vibration device function |
| --- | --- | --- | --- |
| Syntax | Sint32 pdVibIsReady(Uint32 port) |  |  |
| Parameter | port | Port number PDD_PORT_A1/A2/B1/B2/C1/C2/D1/D2 |  |
| Returned Value | TRUE Vibration device is connected. FALSE Vibration device is not connected. |  |  |
| Description |  This function checks to see whether the vibration device is connected. |  |  |
| See also |  |  |  |
| Remarks |  |  |  |
| Example | if (pdVibIsReady(PDD_PORT_A2)) return TRUE; |  |  |

| pdVibGetDirection |  |  | information-getting function |
| --- | --- | --- | --- |
| Syntax | Sint32 pdVibGetDirection(Uint32 port) |  |  |
| Parameter | port | Port number PDD_PORT_A1/A2/B1/B2/C1/C2/D1/D2 |  |
| Returned Value | PDD_VIB_DIRECTION_NORMAL Normal PDD_VIB_DIRECTION_FLIP Flipped PDD_VIB_DIRECTION_LEFT Left PDD_VIB_DIRECTION_RIGHT Right |  |  |
| Description |  This function gets the connection direction of the vibration device. |  |  |
| See also |  |  |  |
| Remarks |  Depending on the connection direction, the direction of the vibration axis changes. |  |  |
| Example |  |  |  |

| pdVibGetInfo |  |  | information-getting function |
| --- | --- | --- | --- |
| Syntax | Sint32 pdVibGetInfo(Uint32 port, PDS_VIBINFO* vibinfo) |  |  |
| Parameter | port vibinfo | Port number PDD_PORT_A1/A2/B1/B2/C1/C2/D1/D2 Address where vibration device information is stored |  |
| Returned Value | PDD_VIBERR_OK Normal termination PDD_VIBERR_NO_VIBRATOR The vibration device is not connected. |  |  |
| Description |  This function gets the vibration device information. |  |  |
| See also | pdVibEnumerateUnit()、PDS_VIBINFO、PDS_VIBUNITINFO |  |  |
| Remarks |  |  |  |
| Example | PDD_VIBINFO info; if (pdVibGetInfo(PDD_PORT_A2, &info) == PDD_VIBERR_OK) { /* Succeed */ } |  |  |

| pdVibEnumerateUnit |  |  | information-getting function |
| --- | --- | --- | --- |
| Syntax | Sint32 pdVibEnumerateUnit(Sint32 port, PD_VIBENUMUNITCALLBACK func, Uint32 param) |  |  |
| Parameter | port func param | Port number PDD_PORT_A1/A2/B1/B2/C1/C2/D1/D2 Callback function address Callback function parameter |  |
| Returned Value | PDD_VIBERR_OK Normal termination PDD_VIBERR_NO_VIBRATOR The vibration device is not connected. |  |  |
| Description |  This function gets information of all vibration units in a vibration device. |  |  |
| See also | pdVibGetInfo()、PD_VIBENUMUNITCALLBACK、PDS_VIBINFO、PDS_VIBUNITINFO |  |  |
| Remarks |  |  |  |
| Example | Sint32 enumfunc(Uint32 unit, Sint32 stat, const PDS_VIBUNITINFO* info, Uint32 param) { return PDD_VIBRET_OK; } ------ if (pdVibEnumerateUnit(PDD_PORT_A2, enumfunc, 0) == PDD_VIBERR_OK) { /* Succeed */ } |  |  |

| PD_VIBENUMUNITCALLBACK |  |  | callback function type |
| --- | --- | --- | --- |
| Syntax | typedef Sint32 (*PD_VIBENUMUNITCALLBACK)(Uint32 unit, Sint32 stat, const PDS_VIBUNITINFO* info, Uint32 param) |  |  |
| Parameter | unit stat info param | Unit number where information is obtained Callback status: PDD_VIBERR_OK Succeed PDD_VIBERR_NO_VIBRATOR The vibration device is not connected. Address where the vibration unit information is stored Callback parameter |  |
| Returned Value | Make sure to return PDD_VIBRET_OK. |  |  |
| Description |  This callback function is called once after one vibration unit's information is obtained, when the pdVibEnumerateUnit() function is used  The callback is called the same number of times as the number of vibration units in the vibration device.  If the vibration unit information can be obtained, the callback status is PDD_VIBERR_OK.  If the vibration device is removed during the operation, the callback status becomes PDD_VIBERR_NO_VIBRATOR, and there will be no more subsequent callback. |  |  |
| See also | pdVibEnumerateUnit()、PDS_VIBUNITINFO |  |  |
| Remarks |  |  |  |
| Example | Sint32 enumfunc(Uint32 unit, Sint32 stat, const PDS_VIBUNITINFO* info, Uint32 param) { if (stat == PDD_VIBERR_OK) { printf("Information of unit %d\n" unit); : } return PDD_VIBRET_OK; } |  |  |

	Structure

| PDS_VIBINFO |  |  | structure |
| --- | --- | --- | --- |
| Definition | typedef struct { Uint8 units; Uint8 se_units; } PDS_VIBINFO; |  |  |
| Description | This structure stores information of a vibration device. |  |  |
| Member | units se_units | Number of vibration units Number of vibration units that can be set at the same time for vibration |  |
| See also | pdVibGetInfo()、PDS_VIBUNITINFO |  |  |

| PDS_VIBUNITINFO |  |  | structure |
| --- | --- | --- | --- |
| Definition | typedef struct { Uint8 position; Uint8 axis; Uint8 pow_enable; Uint8 cont_enable; Uint8 dir_enable; Uint8 wave_enable; Uint8 min_freq; Uint8 max_freq; } PDS_VIBUNITINFO; |  |  |
| Description | This structure stores infomration of a vibration unit. |  |  |
| Member | position axis pow_enable cont_enable dir_enable wave_enable min_freq max_freq | Position of the vibratino unit Vibration axis Can the vibration power be changed? Is continuous vibration allowed? Can vibrtion direction be specified? Is optional waveform allowed? Minimum vibration frequency Maximum vibration frequency |  |
| See also | pdVibEnumerateUnit()、PD_VIBENUMUNITCALLBACK |  |  |

## Starting vibration

To start the vibration, use the pdVibStart() function.  However, the parameters to start the vibration must be set to the structure.  Refer to the vibration unit information obtained by the pdVibEnumerateUnit() function, and then specify the appropriate parameter.

Sint32 ret;

PDS_VIBPARAM param;

param.unit = 1;	/* Unit 1 */

param.flag = PDD_VIB_FLAG_CONTINUOUS;	/* Continuous vibration*/

param.power = 7;	/* Power set to postive, 7 (maximum)*/

param.freq = 20;	/* At 20Hz */

param.inc = 0;	/* No convergence and exhalation */

ret = pdVibStart(PDD_PORT_A2, &param, 1);		/* Start vibration */

if (ret != PDD_VIBERR_OK) {

/* Failed to get it to vibrate  */

}

The above example shows how to enable vibration of vibration unit 1 only.  To enable vibration of multiple vibration units, declare an array of structures as the parameters, as shown in the following example.

Sint32 ret;

PDS_VIBPARAM param[2];

param[0].unit = 1;	/* Unit 1  */

param[0].flag = PDD_VIB_FLAG_CONTINUOUS;	/* Continuous vibration */

param[0].power = 7;	/* Power set to postive, 7 (maximum)*/

param[0].freq = 20;	/* At 20Hz */

param[0].inc = 0;	/* No convergence and exhalation (0Hz) */

param[1].unit = 2;	/* Unit 2 */

param[1].flag = PDD_VIB_FLAG_EXHALATION;	/* Single exhalation */

param[1].power = -7;	/* Power set to negative, 7 (maximum) */

param[1].freq = 15;	/* At 15Hz */

param[1].inc = 1;	/* Exhanlation cycle 1 */

ret = pdVibStart(PDD_PORT_A2, param, 2);	/* Enable vibration */

if (ret != PDD_VIBERR_OK) {

/* Failed to get it to vibrate */

The third parameter in the pdVibStart() function indicates the number of vibration parameters, or in other words the number of vibration units for vibration set up at the same time.  This cannot exceed the " Number of vibration units that can be set at the same time for vibration" in the vibration device information.

## Parameters required for vibration, and setup example

This section describes members of the PDS_VIBPARAM structure used to pass to the pdVibStart()function.  Some examples are also included.  When setting up parameters to each vibration unit, be sure to refer to the vibration unit information obtained by the pdVibEnumerateUnit() function, and then specify the appropriate parameter.  If a parameter is illegal, its vibration unit or all vibration units in the vibration device may not virbrate.  Also, the device hardware may cause unexpected operation.  The library does not perform any check on the parameter.

| Member | Desription of member | Value | Description of each value |
| --- | --- | --- | --- |
| unit | vibration unit number | 1～V.se_units | This member specifies the vibration unit to pass the vibration parameters to. |
| flag | Vibration flag | PDD_VIB_FLAG_CONTINUOUS | This value specifies to use the continuous vibration. The vibration continues until the automatic vibration stop time has come. If the flag is not specified, vibration will be single vibration (one-shot). If U.cont_enable==0, this member cannot be used. |
|  |  | PDD_VIB_FLAG_EXHALATION | This value specifies to gradually increase the vibration (exhalation vibration). This value cannot be used together with PDD_VIB_FLAG_CONVERGENCE. When the continuous vibration reaches its maximum power, the exhalation vibration will start at the strength specified by the "power" member. If U.cont_enable==0, this member cannot be used. |
|  |  | PDD_VIB_FLAG_CONVERGENCE | This value specifies to gradually decrease the vibration (convergence vibration). This value cannot be used together with PDD_VIB_FLAG_EXHALATION When the continuous vibration reaches its minimum power, the convergence vibration will start at the strength specified by the "power". If U.cont_enable==0, this member cannot be used. |
| power | vibration power | -7～0～+7 | A positive value indicates a vibration in the positive direction, and a negative value indicates a vibration in the reversed direction. The larger the value, the stronger is the vibration. If U.pow_enable==0, this member is ignored. If U.dir_enable==0, the sign is ignored. |
| freq | vibration number (vibration frequency) | U.min_freq≦n≦U.max_freq | This member specifies the vibration number (vibration frequency). |
| inc | vibration slope cycle | 1≦n≦255 | This member specifies the cycle for convergence and exhalation vibration. If the vibration is not convergence or exhalation, this member is ignored. |

Table 3-9. Parameter list

※In the table, "V." indicates the PDS_VIBINFO structure.

※In the table, "U." indicates the PDS_VIBUNITINFO structure.

## Stopping the vibration

To stop the vibration, use the VibStart() function, as follows.

Sint32 ret;

PDS_VIBPARAM param;

param.unit = 2;	/* Unit number 2 */

param.flag = 0;	/* No flag */

param.power = 0;	/* Stop vibration */

param.freq = 0;	/* At 0Hz */

param.inc = 0;	/* No convergence and exhalation */

ret = pdVibStart(PDD_PORT_A2, &param, 1);	/* Stop vibration */

if (ret != PDD_VIBERR_OK) {

/* Fail to stop */

}

The above example shows how to stop vibration of vibration unit 2 only.  To enable vibration of multiple vibration units, declare an array of structures as the parameter and set their vibration power to 0.

## Setting automatic vibration stop time

To set the automatic vibration stop time , use the pdVibSetStopTime() function.

Sint32 ret;

Uint8 units[] = {1, 3, 5};		/* Array to store desired unit number */

Uint8 times[] = {10, 20, 30};	/* Array to store value for each unit */

ret = pdVibSetStopTime(PDD_PORT_A2, units, times, 3);	/* Set */

if (ret != PDD_VIBERR_OK) {

/* Fail to set */

}

In the example, vibration units 1, 3, and 5 have their automatic vibration stop time set to 10 (approximately 2.5 seconds), 20 (approximately 5 seconds), and 30 (approximately 7.5 seconds) respectively.

## Description of the vibration functions

| Function | Description |
| --- | --- |
| pdVibStart | This function starts the vibration. |
| pdVibSetStopTime | This function sets the automatic vibration stop time. |

Table 3-10. Function list

	API

| pdVibStart |  |  | vibration device function |
| --- | --- | --- | --- |
| Syntax | Sint32 pdVibStart(Uint32 port, const PDS_VIBPARAM* param, Uint32 num) |  |  |
| Parameter | port param num | Port number PDD_PORT_A1/A2/B1/B2/C1/C2/D1/D2 Address of vibration parameter structure array Number of vibration units |  |
| Returned Value | PDD_VIBERR_OK Normal termination PDD_VIBERR_NO_VIBRATOR The vibration device is not connected. PDD_VIBERR_BUSY The vibration is in the BUSY state. |  |  |
| Description |  This function starts the vibration of the vibration device. |  |  |
| See also | PDS_VIBPARAM |  |  |
| Remarks |  |  |  |
| Example | PDS_VIBPARAM param; param.unit = 1; param.flag = PDD_VIB_FLAG_CONTINUOUS; param.power = -7; param.freq = 15; param.inc = 0; if (pdVibMxStart(PDD_PORT_A2, &param) == PDD_VIBERR_OK) { /* Succeed */ } |  |  |

| pdVibSetStopTime |  |  | vibration device function |  |
| --- | --- | --- | --- | --- |
| Syntax | Sint32 pdVibSetStopTime(Uint32 port, Uint8* unit, Uint8* time, Uint32 num) |  |  |  |
| Parameter | port unit time num | Port number PDD_PORT_A1/A2/B1/B2/C1/C2/D1/D2 Array storing the vibration unit numbers Array storing the automatic vibration stop time s Number of vibration units |  |  |
| Returned Value | PDD_VIBERR_OK Normal termination PDD_VIBERR_NO_VIBRATOR The vibration device is not connected. PDD_VIBERR_BUSY The vibration is in the BUSY state. |  |  |  |
| Description |  This function sets the automatic vibration stop times to stop the vibration of the vibration device. |  |  |  |
| See also |  |  |  |  |
| Remarks |  |  |  |  |
| Example | Uint8 units[] = {1, 3, 5}; Uint8 times[] = {10, 20, 30}; if (pdVibSetStopTime(PDD_PORT_A2, units, times, 3) == PDD_VIBERR_OK) { /* Succeed */ } |  |  |  |

# Optional vibration waveform

This feature is not supported in this versiion.