Cache Principles

Sega Software Technology Development Group

26/07/31

This documentation explains the general caching principles and provides information necessary to understand the cache functions included in the SEGA libraries.

What Is Caching?

Normally, the CPU operates at very high speed (200 MHz in case of the SH7091). Memory access on the other hand is much slower, which would prevent the CPU from delivering its full potential. A cache is a module that serves as a bridge between slow memory and fast CPU. It consists of very high-speed RAM that allows the CPU to develop its full performance.

If a copy of the main memory is kept in the cache, the CPU can quickly obtain memory information without having to go to the slow main memory. For cost reasons, it is not practical to use high-speed memory for the entire main memory, but even with a comparatively small amount of cache RAM, CPU performance can be significantly improved.

How a Cache Works

The amount of memory used as cache is generally much smaller than the capacity of the main memory. (In the SH7091, the instruction cache is 8 KB and the data cache 16 KB). Because it is not possible to fit the entire main memory contents into the cache, the limited amount of cache RAM must be used effectively.

The cache consists of an address array and data array. The address array points to addresses in the main memory, and the data array to data in the main memory. Each address array has 32 bytes of data array. This combination is called an entry. The instruction cache has 256 entries and the data cache 512 entries.

When the CPU performs memory access, it first checks whether the required information is present in the cache. If the address data are in the cache, it retrieves the data from the cache (this is called a cache hit). If the data are not in the cache (cache miss), the CPU must go to the main memory to get the data (penalty). When a cache miss has occurred, 32 bytes of data preceding and following the target address are also read into the cache, to fill the cache entry. A cache miss therefore takes a longer time than a cache hit. This applies both to reading and writing operations. Whenever there is a cache miss, 32 bytes of data in the vicinity of the target address are read to fill the cache entry.

Because the cache is a limited resource, cache entries may not always be available. In this case, the cache controller selects the least recently used cache entry, clears it, and then reads 32 bytes of data from memory into it as a new entry. This cache line selection algorithm is called LRU (least recently used). For details, please refer to the general documentation for CPUs (Pentium, Power PC, etc.).

Write-Back

When the CPU performs memory access and there is a usable cache entry, that entry can not only be read but also be written to. When a write was performed, the so-called dirty bit is raised for that data entry, indicating that there is a discrepancy between the main memory and the cache contents. Of course, simply writing to the cache will not update the main memory, but it allows much faster access to the written data. By writing programs in such a way that access to the main memory is minimized and most operations are performed within the cache is a way to speed up processing. However, the result will be a discrepancy between the cache and the main memory contents. To remedy this, the cache contents must be written to the main memory at regular intervals. This is called write-back.

Cache Coherency

When the CPU performs memory access, it normally will always go through the cache. However, some CPU modules also are capable of direct memory access (DMA) without going through the cache. When an operation such as data write with DMA was carried out for an address that is included in a cache entry, the data as managed by the cache and the actual memory data will be out of sync. The match between the cache and the main memory is called coherency. In the above described case, there is a lack of cache coherency.

Invalidating the Cache

When DMA or other factors have caused a lack of coherency between the cache and the main memory which cannot be corrected by measures such as write-back, the entry in the cache must be invalidated and data must be read in through the cache line again. This process is called cache invalidation. It is divided into entry invalidation and full-cache invalidation.

Invalidation means that the cache data are destroyed and the address array effective bit is turned off, making the entry available for further data.

In the SEGA libraries, GD file access functions are a representative example of functions that use DMA. In the GDFS library, the data cache is invalidated after data have been transferred. The application therefore normally needs not be concerned with invalidation, except when reading in a program as overlay or similar. If a GDFS library function would execute an overlay program without invalidating the instruction cache, the program area would include addresses with entries in the cache, leading to operation errors.

When a program module was read from GD, the instruction cache for the read-in area must be invalidated.

Purging the Cache

After cache write-back was performed, the cache entry is still valid. When the CPU accesses the addresses specified in the entry, the cache entry will be used as is. However, if these addresses are not required for a considerable time, valuable cache space will be wasted. This is not desirable in terms of cache efficiency. Often, it will be more efficient to invalidate the addresses after write- back. This operation is called purging. For purged cache entries, the address array effective bit is turned off, making the entry available for further data.

Cache Pre-Fetch

As explained above, performance during cache access differs greatly depending on whether there is a cache hit or a cache miss. To prevent the performance penalty caused by a cache miss as much as possible, cache efficiency should be optimized. Another method to reduce performance degradation is to predict cache misses and fill the cache entries beforehand with the required data. This is called cache pre-fetch. It can be used only for the data cache. To effectively make use of the pre-fetch technique, sophisticated procedures are required, such as examining the assembler code for places where cache misses are likely to occur, or using a pipeline simulator to check for data load/store pipeline delays.

Obtaining a Cache Entry

When accessing memory for updating the contents of an address regardless of the current contents of the address, such as when a processing result is to be stored in an area reserved as buffer, the following operation will be carried out. The address to be updated is accessed. If it is not in the cache entry, 32 bytes around this address will be used to fill the cache entry, and then the data will be written to the data array in the cache pertaining to the specific address. However, this is not very efficient, because 32 bytes of data have to be retrieved from main memory to fill the cache entry. For such special cases, the following Assembler instruction allows obtaining a cache entry without getting data from memory:

MOVCA.L RO @Rn

The SEGA libraries contain a function that executes this instruction.

Cache RAM Mode

The SH7091 provides a mode that allows using half of the data cache (8 KB) as RAM. Because of the structure of the cache, using cache RAM in place of regular RAM results in very fast operation. However, the SEGA libraries prohibit the use of cache RAM by applications, because normal operation in RAM mode cannot be assured.