Scroll Guide

Ver0.05

SEGA Enterprises, Ltd

Revision Information

# Ver.0.04

The member clip of the scroll structure can not be used.

# Ver.0.05

* “3.2.5” and the description of the member clip in ”5.3 Scroll-related Structure” were

changed.

* “5.4 Color Definition” was modified.

Contents

Chapter 1: Image Units as Related to Scrolling

- 1Overview

1.2 Image Units

Chapter 2: Scroll Rotation, Enlargement, Reduction, and Movement

2.1 Overview

- Scroll Rotation, Resizing, and Movement

Chapter 3: Scroll Programming

- 1 Overview

3.2 Example of Programming a Scroll

Chapter 4: Color

- 1 Overview

4.2 Color Mode

Chapter 5: Scroll functions, Structures, and Definitions

- 1 Overview

5.2 Scroll-related Function

5.3 Scroll-related Structure

5.4 Scroll-related Definitions

5.5 Texture Structures for Use in Cell Programming

Chapter 1: Image Units as Related to Scrolling

1.1 Overview

This chapter explains the image units which Ninja uses in scrolling.

1.2Image Units

Pixel

Cellセル

Mapマップ

Display area表示領域

The smallest component unit of an image

Cell

The smallest unit of an image which makes

up a scrolling screen  Cells in Ninja are

composed of between 8 and 1024 pixels.

The maximum number of Cells which a

program can hold is defined by

NJD_CELL_NUM_MAX.

Map

Maps are composed of collections of Cells

The maximum number of maps which a

program can hold is defined by NJD_MAP_MAX.

Chapter 2 Scroll Rotation, Resizing, and Movement

2.1 Overview

This chapter shows the meanings of the various values used in setting scroll displays and scroll structures, and how those values are calculated.

2.2  Scroll Rotation, Resizing, and Movement

Scroll areaスクロール面

Clip areaクリップ面

Scroll Cell Starting Pointセル書き始め位置

Scroll Rotation, Resizing, and Movement are described as follows.

(0,0)

Origin

(bx,by)

- Both the scroll area and clip area use the

upper left corner of the screen as the origin.

The x and y coordinates which mark the starting point

of a scroll cell are designated bx, by (see Figure 1).

Figure 1

Figure 2図2

(-bx,-by)

Origin原点

(0,0)

- Points on the Scroll Display move in bx, by

from the origin by -bx, -by (see Figure 2).

x - bx

y - by

x‘

y’

=

(3)To perform rotations, move toward

The origin by the difference of the center

Origin

of rotation (cx, cy).  (see Figure 3)

(0,0)

x - bx - cx

y - by - cy

x‘

y’

=

Center of Rotation

(cx,cy)

Figure 3

(4) Make the center of rotation into the origin,

and rotate via the matrix m (see Figure 4)

x‘

y’

=

m

x - bx - cx

y- by - cy

Origin

(0,0)

Center of Rotation

Figure 4

図5Figure 5

(0,0)

Origin原点

Center of Rotation回転中心

(5) After rotation, restore to original by degree

that Scroll Display was moved (see Figure 5).

+

cx

cy

x‘

y’

==

m

x - bx - cx

y - by - cy

(6) Resize by sx, sy, centering on the center of risizing

(spx, spy) (see Figure 6).

図6Figure 6

(0,0)

原点Origin

拡縮中心Center of Resizing

+

sx

sy

+

cx

cy

x‘

y’

=

m

x- bx - cx

y- by - cy

spx

spy

-

spx

spy

(7) Finally, move by px, py (see Figure 7).

sx

sy

+

cx

cy

x‘

y’

=

m

x- bx - cx

y- by - cy

spx

spy

spx

spy

px

py

-

+

+

図7Figure 7

(0,0)

原点Origin

px

py

Chapter 3 Scroll Programming

- 1Overview

In this chapter, we cover everything from drawing of Cells to depicting Scroll area.

3.2 Example of Programming a Scroll

256

256

128

128

1. Draw a Cell Image

Draw a Cell image by following the texture creation rules. Note that Cell size must be from 8 to 1024.

Ex.: Draw four 128x128 textures in one 256x256 texture.

2.Convert Cell Image to pvr format

Use tools to convert textures to PVR format.

3.Create Texture List

Create the texture name structure and texture list structure. Refer to “Texture Guide” for the detail on the way to create them.

4.Creating a Map

Create the following map as an example.

2    3    6    7   17

0    1    4    5   16

8    9   12   13   18

10  11   14   15   19

The map data comes from the previously created files in the following order.

0, 1, 2, 3 from test0.pvr

4, 5, 6, 7 from test1. pvr

8, 9, 10, 11 from test2. pvr

12, 13, 14, 15 from test3. pvr

16, 17, 18, 19 from test4. pvr

Texture numbers are taken in the order they were stored in the list's creation, starting with test0.pvr. test0.pvr is 0, and test4.pvr is 4. We use this and the mapmaking macro NJM_MAP to create maps. NJM_MAP is NJM_MAP(texture number, texture U, texture V). Thus, the 0 area of the map is NJM_MAP(0, 0, 0), 1 is NJM_MAP(0, 128, 0), etc. The map array that results from this is

Uint32 map[4][5] ={

{NJM_MAP(0,0,0),	NJM_MAP(0,128,0),	NJM_MAP(1,0,0),	NJM_MAP(1,128,0),	NJM_MAP(4,0,0)},

{NJM_MAP(0,0,128),NJM_MAP(0,128,128),NJM_MAP(1,0,128),	NJM_MAP(1,128,128),	NJM_MAP(4,128,0)},

{NJM_MAP(2,0,0),	NJM_MAP(2,128,0),	NJM_MAP(3,0,0),	NJM_MAP(3,128,0),	NJM_MAP(4,0,128)},

{NJM_MAP(2,0,128),NJM_MAP(2,128,128),NJM_MAP(3,0,128),	NJM_MAP(3,128,128),	NJM_MAP(4,128,128)}

};

5.Define the Scroll Structure

Define all the elements of the Scroll Structure

celps		assigns cell pixel size between 8 and 1024.

mapw 		assigns map width in number of Cells

maph		assigns map height in number of Cells

sw		assigns horizontal scroll display image size.

sh		assigns vertical scroll display image size

list		assigns pointer to texture list structure

map		assigns pointer to top address of map array.

Make sure that map is at least of dimensions

map[maph][mapw].

Anything smaller will leave this variable undefined

px,py		assigns coordinates for movement of scroll display

bx,by		assigns coordinates for beginning of map draw

pr		assigns scroll priority

sflag		sets resize flag (ON, OFF).

sx,sy		sets the ratio for x- and y-axis resizing

spx,spy		assigns coordinates for center of resizing area

mflag		sets rotation matrix flag (ON, OFF).

cx,cy		assigns coordinates for center of rotation area

m		assigns rotation matrix.

colmode		assigns color mode

colmix		assigns color computations (unimplemented at present)

clip[2]		Not used in this version

attr		attribute (unimplemented at present)

sclc		applies color to entire scroll. Varies according to color mode

6.Use Scroll Functions

Finally, we will try out the scroll functions (using the map and texture list previously created).

First, load the textures.

njInitTexture(&texmemlist,5);

njLoadTexture(&texlist);

Assign scroll structure (see section 5)

scl.celps =  128;

:

:(omitted)

Using the scroll functions, you can draw the scrolls

njDrawScroll(&scl);

Chapter 4 Color

- Overview

This chapter explains about color modes which can be used in colmode of scroll structures

4.2  Color Mode

#### NJD_COLOR_MODE_FLAT_TEXTURE

This mode is used when “No translucent” (RGB565) is set for the textures of all cells.

#### NJD_COLOR_MODE_FLAT_TEXTURE_TRANS

This mode is used when some (even if only one) of textures of the cell is “translucent” (ARGB1555 or ARGB 4444) .

Chapter 5 Scroll function, Structures, and Definitions

5.1Overview

This chapter explains Ninja scroll functions, scroll structures, and scroll definitions.

5.2 Scroll–related Functions

njDrawScroll

Draws 2D scroll

Format

#include <Ninja.h>

void njDrawScroll( *scl )

NJS_SCROLL *scl

Parameters

*scl scroll structure pointer

Return value

none

Function

Draws 2D scroll in clip display

Notes

For details on creating textures, refer to the Texture document.

5.3 Scroll–related Structure

NJS_SCROLLStructure

typedef struct {

Uint16  	celps;		/* Cell Pixel size 			*/

Uint16  	mapw,maph;	/* Number of Cells			*/

Uint16  	sw,sh;		/* Scroll display image size 		*/

NJS_TEXLIST	list;		/* Pointer to texture list structure	*/

Uint16  	*map;		/* Pointer to top address of map array	*/

Float    	px,py;		/* Coordinates for drawing scroll 	*/

Float 	bx,by		/* Coordinates for drawing scroll origin	*/

Float    	pr;		/* Priority 			*/

Sint16 	sflag;		/* Resize flag (ON, OFF)		*/

Float    	sx,sy;		/* x- and y-axis resizing ratio		*/

Float		spx,spy;		/* center of resizing area		*/

Sint16	mflag;		/* Rotation flag (ON, OFF)		*/

Float		cx,cy;		/* Center of rotation area		*/

NJS_SCLMTRX	m;		/* Rotation Matrix 			*/

Uint16  	colmode;		/* Color Mode 			*/

Uint16	colmix;		/* Color Computations (unimplemented at present)*/

NJS_POINT2 	clip[2]		/* Clip point 			*/

NJS_SCLATTR 	attr;		/* Attribute 			*/

NJS_COLOR	sclc;		/* ITE Color			*/

}NJS_SCROLL;

5.4 Scroll-related Definitions

Maximum Values

#define NJD_CELL_NUM_MAX        0xFFFF  /* the maximum of cell's number */

#define NJD_MAP_W_MAX           0xFF    /* the maximum of map's width   */

#define NJD_MAP_H_MAX           0xFF    /* the muximum of map's height  */

#define NJD_MAP_MAX             (NJD_MAP_W_MAX*NJD_MAP_H_MAX)

Color definitions (colormode)

#define	NJD_COLOR_MODE_PACKED_TEXTURE	 	33

#define	NJD_COLOR_MODE_PACKED_TEXTURE_TRANS	41

5.5 Texture Structures for Use in Cell Programming

NJS_TEXINFOStructure

typedef struct{

void*		texaddr;	/* texture memory address cache  	*/

NJS_TEXSURFACE	texsurface;

} NJS_TEXINFO;

NJS_TEXNAMEStructure

typedef struct{

void		*filename;	/* Pointer to filename or NJS_TEXINFO structure	*/

Uint32		attr;	/* Texture Attributes 			*/

Uint32		texaddr;	/* Texture Address				*/

}NJS_TEXNAME;

NJS_TEXLISTStructure

typedef struct {

NJS_TEXNAME     	*textures; /* texture array			*/

Uint32          	nbTexture;	/* texture count			*/

} NJS_TEXLIST;