2-2 Guide of CPU performance profiling (User’s manual)

Using the profiler of CodeScape, you can analyze the performance of the program and tune up the performance.    This document describes the methods of performance analysis and tune-up techniques.

1. Profiler functions

The profiler measures the following data for each function.

(1)	Number of calls

(2)	The value of two counter values measured inside each function

Here the counter means the performance counter of SH7091, which can measure two selected items among the items shown in the following table.

| Item | Meaning | Unit |
| --- | --- | --- |
| Operand access (read/with cache) | Number of accesses (READ) to data cache. | Times |
| Operand access (write/with cache) | Number of accesses (WRITE) to data cache. | Times |
| UTLB miss | Number of TLB misses by data access. | Times |
| Operand cache read miss | Number of data cache misses (READ). | Times |
| Operand cache write miss | Number of data cache misses (WRITE). | Times |
| Instruction fetch ( with cache) | Number of accesses to instruction cache. | Times |
| Instruction TLB miss | Number of exceptions caused by instruction TLB misses. | Times |
| Instruction cache miss | Number of instruction cache misses. | Times |
| All operand access | Number of all the data accesses. | Times |
| All instruction access | Number of all the instruction access. | Times |
| On-chip RAM operand access | Number of data accesses to on-chip RAM. | Times |
| On-chip I/O space access | Number of I/O accesses | Times |
| Operand access (read+write/with cache) | Number of data cache accesses (READ+WRITE). | Times |
| Operand cache read + write miss | Number of data cache misses (READ+WRITE) | Times |
| Branch instruction | Number of branch instructions | Times |
| Branch taken | Number of branches taken. | Times |
| BSR/BSRF/JSR | Number of subroutine calls. | Times |
| Instruction Execution | Number of instructions executed. | Times |
| 2-instruction simultaneous execution | Number of instructions executed in parallel. | Times |
| FPU instruction execution | Number of FPU instructions. | Times |
| Interrupt (normal) | Number of interrupts (general). | Times |
| Interrupt (NMI) | Number of interrupts (NMI) | Times |
| TRAPA instruction execution | Number of software interrupts. | Times |
| UBC-A match | Number of matches of UBC-A. | Times |
| UBC-B match | Number of matches of UBC-B. | Times |
| Instruction cache fill | Number of cycles to fill instruction cache. | Cycles |
| Operand cache fill | Number of cycles to fill data cache. | Cycles |
| Elapsed time | Elapsed time. | Cycles |
| Pipeline freeze (by cache miss/ instruction) | Number of cycles in pipeline freeze (by instruction cache misses). | Cycles |
| Pipeline freeze (by cache miss / data) | Number of cycles in pipeline freeze (by data cache misses) | Cycles |
| Pipeline freeze (by branch instruction) | Number of cycles in pipeline freeze (by branch instructions or exceptions). | Cycles |
| Pipeline freeze (by CPU register) | Number of cycles in pipeline freeze (by CPU register conflict). | Cycles |
| Pipeline freeze (by FPU) | Number of cycles in pipeline freeze (by FPU resource conflict). | Cycles |

Note 1.    When you take the profiling information, the execution speed becomes slow, and the number of interrupts such as timer interrupts becomes relatively frequent.    So notice that the counter value of interrupts or functions called from these interrupts becomes greater than they should actually be.

(In the profiler of CodeScape, the function name of interrupt handler is represented as “VBR+600”).

Note 2.    If the debugging information is not available, the function names are not displayed, but their absolute address values are displayed.    When you profile, make sure to specify debug switch to the compiler.    Library functions without debugging information are also displayed with absolute address values.    Find the correspondence of function names and address values using linkage map.

2. Usage of the profiller

- Select Debug->Profile from CodeScape menu.

Profile window shows the list of function names and counter values.

- Set the break point where you want to measure.

Profiling takes longer time than normal execution.    We recommend profiling in some fixed unit of processing.

For example, in order to profile for each frame processing, set the breakpoint to the function called once in each frame processing.    This enables you to get the profiling information for each frame processing.

(We don’t recommend setting the break point at random.    Because you cannot notice when something unexpected happens and data is incorrect.    We recommend measuring fixed interval such as frame processing).

- Counter selection

In the profile window, click right button and select “setup”.    Select two of the items of the counters.

- Preparation for profiling

Execute the program until the breakpoint of (2) is reached.    Make sure that you are at the breakpoint of (2).

- Profiling

Click “OFF” in the upper left of the profile window.    The display turns into “ON”.    From now on, the program will be measured for profiling.

- Execution of the program

Repeat the execution several times, and accumulate the profiling information for these frames.

- Saving profiler data

In the profile window, click the right button, and select “save” menu to save the profiler information.

- Transformation of profiler data by “prfdump”

Using PREFDUMP.EXE, transform the profiler information and display it.

Usage of PRFDUMP.EXE:

prfdump [switch] filename

Switches:

-h		Sort in descending order by hit count

-c1		Sort in descending order by the counter value C1

-c2		Sort in descending order by the counter value C1

default		Sort in ascending order by address

-f=<symbol file>	By specifying a symbol file, displays the symbols

of the corresponding address

The file specified by “-f=” is the file generated by the linkage editor’s “fsymbol” command.    This file includes the external symbols and their address values.    Using this file, you can see the library functions by name.

To generate this file, specify the following command in linkage.

>lnk

:	:

:	:

: fsymbol      P,PGH		; Specify the section name of the symbols to be

; output

:	:

:exit

By this command, you get a file with extension “.fsy”.    Specify this file with “-f” switch.

3.	 Pointing out performance problems using profiler

This section describes which of the profiling items to select, and how to look at the result in order to find out performanc problems.

3.1 Measurement of execution time and pipeline status

- Method

Set counter 1 to “Instruction execution”, and counter 2 to “Elapsed time”, and profile.

By this, you can find the number of times each function is called, the elapsed time of each function (by cycles), number of instructions executed.    The main target of performance tune-up shall be the functions whose elapsed time is long, or functions frequently called.

By dividing the elapsed time by the number of instructions executed, you find the CPI (Cycle per Instruction) of each function.    If this value is big, the function freezes pipeline frequently.    In this case, you should check if the function suffers from instruction/data cache misses.

(2) Tune-up technique

If the number of calls of the function (hit-count) or the elapsed time is large, and the CPI is big, there is a room for tune up to the function.    Especially, reducing the number of memory accesses (load/store) is effective to reduce the number of execution cycles.

Try the improvement of algorithm or tuning to reduce load/store for these functions.

3.2 Measurement of instruction cache miss

(1) Method

Set counter 1 to “instruction cache miss”, and counter 2 to “Elapsed time” and profile.    This measures the number of instruction cache misses and execution time.

(2) Tune-up technique for instruction cache misses

In order to reduce instruction cache misses, you should re-allocate frequently executed functions in a consecutive area of the cache size.    The following shows the method to implement this.

- Save the profiler data

After the measurement of instruction cache misses, save the profiler data to a file.

(b)	Transformation of profiler data using “prfdmp”

Use PRFDMP to display the profiler data sorted by execution time.

<Command line>

PRFDUMP      -c2      xxx.prf    > xxx.log

<Explanation>

Sorts the Codescape’s profiler data by execution time, and stores it into a log file.

(c)	Selection of functions to reallocate

Design the linkage map, so that functions with long elapsed time doesn’t conflict on the instruction cache.    One simple scheme is to arrange the functions in the descending order of elapsed time.

In the file generated in (b), the list of function names of long elapsed time, with the total size 8K byte is listed.

If you select these frequently executed functions whose total size is within 8K byte, and allocate them in some section, these are allocated in consecutive address.    They don’t use the same cache line, therefore no cache conflict occurs for these functions.

(d)	Re-allocation of functions

In order to allocate selected functions for reduced cache conflict, you must allocate these functions to a separate section.    To change the section name, see the document “How to change section name”.

2.4 Measurement of data cache miss

- Method

Set counter 1 to “Data cache miss”, and set counter 2 to “Elapsed time”.    This shows the number of data cache miss and execution time.

- Tune-up techniques for data cache miss

When you find a function with frequent data cache miss, the following techniques are effective.

(a)	Data structure

Design the structured data so that their size is multiple of 32 bytes, so that unwanted cache miss doesn’t occur.

- Global variable allocation

If a set of global variables causes cache conflict, just like the case of instruction cache, allocate them in a separate section using #pragma section, so that they don’t cause cache conflict.

- Tiling

To use operand cache effectively, it is good to localize data accesses.    Consider techniques for localization, such as tiling.

- Data prefetch

If the function accesses an array data in a loop, you can prefetch array elements for future use to improve cache cache usage.

Prefetching is a technique to fill cache before cache miss occurs.