Ninja Library Image Settings

Last update: 1998/09/04

§1 Revision History	3

§1.1 1998/09/04 Version	3

§2 About Screen Resolution Settings	4

§2.1 Introduction	4

§2.2 Precautions	4

§2.3 Display Cycles	4

§2.4 Flicker Free Processing	4

§2.4.1 Type A	4

§2.4.2 Type B	5

§2.5 Symbol Explanation	5

§2.6 Screen Coverage Ratio in Various VRAM Screen Modes	6

§2.6.1 Frame Buffer Size Calculation	6

§2.6.2 Native Buffer Capacity	6

§3 Screen Modes	7

§3.1 Normal Screen Mode (VGA)	7

§3.2 Anti-Aliasing Screen Mode (VGA)	7

§3.3 Normal Screen Mode (NTSC)	7

§3.4 Normal Screen Mode (PAL)	7

§3.5 Anti-Aliasing Screen Mode (NTSC)	7

§3.6 Anti-Aliasing Screen Mode (PAL)	8

§3.7 Screen Coordinate System	8

§4 Screen Mode Details	9

§4.1 NJD_RESOLUTION_VGA	9

§4.2 NJD_RESOLUTION_VGA_ANTI	9

§4.3 NJD_RESOLUTION_320ｘ240_NTSC (PAL) NI	10

§4.4 NJD_RESOLUTION_320ｘ240_NTSC (PAL) I	10

§4.5 NJD_RESOLUTION_320ｘ480_NTSC (PAL) NI	11

§4.6 NJD_RESOLUTION_320ｘ480_NTSC (PAL) I	11

§4.7 NJD_RESOLUTION_320ｘ480_NTSC (PAL) NI_FF	12

§4.8 NJD_RESOLUTION_640ｘ240_NTSC (PAL) NI	12

§4.9 NJD_RESOLUTION_640ｘ240_NTSC (PAL) I	13

§4.10 NJD_RESOLUTION_640ｘ480_NTSC (PAL) NI	13

§4.11 NJD_RESOLUTION_640ｘ480_NTSC (PAL) I	14

§4.12 NJD_RESOLUTION_640ｘ480_NTSC (PAL) NI_FF	14

§4.13 NJD_RESOLUTION_320ｘ240_NTSC (PAL) NI_ANTI	15

§4.14 NJD_RESOLUTION_320ｘ240_NTSC (PAL) I_ANTI	15

§4.15 NJD_RESOLUTION_320ｘ480_NTSC (PAL) NI_ANTI	16

§4.16 NJD_RESOLUTION_320ｘ480_NTSC (PAL) I_ANTI	16

§4.17 NJD_RESOLUTION_320ｘ480_NTSC (PAL) NI_FF_ANTI	17

§4.18 NJD_RESOLUTION_640ｘ240_NTSC (PAL) NI_ANTI	17

§4.19 NJD_RESOLUTION_640ｘ240_NTSC (PAL) I_ANTI	18

§4.20 NJD_RESOLUTION_640ｘ480_NTSC (PAL) NI_ANTI	18

§4.21 NJD_RESOLUTION_640ｘ480_NTSC (PAL) I_ANTI	19

§4.22 NJD_RESOLUTION_640ｘ480_NTSC (PAL) I_ANTI_FF	19

# Revision History

## 1998/09/04 Version

Due to the addition of a Revision History section, the section numbers have changed.

§2.4  Flicker Free Processing [added]

§2.5  Symbol Explanation [added]

§2.6  Screen Coverage Ratio in Various VRAM Screen Modes [added]

§3.  Screen Mode Settings [added] (Sub-section numbers after 3 incremented by 1)

§3.3  Normal Screen Mode (NTSC) [revised]

NJD_RESOLUTION_640x480_NTSCNI_FF

/*-640x480 Flicker Free Type B(１INT Only)-*/

§3.4  Normal Screen Mode (PAL) [revised]

NJD_RESOLUTION_640x480_PALNI_FF

/*-640x480 Flicker Free Type B(１INT Only)-*/

§3.5  Anti-Aliasing Screen Mode (NTSC) [revised]

NJD_RESOLUTION_640x480_NTSCNI_FF _ ANTI

/*-640x480 Flicker Free Type B(１INT Only)-*/

§3.6  Anti-Aliasing Screen Mode (PAL) [revised]

NJD_RESOLUTION_640x480_PALNI_FF _ ANTI

/*-640x480 Flicker Free Type B(１INT Only)-*/

§4.12  NJD_RESOLUTION_640x480_NTSC(PAL)NI_FF Description [revised]

§4.20  NJD_RESOLUTION_640x480_NTSC(PAL)NI_FF_ANTI Description [revised]

§4.7  NJD_RESOLUTION_320x480_NTSC(PAL)NI_FF Description [added]

§4.17  NJD_RESOLUTION_320x480_NTSC(PAL)NI_FF_ANTI Description [added]

# About Screen Resolution Settings

## Introduction

This document explains the screen modes that can be set with the Dreamcast graphics library Ninja. These settings are realized using the Dreamcast Image Super Sampling function. They allow for a wide range of screen modes.

## Precautions

Compared to normal image drawing modes, the Image Super Sampling method provides an anti-aliasing effect over the entire screen, but the draw area increases by a factor of 2 - 4.

## Display Cycles

･	NTSC standard	:1INT is 1/60 second. (2INT is 1/30 second.)

･	PAL standard	:1INT is 1/50 second. (2INT is 1/25 second.)

･	VGA standard	:1INT is 1/60 second. (2INT is 1/30 second.)

## Flicker Free Processing

A circuit to reduce flicker (vertical fluctuation) during interlaced display has been added to Dreamcast. This section gives a general explanation of how this circuit works.

The flicker free function makes use of the Image Super Sampling feature. In the Power VR architecture, all display processing functions are completed within the video chip, and the result is copied to the frame buffer. At this point, 3-line filtering is carried out, using one of the two types described below.

### Type A

The Y scaling factor for Image Super Sampling is set to close to 1.0, to activate Y filtering. The output data of the 3-line filter is stored in the frame buffer. This means that the result of the 480-line screen draw process will be stored in the frame buffer with all 480 lines. Screen draw therefore needs only to be carried out once per frame (1/30 or 1/25 seconds).

The start address for reading from the frame buffer is shifted by one line for each field, and 1 line each is skipped.

### Type B

Filtering is carried out for three lines of data in Image Super Sampling. The result is scaled to 1/2, and only the line data for the specified field are stored in the frame buffer. This means that screen drawing must be performed for 480 lines, but the pixel data stored in the frame buffer need only cover 240 lines.

Since this kind of processing is performed in tile units when the drawn pixel data are sent to the frame buffer, screen draw needs to be carried out once per field (1/60 or 1/50 seconds).

For display, the data are read from the frame buffer as is.

## Symbol Explanation

This section explains the format of the first argument used for screen mode setting.

(Example 1)  NJD_RESOLUTION_640x480_PALNI_FF_ANTI

640ｘ480	:	Resolution

PAL	:	PAL format

NI	:	Number of frames (60 frames per second in this example)

FF	:	Type B flicker free interlace processing (Type A is set automatically for interlace except single-density)

ANTI	:	Anti-aliasing used

(Example 2)  NJD_RESOLUTION_640x480_NTSCI

NTSC	:	NTSC format

I	:	Number of frames (30 frames per second in this example)
Because of dual-density interlace, type A flicker free processing is selected.

## Screen Coverage Ratio in Various VRAM Screen Modes

The Holly2 VRAM has the following three buffers.

(1)	Native Buffer	:Vertex data buffer

(2)	Texture Buffer	:Texture data buffer

(3)	Frame Buffer	:Screen draw result data buffer

### Frame Buffer Size Calculation

The frame buffer size is calculated as follows.

| Screen resolution | Color size | Screen resolution x color size x buffer number |
| --- | --- | --- |
| 320x240 | 16BPP | (320ｘ240) ｘ 2 Byte ｘ 2 Buffer = 307200 Byte |
|  | 24BPP | (320ｘ240) ｘ 3 Byte ｘ 2 Buffer = 460800 Byte |
|  | 32BPP | (320ｘ240) ｘ 4 Byte ｘ 2 Buffer = 614400 Byte |
| 320x480 | 16BPP | (320ｘ480) ｘ 2 Byte ｘ 2 Buffer = 614400 Byte |
|  | 24BPP | (320ｘ480) ｘ 3 Byte ｘ 2 Buffer = 921600 Byte |
|  | 32BPP | (320ｘ480) ｘ 4 Byte ｘ 2 Buffer = 1228800 Byte |
| 640x240 | 16BPP | (640ｘ240) ｘ 2 Byte ｘ 2 Buffer = 614400 Byte |
|  | 24BPP | (640ｘ240) ｘ 3 Byte ｘ 2 Buffer = 921600 Byte |
|  | 32BPP | (640ｘ240) ｘ 4 Byte ｘ 2 Buffer = 1228800 Byte |
| 640x480 | 16BPP | (640ｘ480) ｘ 2 Byte ｘ 2 Buffer = 1228800 Byte |
|  | 24BPP | (640ｘ480) ｘ 3 Byte ｘ 2 Buffer = 1843200 Byte |
|  | 32BPP | (640ｘ480) ｘ 4 Byte ｘ 2 Buffer = 2457600 Byte |

The above conditions do not change with anti-aliasing, but when type B flicker free processing is used, the data will become halved, as follows.

NJD_RESOLUTION _640ｘ480_NTSCNI_FF

NJD_RESOLUTION _640ｘ480_NTSCNI_FF_ANTI

This is identical to 640x240 resolution.

### Native Buffer Capacity

The native buffer capacity is calculated as follows.

Native buffer capacity = VRAM overall capacity - texture buffer size + display frame buffer size

However, the native buffer (double buffer) overall capacity cannot be more than half that of the frame buffer overall capacity. This means that in mass production systems with an 8 MB frame buffer, the native data buffer is

max. 2 MB x 2

If the result of the above formula exceeds this value, the native buffer (double buffer) will use all the memory up to the limit, and use the remainder as texture area.

# Screen Modes

The screen mode types as defined in NinjaDef.h are as follows. To switch modes, the functions sbInitSystem(), njInitSystem(), and njChangeSystem() are used.

## Normal Screen Mode (VGA)

NJD_RESOLUTION_VGA		/*----VGA mode--------------------------------------*/

## Anti-Aliasing Screen Mode (VGA)

NJD_RESOLUTION_VGA_ANTI	/*----VGA mode--------------------------------------*/

## Normal Screen Mode (NTSC)

NJD_RESOLUTION_320x240_NTSCNI	/* --- 320x240 non-interlaced ------------------------- */

NJD_RESOLUTION_320x240_NTSCI	/* --- 320x240 single-density interlaced ----------- */

NJD_RESOLUTION_320x480_NTSCNI	/* --- 320x480 double-density interlaced 
(Type A) --------- */

NJD_RESOLUTION_320x480_NTSCI	/* --- 320x480 double-density interlaced 
(Type A) --------- */

NJD_RESOLUTION_320x480_NTSCNI_FF	/* --- 320x480 double-density interlaced 
(Type B) --------- */

NJD_RESOLUTION_640x240_NTSCNI	/* --- 640x240 non-interlaced ------------------------- */

NJD_RESOLUTION_640x240_NTSCI	/* --- 640x240 single-density interlaced ----------- */

NJD_RESOLUTION_640x480_NTSCNI	/* --- 640x480 double-density interlaced 
(Type A) --------- */

NJD_RESOLUTION_640x480_NTSCI	/* --- 640x480 double-density interlaced 
(Type A) --------- */

NJD_RESOLUTION_640x480_NTSCNI_FF	/* --- 640x480 double-density interlaced 
(Type B) --------- */

## Normal Screen Mode (PAL)

NJD_RESOLUTION_320x240_PALNI	/* --- 320x240 non-interlaced ------------------------- */

NJD_RESOLUTION_320x240_PALI	/* --- 320x240 single-density interlaced ----------- */

NJD_RESOLUTION_320x480_PALNI	/* --- 320x480 double-density interlaced 
(Type A) ----- */

NJD_RESOLUTION_320x480_PALI	/* --- 320x480 double-density interlaced 
(Type A) ----- */

NJD_RESOLUTION_320x480_PALNI_FF	/* --- 320x480 double-density interlaced 
(Type B) ----- */

NJD_RESOLUTION_640x240_PALNI	/* --- 640x240 non-interlaced ------------------------- */

NJD_RESOLUTION_640x240_PALI	/* --- 640x240 single-density interlaced ----------- */

NJD_RESOLUTION_640x480_PALNI	/* --- 640x480 double-density interlaced 
(Type A) ----- */

NJD_RESOLUTION_640x480_PALI	/* --- 640x480 double-density interlaced 
(Type A) ----- */

NJD_RESOLUTION_640x480_PALNI_FF	/* --- 640x480 double-density interlaced 
(Type B) ----- */

## Anti-Aliasing Screen Mode (NTSC)

NJD_RESOLUTION_320x240_NTSCNI_ANTI	/* --- 320x240 non-interlaced --------------- */

NJD_RESOLUTION_320x240_NTSCI_ANTI	/* --- 320x240 single-density interlaced - */

NJD_RESOLUTION_320x480_NTSCNI_ANTI	/* --- 320x480 double-density interlaced 
(Type A) ----- */

NJD_RESOLUTION_320x480_NTSCI_ANTI	/* --- 320x480 double-density interlaced 
(Type A) ----- */

NJD_RESOLUTION_320x480_NTSCNI_FF_ANTI	/* --- 320x480 double-density interlaced 
(Type B) ----- */

NJD_RESOLUTION_640x240_NTSCNI_ANTI	/* --- 640x240 non-interlaced --------------- */

NJD_RESOLUTION_640x240_NTSCI_ANTI	/* --- 640x240 single-density interlaced - */

NJD_RESOLUTION_640x480_NTSCNI_ANTI	/* --- 640x480 double-density interlaced 
(Type A) ----- */

NJD_RESOLUTION_640x480_NTSCI_ANTI	/* --- 640x480 double-density interlaced 
(Type A) ----- */

NJD_RESOLUTION_640x480_NTSCNI_FF_ANTI 	/* --- 640x480 double-density interlaced 
(Type B) ----- */

## Anti-Aliasing Screen Mode  (PAL)

NJD_RESOLUTION_320x240_PALNI_ANTI	/* --- 320x240 non-interlaced --------------- */

NJD_RESOLUTION_320x240_PALI_ANTI	/* --- 320x240 single-density interlaced - */

NJD_RESOLUTION_320x480_PALNI_ANTI	/* --- 320x480 double-density interlaced 
(Type A) ----- */

NJD_RESOLUTION_320x480_PALI_ANTI	/* --- 320x480 double-density interlaced 
(Type A) ----- */

NJD_RESOLUTION_320x480_PALNI_FF_ANTI	/* --- 320x480 double-density interlaced 
(Type B) ----- */

NJD_RESOLUTION_640x240_PALNI_ANTI	/* --- 640x240 non-interlaced --------------- */

NJD_RESOLUTION_640x240_PALI_ANTI	/* --- 640x240 single-density interlaced - */

NJD_RESOLUTION_640x480_PALI_ANTI	/* --- 640x480 double-density interlaced 
(Type A) ----- */

NJD_RESOLUTION_640x480_PALI_ANTI	/* --- 640x480 double-density interlaced 
(Type A) ----- */

NJD_RESOLUTION_640x480_PALNI_FF_ANTI	/* --- 640x480 double-density interlaced 
(Type B) ----- */

## Screen Coordinate System

The screen coordinate system defined by Ninja is as follows.

# Screen Mode Details

## NJD_RESOLUTION_VGA

This is the PC/AT standard VGA mode. It cannot be displayed on a normal TV. The aspect ratio of the screen coordinate system is set to 1:1 (vertical:horizontal).

## NJD_RESOLUTION_VGA_ANTI

This is the PC/AT standard VGA mode. It cannot be displayed on a normal TV. Image Super Sampling is used to provide anti-aliasing in the horizontal direction. The rendering size is 1280 x 480 dots, while the size that is actually stored in the frame buffer and displayed is 640 x 480 dots. The screen aspect ratio is squashed to

1:0.5 (vertical:horizontal).

For 3D, the function njSetAspect() can be used for compensation, but for 2D, the image must be vertically compressed beforehand.

## NJD_RESOLUTION_320ｘ240_NTSC (PAL) NI

This is a TV display mode. It cannot be displayed on a PC/AT type computer monitor. Because non-interlaced operation prevents the EVEN field from being displayed, there are black horizontal stripes.

The aspect ratio of the screen coordinate system is 1:1 (vertical:horizontal), but when shown on a TV, it becomes as follows.

NTSC format: 1:0.91

PAL format:  1:1.27

For 3D only, the function njSetAspect() can be used for compensation.

## NJD_RESOLUTION_320ｘ240_NTSC (PAL) I

This is a TV display mode. It cannot be displayed on a PC/AT type computer monitor. Single-density interlacing causes both the ODD and EVEN fields to show the same image. This leads to vertical instability (flicker).

The aspect ratio of the screen coordinate system is 1:1 (vertical:horizontal), but when shown on a TV, it becomes as follows.

NTSC format: 1:0.91

PAL format:  1:1.27

For 3D only, the function njSetAspect() can be used for compensation.

## NJD_RESOLUTION_320ｘ480_NTSC (PAL) NI

This is a TV display mode. It cannot be displayed on a PC/AT type computer monitor. Double-density interlacing causes different images to be shown in the ODD and EVEN fields, which improves image quality. Because the image is updated every 1INT in this mode, either the ODD or EVEN image is discarded when displaying from the frame buffer.

The aspect ratio of the screen coordinate system is 1:0.5 (vertical:horizontal), but when shown on a TV, it is squashed by 1/2 in the vertical direction. For 3D only, the function njSetAspect() can be used for compensation.

## NJD_RESOLUTION_320ｘ480_NTSC (PAL) I

This is a TV display mode. It cannot be displayed on a PC/AT type computer monitor. Double-density interlacing causes different images to be shown in the ODD and EVEN fields, which improves image quality. Because the image is updated every 2INT in this mode, no image is discarded when displaying from the frame buffer, but games with 1INT processing cannot be realized. Also, processing losses can cause a drop to 4INT.

The aspect ratio of the screen coordinate system is 1:0.5 (vertical:horizontal), but when shown on a TV, it is squashed by 1/2 in the vertical direction. For 3D only, the function njSetAspect() can be used for compensation.

## NJD_RESOLUTION_320ｘ480_NTSC (PAL) NI_FF

This is a TV display mode. It cannot be displayed on a PC/AT type computer monitor.

Double-density interlacing causes different images to be shown in the ODD and EVEN fields, which improves image quality. In this mode, either the ODD or EVEN image is discarded when storing a rendered image in the frame buffer. Therefore the size of the frame buffer needs only be half that of the normal size, but the image must be fully drawn within 1INT. If there is a CPU processing dropout, the image of two screens earlier will be shown. If there is a draw processing dropout, the image of two tiles earlier will be shown. This causes the image to be impaired. Type B flicker free filter processing is applied.

The aspect ratio of the screen coordinate system is 1:0.5 (vertical:horizontal), but when shown on a TV, it is squashed by 1/2 in the vertical direction. For 3D only, the function njSetAspect() can be used for compensation.

## NJD_RESOLUTION_640ｘ240_NTSC (PAL) NI

This is a TV display mode. It cannot be displayed on a PC/AT type computer monitor. Because non-interlaced operation prevents the EVEN field from being displayed, there are black horizontal stripes.

The aspect ratio of the screen coordinate system is 1:1 (vertical:horizontal), but when shown on a TV, it becomes as follows.

NTSC format: 1:0.45

PAL format:  1:0.64

For 3D only, the function njSetAspect() can be used for compensation.

## NJD_RESOLUTION_640ｘ240_NTSC (PAL) I

This is a TV display mode. It cannot be displayed on a PC/AT type computer monitor. Single-density interlacing causes both the ODD and EVEN fields to show the same image. This leads to vertical instability (flicker).

The aspect ratio of the screen coordinate system is 1:1 (vertical:horizontal), but when shown on a TV, it becomes as follows.

NTSC format: 1:0.45

PAL format:  1:0.64

For 3D only, the function njSetAspect() can be used for compensation.

## NJD_RESOLUTION_640ｘ480_NTSC (PAL) NI

This is a TV display mode. It cannot be displayed on a PC/AT type computer monitor. Double-density interlacing causes different images to be shown in the ODD and EVEN fields, which improves image quality. Because the image is updated every 1INT in this mode, either the ODD or EVEN image is discarded when displaying from the frame buffer.

The aspect ratio of the screen coordinate system is 1:1 (vertical:horizontal), but when shown on a TV, it becomes as follows.

NTSC format: 1:0.91

PAL format:  1:1.27

For 3D only, the function njSetAspect() can be used for compensation.

## NJD_RESOLUTION_640ｘ480_NTSC (PAL) I

This is a TV display mode. It cannot be displayed on a PC/AT type computer monitor. Double-density interlacing causes different images to be shown in the ODD and EVEN fields, which improves image quality. Because the image is updated every 2INT in this mode, no image is discarded when displaying from the frame buffer, but games with 1INT processing cannot be realized. Also, processing losses can cause a drop to 4INT.

The aspect ratio of the screen coordinate system is 1:1 (vertical:horizontal), but when shown on a TV, it becomes as follows.

NTSC format: 1:0.91

PAL format:  1:1.27

For 3D only, the function njSetAspect() can be used for compensation.

## NJD_RESOLUTION_640ｘ480_NTSC (PAL) NI_FF

This is a TV display mode. It cannot be displayed on a PC/AT type computer monitor. Double-density interlacing causes different images to be shown in the ODD and EVEN fields, which improves image quality. In this mode, either the ODD or EVEN image is discarded when storing a rendered image in the frame buffer. Therefore the size of the frame buffer needs only be half that of the normal size, but the image must be fully drawn within 1INT. If there is a draw processing dropout, the image of two tiles earlier will be shown. This causes the image to be impaired. Type B flicker free filter processing is applied.

The aspect ratio of the screen coordinate system is 1:1 (vertical:horizontal), but when shown on a TV, it becomes as follows.

NTSC format: 1:0.91

PAL format:  1:1.27

For 3D only, the function njSetAspect() can be used for compensation.

## NJD_RESOLUTION_320ｘ240_NTSC (PAL) NI_ANTI

This is a TV display mode. It cannot be displayed on a PC/AT type computer monitor. Because non-interlaced operation prevents the EVEN field from being displayed, there are black horizontal stripes.

Image Super Sampling is used to provide anti-aliasing in the horizontal and vertical direction. The rendering size is 640 x 480 dots, while the size that is actually stored in the frame buffer and displayed is 320 x 240 dots.

The aspect ratio of the screen coordinate system is 1:1 (vertical:horizontal), but when shown on a TV, it becomes as follows.

NTSC format: 1:0.91

PAL format:  1:1.27

For 3D only, the function njSetAspect() can be used for compensation.

## NJD_RESOLUTION_320ｘ240_NTSC (PAL) I_ANTI

This is a TV display mode. It cannot be displayed on a PC/AT type computer monitor. Single-density interlacing causes both the ODD and EVEN fields to show the same image. This leads to vertical instability (flicker).

Image Super Sampling is used to provide anti-aliasing in the horizontal and vertical direction. The rendering size is 640 x 480 dots, while the size that is actually stored in the frame buffer and displayed is 320 x 240 dots.

The aspect ratio of the screen coordinate system is 1:1 (vertical:horizontal), but when shown on a TV, it becomes as follows.

NTSC format: 1:0.91

PAL format:  1:1.27

For 3D only, the function njSetAspect() can be used for compensation.

## NJD_RESOLUTION_320ｘ480_NTSC (PAL) NI_ANTI

This is a TV display mode. It cannot be displayed on a PC/AT type computer monitor. Double-density interlacing causes different images to be shown in the ODD and EVEN fields, which improves image quality. Because the image is updated every 1INT in this mode, either the ODD or EVEN image is discarded when displaying from the frame buffer.

Image Super Sampling is used to provide anti-aliasing in the horizontal direction. The rendering size is 640 x 480 dots, while the size that is actually stored in the frame buffer and displayed is 320 x 480 dots.

The aspect ratio of the screen coordinate system is 1:1 (vertical:horizontal), but when shown on a TV, it becomes as follows.

NTSC format: 1:0.91

PAL format:  1:1.27

For 3D only, the function njSetAspect() can be used for compensation.

## NJD_RESOLUTION_320ｘ480_NTSC (PAL) I_ANTI

This is a TV display mode. It cannot be displayed on a PC/AT type computer monitor. Double-density interlacing causes different images to be shown in the ODD and EVEN fields, which improves image quality. Because the image is updated every 2INT in this mode, no image is discarded when displaying from the frame buffer, but games with 1INT processing cannot be realized. Also, processing losses can cause a drop to 4INT.

Image Super Sampling is used to provide anti-aliasing in the horizontal direction. The rendering size is 640 x 480 dots, while the size that is actually stored in the frame buffer and displayed is 320 x 480 dots.

The aspect ratio of the screen coordinate system is 1:1 (vertical:horizontal), but when shown on a TV, it becomes as follows.

NTSC format: 1:0.91

PAL format:  1:1.27

For 3D only, the function njSetAspect() can be used for compensation.

## NJD_RESOLUTION_320ｘ480_NTSC (PAL) NI_FF_ANTI

This is a TV display mode. It cannot be displayed on a PC/AT type computer monitor.

Double-density interlacing causes different images to be shown in the ODD and EVEN fields, which improves image quality. In this mode, either the ODD or EVEN image is discarded when storing a rendered image in the frame buffer. Therefore the size of the frame buffer needs only be half that of the normal size, but the image must be fully drawn within 1INT. If there is a CPU processing dropout, the image of two screens earlier will be shown. If there is a draw processing dropout, the image of two tiles earlier will be shown. This causes the image to be impaired. Type B flicker free filter processing is applied.

Image Super Sampling is used to provide anti-aliasing in the horizontal and vertical direction. The rendering size is 640 x 480 dots, while the size that is actually stored in the frame buffer and displayed is 320 x 480 dots.

The aspect ratio of the screen coordinate system is 1:1 (vertical:horizontal), but when shown on a TV, it becomes as follows.

NTSC format: 1:0.91

PAL format:  1:1.27

For 3D only, the function njSetAspect() can be used for compensation.

## NJD_RESOLUTION_640ｘ240_NTSC (PAL) NI_ANTI

This is a TV display mode. It cannot be displayed on a PC/AT type computer monitor. Because non-interlaced operation prevents the EVEN field from being displayed, there are black horizontal stripes.

Image Super Sampling is used to provide anti-aliasing in the horizontal and vertical direction. The rendering size is 1280 x 480 dots, while the size that is actually stored in the frame buffer and displayed is 640 x 240 dots.

The aspect ratio of the screen coordinate system is 1:1 (vertical:horizontal), but when shown on a TV, it becomes as follows.

NTSC format: 1:0.45

PAL format:  1:0.64

For 3D only, the function njSetAspect() can be used for compensation.

## NJD_RESOLUTION_640ｘ240_NTSC (PAL) I_ANTI

This is a TV display mode. It cannot be displayed on a PC/AT type computer monitor. Single-density interlacing causes both the ODD and EVEN fields to show the same image. This leads to vertical instability (flicker).

Image Super Sampling is used to provide anti-aliasing in the horizontal and vertical direction. The rendering size is 1280 x 480 dots, while the size that is actually stored in the frame buffer and displayed is 640 x 240 dots.

The aspect ratio of the screen coordinate system is 1:1 (vertical:horizontal), but when shown on a TV, it becomes as follows.

NTSC format: 1:0.45

PAL format:  1:0.64

For 3D only, the function njSetAspect() can be used for compensation.

## NJD_RESOLUTION_640ｘ480_NTSC (PAL) NI_ANTI

This is a TV display mode. It cannot be displayed on a PC/AT type computer monitor. Double-density interlacing causes different images to be shown in the ODD and EVEN fields, which improves image quality. Because the image is updated every 1INT in this mode, either the ODD or EVEN image is discarded when displaying from the frame buffer. Type A flicker free filter processing is applied.

Image Super Sampling is used to provide anti-aliasing in the horizontal direction. The rendering size is 1280 x 480 dots, while the size that is actually stored in the frame buffer and displayed is 640 x 480 dots.

The aspect ratio of the screen coordinate system is 1:1 (vertical:horizontal), but when shown on a TV, it becomes as follows.

NTSC format: 1:0.45

PAL format:  1:0.64

For 3D only, the function njSetAspect() can be used for compensation.

## NJD_RESOLUTION_640ｘ480_NTSC (PAL) I_ANTI

This is a TV display mode. It cannot be displayed on a PC/AT type computer monitor. Double-density interlacing causes different images to be shown in the ODD and EVEN fields, which improves image quality. Because the image is updated every 2INT in this mode, no image is discarded when displaying from the frame buffer, but games with 1INT processing cannot be realized. Also, processing losses can cause a drop to 4INT.

Image Super Sampling is used to provide anti-aliasing in the horizontal direction. The rendering size is 1280 x 480 dots, while the size that is actually stored in the frame buffer and displayed is 640 x 480 dots.

The aspect ratio of the screen coordinate system is 1:1 (vertical:horizontal), but when shown on a TV, it becomes as follows.

NTSC format: 1:0.45

PAL format:  1:0.64

For 3D only, the function njSetAspect() can be used for compensation.

## NJD_RESOLUTION_640ｘ480_NTSC (PAL) I_ANTI_FF

This is a TV display mode. It cannot be displayed on a PC/AT type computer monitor. Double-density interlacing causes different images to be shown in the ODD and EVEN fields, which improves image quality. In this mode, either the ODD or EVEN image is discarded when storing a rendered image in the frame buffer. Therefore the size of the frame buffer needs only be half that of the normal size, but the image must be fully drawn within 1INT. If there is a drop of even one image, ODD and EVEN will be reversed which causes the image to be impaired.

Image Super Sampling is used to provide anti-aliasing in the horizontal direction. The rendering size is 1280 x 480 dots, while the size that is actually stored in the frame buffer and displayed is 640 x 480 dots.

The aspect ratio of the screen coordinate system is 1:1 (vertical:horizontal), but when shown on a TV, it becomes as follows.

NTSC format: 1:0.45

PAL format:  1:0.64

For 3D only, the function njSetAspect() can be used for compensation.