- Development of Dreamcast Application

- Peripheral Guide

- Sega Enterprises, Ltd.

- Contents

1	Overview	3

1-1	Types of Peripherals	3

2	Access to the Control Ports	4

2-1	Features of the Dreamcast Control Ports	4

2-2	Control Port Initialization	5

2-3	Assimilation of Types of Controllers Connected to the Ports	7

3	Using Each Device	9

3-1	Control Devices	9

3-2	LCD Device	13

3-3	Timer Device	15

3-4	Sound Input Device	16

3-5	Keyboard device	19

3-6	Gun device	22

3-7	Vibration Device	24

4	Storage device (Memory card)	31

4-1	Incorporation into Typical Applications	32

4-2	File Access in Typical Applications	34

4-3	File Access in Special Applications	42

4-4	Execution Files	46

4-5	Backup File Format	50

4-6	Securing dynamic work buffer	63

5	AV cable peripheral	66

6	Original Information for Each Peripheral	68

6-1	Dreamcast Controller (HKT-7700)	68

6-2	Racing Controller (HKT-7400)	69

6-3	Arcade Stick (HKT-7300)	70

6-4	Dreamcast Keyboard (HKT-7600)	71

6-5	Dreamcast Gun (HKT-7801)	72

6-6	Tsuri Controller (preliminary) (HKT-8700)	73

6-7	Visual Memory (HKT-7002)	74

6-8	Puru Puru Pack (HKT-8600)	75

6-9	Mike Device –Unit for sound recognition and input- (HKT-7200)	75

- Mar. 11, 1999  First Edition, First Print

#### Sega Enterprises, Ltd., Technical Support Center

Copyright © 1999 SEGA ENTERPRISES, LTD.

# Overview

Thank you for your interest in developing applications on SEGA's Dreamcast.

This guide summarizes information about each peripheral necessary for developing applications using Dreamcast.

## 1-1	Types of Peripherals

Controller Peripheral

The type of peripheral inserted in the control port of the Dreamcast system. Includes system accessories "Dreamcast Controller" and "Dreamcast Keyboard".

Expansion Socket Peripheral

The type of peripheral inserted in the expansion socket of such peripherals as "Dreamcast Controller". Includes "Visual Memory" and "Puru Puru Pack".

Compound Peripheral

A peripheral that looks like a single peripheral in which multiple peripherals are combined. A peripheral is not compound because more than one type of peripheral is connected to the same port, but because a peripheral that is inserted in the expansion socket (expansion peripheral) already has an internal peripheral. Includes the tsuri (fishing) controller.

Video Cable Peripheral

Peripherals connected to the video output jack that connects to the back of the Dreamcast unit. Dreamcast can distinguish which video cable is connected from the application.

Extension Bus Peripherals

Peripherals that connect to the extension bus that appears when the modem included with Dreamcast is disconnected. Currently there is no peripheral besides the modem. This guide currently does not have any information available regarding extension bus peripherals.

# Access to the Control Ports

This chapter explains how to access controller peripherals, expansion socket peripherals, and compound peripherals. The control ports have the following features.

## 2-1	Features of the Dreamcast Control Ports

n	Four Peripherals on Control Ports of Dreamcast

Up to four controller peripherals or compound peripherals can be connected to the control ports. Currently peripherals that branch ports like the "Multi terminal 6" port for Saturn are not planned.

n	On each port, up to 5 expansion peripherals can be logically connected.

When there is an expansion socket available through a controller peripheral connected to the control port, it is possible to connect an expansion peripheral. In specs, a maximum of 5 expansion peripherals can be connected. Actually, 1 or 2 peripherals for Dreamcast are generally connected to the expansion sockets. Also, there are peripherals that are internally built-in to expansion peripherals without expansion sockets.。

n	Peripheral Specs are asked from Peripherals

An application can know "What is the peripheral connected?" and "What specs does the connected peripheral have?" by asking the connected peripheral. It is necessary to set each setting for the supported peripheral to base this information on.

n	The maximum current and stand-by current values are recorded in each peripheral

A large amount of power is needed for the vibration peripheral within a peripheral. There is an upper limit to the power that can be supplied to a control port and a peripheral cannot exceed this value. Therefore, it is necessary to observe that the value of the total power that flows to peripherals in all ports that is calculated by the application does not exceed the maximum power that can be supplied.。

n	A library is prepared exclusively for special peripherals

- Gun has an exclusive initialization function. Be aware that when the gun is enabled, other devices may not be able to be used.

- Keyboard can only currently determine if a key is pressed. Auto repeat and FEP must be supported by each application.

- Only sound input is supported in the mike device. Currently sound recognition must be carried out by the application.

## 2-2	Control Port Initialization

There are two types of functions prepared for the initialization that occurs before using a control port. It is necessary to execute an initialization function when starting any kind of application.

- Initialization by pdInitPeripheral() Function

Initialization that assumes peripherals used by the application are in control ports A-D and in the 2 expansion ports in them. With this function, all control ports A-D are initialized, but the necessary work space is about 96KB; it requires a large capacity.

The initial argument of the function pdInitPeripheral() is the argument that chooses the positive and negative logic of digital button information bits of PDS_PERIPHERAL structure and specifies PDD_PLOGIC_ACTIVE (positive logic) or PDD_PLOGIC _NEGATIVE (negative logic).

The remaining two functions specify the send buffer and receive buffer pointers. The send buffer is the buffer to create the send command to the hardware and is the buffer for storing data from the receive buffer peripheral. It is better to secure 32 bytes of free space as it automatically aligns if the buffer address is not within the boundary of 32bytes.

The send and receive buffers necessary for pdInitPeripheral() specify fixed values as shown in the example below.

/* Declare global work */

Uint8 RecvBuf[1024 * 24 * 2 + 32];

Uint8 SendBuf[1024 * 24 * 2 + 32];

/* Intialize controller library */

pdInitPeripheral( PDD_PLOGIC_ACTIVE, RecvBuf, SendBuf );

- Initialization by pdInitPeripheralEx() Function

Initialization that is extended to initialize a control port that has already been specified for use by an application.

By limiting initialization to the control port being used, the work area space is greatly reduced, and internal processing of the library is faster.

The initial argument of the function pdInitPeripheralEx() is the argument that chooses positive logic or negative logic of digital button information bits of the PDS_PERIPHERAL structure, in the same way as the pdInitPeriperal() function.

The next argument specifies the peripheral table. This is the argument that decides which peripheral will be used. The argument specifies by peripheral port number array, as shown in the example array below.

const Sint32 Devs[] = {

PDD_PORT_A0, PDD_PORT_A1, PDD_PORT_A2,    /* Port A0/A1/A2 */

PDD_PORT_B0, PDD_PORT_B1, PDD_PORT_B2,    /* Port B0/B1/B2 */

PDD_PORT_C0, PDD_PORT_C1, PDD_PORT_C2,    /* Port C0/C1/C2 */

PDD_PORT_D0, PDD_PORT_D1, PDD_PORT_D2,    /* Port D0/D1/D2 */

};

The following two arguments specify the send buffer and receive buffer pointers, but they specify the value already calculated from the control port number used, rather than with fixed values. Secure the necessary space using the PDM_WORK_SIZE macro. For each peripheral, 2KB is

necessary.

The PDM_WORK_SIZE macro is the macro that secures the argument x number of ports. In the following example, the work area for 12 ports is secured.

Uint8 SendBuf[PDM_WORK_SIZE(12) + 32];

For the receive buffer as well, 2KB is necessary for each peripheral. In the following example, work area for 12 peripherals is secured. It is not necessary for these buffers to be connected.

- Uint8RecvBuf[12][PDM_WORK_SIZE(1) + 32];

The last argument is the receive buffer address table. This table specifies which peripheral uses which work area. Be sure to allocate a buffer for each specified peripheral with the peripheral table. If a buffer is not allocated for each peripheral, the library cannot operate correctly.

const void* RecvBufTbl[] = {

RecvBuf[0],                 /* Work area of A0, 2KB */

RecvBuf[1],                 /* Work area of A1, 2KB */

RecvBuf[2],                 /* Work area of A2, 2KB */

RecvBuf[3],                 /* Work area of B0, 2KB */

RecvBuf[4],                 /* Work area of B1, 2KB */

RecvBuf[5],                 /* Work area of B2, 2KB */

RecvBuf[6],                 /* Work area of C0, 2KB */

RecvBuf[7],                 /* Work area of C1, 2KB */

RecvBuf[8],                 /* Work area of C2, 2KB */

RecvBuf[9],                 /* Work area of D0, 2KB */

RecvBuf[10],                /* Work area of D1, 2KB */

RecvBuf[11],                /* Work area of D2, 2KB */

};

- Transfer the above parameters to the function pdInitPeripheralEx().

pdInitPeripheralEx(PDD_PLOGIC_ACTIVE, Devs, RecvBufTbl, SendBuf,

sizeof(Devs) / sizeof(Devs[0]));

When initialization finishes, access to the peripheral is possible. Vibration peripherals and the LCD and Memory Card (Backup) libraries can be used. However, you cannot check which peripherals are disabled here. Disabled peripherals will be returned as unconnected.

* Note

- When a port root peripheral (A0, B0, C0, D0) is set not to be used, the expansion port of that port (A1, A2, etc.) cannot be used either.

- List of Function

| Function | Description |
| --- | --- |
| pdInitPeripheral | Initializes peripheral library |
| pdInitPeripheralEx | Initializes peripheral library specially |
| PDM_WORK_SIZE | Macro to calculate work area size |

## 2-3	Assimilation of Types of Controllers Connected to the Ports

To get information about the type of peripherals connected to the control ports, use the pdGetPeripheralInfo() function. This function returns the PDS_PERIPHERALINFO structure pointer by executing the given argument of the port you want to get. In addition to getting information for each control port A-D, the argument port can also specify the peripherals connected to the expansion socket.

- Port numbers specified in the argument port are as follow.

| Definition | Description |
| --- | --- |
| PDD_PORT_A0 | Controller of controller port A |
| PDD_PORT_A1 | Expansion socket 1 of the controller port A |
| PDD_PORT_A2 | Expansion socket 2 of the controller port A |
| PDD_PORT_A3 | Expansion socket 3 of the controller port A |
| PDD_PORT_A4 | Expansion socket 4 of the controller port A |
| PDD_PORT_A5 | Expansion socket 5 of the controller port A |
| PDD_PORT_B0 | Controller of controller port B |
| PDD_PORT_B1 | Expansion socket 1 of the controller port B |
| PDD_PORT_B2 | Expansion socket 2 of the controller port B |
| PDD_PORT_B3 | Expansion socket 3 of the controller port B |
| PDD_PORT_B4 | Expansion socket 4 of the controller port B |
| PDD_PORT_B5 | Expansion socket 5 of the controller port B |
| PDD_PORT_C0 | Controller of controller port C |
| PDD_PORT_C1 | Expansion socket 1 of the controller port C |
| PDD_PORT_C2 | Expansion socket 2 of the controller port C |
| PDD_PORT_C3 | Expansion socket 3 of the controller port C |
| PDD_PORT_C4 | Expansion socket 4 of the controller port C |
| PDD_PORT_C5 | Expansion socket 5 of the controller port C |
| PDD_PORT_D0 | Controller of controller port D |
| PDD_PORT_D1 | Expansion socket 1 of the controller port D |
| PDD_PORT_D2 | Expansion socket 2 of the controller port D |
| PDD_PORT_D3 | Expansion socket 3 of the controller port D |
| PDD_PORT_D4 | Expansion socket 4 of the controller port D |
| PDD_PORT_D5 | Expansion socket 5 of the controller port D |

- PDS_PERIPHERALINFO structures are as follows.

typedef struct {

Uint32 type;		/* Type of peripheral */

Uint32 reserved[3];	/* Reserved */

Uint8 is_root;		/* Root devise flag */

Uint8 area_code;		/* Area code */

Uint8 connector_dir[2];	/* Direction of expansion socket */

Sint8 product_name[32];	/* Name of product */

Sint8 license[64];	/* License letter string */

Uint16 stdby_pow;		/* Consumed electric current at standby position*/

Uint16 max_pow;		/* Maximum consumed electric current */

} PDS_PERIPHERALINFO;

- In the member type that returns the peripheral type, the following values are defined.

| Definition | Description |
| --- | --- |
| PDD_DEVTYPE_CONTROLLER | Controller device |
| PDD_DEVTYPE_STORAGE | Storage device (memory card) |
| PDD_DEVTYPE_LCD | LCD device |
| PDD_DEVTYPE_TIMER | Timer device |
| PDD_DEVTYPE_SOUNDINPUT | Sound input device |
| PDD_DEVTYPE_KEYBOARD | Keyboard device |
| PDD_DEVTYPE_LIGHTGUN | Gun device |
| PDD_DEVTYPE_VIBRATION | Vibration device |

Values are expressed in bits and one peripheral can possess more than one type. For example, Visual Memory returns three types: memory card, LCD, and timer.

In the member area code that displays the area code of the peripheral, the following values are defined.

| Definition | Description |
| --- | --- |
| PDD_DEVAREA_USA | North America |
| PDD_DEVAREA_JAPAN | Japan |
| PDD_DEVAREA_ASIA | Asia |
| PDD_DEVAREA_EUROPE | Europe |

Values are expressed in bits and there are peripherals that support more than one area.

Expansion socket directions are defined with the following values.

In the member connector_dir[2] which shows the direction of the expansion socket of the peripheral, the following values are defined.

| Definition | Description |
| --- | --- |
| PDD_CONDIR_TOPSIDE | Top |
| PDD_CONDIR_BOTTOMSIDE | Bottom |
| PDD_CONDIR_LEFTSIDE | Left |
| PDD_CONDIR_RIGHTSIDE | Right |

These values express the direction of the expansion socket when there is a peripheral in the root device (peripheral directly connected to the port).

In the member product_name[32] that shows the product name of a peripheral, the character string of the registered name is returned, according to the peripheral.

In the member license[64] that shows the license character string, the name of the product's company is returned.

The unit of the members stdby_pow which expresses the stand-by current and max_pow which expresses the maximum current is 0.1 mA (milliamperes). The maximum current for all control ports is 1000mA.

# Using Each Device

This chapter explains how to use peripherals for each device, with the exception of storage devices. The method for using storage devices will be explained in the following chapter.

## 3-1	Control Devices

Getting the States of the Controller Buttons

To get button information, use the function pdGetPeripheral(). You can easily get controller input just by referring to the members of the PDS_PERIPHERAL structure.

PDS_PERIPHERAL  Structure

typedef struct {

Uint32 id;

Uint32 support;

Uint32 on;

Uint32 off;

Uint32 press;

Uint32 release;

Uint16 r;

Uint16 l;

Sint16 x1;

Sint16 y1;

Sint16 x2;

Sint16 y2;

Sint8* name;

void* extend;

Uint32 old;

PDS_PERIPHERALINFO* info;

} PDS_PERIPHERAL;

| Member | Description |
| --- | --- |
| id | Device ID |
| support | Which button / lever are supported |
| on | Status of digital button |
| off | Status of digital button (reverse) |
| press | Digital button down edge |
| release | Digital button up edge |
| r | Value of analog axis R (0 to 255) |
| l | Value of analog axis L (0 to 255) |
| x1 | Value of analog axis X1 (-128 to 127) |
| y1 | Value of analog axis Y1 (-128 to 127) |
| x2 | Value of analog axis X2 (-128 to 127) |
| y2 | Value of analog axis Y2 (-128 to 127) |
| name | Name of device |
| extend | Extended data address (unused) |
| old | Reserved |
| info | Pointer to peripheral information structure |

- Getting Buttons that Support the Controller

For devices related to the controller, there are various button and lever types. Buttons and levers differ on each device, such as 2 directional buttons or 2 channels of analog. To check the button and levers, refer to the PDS_PERIPHERAL member support.

Bit assign is fixed as shown on the following table. 1 is stored when there is a button or lever, 0 is stored when there is none.

Devices related to the controller must have directional button A, start button, A button, and B button, so it is possible to maintain compatibility by building applications using only these buttons.

| Button / Lever bit position | Bit assign constants |
| --- | --- |
| Direction key A up | PDD_DEV_SUPPORT_KU |
| Direction key A down | PDD_DEV_SUPPORT_KD |
| Direction key A left | PDD_DEV_SUPPORT_KL |
| Direction key A right | PDD_DEV_SUPPORT_KR |
| Direction key B up | PDD_DEV_SUPPORT_KUB |
| Direction key B down | PDD_DEV_SUPPORT_KDB |
| Direction key B left | PDD_DEV_SUPPORT_KLB |
| Direction key B right | PDD_DEV_SUPPORT_KRB |
| Start button | PDD_DEV_SUPPORT_ST |
| A button | PDD_DEV_SUPPORT_TA |
| B button | PDD_DEV_SUPPORT_TB |
| C button | PDD_DEV_SUPPORT_TC |
| X button | PDD_DEV_SUPPORT_TX |
| Y button | PDD_DEV_SUPPORT_TY |
| Z button | PDD_DEV_SUPPORT_TZ |
| D button | PDD_DEV_SUPPORT_TD |
| Analog axis R | PDD_DEV_SUPPORT_AR |
| Analog axis L | PDD_DEV_SUPPORT_AL |
| Analog axis X1 | PDD_DEV_SUPPORT_AX1 |
| Analog axis Y1 | PDD_DEV_SUPPORT_AY1 |
| Analog axis X2 | PDD_DEV_SUPPORT_AX2 |
| Analog axis Y2 | PDD_DEV_SUPPORT_AY2 |

- Refer to button and lever support information for an application and handle data appropriately.

- States of Digital Buttons

Digital button information is stored in the four members on, off, press and release, and is used properly according to use. Buttons are allocated in bits of each member and when a button is pressed, the corresponding bit is 1, when not pressed, 0 (positive logic). Also, when the initialization function pdInitPeripheral() is called, the logic state can be set to negative logic.

| Digital button | Bit assign constants |
| --- | --- |
| Direction key A up | PDD_DGT_KU |
| Direction key A down | PDD_DGT_KD |
| Direction key A left | PDD_DGT_KL |
| Direction key A right | PDD_DGT_KR |
| A button | PDD_DGT_TA |
| B button | PDD_DGT_TB |
| C button | PDD_DGT_TC |
| D button | PDD_DGT_TD |
| X button | PDD_DGT_TX |
| Y button | PDD_DGT_TY |
| Z button | PDD_DGT_TZ |
| L button | PDD_DGT_TL |
| R button | PDD_DGT_TR |
| Start button | PDD_DGT_ST |
| Direction key B up | PDD_DGT_KUB |
| Direction key B down | PDD_DGT_KDB |
| Direction key B left | PDD_DGT_KLB |
| Direction key B right | PDD_DGT_KRB |

| Member | Description |
| --- | --- |
| Uint32 on | When a button is pressed (button down), the corresponding bit becomes 1. When the button is not pressed, the bit is 0. |
| Uint32 off | Inverse of the on member bit. In other words, when the button is not pressed (button up), the corresponding bit becomes 1. |
| Uint32 press | When the state of the button changes from not being pressed to being pressed (button down edge), the corresponding bit becomes 1. Otherwise, the bit is 0. |
| Uint32 release | When the state of the button changes from being pressed to not being pressed (button up edge), the corresponding bit becomes 1. Otherwise, the bit is 0. |

* When negative logic is set, information is stored as the inverse for all bits of button, on, off, press, and release.

Digital LR button information is created as simulated data by the software from L/R trigger information. Note that when a device doesn't have L/R triggers, the bit doesn't change.

(There are no devices that physically have digital LR buttons.)

- States of Analog Buttons

Analog axis information is stored in the 6 members r, l, x1, y1, x2, and y2. For devices that are not connected or don't have buttons or levers, information is stored as the same data as the center position.

| Member | Sign | Range | Center position | Lever / Button |
| --- | --- | --- | --- | --- |
| Uint16 r | Unsigned | 0 to 255 | 0 | R trigger |
| Uint16 l | Unsigned | 0 to 255 | 0 | L trigger |
| Sint16 x1 | Signed | -128 thru 0 thru 127 | 0 | Analog direction key X1 |
| Sint16 y1 | Signed | -128 thru 0 thru 127 | 0 | Analog direction key Y1 |
| Sint16 x2 | Signed | -128 thru 0 thru 127 | 0 | Analog direction key X2 |
| Sint16 y2 | Signed | -128 thru 0 thru 127 | 0 | Analog direction key Y2 |

- Errors when Getting Data

An error may generate when communicating with a device. If an error occurs, the structure is set with the values shown in the table below because data cannot be normally retrieved from buttons, etc.

| Member | Setting |
| --- | --- |
| id,name,extend | Former value |
| Digital button | All the buttons are released |
| Analog axis | Center position |

- You can check if an error has been generated or not with the function pdGetPeripheralError().

| Error code | Content |
| --- | --- |
| PDD_ERR_OK | No error |
| PDD_ERR_RETRY | Unable to get button information normally |
| PDD_ERR_GENERIC | Undefined error |

If an error has been generated, there are no steps for recovery in the library. Handle the error using the appropriate process in the application.

- List of Functions

| Function | Description |
| --- | --- |
| PdGetPeripheral | Gets status of controller button |
| PdGetPeripheralError | Gets error of controller |

## 3-2	LCD Device

This function group is provided to control monochrome LCD of Visual Memory. Currently, Visual Memory is the only peripheral with LCD device

- Visual Memory LCD Specifications

| Item | Contents |
| --- | --- |
| Resolution | 48 (H) x 32 (V) pixels |
| Number of gradations | Two gradations (monochrome) |
| LCD base color | White |
| Contrast adjustment | None |

- Bit Assign of Pixel Data

In order to make the pixel data easier to handle through the software, 8 bits (one byte) are used to represent one pixel. The lower four bits are used to express the gradation, but this library only refers to bit 3.

When storing pixel data, set bit 3 to "0" (0x00, 0x07, etc.) for pixels that are to be displayed in white (the base color of the LCD), and set bit 3 to "1" for pixels that are to be displayed in black (0x0f, 0x08, etc.).

bit

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Application dependent |  |  |  | Gradation data (0 to 15) |  |  |  |

- Pixel Data Format

The entire LCD screen is data of 1536 bytes (48 x 32). The pixel data is stored consecutively, from left to right, top to bottom. The memory image is shown in the diagram below.

| 0x8C100000 | ( 0, 0 ) | ( 1, 0 ) | ( 2, 0 ) | ... | ( 47, 0 ) |
| --- | --- | --- | --- | --- | --- |
| 0x8C100030 | ( 0, 1 ) | ( 1, 1 ) | ( 2, 1 ) | ... | ( 47, 1 ) |
| : |  |  |  |  | : |
| 0x8C1005D0 | ( 0, 31 ) | ( 1, 31 ) | ( 2, 31 ) | ... | ( 47, 31 ) |

Pixel data can be described in software in the following manner (The addresses that are indicated are examples) :

const Uint8 cgdata[48 * 32] = {

0x00, 0x00, 0x0f, 0x0f, ...

:

};

- LCD Orientation

Depending on the orientation of the controller's expansion socket, the LCD screen may be upside down or sideways to the user. For example, when Visual Memory is connected to a standard controller, the LCD screen appears upside down.

| Orientation | Definition |  |
| --- | --- | --- |
| Normal | PDD_LCD_DIRECTION_NORMAL | ↑ |
| Upside-down | PDD_LCD_DIRECTION_FLIP | ↓ |
| Facing left | PDD_LCD_DIRECTION_LEFT | ← |
| Facing right | PDD_LCD_DIRECTION_RIGHT | → |

- List of Functions

| Function | Description |
| --- | --- |
| PdVmsLcdWrite | Displays data on the Visual Memory LCD. |
| PdLcdGetDirection | Gets the orientation of the LCD. |
| PdVmsLcdIsReady | Checks whether the Visual Memory is connected or not. |

## 3-3	Timer Device

This function group provides the ability to handle a timer device connected to the controller. Currently, visual memory is the only peripheral with timer device. The following is a structure to store the set time of the timer device.

- Definition

typedef struct {

Uint16 year;

Uint8 month;

Uint8 day;

Uint8 hour;

Uint8 minute;

Uint8 second;

Uint8 dayofweek;

} PDS_TIME;

| Member | Description |
| --- | --- |
| year | Year |
| month | Month |
| day | Day |
| hour | Time |
| minute | Minute |
| second | Second |
| dayofweek | Day of the week (Sunday=0, Monday=1,…, Saturday=6) |

Be sure to set the “dayofweek” member to a valid value when setting the time. When getting the time, if its value is not looked up from here but calculated from the year-month-day value, then this will enable support of future timer devices.

- List of Function

| Function | Description |
| --- | --- |
| PdTmrSetTime | Sets the time |
| PdTmrGetTime | Gets the time information |
| pdTmrAlarm | Controls the alarm directly |
| pdTmrIsReady | Checks whether there is a timer device |

## 3-4	Sound Input Device

Sound Input Device Specifications

Data input from the sound input device being used by Dreamcast is processed by the dedicated sound sampling library. In this library, sampling of data that has been input is carried out. Sound recognition must be processed in the actual application.

Wave sampling library is made from two mudules.

- WSB : Wave Sampling Buffer Module

Set sampling data to a ring buffer from SIP manufacture.

- WSS : Wave Sampling Stream Module

Get sampling data from a ring buffer to user buffer.

"Setting" is to transfer data from work area in controll port driver into a ring buffer in this library. "Getting" is to transfer data from a ring buffer in a ring buffer in this library into work area that some sound recognition engine or more uses. And, "Writing" is to set data into a ring buffer. "Reading" is to get data from a ring buffer.

- This library uses a ring buffer to operate sampling data. The buffer has two features.

- Data are overwritten except for confirmation the data read, because sampling data are set in a ring buffer continually.

- It is possible that you can get data from a ring buffer at any reading position asynchronously, because of making full use of data in a ring buffer from SIP manufacture.

- The following shows a model of getting sampling sound data from SIP manufacture.

SIP

Controller

Port

Driver

Setting

Module

(WSB)

Ring

Buffer

Getting

Module

(WSS)

Writing

Reading

Voice recognition engine  etc.

Wave Sampling Library

Setting                       Getting

- How to get sampling data

Setting into a ring buffer is automatically operated by calling a server function (wsBufExecServer) every V-Sync interrupt. Setting from a ring buffer is controlled by wsBufStart() function to start and wsBufStop() function to stop.

Getting data from a ring buffer is operated by calling wsStmCopyPcm() function when you need.

SIP

Controller

Port

Driver

Ring

Buffer

WSS#2

wsStmCopyPcm

WSB

wsBufStart

wsBufStop

Voice recognition

etc.

Wave Sampling Library

Sound codec

etc.

WSS#1

wsStmCopyPcm

- List of Functions

- Initialize and finish library

| Function | Description |
| --- | --- |
| wsInit | Initialize of library |
| wsFinish | Finish of library |

- Setting sampling data into a ring buffer (WSB)

| Function | Description |
| --- | --- |
| wsBufCreate | Creats a WSBUF handle |
| wsBufDestroy | Destroys a WSBUF handle |
| wsBufStart | Starts sampling into a ring buffer |
| wsBufStop | Stops sampling into a ring buffer |
| wsBufExecServer | Server function |
| wsBufGetStat | Gets status of a WSB module |
| wsBufGetErr | Gets an error code in WSB module |
| wsBugGetNumSmpl | Gets a number of samples in a ring buffer |
| wsBufGetWrPos | Gets a writing position in a ring buffer |
| wsBufGetSfreq | Gets sampling frequency information |
| wsBufGetBitPerSmpl | Gets bits per sample information |
| wsBufGetAmpGain | Gets AMP gain of SIP |
| wsBufSetAmpGain | Sets AMP gain of SIP |
| wsBufGetSBFOV | Gets overflow bit about SIP buffer |

- Getting sampling data from a ring buffer (WSS)

| Function | Description |
| --- | --- |
| wsStmCreate | Creats a WSSTM handle |
| wsStmDestroy | Destroys a WSSTM handle |
| wsStmGetWsbuf | Gets a WSBUF handle to the WSSTM handle |
| wsStmGetNumSmpl | Gets a number of sample in a ring buffer |
| wsStmSeekRdPos | Changes a seek position in a ring buffer |
| wsStmGetRdPos | Gets a seek position information |
| wsStmAddRdPos | Advances a seek position |
| wsStmIsOverFlow | Gets overflow information |
| wsStmClearOverflow | Clears overflow flag in a handle |
| wsStmCopyPcm | Gets sampling data from a ring buffer |

List of Structures

| Structure | Description |
| --- | --- |
| WSS_POS | Structure for an index of sample |
| WSS_BUF_PRM | Structure for setting a MSB handle |

- Sample program

/* application main function */

void main(void)

{

WSBUF          wsbuf;                   /* a handle of WSB control      */

WSSTM          wsstm;                   /* a handle of WSS control      */

WSS_BUF_PRM   prm;                     /* parameters for WSB handle    */

Sint32         pno = 1;                 /* a number of a device         */

Sint16         data[DATA_SIZE];       /* a buffer for getting data    */

Sint32         nsmpl, cpsmpl;

/* setting parameters for WSB */

prm.sfreq   = WSD_BUF_SFREQ_11KHZ;  /* sampling frequency.F11.025kHz */

prm.bps     = WSD_BUF_SFREQ_16BIT;     /* bits per sample�F16bit       */

prm.gain    = (WSD_BUF_PRM_AMPMAX-WSD_BUF_PRM_AMPMIX)>>2;

/* medium AMP GAIN　*/

prm.blksize = 11025*5;                    /* 5 seconds buffer size        */

prm.nblk    = 1;                             /* one block                      */

prm.bufptr  = (Sint16 *)0xC100000;       /* address of a ring buffer      */

wsbuf = wsBufCreate(pno, &prm);           /* creation of a WSB handle     */

wsstm = wsStmCreate(wsbuf);                /* creation of a WSS handle     */

wsBufStart(wsbuf);                          /* start sampling               */

while(1) {

nsmpl = wsStmGetNumSmpl(wsstm);

if  (nsmpl > 0) {

cpsmpl = wsStmCopyPcm(wsstm, &data, 256);/* get 256 samples      */

wsStmAddRdPos(wsstm, 128);/* advance read pointer 128 samples  */

}

/* process the data for a user*/

}

wsBufStop(wsbuf);                       /* stop sampling               */

wsStmDestroy(wsstm);                      /* destroy a WSS handle         */

wsBufDestroy(wsbuf);                      /* destroy a WSB handle         */

}

## 3-5	Keyboard device

Keyboard Specifications

The keyboard hardware that is used in Dreamcast differs from the system that returns so-called MAKE, BREAK code. Instead, it returns multiple acquisitions of key codes that are pressed.

When inputting keys from the application, data is gotten once per 1 frame (1INT) and processing key up, key down, and key repeat based on the differential of the previous time is required.

Definition

typedef struct {

Uint8 ctrl;

Uint8 led;

Uint8 key[6];

} PDS_KEYBOARD;

| Member | Description |
| --- | --- |
| Ctrl | Status of special keys (shift, alt, ctrl, etc.) |
| Led | On / Off status of LED |
| Key | Key data |

- Control Key

In the member ctrl that shows the state of special keys, information of whether a control key is currently being pressed is stored. If the key is being pressed, the corresponding bit is 1; if it not being pressed, the bit is 0.

Corresponding bits are defined as follow.

| Bit assign constants | Key pressed |
| --- | --- |
| PDD_KEY_CTRL_RGUI | Right GUI (S2 key) |
| PDD_KEY_CTRL_RALT | Right ALT |
| PDD_KEY_CTRL_RSHIFT | Right shift |
| PDD_KEY_CTRL_RCTRL | Right control |
| PDD_KEY_CTRL_LGUI | Left GUI (S1 key) |
| PDD_KEY_CTRL_LALT | Left ALT |
| PDD_KEY_CTRL_LSHIFT | Left shift |
| PDD_KEY_CTRL_LCTRL | Left control |

- LED Lamp State

In the member led that shows the state of the LED lamps, information of which LED is currently lit is stored. If an LED is lit, the corresponding bit for that LED is 1; if it is not lit, the corresponding bit is 0. Corresponding bits are defined as follow.

| Bit assign constants | LED |
| --- | --- |
| PDD_LED_SHIFT | Shift |
| PDD_LED_POWER | Power |
| PDD_LED_KANA | “Kana” (Japanese character) |
| PDD_LED_SCRLOCK | Scroll Lock |
| PDD_LED_CAPLOCK | Caps Lock |
| PDD_LED_NUMLOCK | Num Lock |

For keyboards without LED lamps, this member is meaningless.

- Key codes

In the member key that shows key data, the key code that is pressed is stored. Key code from the first pressed key to the sixth pressed key is stored in order from key[0] to key[5].

If six keys are not pressed, 0x00 is stored in the remaining members. Detection of more than 7 keys pressed simultaneously is not possible. When more than 7 keys are pressed, 0x01 is stored in key [0]. Also, depending on the combination of keys being pressed on the hardware configuration, if more than 3 keys are pressed simultaneously, key code also cannot be detected and 0x01 is stored in key [0].

The chart below shows the codes of keys that are returned. The horizontal axis represents upper 4 bits and the vertical axis represents lower four bits.

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  | Mも | #ぁ3あ | {「[゜ | F7 | ← |  |  |  |
| 1 |  | Nみ | $ぅ4う |  | F8 | ↓ |  |  |  |
| 2 |  | Oら | %ぇ5え | }」]む | F9 | ↑ |  |  |  |
| 3 |  | Pせ『 | &ぉ6お | +』；れ | F10 |  |  |  |  |
| 4 | Aち | Qた | 'ゃ7や | *ヶ:け | F11 |  |  |  |  |
| 5 | Bこ | Rす | (ゅ8ゆ | 半角/ 全角 | F12 |  | S3 |  |  |
| 6 | Cそ | Sと | )ょ9よ | <、,ね | Print Screen |  |  |  |  |
| 7 | Dし | Tか | ~を0わ | >。.る | Scroll Lock |  |  |  |  |
| 8 | Eい | Uな | Enter | ?・/め | Pause |  |  |  | カタカナ ひらがな |
| 9 | Fは | Vひ | Esc | CapsLock 英数 | Insert |  |  |  | ¦￢ ￥－ |
| A | Gき | Wて | Back space | F1 | Home |  |  |  | 前候補 変換 |
| B | Hく | Xさ | Tab | F2 | Page Up |  |  |  | 無変換 |
| C | Iに | Yん | <SPACE> | F3 | Delete |  |  |  |  |
| D | Jま | Zつっ | =￡-ほ | F4 | End |  |  |  |  |
| E | Kの | !1ぬ | ￣々^へ | F5 | Page Down |  |  |  |  |
| F | Lり | "2ふ | `￠@゛ | F6 | → |  |  |  |  |

*Note : Please be noted keyboard device here is spec. for Japanese.

- List of Function

| Function | Description |
| --- | --- |
| PdKbdGetData | Gets keyboard data. |

- Example

Sint32 get_key(void)

{

PDS_KEYBOARD* kbd;

Sint32 i;

/* Gets keyboard data of port A */

kbd = pdKbdGetData(PDD_PORT_A0);

/* If keyboard is connected, */

/* pressed key is displayed. */

if (kbd != NULL) {

for (i = 0; i < 6; i++) {

njPrintH(NJM_LOCATION(i * 4, 10), kbd->key[i]);

}

}

return NJD_USER_CONTINUE;

}

## 3-6	Gun device

Dreamcast Gun employs the same system as Saturn's of detecting scan lines on a TV screen. To use the gun, it is necessary to set the special state called "Gun Mode" in the controller library. This special designation is necessary because the gun has an access system that differs greatly from the regular controller and keyboard, and peripherals like the storage device. There are some limitations while in gun mode.

- Limitations with Expansion Peripherals

While in gun mode, there are great limitations for accessing expansion peripherals. The following devices cannot be used in gun mode.

- Storage device (Memory card)

- LCD device

- Timer device

When using storage devices, exit from the memory card (backup) library using the function buExit(). If this library is used, mount processing will be lost and the memory card won't be recognized.

Because storage devices cannot be used in gun mode, save memory with the following methods in the application.

- Operate the gun mode normally and set the regular mode to only save and load screens.

- Initialize the backup library at the input phase on the save screen and exit when the save screen is exited.。

- Screen Flash

When the gun trigger is pulled while in gun mode, the library automatically makes a flash on the screen for 1INT internally to detect the gun coordinates. Gun coordinates are the coordinates detected at the point when the gun is pointed at the screen and the trigger is pulled.

The screen flash color is white (0x00c0c0c0) as the default. When the screen does not flash, gun coordinates have not been properly detected.

- Limitions of the Gun Position when Getting Gun Coordinates

Gun coordinates of the left, right and lower edges on the screen may not be able to be gotten with Dreamcast Gun. This occurs for the left and right edges because the loopback point cannot be pinpointed by offset and for the lower edge because the lowest coordinate always becomes data for the screen angle of the lens. Also, unless the tv monitor has the same display in all directions, gun coordinates must be at least 16 pixels for all the edges on a 640x480 screen and 8 pixels for a 320x240 screen.

- Gun Priority Sequencing

The gun can only get data of one port in 1INT on hardware specs. In the library, data is gotten with Port A given the highest priority. Therefore, when more than one gun trigger is pulled simultaneously, triggers from low port numbers are given priority and distinguished first.

- List of Function

| Function | Description |
| --- | --- |
| pdGunEnter | Starts gun mode |
| pdGunLeave | Ends gun mode |
| pdGunGetLatchedPort | Gets controller port which has detected coordinates |
| pdGunGetPosition | Gets coordinates of gun position |
| pdGunSetFlashColor | Sets both-sided flash color |
| pdGunSetCallback | Registers callback function for trigger information setting |
| pdGunSetTrigger | Sets trigger information |

## 3-7	Vibration Device

In the vibration device library, only general functions and simple functions supported by "Puru Puru Pack" are accommodated.

- Vibration device's connection direction

Depending on the direction of the extension connector on the controller, the user may see a different vibration direction. For example, if the vibration pack is connected to the standard controller, the vibration direction is reversed. To check the connected direction of the vibration device, use the pdVibGetDirection() function.

Sint32 dir;

dir = pdVibGetDirection(PDD_PORT_A2)

switch (dir) {

case PDD_VIB_DIRECTION_NORMAL:	/* Normal */

break;

case PDD_VIB_DIRECTION_FLIP:		/* Flipped */

break;

case PDD_VIB_DIRECTION_LEFT:		/* Left */

break;

case PDD_VIB_DIRECTION_RIGHT:	/* Right */

break;

}

- Vibration unit

In this library, hardware which exists in a vibration device and serves as a vibration source is called a "vibration unit". Multiple vibration units can exist in a vibration device. This library supports from 1 to 15 vibration units. A vibration unit has fixed parameters as well as other parameters, as shown in the following table.

| Parameter of vibration unit | Description |
| --- | --- |
| Vibration position | This parameter indicates the position of a vibration unit within the vibration device. There are four types: front, back, left, and right. |
| Vibration axis | This parameter indicates the vibration axis (direction) in a vibration unit. The possible values are: no axis, X axis, Y axis, and Z axis. |
| Vibration power | Some vibration units use only a fixed power level, but other may allow up to 8 levels of vibration power. |
| Continuous vibration | Some vibration units support the use of continuous vibration until the next vibration setting. |
| Vibration direction | In some vibration units, the vibration direction can be specified: positive or negative. |
| Optional vibration waveform | In some vibration units, optional vibration waveform can be created and set to vibrate accordingly. |
| Vibration number | Vibration frequency |

- Number of vibration units and number of them that can be set at the same time

To get the number of vibration units that can be set at the same time for vibration, use the pdVibGetInfo() function

PDS_VIBINFO info;

if (pdVibGetInfo(PDD_PORT_A2, &info)) == PDD_VIBERR_OK) {

- /* Succeed in getting information */

printf("Number of vibration units builit in: %d.\n", info.units);

printf("Number of vibration units that can be set for vibration

at the same time:%d.\n",info.se_units);

- } else {

/* Vibration device is not connected. */

}

The number of vibration units that can be set for vibration at the same time refers to the number of units that can be set to vibrate when the command is issued once (the pdVibStart() function is called once).

- Getting the vibration unit information

Use the pdVibEnumerateUnit() function to get the information of all the vibration units in a vibration device. To get this information, communication with the device is required, which means for each unit 1INT of time is required.

The following shows the member taken by PDS_VIBUNITINFO structure.

typedef struct {

Uint8 position;		/* Vibration peripheral unit position */

Uint8 axis;		/* Vibration axis */

Uint8 pow_enable;	/* Intensity variable flag */

Uint8 cont_enable;	/* Continuous vibration enable flag */

Uint8 dir_enable;	/* Direction specification enable flag */

Uint8 wave_enable;	/* Optional waveform setting enable flag */

Uint8 min_freq;		/* Minimum vibration frequency */

Uint8 max_freq;		/* Maximum vibration frequency */

} PDS_VIBUNITINFO;

- Vibration peripheral unit position indicated by member “position” returns the following values.

| Value | Description |
| --- | --- |
| PDD_VIB_POS_FRONT | Positioned in front part of the device |
| PDD_VIB_POS_BACK | Positioned in back part of the device |
| PDD_VIB_POS_LEFT | Positioned in left side of the device |
| PDD_VIB_POS_RIGHT | Positioned in right side of the device |

- Vibration axis described by member “axis” returns the following values.

| Value | Description |
| --- | --- |
| PDD_VIB_AXIS_NONE | No axis direction (rotate movement etc.) |
| PDD_VIB_AXIS_X | Direction of X axis (horizontal direction against player) |
| PDD_VIB_AXIS_Y | Direction of Y axis (vertical direction against player) |
| PDD_VIB_AXIS_Z | Direction of Z axis (plus and minus direction against player) |

The member pow_enable which shows the vibration intensity variable flag is 0 when the flag is fixed, and 1 for an 8 phase variable flag.

The member count_enable which shows the continuous vibration flag is 0 when continuous vibration is impossible, and 1 when continuous vibration is possible.

The member dir_enable which shows the direction specification flag is 0 when the vibration direction is fixed, and 1 when vibration direction specification is possible (forward direction, reverse direction specification possible with +, -.)

The member wave_enable which shows the optional vibration waveform flag is 0 when optional waveform specification is impossible, and 1 when it is possible.

The member min_freq which shows the minimum vibration frequency shows a positive number for the frequency when vibrating, 0 when specifying the maximum frequency is impossible.

The member max_freq which shows the maximum vibration frequency shows a positive number for the frequency when vibrating, 0 when specifying the maximum frequency is impossible.

- Example

Uint32 flag;

Sint32 user_enum_unit_func(Uint32 unit, Sint32 stat,

const PDS_VIBUNITINFO* info, Uint32 param)

{

switch (stat) {

case PDD_VIBERR_NO_VIBRATOR:

printf("No vibration device\n");

flag = TRUE;		/* No callback after this line */

break;

case PDD_VIBERR_OK:

printf("Information of unit %d\n", unit);

printf("Position : %d\n", info->position);

:

:

if (unit == param) flag = TRUE;

/* Callback of all units completed */

break;

}

return PDD_VIBRET_OK;

}

Sint32 ret;

flag = FALSE;

if (pdVibGetInfo(PDD_PORT_A2, &info) != PDD_VIBERR_OK) return NG;

if (pdVibEnumerateUnit(PDD_PORT_A2, user_enum_unit_func, info.units) !=

PDD_VIBERR_OK) return NG;

while (!flag);	     /* Wait for callback of all units */

In this way, the user_enum_unit_func() function is called the number of times equal to the number of the vibration units, to get information of each of the vibration units. However, be aware that the pdVibEnumerateUnit() function returns immediately, while the callback triggers once for every 1INT.

- List of Functions

| Function | Description |
| --- | --- |
| pdVibIsReady | Checks to see whether the vibration device is connected. |
| pdVibGetInfo | Gets the vibration device information. |
| pdVibEnumerateUnit | Gets information of all vibration units in a vibration device. |
| pdVibGetDirection | Gets the direction of the vibration device that is connected. |
| PD_VIBENUMUNITCALLBACK | Callback function type |

- Starting vibration

To start the vibration, use the pdVibStart() function. However, the parameters to start the vibration must be set to the structure. Refer to the vibration unit information obtained by the pdVibEnumerateUnit() function, and then specify the appropriate parameter.

Sint32 ret;

PDS_VIBPARAM param;

param.unit = 1;	   /* Unit 1 */

param.flag = PDD_VIB_FLAG_CONTINUOUS;	   /* Continuous vibration */

param.power = 7;	   /* Power set to postive direction,

7 (maximum)*/

param.freq = 20;	   /* At 10.5Hz */

param.inc = 0;	   /* No convergence and exhalation */

ret = pdVibStart(PDD_PORT_A2, &param, 1);	/* Start vibration */

if (ret != PDD_VIBERR_OK) {

/* Fail to get it to vibrate */

}

The above example shows how to enable vibration of vibration unit 1 only.  To enable vibration of multiple vibration units, declare an array of structures as the parameters, as shown in the following example.

Sint32 ret;

PDS_VIBPARAM param[2];

param[0].unit = 1;	   /* Unit 1 */

param[0].flag = PDD_VIB_FLAG_CONTINUOUS;   /* Continuous vibration */

param[0].power = 7;	   /* Power set to positive direction,

7 (maximum) */

param[0].freq = 20;	   /* At 10.5Hz */

param[0].inc = 0;	   /* No convergence and exhalation

(0Hz) */

param[1].unit = 2;	   /* Unit 2 */

param[1].flag = PDD_VIB_FLAG_EXHALATION;   /* Single exhalation */

param[1].power = -7;	    /* Power set to negative direction,

7 (maximum) */

param[1].freq = 15;	   /* At 8Hz */

param[1].inc = 1;	   /* Exha1ation cycle 1 */

param[1].power = -7;

ret = pdVibStart(PDD_PORT_A2, param, 2);	/* Start vibration */

if (ret != PDD_VIBERR_OK) {

- /* Fail to get it to vibrate */

}

The third parameter in the pdVibStart() function indicates the number of vibration parameters, or in other words the number of vibration units for vibration set up at the same time. This cannot exceed the " Number of vibration units that can be set at the same time for vibration" in the vibration device information.

- Parameters required for vibration, and setup example

This section describes members of the PDS_VIBPARAM structure which is passed on to pdVibStart() function. Some examples are also included. When setting up parameters to each vibration unit, be sure to refer to the vibration unit information obtained by the pdVibEnumerateUnit() function, and then specify the appropriate parameter. If a parameter is illegal, its vibration unit or all vibration units in the vibration device may not vibrate. Also, the device hardware may cause unexpected operation. The library does not perform any check on the parameter.

typedef struct {

Uint8 unit;			/* Unit number */

Uint8 flag;			/* Vibration flag */

Sint8 power;			/* Vibration power */

Uint8 freq;			/* Vibration frequency */

Uint8 inc;			/* Vibration slope cycle */

Uint8 reserved[3];		/* Reserved */

} PDS_VIBPARAM;

Vibration unit number described by member “unit” specifies which vibration unit among them to pass the vibration parameters.

Vibration flag described by member “flag” takes the following values.

| Definition | Description |
| --- | --- |
| PDD_VIB_FLAG_CONTINUOUS | This value specifies to use the continuous vibration. The vibration continues until the automatic vibration stop time has come. If the flag is not specified, vibration will be single vibration (one-shot). This cannot be used when “count_enable” is set to 0 in PDS_VIBUNITINFO structure. |
| PDD_VIB_FLAG_EXHALATION | This value specifies to gradually increase the vibration (exhalation vibration). When the continuous vibration reaches its maximum power, the exhalation vibration will start at the strength specified by the "power". This cannot be used when “pow_enable” is set to 0 in PDS_VIBUNITINFO structure. |
| PDD_VIB_FLAG_CONVERGENCE | This value specifies to gradually decrease the vibration (convergence vibration). When the continuous vibration reaches its minimum power, the convergence vibration will start at the strength specified by the "power". This cannot be used when “pow_enable” is set to 0 in PDS_VIBUNITINFO structure. |

* PDD_VIB_FLAG_CONVERGENCE and PDD_VIB_FLAG_EXHALATION cannot be used together.

The range of vibration power, described by member “power”, is –7 thru 0 thru +7. A positive value indicates a vibration in the positive direction, and a negative value indicates a vibration in the reverse direction. The larger the value, the stronger is the vibration. If “pow_enable” in PDS_VIBUNITINFO structure is 0, this member is ignored. If “dir_enable” is 0, the sign is ignored.

The member freq which shows vibration frequency specifies a range from the minimum vibration frequency to the maximum vibration frequency gotten in the PDS_VIBUNITINFO structure. The actual frequency for a specified value is the value of the specified value + 1 divided by 2. For "Puru Puru Pack", specification is 15<n<59, but the actual frequency is 8Hz-30Hz.

This member , indicated by “inc”, specifies the cycle for convergence and exhalation vibration. If the vibration is not convergence or exhalation, this member is ignored. The range to be taken is between 1 and 255.

- Stopping the vibration

- To stop the vibration, use the VibStart() function, as follows.

Sint32 ret;

PDS_VIBPARAM param;

param.unit = 2;		/* Unit number 2 */

param.flag = 0;		/* No flag */

param.power = 0;		/* Stop vibration */

param.freq = 0;		/* At 0Hz */

param.inc = 0;		/* No convergence and exhalation */

ret = pdVibStart(PDD_PORT_A2, &param, 1);	        /* Stop vibration */

if (ret != PDD_VIBERR_OK) {

- /* Fail to stop */

}

The above example shows how to stop vibration of vibration unit 2 only. To stop vibration of multiple vibration units, declare an array of structures as the parameter, just as when starting the vibration, and set their vibration power to 0.

- Setting automatic vibration stop time

- To set the automatic vibration stop time, use the pdVibSetStopTime() function.

Sint32 ret;

Uint8 units[] = {1, 3, 5};		/* Array to store desired unit number */

Uint8 times[] = {10, 20, 30};	/* Array to store value for each unit */

ret = pdVibSetStopTime(PDD_PORT_A2, units, times, 3);	/* Set */

if (ret != PDD_VIBERR_OK) {

- /* Fail to set */

}

In the example, vibration units 1, 3, and 5 have their automatic vibration stop time set to 10 (approximately 2.5 seconds), 20 (approximately 5 seconds), and 30 (approximately 7.5 seconds) respectively.

- List of Functions

| Function | Description |
| --- | --- |
| pdVibStart | Starts the vibration. |
| pdVibSetStopTime | Sets the automatic vibration stop time. |

- Exclusive functions for “Puru Puru Pack”

For ease of use of the vibration device, dedicated functions are included in "Puru Puru Pack". Basically, the general function group and the way of using it is the same, but because these functions are already separated in hardware specs, it is not necessary to get vibration unit information. Also, only one vibration unit is installed, so setting parameters is also easy.

- Specification of "Puru Puru Pack"

| Item | Description |
| --- | --- |
| Number of vibration units | 1 |
| Position of the vibration unit | Front |
| Vibration axis of the vibration unit | No |
| Continuous vibration | Allowed |
| Set vibration direction (positive, reverse) | Allowed |
| Vibration strength that can be set | -7 thru 0 thru +7 |
| Set up optional vibration waveform | not allowed |
| Vibration frequently that can be set | 0FH(15) to 3BH(59) (8Hz to 30Hz) |

To start vibration of “Puru Puru Pack”, pdVibMxStart() function needs to be called.  Value to be taken in PDS_VIBPARAM structure which is specified here are the same as the ones taken in pdVibStart() function.

- List of exclusive functions for “Puru Puru Pack”

| Function | Description |
| --- | --- |
| pdVibMxIsReady | Checks connection to "Puru Puru Pack". |
| pdVibMxStart | Starts the vibration. |
| pdVibMxStop | Stops the vibration. |
| pdVibMxSetStopTime | Sets the automatic vibration stop time. |

# Storage device (Memory card)

The storage device has more complicated elements compared to other peripherals, so how to handle it is explained in this chapter.

On Dreamcast, it is possible to connect up to 2 memory cards for each peripheral with expansion sockets in control ports A-D. Up to a total of 8 memory cards can be physically attached. During initialization, accessing optional specified storage devices is possible. Hereafter, for convenience's sake, storage devices may be referred to as "drive" in this explanation.

| Drive No. | Constant | Connection |
| --- | --- | --- |
| 0 | BUD_DRIVE_A1 | Storage device connected to expansion socket 1 on the controller port 1 |
| 1 | BUD_DRIVE_A2 | Storage device connected to expansion socket 2 on the controller port 1 |
| 2 | BUD_DRIVE_B1 | Storage device connected to expansion socket 1 on the controller port 2 |
| 3 | BUD_DRIVE_B2 | Storage device connected to expansion socket 2 on the controller port 2 |
| 4 | BUD_DRIVE_C1 | Storage device connected to expansion socket 1 on the controller port 3 |
| 5 | BUD_DRIVE_C2 | Storage device connected to expansion socket 2 on the controller port 3 |
| 6 | BUD_DRIVE_D1 | Storage device connected to expansion socket 1 on the controller port 4 |
| 7 | BUD_DRIVE_D2 | Storage device connected to expansion socket 2 on the controller port 4 |

- Capacity of Memory Card

This library supports the amount of memory card that was specified during initialization. Storage device is accessed by block units which 1 block is 512 bytes.

Currently, the following is the only peripheral with storage device.

| Capacity | Name | Maximum number of files (Maximum number of blocks) |
| --- | --- | --- |
| 128KB | VM (Visual memory) | 200 |

Visual memory is the only peripheral on sale now with memory card.

- Function Types

This library contains two types of functions, TYPE A and TYPE B. The features of each type are listed below.

| Type | Description |
| --- | --- |
| TYPE A | - This type returns immediately after ending. There is no need to use buStat() to acquire status information, and there is no call back. - Other file system functions can be called immediately after this type of function ends. - Most functions of this type just reference cached information when storage device is connected, so they can return immediately after ending. |
| TYPE B | - Although this type returns immediately, processing does not end at that point; instead, it is necessary to use buStat() to acquire status information after each frame and wait for processing to end. - As long as the status is not shown as "complete," a file system function that uses the same drive cannot be called. If such a function is called, an error is returned. - When processing ends, a callback is performed with the specified "complete" callback function. - While processing is in progress, a callback is performed using the "progress" callback function. - The value that is returned by this type of function is either BUD_ERR_OK or BUD_ERR_BUSY. - In the case of a function that gets data from addresses that are indicated by a parameter (such as buLoadFile(), etc.), it is important to note that the data that is stored in addresses is not valid until the status is "complete." - In the case of a function that writes data from addresses that are indicated by a parameter into storage device (such as buSaveFile(), etc.), the data must be guaranteed until the status is "complete." - Last error can be detected by using buGetLastError() function. |

## 4-1	Incorporation into Typical Applications

Although this library includes a large number of functions, most are intended for use only in BOOT ROM and for applications that include some file utility-type functions. In the case of a typical game application, storage device file access can be performed easily using just a few of these functions, such as load data, save data, and get status.

- Incorporating the Backup Library

- 1. Backup Library Initialization

In order to initialize the backup library, the library work area starting address, the number of drives supported, the maximum capacity, and the initialization complete callback function are needed.

#include <shinobi.h>

#define MAX_DRIVES 8

Uint32 bupWork[BUM_WORK_SIZE(BUD_CAPACITY_128KB, MAX_DRIVES) / sizeof(Uint32)];

static void init_callback(void)

{

}

void BackupInit(void)

{

buInit(BUD_CAPACITY_128KB, BUD_USE_DRIVE_ALL, bupWork, init_callback);

}

void BackupExit(void)

{

buExit();

}

In this example, the library is being initialized to support eight drives, each with up to 128K of storage device. To support only the two storage device units that are connected to controller A, use the following:

#define MAX_DRIVES 2

void BackupInit(void)

{

buInit(BUD_CAPACITY_128KB, BUD_USE_DRIVE_A1 | BUD_USE_DRIVE_A2,

bupWork, init_callback);

}

If storage device is connected to a drive that was not specified during initialization, that storage device is not recognized. However, avoid specifying any drives that are not needed by the application. Not specifying unnecessary drives reduces the amount of work and the library load.

- 2. Callback Function Registration

When file access processing is completed, it is possible to register the progress callback functions, if necessary. This is accomplished in the following manner:

Sint32 complete_callback(Sint32 drive, Sint32 op, Sint32 stat, Uint32 param)

{

return BUD_CBRET_OK;

}

Sint32 progress_callback(Sint32 drive, Sint32 op, Sint32 count, Sint32 max)

{

return BUD_CBRET_OK;

}

static void init_callback(void)

{

buSetCompleteCallback(complete_callback);

buSetProgressCallback(progress_callback);

}

As shown in the example, use buSetCompleteCallback() to register functions that you want to call when processing is completed, and use buSetProgressCallback() to register functions that you want to call once a certain stage of progress has been reached in processing. Registration should be performed within the initialization callback function described earlier. In this example, the callback function does nothing and returns BUD_CBRET_OK.

Callbacks can be used to easily detect the end of a load/save operation or the connection/disconnection of storage device. They can also be used to determine the state of progress in processing that lasts a long time.

Details on callback functions are provided in the next chapter.

- Functions Used for Incorporating

| Function | Description | Type |
| --- | --- | --- |
| buInit | Initializes the library. | A |
| buExit | Exits the library. | A |
| buSetCompleteCallback | Registers callback functions for when processing is complete. | A |
| buSetProgressCallback | Registers callback functions for when a certain stage of progress has been reached in processing. | A |
| BU_INITCALLBACK | Callback function type when initialization is completed. | - |
| BU_COMPLETECALLBACK | Callback function type when processing is completed. | - |
| BU_PROGRESSCALLBACK | Callback function type when a certain stage of progress has been reached in processing. | - |

## 4-2	File Access in Typical Applications

Although this library includes a large number of functions, most are intended for use only in BOOT ROM and for applications that include some file utility-type functions. In the case of a typical game application, storage device file access can be performed easily using just a few of these functions, such as load data, save data, and get status.

- Comfirming the Storage Device Connection

- Use the buIsReady() function to check whether storage device is connected or not.

if (buIsReady(BUD_DRIVE_A1)) {

- /* Storage device is connected */

}

- Getting Storage Device Information

- 1. Format Status

Use the buIsFormatDisk() function to check whether storage device has been formatted or not.

switch (buIsFormatDisk(BUD_DRIVE_A1)) {

case BUD_ERR_OK:

/* Storage device has been formatted */

break;

case BUD_ERR_NO_DISK:

/* Storage device is not connected */

break;

case BUD_ERR_BUSY:

/* Storage device is busy and could not be checked */

break;

}

- If storage device has not been formatted, no processing other than formatting is possible.

2. Free Space

Use the buGetDiskFree() function to check the amount of free space in storage device.

Sint32 free;

free = buGetDiskFree(BUD_DRIVE_A1, BUD_FILETYPE_NORMAL);

if (free < 0) return NG;

If free space is successfully acquired, a value of 0 or more is returned in "free". This number represents the number of free blocks in that storage device. (One block is 512 bytes.) If an error occurs, a negative value is returned.

- 3. Maximum Capacity and Other Information

The buGetDiskInfo() function can also be used to get backup information. For details on the BUS_DISKINFO structure, refer to the structure reference.

BUS_DISKINFO info;

Sint32 ret;

ret = buGetDiskInfo(BUD_DRIVE_A1, &info);

if (ret < 0) return NG;

- 4. Confirming the Existence of a File

It is possible to check whether a specified file exists before accessing that file.

Sint32 ret;

ret = buIsExistFile(BUD_DRIVE_A1, "SAVEDATA_001");

switch (ret) {

case BUD_ERR_OK:

/* File exists */

break;

case BUD_ERR_FILE_NOT_FOUND:

/* File not found */

break;

defalut:

/* Other errors */

break;

}

- 5. Storage Device Access State

Use the following in order to check whether storage device is being accessed or not (either for some type of file access, or for other processing such as formatting):

if (buStat(BUD_DRIVE_A1) == BUD_STAT_BUSY) {

/* busy */

}

If storage device is busy, other processing cannot be performed with that drive. If a function that attempts to perform other processing with that drive is called, an error is returned.

- File Save

- 1. Save Request

Use the buSaveFile() function to save a file.

Sint32 ret;

Sint32 blocks, flag;

BUS_TIME time;

extern Uint8 SaveData[];	   /* Data to be saved (512 * 10 bytes) */

blocks = 10;		   /* File size: 10 blocks */

flag = BUD_FLAG_VERIFY | BUD_FLAG_COPY(0x00);

/* Verify on; set copy flag to 00H */

time.year = 1998;		   /* Time stamp setting */

time.month = 6;		   /* 1998/6/25(Thurs.) 23:59:59 */

time.day = 25;

time.hour = 23;

time.minute = 59;

time.second = 59;

time.dayofweek = 4;

ret = buSaveFile(BUD_DRIVE_A1, "SAVEDATA", SaveData, blocks, &time, flag);

if (ret == BUD_ERR_OK) {

/* Save request was successful */

} else {

/* Save request failed (BUSY) */

}

- 2. Save End Determination and Error Check

Use the buStat() function to check whether the file save operation was completed or not.

while (buStat(BUD_DRIVE_A1) == BUD_STAT_BUSY) {

/* Busy */

}

Use the buGetLastError() function to check whether an error occurred during the file save operation or not.

if (buGetLastError(BUD_DRIVE_A1) == BUD_ERR_OK) {

/* Save was successful */

} else {

/* Save failed */

}

- 3. Time Required for Save

The time required for a file save operation in the case of Visual Memory can be roughly calculated according to the following formula:

time (number of frames) = (number of blocks + 2) * 5

In the example shown above, ten blocks are being saved, which will require about 60 frames (one second). This time is only an estimate, however, and can vary from the actual time, depending on accesses to other drives and the occurrence of errors. Furthermore, if the verify operation is performed, the save time will be somewhat longer since the verify operation entails reading the data.

- File Load

- 1. Load Request

Use the buLoadFile() function to load a file.

Sint32 ret;

Sint32 blocks;

extern Uint8 SaveData[];	   /* Load buffer */

blocks = buGetFileSize(BUD_DRIVE_A1, "SAVEDATA");	/* Get file size */

if (blocks <= 0) return NG;

ret = buLoadFile(BUD_DRIVE_A1, "SAVEDATA", SaveData, blocks);

if (ret == BUD_ERR_OK) {

/* Load request was successful */

} else {

/* Load request failed (BUSY) */

}

- 2. Load End Determination and Error Check

Use the buStat() function to check whether the file load operation was completed or not.

while (buStat(BUD_DRIVE_A1) == BUD_STAT_BUSY) {

/* Busy */

}

Use the buGetLastError() function to check whether an error occurred during the file load operation or not.

if (buGetLastError(BUD_DRIVE_A1) == BUD_ERR_OK) {

/* Load was successful */

} else {

/* Load failed */

}

3. Time Required for Load

The time required for a file load operation in the case of Visual Memory can be roughly calculated according to the following formula:

time (number of frames) = (number of blocks)

In the example shown above, ten blocks are being loaded, which will require about 10 frames (167ms). This time is only an estimate, however, and can vary from the actual time, depending on accesses to other drives and the occurrence of errors.

- File Delete

- 1. Delete Request

Use the buDeleteFile() function to delete a file.

Sint32 ret;

ret = buDeleteFile(BUD_DRIVE_A1, "SAVEDATA");

if (ret == BUD_ERR_OK) {

/* Delete request was successful */

} else {

/* Delete request failed (BUSY) */

}

- 2. Delete End Determination

Use the buStat() function to check whether the file delete operation was completed or not.

while (buStat(BUD_DRIVE_A1) == BUD_STAT_BUSY) {

- /* Busy */

}

Use the buGetLastError() function to check whether an error occurred during the file delete operation or not.

if (buGetLastError(BUD_DRIVE_A1) == BUD_ERR_OK) {

- /* Delete was successful */

} else {

- /* Delete failed */

}

- 3. Time Required for Delete

The time required for a file delete operation, in the case of Visual Memory is roughly 5 frames. This time is only an estimate, however, and can vary from the actual time, depending on accesses to other drives and the occurrence of errors.

- Callback Function

Callback functions are supported in order to handle processing easily that requires time, such as load or save operations, in the background.

- 1. Complete Callback

In a complete callback, a function that was previously registered by the user is called as soon as the load or save processing is completed. An example of a description using a complete callback function is shown below.

Sint32 complete_callback(Sint32 drive, Sint32 op, Sint32 stat, Uint32 param)

{

switch (op) {

case BUD_OP_MOUNT:

printf("Storage device %d was connected. \n", drive);

break;

case BUD_OP_UNMOUNT:

printf("Storage device %d was disconnected. \n", drive);

break;

case BUD_OP_LOADFILE:

printf("Load was completed. \n");

break;

case BUD_OP_SAVEFILE:

printf("Save was completed. \n");

break;

case BUD_OP_DELETEFILE:

printf("Delete was completed. \n");

break;

}

return BUD_CBRET_OK;

}

The drive number is passed in the "drive" parameter, and the opcode is passed in the "op" parameter. There are different opcodes for each function.

| Opcode | Processing | Corresponding function |
| --- | --- | --- |
| BUD_OP_MOUNT | Storage device connect | Connect storage device, buRemount() |
| BUD_OP_UNMOUNT | Storage device disconnect | Disconnect storage device |
| BUD_OP_DELETEFILE | File delete | buDeleteFile() |
| BUD_OP_LOADFILE | File load | buLoadFile() |
| BUD_OP_SAVEFILE | File save | buSaveFile() |

The end status, which indicates whether the processing was completed successfully or not, and whether an error occurred or not, is passed through "stat". For details on what end statuses can be returned for which processing, refer to the function reference.

User-specified parameters are passed in "param".

File system functions can be called from within a callback function. Therefore, from within the storage device connection callback function it is possible to use the buGetDiskInfo() function, etc., to acquire information; to check the existence of a file; or to use buLoadFile() to automatically load a file.

- 2. Storage Device Connection and Disconnection

A complete callback can be used to easily detect the connection/disconnection of storage device.

- Uint32 gBackupRAMReady[8];

Sint32 complete_callback(Sint32 drive, Sint32 op, Sint32 stat, Uint32 param)

{

switch (op) {

case BUD_OP_MOUNT:

if (buIsFormatDisk(drive) == BUD_ERR_OK) {

gBackupRAMReady[drive] = TRUE;

}

break;

case BUD_OP_UNMOUNT:

gBackupRAMReady[drive] = FALSE;

break;

}

return BUD_CBRET_OK;

}

- 3. Progress Callback

A progress callback can be used to easily determine the extent of progress that has been made in processing.

- Uint32 gProgress[8];

Sint32 progress_callback(Sint32 drive, Sint32 op, Sint32 count, Sint32 max)

{

gProgress[drive] = count * 100 / max;

printf("%d %% completed.\n", gProgress[drive]);

return BUD_CBRET_OK;

}

4. Notes on Callbacks

- A callback function is called using a Maple interrupt as a trigger (as an interrupt handler). Do not call other library functions or application functions that do not support calls from multiple processes (threads).

- If storage device is disconnected while a file access is in progress, a complete callback is generated first for the processing that is currently in progress. In this instance, the end status is BUD_ERR_NO_DISK. Subsequently, a backup RAM disconnection callback (BUD_OP_UNMOUNT) is generated during the same interrupt.

- When the return value of TYPE B function is BUD_ERR_BUSY, callback is not generated. Avoid calling that waits for unconditional callbacks without checking the TYPE B function return value.

- Error Processing

Unforeseen errors may occur during file access. Because this storage device utilizes removable media, it is conceivable that the storage device might be removed while a load or save operation is in progress.

- Functions for Getting Information

| Function | Description | Type |
| --- | --- | --- |
| buIsReady | Checks whether storage device is connected or not | A |
| buIsFormat | Checks whether storage device has been formatted or not | A |
| buGetDiskFree | Checks the amount of free space | A |
| buGetDiskInfo | Gets information concerning storage device | A |
| buIsExistFile | Checks whether the specified file exists or not | A |
| buGetFileSize | Checks the size of the specified file | A |
| buGetFileInfo | Gets information concerning the specified file | A |
| buStat | Checks whether processing has been completed or not | A |
| buGetLastError | Gets the last error that occurred | A |

| Structure | Description |
| --- | --- |
| BUS_DISKINFO | Stores storage device information |
| BUS_FILEINFO | Stores file information |
| BUS_TIME | Stores the time stamp |

## 4-3	File Access in Special Applications

In the case of an application such as a file management utility, it is necessary to be able to perform tasks such as getting the names of all of the files in storage device, copying files, and displaying file information. This chapter describes the functions that are provided for this type of application.

- Getting Directory Information

Use the buFindFirstFile() and buFindNextFile() to get the names of all of the files in storage device.

Sint32 ret, files, blocks, totel;

char fname[16];

BUS_FILEINFO info;

files = blocks = total = 0;

/* Get name of first file */

ret = buFindFirstFile(drive, fname);

if (ret < 0) {

if (ret == BUD_ERR_FILE_NOT_FOUND) goto end;

else goto err;

}

if (buGetFileInfo(drive, fname, &info) < 0) goto err;

blocks = info.blocks;

total += blocks;

files++;

do {

printf("%12s %10d bytes(%3d blocks)\n", files, blocks * 512, blocks);

- /* Get name of second and subsequent files */

ret = buFindNextFile(drive, fname);

if (ret < 0) {

if (ret == BUD_ERR_FILE_NOT_FOUND) goto end;

else goto err;

}

if (buGetFileInfo(drive, fname, &info) < 0) goto err;

blocks = info.blocks;

total += blocks;

files++;

} while (files < FILE_MAX);

end:

return OK;

err:

return NG;

- Copying Files

There is no file copying function provided. Use the buLoadFile() and buSaveFile() functions to copy files.

Sint32 ret;

Sint32 blocks;

Sint32 saveflag;

extern Uint8 CopyBuffer[];	/* Copy buffer */

BUS_FILEINFO info;

/* Get file information */

ret = buGetFileInfo(BUD_DRIVE_A1, "SAVEDATA_001", &info);

if (ret < 0) return NG;

blocks = info.blocks;

/* Load file */

ret = buLoadFile(BUD_DRIVE_A1, "SAVEDATA_001", CopyBuffer, blocks);

if (ret != BUD_ERR_OK) return NG;

while (buStat(BUD_DRIVE_A1) == BUD_STAT_BUSY) {

}

if (buGetLastError(BUD_DRIVE_A1) != BUD_ERR_OK) return NG;

/* Set save flag; set verify on, and copy flag to 00H */

saveflag = BUD_FLAG_VERIFY | BUD_FLAG_COPY(0x00);

/* Save file */

ret = buSaveFile(BUD_DRIVE_A2, "SAVEDATA_001", CopyBuffer, blocks,

&info.time, saveflag);

if (ret != BUD_ERR_OK) return NG;

while (buStat(BUD_DRIVE_A1) == BUD_STAT_BUSY) {

}

if (buGetLastError(BUD_DRIVE_A1) != BUD_ERR_OK) return NG;

return OK;

In the above example, a file is copied from drive A1 to B1. In an actual application, the following points must be noted:

1)	Check the connection and formatting of the destination storage device.

2)	Check the free space in the destination.

3)	Check for an existing file with the same name in the destination.

Furthermore, in order to preserve compatibility with file copying on the boot ROM and visual memory file management screens, the following points must be noted:

1)	Make sure that the date of the source file is reflected in the destination file.

2)	The copy flag of the source file is not reflected in the destination file. Set the copy flag of the destination file to 00H.

3)	Files for which the copy flag is set to FFH (copying prohibited) cannot be copied.

- Loading a Portion of a File

It is sometimes necessary to load a specific block of all files and analyze the header. In such a case, use the buLoadFileEx() function, which is capable of loading a portion of a file.

Sint32 ret;

Sint32 blocks;

Sint32 start_block;

extern Uint8 LoadBuffer[];	/* Load buffer */

/* Load a portion of a file */

start_block = 0;

blocks = 1;

ret = buLoadFileEx(BUD_DRIVE_A1, "SAVEDATA_001", LoadBuffer,

start_block, blocks);

if (ret != BUD_ERR_OK) return NG;

while (buStat(BUD_DRIVE_A1) == BUD_STAT_BUSY) {

}

if (buGetLastError(BUD_DRIVE_A1) != BUD_ERR_OK) return NG;

return OK;

- In this example, just the first block of a file is loaded.

- Formatting

Although storage device is already formatted when it is shipped, the buFormatDisk() function can be used to format storage device again if necessary.

Sint32 ret;

BUS_TIME time;

Uint8 volume[32];

Sint32 icon_no;

time.year = 1998;			/* Time stamp setting */

time.month = 6;			/* 6/25/98 (Thurs.) 23:59:59 */

time.day = 25;

time.hour = 23;

time.minute = 59;

time.second = 59;

time.dayofweek = 4;

icon_no = 0;			/* Icon number setting */

/* Set body color to blue */

memset(volume, 0, sizeof(volume));

volume[0] = 0x01;		    /* Body color information exists */

volume[1] = 0xbf;		    /* B */

volume[2] = 0x00;		    /* G */

volume[3] = 0x00;		    /* R */

volume[4] = 0xff;		    /* A */

ret = buFormatDisk(BD_DRIVE_A1, volume, icon_no, &time, TRUE);

while (buStat(BUD_DRIVE_A1) == BUD_STAT_BUSY) {

}

if (buGetLastError(BUD_DRIVE_A1) != BUD_ERR_OK) return NG;

return OK;

- 1. Icon Number

This specifies the number of the "default icon" that is displayed on the boot ROM file management screen. Since boot ROM has 124 different default icons (as of 8/3/98), specify a value from 0 to 123. If an application has its own icon, it can be displayed on the boot ROM file management screen instead of the default icon.

- 2. Body Color Information

It is possible to specify body color information for storage device that is displayed on the boot ROM file management screen. This information is specified in the volume data in the following format.

- When specifying body color information

|  | +0 | +1 | +2 | +3 | +4 | +5 | … | +31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01H | BLUE | GREEN | RED | ALPHA | 00H | … | 00H |

Store 01H at the start of the volume data. Any other value is prohibited. Set the blue, green and red components individually with a value from 0 to 255. Set a value from 128 to 255 for the alpha (transparency) value. Specifying a value of 127 or below is prohibited, since the resulting image will be difficult to see on the boot ROM display.

- When not specifying body color information

|  | +0 | +1 | +2 | +3 | +4 | +5 | … | +31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00H | 00H | 00H | 00H | 00H | 00H | … | 00H |

Specify 00H for all. When body color information is not displayed, the default color (white) is used on the boot ROM file management screen.

- File Access Functions

| Function | Description | Type |
| --- | --- | --- |
| buFindFirstFile | Gets the name of the first file. | A |
| buFindNextFile | Gets the name of the next file. | A |
| buLoadFileEx | Loads a file, specifying the loading start block and the number of blocks. | B |
| buFormatDisk | Formatting storage device. | B |

## 4-4	Execution Files

Types of storage device, such as Visual Memory, that also can serve as portable game machines can store execution files. A Visual Memory in which an execution file has been stored functions as a standalone game machine.

- Features of Execution Files

Execution files include the following features:

1)	An execution files is always stored continuously and in sequence, starting from the first block in storage device.

2)	The maximum size is 128 blocks (64K).

3)	Only one execution file can exist in one storage device unit.

- Confirming the Existence of an Execution File

Use the buFindExecFile() function to check whether an execution file exists in storage device or not.

Sint32 ret;

char fname[16];

/* Get name of execution file */

ret = buFindExecFile(BUD_DRIVE_A1, fname);

switch (ret) {
	case BUD_ERR_OK:

/* An execution file exists. The file name is stored in "fname"*/

printf("Execution file:%s\n", fname);

break;

case BUD_ERR_FILE_NOT_FOUND:

printf("Execution file not found.\n");

break;

default:

printf("Error\n");

break;

}

- Getting the Execution File Free Space

Use the buGetDiskFree() function to check whether there is any free space in storage device in which an execution file can be written.

- Sint32 free;

free = buGetDiskFree(BUD_DRIVE_A1, BUD_FILETYPE_EXECUTABLE);

if (free < 0) return NG;

If the free space is gotten successfully, a value of "0" or greater is stored in "free". This number represents the number of free blocks in which an execution file can be written in that storage device.

If an error occurs, a negative value is returned.

- Allocating Execution File Free Space

In order to write an execution file, consecutive free space starting from the first block is required. Use buGetDiskFree (drive, BUD_FILETYPE_EXECUTABLE), which was described earlier, to check the number of free blocks available, starting from the first block.

Even if the number of consecutive free blocks available is smaller than the size of the execution file, it may be possible to use defragmentation processing to allocate the required number of consecutive blocks if there is enough total free space in storage device as a whole. Perform defragmentation processing according to the following procedure:

1)	Use buGetDiskFree(drive, BUD_FILETYPE_EXECUTABLE) to get the number of consecutive blocks in which an executable file can be written.

2)	If the number of blocks found in step 1 is sufficient, the execution file can be saved immediately. There is no need to perform defragmentation processing.

3)	If there are not enough consecutive blocks, use buGetDiskFree(drive, BUD_FILETYPE_NORMAL) to get the total number of free blocks in storage device.

4)	If the total number of free blocks is insufficient, performing defragmentation processing is meaningless; therefore, the execution file cannot be saved.

5)	If there are a sufficient number of free blocks, defragmentation processing can be used to allocate the required number of consecutive blocks. Use buDefragDisk() to perform defragmentation processing.

Sint32 ret;

Uint32 DefragWork[512 / sizeof(Uint32)];

ret = buDefragDisk(BUD_DRIVE_A1, DefragWork);

if (ret != BUD_ERR_OK) return NG;

while (buStat(BUD_DRIVE_A1) == BUD_STAT_BUSY) {

}

if (buGetLastError(BUD_DRIVE_A1) != BUD_ERR_OK) return NG;

return OK;

- *Note

Defragmentation processing overwrites all of storage device. This requires considerable processing time. If the storage device is removed while defragmentation processing is in progress, in most cases the contents of the storage device will be destroyed.

Defragmentation processing needs to be done at the point of saving execution files following the above procedure, only when it is required.

- Execution File Save

- Use the buSaveExecFile() function to save an execution file.

Sint32 ret;

Sint32 blocks, flag;

BUS_TIME time;

extern Uint8 SaveData[];	    /* Data to be saved (512 * 10 bytes) */

blocks = 10;			/* File size is 10 blocks */

flag =　BUD_FLAG_VERIFY | BUD_FLAG_COPY(0xff);

/* Verify on | set copy flag to 00H */

time.year = 1998;		    /* Time stamp setting */

time.month = 6;		    /* 6/25/98 (Thurs.) 23:59:59 */

time.day = 25;

time.hour = 23;

time.minute = 59;

time.second = 59;

time.dayofweek = 4;

ret = buSaveExecFile(BUD_DRIVE_A1, "SAVEDATA", SaveData, blocks, &time, flag);

if (ret != BUD_ERR_OK) return NG;

while (buStat(BUD_DRIVE_A1) == BUD_STAT_BUSY) {

}

if (buGetLastError(BUD_DRIVE_A1) != BUD_ERR_OK) return NG;

return OK;

- Partial update of the executable file

The executable file may rewrite part of itself due to operation on portable game machines such as Visual Memory and data saving. To work together with the applications on the Dreamcast's main unit, there may be need to re-write part of the executable file from the main unit.

There are many cases where the executable file is bigger than the usual data file, and so normal use of the buSaveExecFile function to re-write the whole file will take time.

Also, when it is removed during a save operation, the executable file will mostly be not able to run.

The following function is provided for this purpose and it allows partial re-write of the executable file.

Sint32 ret;

ret = buRewriteExecFile(BUD_DRIVE_A1, "GAMEFILE_VM", buf, 15, 2);

In this example, the two contiguous blocks beginning from block 15 of the file will be directly re-written. (The leading block in the file is taken as block 0.)

- Execution File Functions

| Function | Description | Type |
| --- | --- | --- |
| buFindExecFile | Gets name of execution file. | A |
| buSaveExecFile | Saves execution file. | B |
| buDefragDisk | Allocates consecutive free blocks. | B |
| buRewriteExecFile | Directly rewrites a part of executable file. | B |

## 4-5	Backup File Format

It is not possible to store just any type of file in storage device. A file saved in storage device must have the correct format, based on the Dreamcast software development standards. This section describes the external interface and internal format for backup files. Refer to this section in conjunction with the Dreamcast software development standards.

- External Interface for Files

- 1. File Names

Twelve characters must be specified for file names. File names may consist of normal-sized alphabetic characters, certain symbols, and other character codes; the usable codes are indicated by the shaded region on the table below. Any other characters are prohibited. Thirteen bytes are needed to specify a library function, when the NULL character is included.

When creating multiple files for one application, always keep the first nine characters constant; vary only the last three characters. The boot ROM file management screen includes functions that, for example, can copy all files for one game as a group. Because the determination as to whether files belong to one game or not is made on the basis of the first nine characters of the file name, these functions will not operate properly if this file naming convention is not followed.

- Example file names : ”SAVEDATA_SYS” ”SAVEDATA_001” “SAVEDATA_002” etc.

- 2. File Attributes

The file attributes are described below:

- 1)	Copy flag

The copy flag can have a value ranging from 00H to FFH. This flag can be set as to any value when saving a file.

A file for which this value is set to FFH cannot be copied on the boot ROM file management screen or the Visual Memory standalone file management screen. In other words, this flag can be used to prohibit copying.

If a file for which this value is set between 00H and FEH is copied on the boot ROM file management screen or the Visual Memory standalone file management screen, the copy flag for the destination file is set to 00H.

2)	Executable

In the case of storage device that also serves as a portable game machine, such as Visual Memory, a single executable file can be stored in that storage device. The maximum size for an executable file is 128 blocks.

- 3. Internal Format of Files

A backup file must include data such as comments and icon information that will be displayed on the boot ROM file management screen or the Visual Memory standalone file management screen.

The format of a backup file is shown below. Note that the word data and long-word data are stored in Little Endian format.

Note that the position of the file header in the format of a normal file differs from the position in the format of an executable file.

|  | +00 | +01 |  | +02 | +03 |  | +04 | +05 |  | +06 | +07 |  | +08 | +09 | +0A | +0B |  | +0C | +0D | +0E | +0F |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| +0000 | VM comment (16 bytes) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Fixed length header area |  |
| +0010 | Boot ROM comment (32 bytes) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| +0030 | Game name (sort item) (16 bytes) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| +0040 | Number of icons |  | Icon speed |  |  | Visual type |  |  | CRC |  |  | Save data size |  |  |  |  | (Reserved) |  |  |  |  |  |  |  |
| +0050 | (Reserved) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| +0060 | Icon palette data (32 bytes) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| +0080 | Icon pixel data 1 (512 bytes) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| +0280 | Icon pixel data 2 (512 bytes) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Variable length |  |
| +0480 | Icon pixel data 3 (512 bytes) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| +nnnn | Visual data (palette/pixel) (2048/4544/8064 bytes) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| +nnnn | Application save data |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | User area |  |

|  | +00 | +01 |  | +02 | +03 |  | +04 | +05 |  | +06 | +07 |  | +08 | +09 | +0A | +0B |  | +0C | +0D | +0E | +0F |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| +0000 | Execution file reset vector, program code, etc. (512 bytes) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | User area 1 |  |
| +0200 | VM comment (16 bytes) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Fixed length header area |  |
| +0210 | Boot ROM comment (32 bytes) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| +0230 | Game name (sort item) (16 bytes) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| +0240 | Number of icons |  | Icon speed |  |  | Visual type |  |  | CRC |  |  | Save data size |  |  |  |  | (Reserved) |  |  |  |  |  |  |  |
| +0250 | (Reserved) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| +0260 | Icon palette data (32 bytes) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| +0280 | Icon pixel data 1 (512 bytes) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| +0480 | Icon pixel data 2 (512 bytes) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Variable length |  |
| +0680 | Icon pixel data 3 (512 bytes) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| +nnnn | Visual data (palette/pixel) (2048/4544/8064 bytes) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| +nnnn | Program code, data |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | User area 2 |  |

- The fixed length header (the shaded portion) is required data. All fields must be specified correctly (except for the CRC and save data size).

- Specify "00H" for all "reserved" areas.

- Within the fixed length header, only the VM comment is referenced for the Visual Memory standalone file management screen.

- If no data is to be stored in the variable length header, the application save data can be stored starting from +280H (in the case of a normal file).

- Files are always accessed in units of blocks. Arrange the application save data so that the size of the entire file is a multiple of 512 bytes, and ensure that undefined data is not stored in the final block of a file.

- Because the fixed length header is 280H bytes long, a backup file will consume a minimum of 2 blocks.

4. VM Comment

This field contains comment data that is displayed on both the boot ROM and Visual Memory standalone file management screens. Use no more than 16 bytes for the comment. The characters that are shaded in the table below can be used in comments. Use of any other characters is prohibited.

- 5. Boot ROM Comment

This field contains comment data that is displayed on the boot ROM file management screen. A maximum of 32 bytes may be used, and may consist of a mixture of double-width (2-byte) characters and normal-width (1-byte) characters. Alphanumerics, hiragana, katakana, and Level 1 and Level 2 shift-JIS codes may be used as double-width characters.

6. Game Name (Sort Item)

This field is used as the sorting key when displaying files on the boot ROM file management screen. This field may contain up to 16 bytes, taken from the table shown below. If the game name is shorter than 16 bytes, pad the field with 00H.

| Decimal | Hexadecimal | Font |  |  | Decimal | Hexadecimal | Font |  |  | Decimal | Hexadecimal | Font |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | Space |  |  | 32 | 20 | タ |  |  | 64 | 40 | ミ |
| 1 | 1 | ア |  |  | 33 | 21 | ダ |  |  | 65 | 41 | ム |
| 2 | 2 | ァ |  |  | 34 | 22 | チ |  |  | 66 | 42 | メ |
| 3 | 3 | イ |  |  | 35 | 23 | ヂ |  |  | 67 | 43 | モ |
| 4 | 4 | ィ |  |  | 36 | 24 | ツ |  |  | 68 | 44 | ヤ |
| 5 | 5 | ウ |  |  | 37 | 25 | ヅ |  |  | 69 | 45 | ャ |
| 6 | 6 | ヴ |  |  | 38 | 26 | ッ |  |  | 70 | 46 | ユ |
| 7 | 7 | ゥ |  |  | 39 | 27 | テ |  |  | 71 | 47 | ュ |
| 8 | 8 | エ |  |  | 40 | 28 | デ |  |  | 72 | 48 | ヨ |
| 9 | 9 | ェ |  |  | 41 | 29 | ト |  |  | 73 | 49 | ョ |
| 10 | 0A | オ |  |  | 42 | 2A | ド |  |  | 74 | 4A | ラ |
| 11 | 0B | ォ |  |  | 43 | 2B | ナ |  |  | 75 | 4B | リ |
| 12 | 0C | カ |  |  | 44 | 2C | ニ |  |  | 76 | 4C | ル |
| 13 | 0D | ガ |  |  | 45 | 2D | ヌ |  |  | 77 | 4D | レ |
| 14 | 0E | キ |  |  | 46 | 2E | ネ |  |  | 78 | 4E | ロ |
| 15 | 0F | ギ |  |  | 47 | 2F | ノ |  |  | 79 | 4F | ワ |
| 16 | 10 | ク |  |  | 48 | 30 | ハ |  |  | 80 | 50 | ヰ |
| 17 | 11 | グ |  |  | 49 | 31 | バ |  |  | 81 | 51 | ヱ |
| 18 | 12 | ケ |  |  | 50 | 32 | パ |  |  | 82 | 52 | ヲ |
| 19 | 13 | ゲ |  |  | 51 | 33 | ヒ |  |  | 83 | 53 | ン |
| 20 | 14 | コ |  |  | 52 | 34 | ビ |  |  | 84 | 54 | 0 |
| 21 | 15 | ゴ |  |  | 53 | 35 | ピ |  |  | 85 | 55 | 1 |
| 22 | 16 | サ |  |  | 54 | 36 | フ |  |  | 86 | 56 | 2 |
| 23 | 17 | ザ |  |  | 55 | 37 | ブ |  |  | 87 | 57 | 3 |
| 24 | 18 | シ |  |  | 56 | 38 | プ |  |  | 88 | 58 | 4 |
| 25 | 19 | ジ |  |  | 57 | 39 | ヘ |  |  | 89 | 59 | 5 |
| 26 | 1A | ス |  |  | 58 | 3A | ベ |  |  | 90 | 5A | 6 |
| 27 | 1B | ズ |  |  | 59 | 3B | ペ |  |  | 91 | 5B | 7 |
| 28 | 1C | セ |  |  | 60 | 3C | ホ |  |  | 92 | 5C | 8 |
| 29 | 1D | ゼ |  |  | 61 | 3D | ボ |  |  | 93 | 5D | 9 |
| 30 | 1E | ソ |  |  | 62 | 3E | ポ |  |  |  |  |  |
| 31 | 1F | ゾ |  |  | 63 | 3F | マ |  |  |  |  |  |

7. Number of Icons

The number of icons can be specified from 1 to 3. At least one icon must be stored.

- 8. Icon Animation Speed

When storing 2 or 3 icons, specify the pattern switching speed in this field in terms of frames per second. (in units of 1/30 second).

- 9. Visual Type

This field specifies the type of the visual comment that is stored.

| Visual type | Data that is stored |
| --- | --- |
| No visual data | 0000H |
| TYPE A | 0001H |
| TYPE B | 0002H |
| TYPE C | 0003H |

- 10. CRC Code

The CRC code is applied to the entire file, from start to finish.

However, use of the CRC code is left up to the application. The CRC code is not referenced by the boot ROM file management screen.

The reason is that, in the case of an execution file, the Visual Memory operates on its own, and may overwrite itself in order to store data.

If CRC is not to be used, store 0000H in this field. If the utility functions that are described later are used, CRC generation and checking are performed automatically.

- 11. Save Data Size/File Size

This field specifies the size of the application save data in bytes. In the case of an executable file, specify the number of bytes in the entire file. This field is not referenced by the boot ROM file management screen.

12. Icon Data (Palette Data/Pixel Data 1, 2, 3)

The icon is displayed on the boot ROM file management screen. At least one icon must be stored in save data. Create this data in the format described below:

| Item | Description |
| --- | --- |
| Size | 32 x 32 pixels (W x H) |
| Color format | 4 bits (16-color palette) |
| Palette format | ARGB4444 format x 16 colors One color is expressed by 2 bytes (16 bits) Little Endian format |
| Number of icons | Minimum of 1 required; up to 3 can be stored Palette is shared |
| Storage format | 16 colors are stored for palette data. This is followed by pixel data, expressed consecutively from the upper left corner to the lower right corner. One byte represents two pixels. The first four bits correspond to the left dot, and the last four bits correspond to the right dot. |
| Size | One icon: 544 bytes Two icons: 1056 bytes Three icons: 1566 bytes |

When two or three icons are stored, animation can be implemented by switching between the icons at a specified number of frames (in units of 1/30 second).

- 13. Visual Comments

This field contains the visual comment that is displayed on the boot ROM file management screen. Whether or not to store a visual comment in a file is discretionary. If it is to be stored, create the data in the format shown below.

There are three types of visual comments, Type A, B, and C, depending on the pixel data format. These formats are described below.

- a)	Visual comment type A

| Item | Description |
| --- | --- |
| Size | 72 x 56 pixels (W x H) |
| Color format | 16-bit direct color (ARGB4444) |
| Palette format | None |
| Number of icons | One or none |
| Storage format | Pixel data expressed consecutively from the upper left corner to the lower right corner. Two bytes (16 bits) represent one pixel. Little Endian format. |
| Size | 8064 bytes (16 blocks) |

b)	Visual comment type B

| Item | Description |
| --- | --- |
| Size | 72 x 56 pixels (W x H) |
| Color format | 8-bit color (256-color palette) |
| Palette format | ARGB4444 format x 16 colors One color is expressed by 2 bytes (16 bits) Little Endian format |
| Number of icons | One or none |
| Storage format | 256 colors are stored for palette data. This is followed by pixel data, expressed consecutively from the upper left corner to the lower right corner. One byte represents one pixel. |
| Size | 4544 bytes (9 blocks) |

c)	Visual comment type C

| Item | Description |
| --- | --- |
| Size | 72 x 56 pixels (W x H) |
| Color format | 4-bit color (16-color palette) |
| Palette format | ARGB4444 format x 16 colors One color is expressed by 2 bytes (16 bits) Little Endian format |
| Number of icons | One or none |
| Storage format | 16 colors are stored for palette data. This is followed by pixel data, expressed consecutively from the upper left corner to the lower right corner. One byte represents two pixels. The first four bits correspond to the left dot, and the last four bits correspond to the right dot. |
| Size | 2048 bytes (4 blocks) |

Because visual type A uses a large number of blocks but does not offer particularly higher quality compared to visual types B and C, visual type A should not be used.

- 14. Application Save Data

This field stores the application data. Files are always accessed in units of blocks. Arrange the application save data so that the size of the entire file is a multiple of 512 bytes, and ensure that undefined data is not stored in the final block of a file.

- Backup File Utility Functions

Utility functions are provided that create the image of a backup file in memory and analyze a file loaded into memory. Note that these functions can only be used with normal files, not with executable files.

- 1. Creating a Backup File Image

An example of how to declare the BUS_BACKUPFILEHEADER structure and set the parameters is shown below.

extern Uint8 game_name[];		/* Game name (sort item) */

extern Uint16 icon_palette[];	/* Icon palette data */

extern Uint8 icon_data[];		/* Icon pixel data */

extern Uint8 visual_data[];	/* Visual data */

extern Uint8 save_data[];		/* Application save data */

BUS_BACKUPFILEHEADER hdr;		/* Backup file header */

memset(&hdr, 0, sizeof(hdr));

strcpy(hdr.vms_comment, "VMS_COMMENT");	/* VM comment setting */

strcpy(hdr.btr_comment, " BOOT ROM Double-width comment;

- /* BOOT ROM comment setting */

memcpy(hdr.game_name,   game_name, 16);	/* Game name(sort item) setting */

hdr.icon_palette = icon_palette;		/* Icon palette data setting */

hdr.icon_data    = icon_data;		/* Icon pixel data setting */

hdr.icon_num     = 1;			/* Icon number setting */

hdr.icon_speed   = 1;			/* Icon speed setting */

hdr.visual_data  = visual_data;		/* Visual comment setting */

hdr.visual_type  = BUD_VISUALTYPE_C;	/* Visual type setting */

hdr.save_data    = save_data;		/* Application save data */

hdr.save_size    = 0x400;		  /* Application save data size */

This example sets up a structure, assuming that it stores one icon, type C visual data, and 400H of application data.

After the structure is set up, the buCalcBackupFileSize() function is used to calculate how many blocks the backup file that is to be created will consume. When the number of bytes in the whole file is not a multiple of 512, this function calculates the number of blocks up to the next 512 byte boundary.

Sint32 nblock;

nblock = buCalcBackupFileSize(hdr.icon_num, hdr.visual_type, hdr.save_size);

When creating a file image, work memory is naturally needed where the file is to be created. In the following example, the syMalloc() function is used to allocate memory:

void* buf;

buf = syMalloc(nblock * 512);

- /* Allocates (the number of blocks used x 512 bytes) */

if (buf == NULL) {

/* Memory could not be allocated */

return NG;

}

Use the buMakeBackupFileImage() function to create a file image in memory. When the number of bytes in the whole file is not a multiple of 512, this function fills the remaining area with 00H up to the next 512-byte boundary. This area has no effect on CRC calculation.

nblock = buMakeBackupFileImage(buf, &hdr);

if (nblock < 0) {

- /* There is an illegal parameter in the structure */

return NG;

}

Finally, use the buSaveFile() function to actually save the contents of memory to storage device. Be sure to release the memory that was allocated once the file save operation is completed.

2. Getting a Backup File Image

This function analyzes a file that was loaded into memory, and then creates the BUS_BACKUPFILEHEADER structure.

extern Uint8 save_data[];		/* Application save data */

Sint32 ret;

BUS_BACKUPFILEHEADER hdr;		/* Backup file header */

ret = buAnalyzeBackupFileHeader(&hdr, save_data);

switch (ret) {

case BUD_ERR_OK:

- /* Analysis was completed correctly */

break;

case BUD_ERR_BUPFILE_CRC:

/* Analysis was completed correctly, but CRC differs */

break;

case BUD_ERR_BUPFILE_ILLEGAL:

/* Incorrect file format */

break;

}

If the return value is BUD_ERR_OK, each member of the structure has been set correctly.

If the return value is BUD_ERR_BUPFILE_ILLEGAL, it is possible that the file that was loaded is not a backup file with the proper format.

If the return value is BUD_ERR_BUPFILE_CRC, the CRC value that is written in the fixed length file header does not agree with the CRC value that was actually calculated. In this case, it is possible that a portion of the file was not loaded correctly.

- List of Functions

| Function | Description | Type |
| --- | --- | --- |
| buMakeBackupFileImage | Creates backup file-format memory image | A |
| buCalcBackupFileSize | Calculates block size of backup file image | A |
| buAnalyzeBackupFileImage | Analyzes file image in memory | A |

- Volume Icon

Completely different from the backup files described until this point, another type of file (called "volume icon") can also be stored in storage device.

If a file with the format shown below and with the file name "ICONDATA_VMS" is stored, this icon is displayed instead of the default icon on the boot ROM file management screen. This can be used to give applications their own icons.

A volume icon can consist of a monochrome icon that is used for display on the LCD of a memory card unit that has an LCD, such as Visual Memory, and a color icon that is used for display on a TV screen.

The file format of a volume icon is shown below. Note that the word and long-word data that are shown are Little Endian format.

- *	Unlike other files, volume icon files are not displayed on the boot ROM file management screen, so they cannot be copied, deleted, etc.

|  | +00 | +01 | +02 | +03 |  | +04 | +05 | +06 | +07 |  | +08 | +09 | +0A | +0B | +0C | +0D | +0E | +0F |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| +0000 | VM comment (16 bytes) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| +0010 | Monochrome icon offset |  |  |  | Color icon offset |  |  |  |  | (Reserved: 00H) |  |  |  |  |  |  |  |  |  |  |  |
| +0020 | Monochrome icon data |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| +00A0 | Color icon palette data |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| +00C0 | Color icon pixel data |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- 1. VM Comment

This conforms with the VM comment field for a backup file.

- 2. Monochrome Icon Offset

Always store 00000020H in this field.

- 3. Color Icon Offset

Always store 000000A0H in this field.

- 4. Monochrome Icon Data

This data represents the monochrome icon that is displayed on the visual memory LCD on the boot ROM file management screen.

| Item | Description |
| --- | --- |
| Size | 32 x 32 pixels (W x H) |
| Storage format | Pixel data is stored consecutively as data from the upper left corner to the lower right corner. One byte represents eight pixels. The LSB is on the right, the MSB is on the left. A "1" represents a black dot, a "0" represents the background color. |
| Size | 128 bytes |

5. Color Icon (Palette Data/Pixel Data)

This data represents the color icon that is displayed on a TV screen on the boot ROM file management screen.

| Item | Description |
| --- | --- |
| Size | 32 x 32 pixels (W x H) |
| Color format | 4 bits (16-color palette) |
| Palette format | ARGB4444 format x 16 colors One color is expressed by 2 bytes (16 bits) Little Endian format |
| Storage format | 16 colors are stored for palette data. This is followed by pixel data, expressed consecutively from the upper left corner to the lower right corner. One byte represents two pixels. The first four bits correspond to the left dot, and the last four bits correspond to the right dot. |
| Size | 544 bytes |

## 4-6	Securing dynamic work buffer

With the normal method explained so far, the backup library requires considerable amount of work buffer. In particular, to handle 8 units of 1 Mbyte large capacity storage device, a work buffer with more than 400 Kbytes is required.

By using the dynamic work buffer, only the required storage device needs to be mounted at the required time, thus dramatically reducing the work buffer size.

- Method to secure the dynamic work buffer

- 1. Initialization

Assigning NULL to the work buffer address specified at the initialization function buInit() enables the dynamic work buffer mode so that the new feature can be used. Perform initialization using the following method.

Note that the maximum capacity (the second parameter) is ignored.

void BackupInit(void)

{

buInit(BUD_CAPACITY_128KB, USE_DRIVE_ALL, NULL, init_callback);

}

- 2. Connecting the storage device

When the storage device is connected, the completion callback function is called.

At this moment, the following parameters are used:

| Parameter | Description |
| --- | --- |
| drive | Drive number (0 to 7) |
| op | BUD_OP_CONNECT (new callback operation code) |
| stat | Connected storage device's capacity flag BUD_CAPACITY_128KB:capacity 128KB BUD_CAPACITY_256KB:capacity 256KB BUD_CAPACITY_512KB:capacity 512KB BUD_CAPACITY_1MB: capacity 1MB |
| param | User parameter |

The BUD_OP_CONNECT callback is a callback which occurs only in the dynamic work mode.

- 3. Securing work buffer and mounting drive

/* Secure amount of work buffer for 1 drive indicated at stat */

work = syMalloc(BUM_WORK_SIZE(stat, 1));

if (work == NULL) return NG;

In this work buffer, be sure, as responsibility of the application, to secure the necessary amount of memory from the actually usable memory region. While the drive is being accessed, if the work buffer is released or its content destroyed, the operation will not be guaranteed.

If the work buffer can be secured, mount the drive.

- Sint32 err;

err = buMountDisk(drive, work, BUM_WORK_SIZE(stat, 1));

if (err != BUD_ERR_OK) return NG;

This is how the drive is mounted, which conventionally is automatically performed when the memory card is connected. When the drive mount operation is complete, the BUD_OP_MOUNT callback occurs.

After the callback occurs, files can be accessed as usual.

- 4. Unmounting drive and releasing work buffer

When the required file access is complete and there is no need to access the drive, the drive can be unmounted. After the drive is unmounted, the work buffer can be released and the memory can be efficiently used.

- Sint32 err;

err = buUnmount();

if (err != BUD_ERR_OK) return NG;

Since buUnmount() is a function which is completed instantly and control is also returned immediately, when the function is called and control is returned, the unmount operation is complete. Therefore, work buffer can be released right after buUnmount() function is called.

In this example, because the work buffer was secured by using the syMalloc() function, it can be released by using the syFree() function. However, when BUSY is returned, it indicates unmount operation is not completed due to some kind of file access to the driver. In this case, work buffer should not be released.

syFree(work);

Also, as a matter of fact, it is completely impossible to predict which drive has the storage device connected and when the connection/disconnection is made, If syMalloc() and syFre(), or malloc() and free() are to be used to secure and release the work buffer, a thorough understanding of the algorithm in the memory management functions is required. Be aware that depending on how the functions are used, memory fragmentation may occur.

- Disconnecting memory card

When mounted storage device is removed, as usual the BUD_OP_UNMOUNT callback occurs. Here because the unmount operation is carried out automatically, releasing the work buffer while callback is taking place will not result in any problem.

switch (op) {

case BUD_OP_UNMOUNT:

if (work) syFree(work);

break;

:

}

- Functions for securing dynamic work buffer

| Function | Description | Type |
| --- | --- | --- |
| buMountDisk | Mounts storage device. | B |
| buUnmount | Unmounts storage device. | A |

# AV cable peripheral

Dreamcast can determine types of AV cable peripherals connected to the AV jack. The application must set the possible screen mode by checking this information.

In this chapter, how to distinguish AV cable peripherals and the direct relationship of them to deep screen mode are explained.

- DIP SW. of Dev.Box and cable type

The developing tool Dev.Box does not have the same type of AV jack as the Dreamcast unit. Therefore, emulate the cable type with the DIP SW. (dip switch) on the front screen panel.

| Switch1 | Switch2 | Cable type |
| --- | --- | --- |
| OFF | OFF | Composite cable |
| OFF | ON | RESERVED |
| ON | OFF | RGB cable |
| ON | ON | VGA cable |

- ROTARY SW. of Dev.Box and Broadcast system type

Broadcast systems (NTSC/PAL) are already set in Dreamcast internal system information. Broadcast systems are registered in system memory for the Dreamcast unit. For the developing tool Dev.Box, emulate the broadcast system type by operating the ROTARY SW. (rotary switch) on the front screen panel.

| Number | Description |
| --- | --- |
| 0 | Japan : NTSC |
| 4 | North America : NTSC |
| 6 | Brazil : PAL-M |
| 7 | Argentina : PAL-N |
| 9 | Europe : PAL |

- Classifying cable types

In order to determine type of currently connected cable, either syCblCableCheck() or syCblCheck() function is used. The following values are returned by syCblCableCheck() function.

| Definition | Description |
| --- | --- |
| SYE_CBL_CABLE_VGA | VGA cable |
| SYE_CBL_CABLE_VBS | VBS/S, RF output |
| SYE_CBL_CABLE_RGB | RGB cable (Multi21P) |

Broadcast system returned by syCblCheck() function is only two types, NTSC and PAL due to the information required by njInitSystem() funciton. In the case of PAL-M, “NTSC”is returned. And in the case of PAL-N, “PAL” is returned

| Definition | Description |
| --- | --- |
| SYE_CBL_NTSC | NTSC |
| SYE_CBL_VGA | VGA |
| SYE_CBL_PAL | PAL |

- The following is an example on how to use syCblCheck() function.

- Example

void Init_Cable(void)

{

switch( syCblCheck() ) {

case SYE_CBL_VGA:

/* Setting for VGA */

njInitSystem(NJD_RESOLUTION_VGA, NJD_FRAMEBUFFER_MODE_RGB565, 1);

break;

case SYE_CBL_NTSC:

- /* Setting for NTSC */

njInitSystem(NJD_RESOLUTION_320x240_NTSCNI,

- NJD_FRAMEBUFFER_MODE_RGB565, 1);

break;

case SYE_CBL_PAL:

- /* Setting for PAL */

njInitSystem(NJD_RESOLUTION_320x240_PALNI,

- NJD_FRAMEBUFFER_MODE_RGB565, 1);

break;

}

}

- Types of Broadcast System

- The following values are returned by syCblCheckBroadcast() function.

| Definition | Description |
| --- | --- |
| SYE_CBL_BROADCAST_NTSC | NTSC (Japan, USA) |
| SYE_CBL_BROADCAST_PAL | PAL (EC) |
| SYE_CBL_BORADCAST_PALM | PAL-M (Brazil) |
| SYE_CBL_BROADCAST_PALN | PAL-N (Argentina) |

- List of Functions

| Function | Description |
| --- | --- |
| SyCblCheckCable | Gets type of video cable. |
| SyCblCheckBroadcast | Gets type of broadcast system. |
| SyCblCheck | Classifies type of cable for simple monitor selection. |

# Original Information for Each Peripheral

This chapter explains the specifications for peripherals currently on the market or in the preliminary stage of marketing.

## 6-1	Dreamcast Controller (HKT-7700)

The controller included with the Dreamcast product. Comes with 2 expansion sockets.

- Peripheral information

| Member | Information |
| --- | --- |
| Type | CONTROLLER |
| is_root | 1 |
| area_code | PDD_DEVAREA_USA / -_JAPAN / -_ASIA / -_EUROPE |
| Connector_dir | 2 |
| Product_name | "Dreamcast Controller" |
| License | "Produced By or Under License From SEGA ENTERPRISES, LTD." |
| stdby_pow | 43.0mA |
| max_pow | 50.0mA |

(L Trigger)

(X Button)

(R Trigger)

(Y Button)

(B Button)

)

(Analog Direction Key)

(A Button)

(Start Button)

(Direction Button / digital)

- Definition of each button

| Button / Trigger | Definition |
| --- | --- |
| Ａ button | PDD_DGT_TA |
| Ｂ button | PDD_DGT_TB |
| Ｘ button | PDD_DGT_TX |
| Ｙ button | PDD_DGT_TY |
| Direction button | Up : PDD_DGT_KU / Down : PDD_DGT_KD Right : PDD_DGT_KR / Left : PDD_DGT_KL |
| Analog direction key | X direction : x1 member of PDS_PERIPHERAL structure Y direction : y1 member of PDS_PERIPHERAL structure |
| R trigger | R member of PDS_PERIPHERAL structure / PDD_DGT_TR |
| L trigger | L member of PDS_PERIPHERAL structure / PDD_DGT_TL |
| Start button | PDD_DGT_ST |

## 6-2	Racing Controller (HKT-7400)

A controller with a car steering wheel device. Mainly used for racing games. Comes with 1 expansion socket.

| Member | Information |
| --- | --- |
| type | CONTROLLER |
| is_root | 1 |
| area_code | PDD_DEVAREA_USA / -_JAPAN / -_ASIA / -_EUROPE |
| connector_dir | 2 |
| product_name | "Racing Controller" |
| license | "Produced By or Under License From SEGA ENTERPRISES, LTD." |
| stdby_pow | 44.0mA |
| max_pow | 55.0mA |

(- Button)

)

(+ Button)

)

(R Button)

)

(L Button)

)

(Extention Terminal)

(A Button)

(Start Button)

(B Button)

)

- Definition of each button

| Button / Trigger | Definition |
| --- | --- |
| Ａ button | PDD_DGT_TA |
| Ｂ button | PDD_DGT_TB |
| ＋ button | PDD_DGT_KU |
| － button | PDD_DGT_KD |
| Ｒ button | R member of PDS_PERIPHERAL structure / PDD_DGT_TR |
| Ｌ button | L member of PDS_PERIPHERAL structure / PDD_DGT_TL |
| Steering wheel | Right rotation : x1 member of PDS_PERIPHERAL structure (0 to 127) / PDD_DGT_KR Light rotation : x1 member of PDS_PERIPHERAL structure (-128 to 0) / PDD_DGT_KL |
| Start button | PDD_DGT_ST |

## 6-3	Arcade Stick (HKT-7300)

A controller with a joystick used in arcade games. Created to be able to withstand strong movement by a player. Comes with 1 expansion socket.

| Member | Information |
| --- | --- |
| Type | CONTROLLER |
| is_root | 1 |
| area_code | PDD_DEVAREA_USA / -_JAPAN / -_ASIA / -_EUROPE |
| Connector_dir | 2 |
| Product_name | "Arcade Stick" |
| License | "Produced By or Under License From SEGA ENTERPRISES, LTD." |
| stdby_pow | 17.0mA |
| max_pow | 30.0mA |

(Start Button)

(B Button)

(A Button)

(X Button)

(Y Button)

(Z Button)

(C Button)

(Digital Stick)

- Definition of each button

| Button / Trigger | Definition |
| --- | --- |
| Ａ button | PDD_DGT_TA |
| Ｂ button | PDD_DGT_TB |
| Ｃ button | PDD_DGT_TC |
| Ｘ button | PDD_DGT_TX |
| Ｙ button | PDD_DGT_TY |
| Ｚ button | PDD_DGT_TZ |
| Digital stick | Up : PDD_DGT_KU / Down : PDD_DGT_KD Right : PDD_DGT_KR / Left : PDD_DGT_KL |
| Start button | PDD_DGT_ST |

## 6-4	Dreamcast Keyboard (HKT-7600)

A controller which closely resembles a PC keyboard. For compactibility and ease of use, there are no number keys (Num key). Also, it does not have any expansion sockets.

| Member | Information |
| --- | --- |
| Type | KEYBOARD |
| is_root | 1 |
| area_code | PDD_DEVAREA_JAPAN |
| Connector_dir |  |
| product_name | "Keyboard" |
| License | "Produced By or Under License From SEGA ENTERPRISES, LTD." |
| stdby_pow | 35.0mA |
| max_pow | 40.0mA |

For the SEGA Library environment, dedicated functions are accommodated for using this peripheral. These functions can only determine whether the same keys are being pressed as other functions. Therefore, it is necessary to prepare additional temporary storage for typed keys and kanji exchange (FEP).

| Function / Definition | Description |
| --- | --- |
| pdKbdGetData | Get keyboard information function |
| PDS_KEYBOARD | Keyboard data structure |

## 6-5	Dreamcast Gun (HKT-7801)

An input device used mainly in gun shooting games. Comes with 1 expansion socket.

Dedicated functions are accommodated to use this peripheral in the SEGA Library environment.

| Member | Information |
| --- | --- |
| type | CONTROLLER / LIGHT-GUN |
| is_root | 1 |
| area_code | PDD_DEVAREA_USA / -_JAPAN / -_ASIA / -_EUROPE |
| connector_dir | 1 |
| product_name | "Dreamcast Gun" |
| license | "Produced By or Under License From SEGA ENTERPRISES, LTD." |
| stdby_pow | 22.0mA |
| max_pow (Start Button) | 30.0mA |

(Expansion Socket)

(Direction Button)

(B Button)

)

(Trigger / A button)

- Dreamcast Gun employs the same system of Saturn's of detecting scan lines on a TV screen.

| Button / Trigger | Definition |
| --- | --- |
| Trigger (A button) | PD_DGT_TA |
| Ｂ button | PD_DGT_TB |
| Direction button | Up : PDD_DGT_KU / Down : PDD_DGT_KD Right : PDD_DGT_KR / Left : PDD_DGT_KL |
| Start button | PD_DGT_ST |

## 6-6	Tsuri Controller (preliminary) (HKT-8700)

A controller with a reel mainly for use in fishing games. A vibration device like "Puru Puru Pack" is internally installed. The vibration device is in expansion port number 5. Does not come with any expansion sockets.

- CONTROLLER device (Port 0)

| Member | Information |
| --- | --- |
| type | CONTROLLER |
| is_root | 1 |
| area_code | PDD_DEVAREA_USA / -_JAPAN / -_ASIA / -_EUROPE |
| connector_dir |  |
| product_name | "Dreamcast Fishing Controller" |
| license | "Produced By or Under License From SEGA ENTERPRISES,LTD." |
| stdby_pow | 60mA |
| max_pow | 240mA |

VIBRATION device (Port 5)

| Member | Information |
| --- | --- |
| Type | VIBRATION |
| is_root | 0 |
| area_code | PDD_DEVAREA_USA / -_JAPAN / -_ASIA / -_EUROPE |
| Connector_dir |  |
| Product_name | "Dreamcast Fishing Controller" |
| License | "Produced By or Under License From SEGA ENTERPRISES,LTD." |
| stdby_pow | 60mA |
| max_pow | 240mA |

(Start Button)

(Reel Handle)

(X Button)

(Y Button)

(B Button)

(Analog Direction Key)

(A Button)

- Definition of each button

| Button / Trigger | Definition |
| --- | --- |
| Ａ button | PD_DGT_TA |
| Ｂ button | PD_DGT_TB |
| Ｘ button | PDD_DGT_TX |
| Ｙ button | PDD_DGT_TY |
| Reel handle | Rotation speed : R member of PDS_PERIPHERAL structure / PDD_DGT_TR |
| Analog direction key | X direction : x1 member of PDS_PERIPHERAL structure Y direction : y1 member of PDS_PERIPHERAL structure Up : PDD_DGT_KU / Down : PDD_DGT_KD Right : PDD_DGT_KR / Left : PDD_DGT_KL |
| Ｘ transfer accelaration | X2 member of PDS_PERIPHERAL structure |
| Ｙ transfer accelaration | Y2 member of PDS_PERIPHERAL structure |
| Ｚ transfer accelaration | L member of PDS_PERIPHERAL structure / PDD_DGT_TL |
| Start button | PD_DGT_ST |

## 6-7	Visual Memory (HKT-7002)

| Member | Information |
| --- | --- |
| Type | STORAGE / LCD / TIMER |
| is_root | 0 |
| Area_code | PDD_DEVAREA_JAPAN |
| Connector_dir |  |
| Product_name | "Visual Memory" |
| License | "Produced By or Under License From SEGA ENTERPRISES, LTD." |
| Stdby_pow | 12.4mA |
| max_pow | 13.4mA |

## 6-8	Puru Puru Pack (HKT-8600)

Vibration peripheral. In addition to functions for vibration peripherals, dedicated functions for using this peripheral are accommodated for the SEGA Library environment.

| Member | Information |
| --- | --- |
| Type | VIBRATION |
| is_root | 0 |
| area_code | PDD_DEVAREA_USA / -_JAPAN / -_ASIA / -_EUROPE |
| Connector_dir |  |
| Product_name | "Puru Puru Pack" |
| License | "Produced By or Under License From SEGA ENTERPRISES,LTD." |
| stdby_pow | 20.0mA |
| max_pow | 160.0mA |

## 6-9	Mike Device –Unit for sound recognition and input- (HKT-7200)

Sound sampling peripheral. Used by mounting a mike to the front. Dedicated functions for using this peripheral are accommodated for the SEGA Library environment.

| Member | Information |
| --- | --- |
| Type | SOUNDINPUT |
| is_root | 0 |
| Area_code | PDD_DEVAREA_USA / -_JAPAN |
| Connector_dir |  |
| Product_name | "SoundInputPeripheral (S.I.P.)" |
| license | "Produced By or Under License From SEGA ENTERPRISES,LTD." |
| stdby_pow | 30.0mA |
| max_pow | 30.0mA |