GDFS (GD File System)
Ver 1.05

1998/12/02

Limitations:

Revision history:

1998/12/02	ver 1.05

- Modified gdFsRead function so as not to return normal termination even it encounters busy drive.

- Added workaround code to gdFsStopRd function. (Drive problem fixed.)

1998/10/27	ver 1.02

- Changed the internal processing of gdFsReqDrvStat function. (The command to issue is changed.)

1998/10/12	ver 1.01

- Provided workaround to incorrect operation in CDDA Release.

- Added global error handle registration function (gdFsEntryErrFuncAll)

1998/09/28	ver 1.00

- Modified so as to return GDD_ERR_CHECKBUSY when drive media is checked.

- Added function to get time elapsed in drive (gdFsDaGetInfo).

1998/09/15	ver 0.54

- Added function to update Drive Status (gdFsReqDrvStat).

- Added function to get System handle (gdFsGetSysHn)

1998/08/28 ver 0.53

- Changed DA-type functions to return immediately after calling. (API continues to use conventional type.)

1998/07/06 ver 0.52

- Corrected problem where cache becomes invalid when A0000000 address is passed as buffer.

- Changed so that ReqRd32 function returns 0 for EOF and BUSY.

- Added gdFsReinit function.

- Released gdFsReqGdRd and gdFsTrans32 functions.

- Added GDD_ERR_SIZEOVER error constant.

1998/06/26 ver 0.51

- Corrected problem where gdFsGetDirectSize macro function cannot return correct value.

- Changed parameters of gdFsDaPlay function as a result of change in GDC specification.

- Added gdFsGetErrStat function.

- Removed gdFsIsTransReady function (replaced by gdFsGetTransStat).

- Added gdFsGetWorkHn function (able to get handle of current process).

- Removed gdFsIsEof function (replaced by gdFsCheckEof).

- Changed return value of gdFsGetFile* function to Bool.

- Removed GDD_STAT_PLAY.

1998/06/01	ver 0.50

- Released Set5 alpha version (as ver 0.49)

1. Outline	5

1.1 Basic Functions	5

1.2 Module Composition	5

2. Function Description	6

2.1 Terms	6

2.2 Initialization	6

2.3 Directory Operations	7

2.4 File Operations	9

2.5 File Access	9

2.6 Getting Drive Information	13

2.7 Callback Registration	14

2.8 Digital Audio Playback	15

2.9 Server Functions	15

3. Data Specifications	16

3.1 Constants	16

3.2 Data Type	18

4. Function Specifications	19

4.1 Function List	19

4.2 Function (API)	21

Library Initialization/Shutdown Functions	21

Directory Operation Functions	24

File Operation Functions	28

Compretion Return Function	32

Immediate Return Functions	33

Drive Status Functions	39

Callback Registration Functions	41

Digital Audio Playback Functions	43

Server Function	47

A. Extras	48

A.1. Description of Error Constants 	48

# Outline

## Basic Functions

This library provides the following functions.

- GD-ROM high-density area access

The main functions are as follows:

- Directory access

- File open/close

- Synchronous/asynchronous file read

- GD Digital Audio (DA) playback

The main functions are as follows:

- Track-specified playback

- Sector-specified playback

## Module Composition

The composition of modules for GD access, including the file system, is as follows:

Fig. 1  Module Composition

This library requires the following library groups.

- sy_cache.lib (RAW CACHE library)

- sy_chain.lib (interrupt chain handler library)

- sg_ov.lib (Overlay Library)

- kabuto.lib (Access Librar for kabuto)

# Function Description

## Terms

The following terms are used in this document.

| Term | Meaning |
| --- | --- |
| FAD | Frame address. Physical sector address on GD-ROM. |
| File handle | Necessary information (pointers) for handling files |
| GD buffer | Temporary storage space for data read off the GD-ROM |
| Callback | A means of calling up a registered process when certain conditions are met |
| Sector | Smallest unit for handling data on a GD-ROM (fixed to 2048 bytes on a GD-ROM) |
| Directory record | Location of data giving information about GD-ROM directory structure |
| Directory record handle | File information (pointers) extracted from the directory record |
| Library work area | Work space required by library |

Table 1  Definition of Term

## Initialization

Before using this library, be sure to call the gdFsInit function. This function performs the following actions.

- Initialization of library work area

- Device initialization (GD-ROM)

- Mount processing

When the library is initialized, a space for storing information about the work area and current directory is required. This space must be provided by the library user.

The size of the library work area changes according to the maximum number of simultaneously open files. The size of the space for storing current directory information varies according to the number of stored files.

The following macro can be used to obtain the required size for the above items.

- gdFsGetWorkSize(max_open)

- gdFsGetDirrecSize(max_dirent)

The library can be initialized as follows.

Initialization sequence (example):

Uint32 gdfswork[gdFsGetWorkSize(8)/4];

Uint32 gdfscurdir[gdFsGetDirrecSize(64)/4];

gdFsInit(8, gdfswork, 64, gdfscurdir);

In this case, gdfswork must be placed on a 32-byte boundary.

When Init finishes or before Finished is called, even if Init is called again, SUCCESS is always returned unconditionally. (This is solution to handle the Overlay.)

## Directory Operations

This library uses directory record handles for directory operations. Directory record handles store information about files in directories on the GD-ROM. They provide vital information such as file name and file size to allow use of the file system.

Directory record handles correspond to the directory cache in a conventional operating system.

Fig. 2  Directory record handle

When the library is initialized, a directory record handle for storing information on the current directory is automatically created.

When the current directory is changed to the work directory, there is no need to create another handle.

To change the current directory, use gdFsChangeDir.

When alternately accessing files from several directories, the reader will have to access the GD-ROM every time which can result in a marked decrease in performance.

Fig. 3  Switching between multiple directories

In such a case, creating directory record handles for directories that are to be accessed frequently will serve to store required file information and thereby reduce the number of disc accesses.

Switching between several directories (example)

Uint32 dirbuf1[gdFsGetDirrecSize(64)];

Uint32 dirbuf2[gdFsGetDirrecSize(64)];

GDFS_DIRREC gf_moviedir;

GDFS_DIRREC gf_gradir;

GDFS gdfs1, gdfs2;

gf_moviedir = gdFsCreateDirhn(dirbuf1, 64);

gf_gradir = gdFsCreateDirhn(dirbuf2, 64);

gdFsLoadDir("MOVIE", gf_moviedir);

gdFsLoadDir("GRA", g_gradir);

gdfs1 = gdFsOpen(“ABC.MOV”, gf_moviedir);

gdfs2 = gdFsOpen(“DEF.GRA”, gf_gradir);

/File read processing etc. */

gdFsClose(gdfs1);

gdfsClose(gdfs2);

## File Operations

For read-in and other file operations, File handles are required. File handles can be generated with the gdFsOpen function.

File handles can be used to obtain file size and other information.

(gdFsGetFileSize, gdFsFileSizeSctSize, gdFsGetFad)

Files handles can also be used to change or get the read position of the file and to check whether the end of the file is reached.

(gdFsSeek, gdFsTell, gdFsCheckEof)

File handles that are no longer required can be freed with gdFsClose.

## File Access

File access is performed by one of the two following methods.

- Completion return access

Control is blocked until processing is completed.

- Immediate return access

Control returns to the application as soon as the request is received.

### Completion Return Access

The completion return access function simplifies control, but since control is blocked until processing is finished, performance may decrease, especially in games etc. where real-time control is required.

Completion return access (example)

GDFS gf;

Uint32 buf[2048/4];

gf = gdFsOpen("ABC.DAT", NULL);

gdFsRead(gf, 1, buf);

gdFsClose(gf);

### Immediate Return Access

When immediate return access is used, control is more complicated than with completion return access, but other operations can be carried out in parallel while the read operation is in progress.

Immediate return access (example)

GDFS gf;

Uint32 buf[2048/4];

gf = gdFsOpen("ABC.DAT", NULL);

gdFsReqRd32(gf, 1, buf);

do {

stat = gdFsGetStat(gf);

} while (stat == GDD_STAT_READ);

gdFsClose(gf);

The state transition during read-in as as shown below.

Fig. 4  File handle State Transition

Functions where the read-in and transfer operations are separate are also available, mainly for streaming. (gdFsReqGdRd and gdFsTrans32)

Fig. 5  gdFsReqGdRd and gdFsTrans32 Functions

ReqGdRd access (example)

gdFsReqGdRd(gf, 128);

while ((stat = gdFsGetStat(gf)) == GDD_STAT_READ) {

if (gdFsGetTransStat(gf) == GDD_TRANS_READY)

gdFsTrans32(gf, 2048, buf);

}

### Access Precautions

#### Cache

Katana incorporates a 16KB operand cache (OC) and an 8KB instruction cache (IC).

When these are active, the following problems can occur:

- When re-reading modules of a program to the same address, such as when rereading overlay modules, the previous code will still be present in the instruction cache (IC) also when the next module is read, which can lead to operation errors.

- When re-reading data to the same address, data remaining in the operand cache (OC) may be read instead.

To prevent such problems, the instruction cache (IC) or operand cache (OC) must be cleared before the read-in operation.

However, in the file system it is not possible to determine whether the file to be read is a program or data.

Therefore the file system is designed only to assure the coherency of the operand cache (OC).

In practice, when a file read request is received, the cache for the relevant memory space should be invalidated.

(Except　when P2 space (0xa0000000) is specified, in which case the cache is not invalidated.)

However, if the relevant memory were accessed before actual data had been transferred to memory, the data would be read into the cache.

Therefore access to the relevant memory is restricted until the read-in process is completed.

## Getting Drive Information

The drive information functions allow obtaining the drive status and TOC information.

The drive status function can be used to determine if the GD tray is open.

Drive status function (example)

Sint32 dstat;

dstat = gdFsGetDrvStat();

if (dstat == GDD_DRVSTAT_OPEN) {

/abort processing */

}

However, the drive status is updated when the command to access the drive status is issued.

If the drive is not accessed for a long time, be sure to use the drive status request function (gdFsReqDrvStat) to update the status and then get the drive status.

Getting the TOC data provides information about the various tracks. This can be used to identify a DA (Digital Audio) track.

TOC function (example)

Sint32 tocbuf[102];

gdFsGetToc(1, &tocbuf);

TOC data can be obtained separately for the single-density and high-density areas.

Normally, DA tracks for games will be located in the high-density area.

The structure is as follows.

| Bytes | Contents |
| --- | --- |
| 0-3 | Track 1 information |
| 4-7 | Track 2 information |
|  | … |
| 392-395 | Track 99 information |
| 396-399 | Start track information |
| 400-403 | End track information |
| 404-407 | Lead-out information |

Fig. 6  TOC Data Format

| High-order | Low-order |  |  |  |
| --- | --- | --- | --- | --- |
| 1st Byte |  | 2nd byte | 3rd byte | 4th byte |
| Ctr | 0x1 | Track start FAD |  |  |
| Ctr | 0x1 | Start track number | 0x00 | 0x00 |
| Ctr | 0x1 | End track number | 0x00 | 0x00 |
| Ctr | 0x1 | Lead-out start FAD |  |  |

Fig. 7  TOC Data Format Details

| Control Field | Description |
| --- | --- |
| 00x0 | 2 Audio without Pre-emphansis |
| 00x1 | 2 Audio with Pre-emphansis |
| 0x0x | Copy Prohibited |
| 0x1x | Copy Permitted |
| 01x0 | Digital Data |
| 1xxx | Broadcast Data |

Fig. 8  Control (Ctr) Bit Value

## Callback Registration

This library provides the following callback items.

- Read-in complete callback

- Transfer complete callback

- Error occurrence callback

The gdFsEntry- functions are supplied for callback functions registered by individual library users.

Since these callback functions are called from within interrupts, they should be kept as simple and short as possible.

If callback functions require a long processing time, this may adversely affect other processes. Please exercise due care.

Also, within the transfer termination callback, all functions except gdFsTrans32 and fsFsStopRd cannot be used.

If both error callback and global error callback are registered, then the global callback will be executed first.

## Digital Audio Playback

The following functions are provided for DA (Digital Audio) playback.

- Track-specified playback

- Sector-specified playback

- Playback stop

- Playback pause

- Playback restart

These are implemented with the gdFsDA- functions.

If a file read process etc. is initiated while DA playback is in progress, playback will be interrupted. While file read is being carried out, a DA playback command is invalid.

## Server Functions

This library comes with the server functions for background processing of requested tasks.

However, because these server functions are automatically registered in the VblankIn interrupt when the library is initialized, the library user does not need to use them if there is no such need in particular.

# Data Specifications

## Constants

| Constant | GDD_SEEK_~ | Seek Mode |
| --- | --- | --- |

| Constant Name | Description |
| --- | --- |
| GDD_SEEK_SET | File start |
| GDD_SEEK_CUR | Current file read position |
| GDD_SEEK_END | File end |

| Constant | GDD_STAT_~ | Status |
| --- | --- | --- |

| Constant Name | Description |
| --- | --- |
| GDD_STAT_IDLE | Idle |
| GDD_STAT_COMPLETE | Operation complete |
| GDD_STAT_READ | Read in progress |
| GDD_STAT_ERR | Error has occurred |
| GDD_STAT_FATAL | Fatal error has occurred |

| Constant | GDD_DRVSTAT_~ | Drive Status |
| --- | --- | --- |

| Constant Name | Description |
| --- | --- |
| GDD_DRVSTAT_BUSY | Busy |
| GDD_DRVSTAT_PAUSE | Pause |
| GDD_DRVSTAT_STANDBY | Standby |
| GDD_DRVSTAT_PLAY | Playback |
| GDD_DRVSTAT_SEEK | Seek |
| GDD_DRVSTAT_SCAN | Scan playback |
| GDD_DRVSTAT_OPEN | Tray open |
| GDD_DRVSTAT_NODISC | No disc |
| GDD_DRVSTAT_RETRY | Retry |
| GDD_DRVSTAT_ERROR | Error |

| Constant | GDD_FS_TRANS_~ | Transfer status |
| --- | --- | --- |

| Constant Name | Description |
| --- | --- |
| GDD_FS_TRANS_READY | Transfer ready |
| GDD_FS_TRANS_BUSY | Transferring |
| GDD_FS_TRANS_COMPLETE | Transfer complete |
| GDD_FS_TRANS_ERROR | Transfer error |

| Constant | GDD_ERR_~ | Error Code |
| --- | --- | --- |

| Constant Name | Description |
| --- | --- |
| GDD_ERR_OK | Normal end |
| GDD_ERR_INIT | Library initialization failed |
| GDD_ERR_RESET | Drive initialization failed |
| GDD_ERR_LIBOV | Overlay initialization failed |
| GDD_ERR_MOUNT | Mounting failed |
| GDD_ERR_DISC | Bad disc |
| GDD_ERR_DIRREC | Incorrect directory record handle |
| GDD_ERR_CANTOPEN | Could not open file |
| GDD_ERR_NOTFOUND | File not found |
| GDD_ERR_NOHNDL | No free handle available |
| GDD_ERR_ILLHNDL | Illegal handle |
| GDD_ERR_NOTDIR | Not a directory |
| GDD_ERR_DIROVER | Number of entries exceeded |
| GDD_ERR_BUSY | Busy |
| GDD_ERR_32ALIGN | Not 32-byte boundary |
| GDD_ERR_SIZE | Not 32-byte unit |
| GDD_ERR_SEEK | Incorrect seek specification |
| GDD_ERR_OFS | Incorrect position specification |
| GDD_ERR_ILLTMODE | Incorrect transfer mode |
| GDD_ERR_READ | Read failure |
| GDD_ERR_NOTREAD | Read not in progress |
| GDD_ERR_TOUT | Timeout |
| GDD_ERR_EOF | End of file |
| GDD_ERR_TRAYOPEND | Tray is open |
| GDD_ERR_SIZEOVER | Requested size too large |
| GDD_ERR_FATAL | Fatal error |
| GDD_ERR_UNDEF | Undefined error |
| GDD_ERR_NOERR | No error has occurred |
| GDD_ERR_RECOVER | Error successfully recovered |
| GDD_ERR_NOTREADY | Drive not ready |
| GDD_ERR_MEDIA | Media error |
| GDD_ERR_HWARE | Hardware error |
| GDD_ERR_ILLREQ | Illegal request |
| GDD_ERR_UNITATTENT | Media change was detected |
| GDD_ERR_PROTECT | Protected |
| GDD_ERR_ABORT | Aborted |
| GDD_ERR_NOREADABLE | Not readable |
| GDD_ERR_CHECKBUSY | Media is being checked. |

| Constant | GDD_FF_~ | File Flag |
| --- | --- | --- |

| Bit7 | Bit6 | Bit5 | Bit4 | Bit3 | Bit2 | Bit1 | Bit0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Multi Extent | **** | **** | Protection | Record | Associated | Directory | Existence |

## Data Type

| Data Type | GDFS | File handle |
| --- | --- | --- |

| Data Type | GDFS_DIRREC | Directory Handl |
| --- | --- | --- |

| Data Type | GDFS_DIRINFO | Directory Information |
| --- | --- | --- |

|  | Type | Name | Description |
| --- | --- | --- | --- |
|  | Sint32 | fad | Start FAD |
|  | Sint32 | fsize | File size |
|  | Uint8 | flag | File flag |
|  | Uint8 | pad[3] | Reserved |

| Data Type | GDFS_FUNC | Callback Function |
| --- | --- | --- |

|  | Type |  | Name | Description |
| --- | --- | --- | --- | --- |
|  | void * |  | obj | First argument for callback function |
| Function example |  | void gdfs_func(void *obj) |  |  |

| Data Type | GDFS_ERRFUNC | Error Callback Function |
| --- | --- | --- |

|  | Type |  | Name | Description |
| --- | --- | --- | --- | --- |
|  | void * |  | obj | First argument for callback function |
|  | Sint32 |  | errcode | Error Code |
| Function example |  | void gdfs_errfunc(void *obj, Sint32 errcode) |  |  |

| Data Type | GDFS_DAINFO | DA Playback Information |
| --- | --- | --- |

|  | Type | Name | Description |
| --- | --- | --- | --- |
|  | Sint32 | track | Track number |
|  | Sint32 | min | Elapsed time (minute) |
|  | Sint32 | sec | Elapsed time (second) |
|  | Sint32 | frame | Elapsed time (frame) |
|  | Sint32 | fad | Frame address |

# Function Specifications

## Function List

| Library initialization/shutdown functions |  |  |
| --- | --- | --- |
|  | gdFsInit | Initialize library |
|  | gdFsReinit | Re-initialize library |
|  | gdFsFinish | Finish library |
|  | gdFsGetWorkSize | Get work area size (macro) |
|  | gdFsGetDirrecSize | Get directory buffer size (macro) |

| Directory operation functions |  |  |
| --- | --- | --- |
|  | gdFsCreateDirhn | Create directory handle |
|  | gdFsLoadDir | Load directory |
|  | gdFsSetDir | Set current directory |
|  | gdFsChangeDir | Change current directory |
|  | gdFsGetDirInfo | Get file information |

| File operation functions |  |  |
| --- | --- | --- |
|  | gdFsOpen | Open file |
|  | gdFsOpenRange | Open specified sector range |
|  | gdFsClose | Close file |
|  | gdFsSeek | Move read-in pointer |
|  | gdFsTell | Get read-in pointer position |
|  | gdFsCheckEof | Check for end of file |
|  | gdFsGetFileSize | Get file size |
|  | gdFsGetFileSctSize | Get file sector size |
|  | gdFsGetFad | Get file FAD |
|  | gdFsCalcSctSize | Calculate sector size from bytes (macro) |

| Completion return function |  |  |
| --- | --- | --- |
|  | gdFsRead | Read file |

| Immediate return functions |  |  |
| --- | --- | --- |
|  | gdFsReqRd32 | Read file |
|  | gdFsGetStat | Get file status |
|  | gdFsGetErrStat | Get file error status |
|  | gdFsGetNumRd | Get number of read bytes |
|  | gdFsStopRd | Stop file read |
|  | gdFsMovePickup | Move pickup |
|  | gdFsReqGdRd | Request read-ahead into GD buffer |
|  | gdFsTrans32 | Specify transfer from GD buffer |
|  | gdFsGetTransStat | Get transfer status |
|  | gdFsGetWorkHn | Get work handle |
|  | gdFsGetSysHn | Gets system handle. |

| Drive status functions |  |  |
| --- | --- | --- |
|  | gdFsGetToc | Get TOC |
|  | gdFsGetDrvStat | Get drive status |
|  | gdFsReqDrvStat | Updates drive status. |

| Callback registration functions |  |  |
| --- | --- | --- |
|  | gdFsEntryRdEndFunc | Enter read end callback function |
|  | gdFsEntryTrEndFunc | Enter transfer end callback function |
|  | gdFsEntryErrFunc | Enter error callback function |
|  | gdFsEntryErrFuncAll | Register callback function present when error occurs on all handle |

| Digital Audio playback functions |  |  |
| --- | --- | --- |
|  | gdFsDaPlay | Start track-specified DA playback |
|  | gdFsDaPlaySct | Start sector-specified DA playback |
|  | gdFsDaStop | Stop DA playback |
|  | gdFsDaPause | Pause DA playback |
|  | gdFsDaRelease | Release DA playback pause |
|  | gdFsDaGetInfo | Get DA's playback information. |

| Server function |  |  |
| --- | --- | --- |
|  | gdFsExecServer | Server function |

## Function (API)

##### Library Initialization/Shutdown Functions

| Function | gdFsInit | Initialize library |
| --- | --- | --- |

Format	Sint32  gdFsInit(Sint32 max_open, void *gdfs_work, Sint32 max_dirent, void *dirbuf)

Input	max_open		Maximum number of simultaneously open files
gdfs_work		Work area pointer (supplied by user area)
max_dirent		Number of entries in current directory
dirbuf		Current directory buffer (supplied by user area)

Output	None

Return value	Error codes
Main error codes returned by this function are listed below.
GDD_ERR_OK
GDD_ERR_32ALIGN
GDD_ERR_RESET
GDD_ERR_TRAYOPEND
GDD_ERR_DISC	
GDD_ERR_MOUNT
GDD_ERR_DIROVER

Function	Initializes the library.

Usage example	Uint32 gdfswork[gdFsGetWorkSize(8)/4];
Uint32 gdfscurdir[gdFsGetDirrecSize(64)/4];

gdFsInit(8, gdfswork, 64, gdfscurdir);

Remarks	gdfs_work must be 32-byte aligned. (dirbuf may be 4-byte aligned.)

When initialization has been completed, renewed initialization with gdFsInit is not possible before gdFsFinish has been called.

gdFsInit performs the following main processes:

- GD driver initialization

- Work area initialization

- Device initialization

- Mount processing

Note	This is a completion return function.  If this function is executed during interrupt inhabited period, the process gets stuck without being able to time-out.

| Function | gdFsReinit | Re-initialize library |
| --- | --- | --- |

Format	Sint32  gdFsReinit(void)

Input	None

Output	None

Return value	Error codes
The following are main error codes returned by this function.
GDD_ERR_OK
GDD_ERR_INIT
GDD_ERR_32ALIGN
GDD_ERR_RESET
GDD_ERR_TRAYOPEND
GDD_ERR_DISC
GDD_ERR_MOUNT
GDD_ERR_DIROVER

Function	Re-initializes the library.
Performs the steps from device initialization to mount processing.
Use this after a media change, etc.

Usage example	gdFsReinit();

Note	This is a completion return function.  If this function is executed during interrupt inhabited period, the process gets stuck without being able to time-out.

| Function | gdFsFinish | Finish library |
| --- | --- | --- |

Format	void  gdFsFinish(void)

Input	None

Output	None

Return value	None

Function	Terminates use of the library.

Usage example	Uint32 gdfswork[gdFsGetWorkSize(8)/4];
Uint32 gdfscurdir[gdFsGetDirrecSize(64)/4];

gdFsInit(8, gdfswork, 64, gdfscurdir);
gdFsFinish();

| Macro | gdFsGetWorkSize | Get work area size |
| --- | --- | --- |

Macro	gdFsGetWorkSize(max_open)

Argument	max_open		Maximum number of simultaneously open files

Macro value	Required library work area (bytes)

Function	Gets the number of bytes that are required for the library work area (macro function).

Usage example	Uint32 gdfswork[gdFsGetWorkSize(8)/4];

| Macro | gdFsGetDirrecSize | Get directory buffer size |
| --- | --- | --- |

Macro	gdFsGetDirrecSize(max_dirent)

Argument	max_dirent		Maximum number of directory entries

Macro value	Required directory buffer size (bytes)

Function	Gets the number of bytes that are required for the directory buffer (macro function).

Usage example	Uint32 gdfscurdir[gdFsGetDirrecSize(64)/4];

##### Directory Operation Functions

| Function | gdFsCreateDirhn | Create directory handle |
| --- | --- | --- |

Format	GDFS_DIRREC gdFsCreateDirhn(void *dirbuf, Sint32 max_dirent)

Input	dirbuf		Directory buffer
max_dirent		Maximum number of directory entries

Output	None

Return value	Directory record handle

Function	Creates the directory handle.

Usage example	Uint32 dirbuf[gdFsGetDirrecSize(64)];
GDFS_DIRREC g_dir;

g_dir = gdFsCreateDirhn(dirbuf, 64);

Note	If discs are exchanged, the directory handle before the exchange cannot be used. Therefore, regenerate it again.

| Function | gdFsLoadDir | Load directory |
| --- | --- | --- |

Format	Sint32  gdFsLoadDir(const char *dirname, GDFS_DIRREC gf_dirrec)

Input	dirname		Directory name

Output	gf_dirrec		Directory record handle
			(If NULL go to current directory)

Return value	Error codes
The following are main error codes returned by this function.
GDD_ERR_OK
GDD_ERR_NOTFOUND
GDD_ERR_NOTDIR
GDD_ERR_DIROVER
GDD_ERR_BUSY
GDD_ERR_TOUT
GDD_ERR_NOERR
GDD_ERR_RECOVER
GDD_ERR_NOTREADY
GDD_ERR_MEDIA
GDD_ERR_HWARE
GDD_ERR_UNITATTENT
GDD_ERR_TRAYOPEND
GDD_ERR_CHECKBUSY

Function	Loads the directory record (no distinction between upper/lower case).

Usage example	/Sample 1*/
/Load MOVIE directory to g_dir */
Uint32 dirbuf[gdFsGetDirrecSize(64)];
GDFS_DIRREC g_dir;

g_dir = gdFsCreateDirhn(dirbuf, 64);
gdFsLoadDir("MOVIE", g_dir);

/Sample 2 */
/Change current directory to DATA directory */
gdFsLoadDir("DATA", NULL);

Note	This is a completion return function.  If this function is executed during interrupt inhabited period, the process gets stuck without being able to time-out.

| Function | gdFsSetDir | Set current directory |
| --- | --- | --- |

Format	Sint32 gdFsSetDir(GDFS_DIRREC gf_dirrec)

Input	gf_dirrec		Directory record handle

Output	None

Return value	Error codes
GDD_ERR_OK
GDD_ERR_DIROVER

Function	Sets the current directory.

Usage example	/Load MOVIE directory to g_dir */
Uint32 dirbuf[gdFsGetDirrecSize(64)];
GDFS_DIRREC g_dir;

g_dir = gdFsCreateDirhn(dirbuf, 64);
gdFsLoadDir("MOVIE", g_dir);
gdFsSetDir(g_dir);

| Function | gdFsChangeDir | Change current directory |
| --- | --- | --- |

Format	Sint32 gdFsChangeDir(const char *dirname)

Input	dirname		Directory name

Output	None

Return value	Error codes
The following are main error codes returned by this function.
GDD_ERR_OK
GDD_ERR_NOTFOUND
GDD_ERR_NOTDIR
GDD_ERR_DIROVER
GDD_ERR_BUSY
GDD_ERR_TOUT
GDD_ERR_NOERR
GDD_ERR_RECOVER
GDD_ERR_NOTREADY
GDD_ERR_MEDIA
GDD_ERR_HWARE
GDD_ERR_UNITATTENT
GDD_ERR_TRAYOPEND
GDD_ERR_CHECKBUSY

Function	Changes the current directory (no distinction between upper/lower case).

Usage example	/Change current directory to DATA directory */
gdFsChangeDir("DATA");

Note	This is a completion return function.  If this function is executed during interrupt inhabited period, the process gets stuck without being able to time-out.

| Function | gdFsGetDirInfo | Get file information |
| --- | --- | --- |

Format	Sint32 gdFsGetDirInfo(const char *name, GDFS_DIRINFO dirinfo)

Input	name			File name/directory name

Output	dirinfo		File information

Return value	Error codes
GDD_ERR_OK
GDD_ERR_NOTFOUND

Function	Gets file information.

Usage example	GDFS_DIRINFO dinfo;
gdFsGetDirInfo(“ABC.DAT”, &dinfo);

##### File Operation Functions

| Function | gdFsOpen | Open file |
| --- | --- | --- |

Format	GDFS  gdFsOpen(const char *fname, GDFS_DIRREC gf_dirrec)

Input	fname		File name
gf_dirrec		Directory record handle for searching file name
			(If NULL search current directory)

Output	None

Return value	NULL	...  Failed
Other	...  File handle

Function	Opens a file.

Usage example	/Sample 1 *
GDFS gf;

gf = gdFsOpen("A.BIN", NULL);

/Sample 2 */
GDFS gf;
Uint32 dirbuf[gdFsGetDirSize(64)];
GDFS_DIRREC g_dir;

g_dir = gdFsCreateDirhn(dirbuf, 64);
gdFsLoadDir("MOVIE", g_dir);
gf = gdFsOpen("SMP.MOV", g_dir);

| Function | gdFsOpenRange | Open specified sector range |
| --- | --- | --- |

Format	GDFS  gdFsOpenRange(Sint32 stsct, Sint32 nsct)

Input	stsct			Start sector
endsct		End sector

Output	None

Return value	NULL	...  Failed
Other	...  File handle

Function	Opens a file by specifying a sector range.

Usage example	GDFS gf;

gf = gdFsOpenRange(0xB555, 0x10);
gdFsClose(gf);

Note	The single-density area cannot be accessed.

| Function | gdFsClose | Close file |
| --- | --- | --- |

Format	void  gdFsClose(GDFS gdfs);

Input	gdfs		File handle

Output	None

Return value	None

Function	Closes a file.

Usage example	GDFS gf;

gf = gdFsOpen("TEST.BIN", NULL);
gdFsClose(gf);

| Function | gdFsSeek | Move read-in pointer |
| --- | --- | --- |

Format	Sint32  gdFsSeek(GDFS gdfs, Sint32 sctno, Sint32 type)

Input	gdfs		File handle
sctno		Sector number
type		GDD_SEEK_SET, _CUR, _END

Output	None

Return value	Error codes
GDD_ERR_OK
GDD_ERR_ILLHNDL
GDD_ERR_OFS
GDD_ERR_SEEK

Function	Moves the read-in pointer.

Usage example	GDFS gf;

gf = gdFsOpen("TEST.BIN", NULL);

/Perform seek to 5th sector from start */
gdFsSeek(gf, 5, SEEK_SET);

| Function | gdFsTell | Get read-in pointer position |
| --- | --- | --- |

Format	Sint32  gdFsTell(GDFS gdfs);

Input	gdfs		File handle

Output	None

Return value	Read-in pointer position

Function	Gets the read-in pointer position (in sector units).

Usage example	Sint32 pos;
GDFS gf;

gf = gdFsOpen("TEST.BIN", NULL);

pos = gdFsTell(gf);

| Function | gdFsCheckEof | Check for end of file |
| --- | --- | --- |

Format	Sint32  gdFsCheckEof(GDFS gdfs, Bool *iseof)

Input	gdfs		File handle

Output	iseof

Return value	Error codes
GDD_ERR_OK
GDD_ERR_ILLHNDL

Function	Checks whether end of file is reached.

Usage example	GDFS gf;
Bool flag;
Sint32 ret;
	
	gf = gdFsOpen("TEST.BIN", NULL);

/After read process */

ret = gdFsCheckEof(gf, &flag);

if (ret == GDD_ERR_OK && flag == TRUE) {

gdFsClose(gf);

return;

}

| Function | gdFsGetFileSize | Get file size |
| --- | --- | --- |

Format	Bool  gdFsGetFileSize(GDFS gdfs, Sint32 *fsize)

Input	gdfs		File handle

Output	fsize		File size (bytes)

Return value	True / False

Function	Gets the file size.

Usage example	GDFS gf;
Sint32 flen;

gf = gdFsOpen("TEST.BIN", NULL);
gdFsGetFileSize(gdfs, &flen);

| Function | gdFsGetFileSctSize | Get file sector size |
| --- | --- | --- |

Format	Bool  gdFsGetFileSctSize(GDFS gdfs, Sint32 *fsctsize)

Input	gdfs		File handle

Output	fsctsize	File size (bytes)

Return value	True / False

Function	Gets the file sector size.

Usage example	GDFS gf;
Sint32 fslen;

gf = gdFsOpen("TEST.BIN", NULL);
gdFsGetFileSctSize(gdfs, &fslen);

| Function | gdFsGetFad | Get file FAD |
| --- | --- | --- |

Format	Bool  gdFsGetFad(GDFS gdfs, Sint32 *fad)

Input	gdfs		File handle

Output	fad		Frame address (physical sector)

Return value	True / False

Function	Gets the frame address (start sector) of the file.

Usage example	Sint32 fad;
Sint32 gf;

gf = gdFsOpen("TEST.BIN", NULL);
gdFsGetFad(gf, &fad);

| Macro | gdFsCalcSctSize | Calculate sector size from bytes |
| --- | --- | --- |

Macro	gdFsCalcSctSize(bytes)

Input	bytes		Number of bytes in file

Macro value	Sector size

Function	Calculates the sector size from the number of bytes (macro function).

Usage example	GDFS gf;
Sint32 flen;
Sint32 fslen;

gf = gdFsOpen("TEST.BIN", NULL);
gdFsGetFileSize(gdfs, &flen);
fslen = gdFsCalcSctSize(flen);

##### Completion Return Function

| Function | gdFsRead | Read file |
| --- | --- | --- |

Format	Sint32  gdFsRead(GDFS gdfs, Sint32 nsct, void *buf)

Input	gdfs		File handle
nsct		Number of sectors to read

Output	buf		Store buffer

Return value	Error codes

Function	Reads a file (completion return)
The following are main error codes returned by this function.
GDD_ERR_OK
GDD_ERR_TOUT
GDD_ERR_ILLHNDL
GDD_ERR_RECOVER
GDD_ERR_NOTREADY
GDD_ERR_MEDIA
GDD_ERR_HWARE
GDD_ERR_UNITATTENT
GDD_ERR_PROTECT
GDD_ERR_ABORT
GDD_ERR_NOREADABLE
GDD_ERR_TRAYOPEND
GDD_ERR_CHECKBUSY

Usage example	GDFS gf;
Sint32 buf[32*2048/4];

gf = gdFsOpen("TEST.BIN", NULL);
gdFsRead(gf, 32, buf);
gdFsClose(gf);

Note	This is a completion return function.  If this function is executed during interrupt inhabited period, the process gets stuck without being able to time-out.

##### Immediate Return Functions

| Function | gdFsReqRd32 | Read file |
| --- | --- | --- |

Format	Sint32  gdFsReqRd32(GDFS gdfs, Sint32 nsct, void *buf)

Input	gdfs		File handle
nsct		Number of sectors to read

Output	buf		Store buffer

Return value	Actually requested number of sectors (>0). When negative, error code is returned.
GDD_ERR_ILLHNDL
GDD_ERR_32ALIGN

Function	Requests a file read (immediate return)

Note	buf must be 32-bytes aligned. Also, if another command is being executed or when the file end is reached, 0 is returned.

Remarks	Only one request can be accepted at a time.

Usage example	GDFS gf;
Uint32 buf[32*2048/4];

gf = gdFsOpen("TEST.BIN", NULL);
gdFsReqRd32(gf, 32, buf);

while (gdFsGetStat(gf) == GDD_STAT_READ);

gdFsClose(gf);

| Function | gdFsGetStat | Get file status |
| --- | --- | --- |

Format	Sint32  gdFsGetStat(GDFS gdfs)

Input	gdfs		File handle

Output	None

Return value	Status. A negative value indicates it is an error code (GDD_ERR_ILLHNDL only).

Function	Gets the file handle status.

Usage example	GDFS gf;
Uint32 buf[32*2048/4];

gf = gdFsOpen("TEST.BIN", NULL);
gdFsReqRd32(gf, 32, buf);

while (gdFsGetStat(gf) == GDD_STAT_READ);

gdFsClose(gf);

| Function | gdFsGetErrStat | Get file error status |
| --- | --- | --- |

Format	Sint32  gdFsGetErrStat(GDFS gdfs)

Input	gdfs		File handle

Output	None

Return value	Error codes
GDD_ERR_NOERR
GDD_ERR_RECOVER
GDD_ERR_NOTREADY
GDD_ERR_MEDIA
GDD_ERR_HWARE
GDD_ERR_ILLREQ
GDD_ERR_UNITATTENT
GDD_ERR_PROTECT
GDD_ERR_ABORT
GDD_ERR_NOREADABLE
GDD_ERR_TRAYOPEND
GDD_ERR_CHECKBUSY

Function	Gets the file handle error status.

Usage example	GDFS gf;
Uint32 buf[32*2048/4];
Sint32 stat;
Sint32 err;

gf = gdFsOpen("TEST.BIN", NULL);
gdFsReqRd32(gf, 32, buf);

while ((stat = gdFsGetStat(gf)) == GDD_STAT_READ);

if (stat == GDD_STAT_ERR) {
	err = gdFsGetErrStat(gf);
	/error processing */
}
gdFsClose(gf);

| Function | gdFsGetNumRd | Get number of read bytes |
| --- | --- | --- |

Format	Sint32  gdFsGetNumRd(GDFS gdfs);

Input	gdfs		File handle

Output	None

Return value	0 or higher	Number of read bytes

Function	Gets the number of read bytes.

Usage example	GDFS gf;
	Uint32 buf[32*2048/4];

gf = gdFsOpen("TEST.BIN", NULL);
gdFsReqRd32(gf, 32, buf);

while (gdFsGetStat(gf) != GDD_STAT_COMPLETE) {
  readnum = gdFsGetNumRd(gdfs);
}

gdFsClose(gf);

| Function | gdFsStopRd | Stop file read |
| --- | --- | --- |

Format	Sint32  gdFsStopRd(GDFS gdfs)

Input	gdfs		...  File handle

Output	None

Return value	Error codes
GDD_ERR_OK
GDD_ERR_TOUT
GDD_ERR_ILLHNDL
GDD_ERR_BUSY
GDD_ERR_RECOVER
GDD_ERR_NOTREADY
GDD_ERR_MEDIA
GDD_ERR_HWARE
GDD_ERR_ILLREQ
GDD_ERR_UNITATTENT
GDD_ERR_PROTECT
GDD_ERR_ABORT
GDD_ERR_NOREADABLE
GDD_ERR_TRAYOPEND
GDD_ERR_CHECKBUSY

Function	Interrupts the request.

Usage example	gdFsStopRd(gf);

Note	This is a completion return function.  If this function is executed during interrupt inhabited period, the process gets stuck without being able to time-out.

| Function | gdFsMovePickup | Move pickup |
| --- | --- | --- |

Format	Sint32  gdFsMovePickup(GDFS gdfs)

Input	gdfs		File handle

Output	None

Return value	Error codes
GDD_ERR_OK
GDD_ERR_TOUT
GDD_ERR_ILLHNDL
GDD_ERR_BUSY
GDD_ERR_RECOVER
GDD_ERR_NOTREADY
GDD_ERR_MEDIA
GDD_ERR_HWARE
GDD_ERR_ILLREQ
GDD_ERR_UNITATTENT
GDD_ERR_PROTECT
GDD_ERR_ABORT
GDD_ERR_NOREADABLE
GDD_ERR_TRAYOPEND
GDD_ERR_CHECKBUSY

Function	Moves the pickup to the next read-in position.

Usage example	gdFsMovePickup(gf);

Note	This is a completion return function.  If this function is executed during interrupt inhabited period, the process gets stuck without being able to time-out.

| Function | gdFsReqGdRd | Request read-ahead into GD buffer |
| --- | --- | --- |

Format	Sint32  gdFsReqGdRd(GDFS gdfs, Sint32 nsct)

Input	gdfs		...  File handle

nsct		...  Number of read sectors

Output	None

Return value	Actually requested number of sectors (>0).
When negative, error code is returned. (GDD_ERR_ILLHNDL only)

Function	Requests a file read into the GD buffer.

Usage example	gdFsReqGdRd(gf, 128);

| Function | gdFsTrans32 | Specify transfer from GD buffer |
| --- | --- | --- |

Format	Sint32  gdFsTrans32(GDFS gdfs, Sint32 nbytes, void *buf)

Input	gdfs		...  File handle

nbytes	...  Number of bytes to be transferred (32-byte units)

Output	buf		...  Transfer target address

Return value	Error codes
GDD_ERR_OK
GDD_ERR_ILLHNDL
GDD_ERR_32ALIGN
GDD_ERR_SIZE
GDD_ERR_NOTREAD
GDD_ERR_SIZEOVER
GDD_ERR_BUSY

Function	Specifies a transfer from the GD buffer.

Usage example	while ((stat = gdFsGetStat(gf)) == GDD_STAT_READ) {
	if (gdFsGetTransStat(gf) == GDD_TRANS_READY)
	gdFsTrans32(gf, 2048, buf);
}

| Function | gdFsGetTransStat | Get transfer status |
| --- | --- | --- |

Format	Sint32  gdFsGetTransStat(GDFS gdfs)

Input	gdfs		File handle

Output	None

Return value	Transfer status.
When negative, error code is returned. (GDD_ERR_ILLHNDL only)

Function	Gets the transfer status.

Usage example	gdFsGetTransStat(gf);

| Function | gdFsGetWorkHn | Get work handle |
| --- | --- | --- |

Format	GDFS  gdFsGetWorkHn(void)

Input	None

Output	None

Return value	Current work handle. NULL if none.

Function	Gets the current work handle.

Usage example	GDFS gf;
gf = gdFsGetWorkHn();

| Function | gdFsGetSysHn | Get system handle |
| --- | --- | --- |

Format	GDFS  gdFsGetSysHn(void)

Input	None

Output	None

Return value	System Handle

Funtion	Get system handle

Remarks	Used for checking if system handle is executed or not.

Usage example	GDFS sys;
sys = gdFsGetSysHn();
stat = gdFsGetStat(sys);

##### Drive Status Functions

| Function | gdFsGetToc | Get TOC |
| --- | --- | --- |

Format 	Sint32  gdFsGetToc(Sint32 type, void *buf)

Input	type		...  TOC type (0: single-density  1: high-density)

Output	buf		...  TOC read-in buffer (required size: 408 bytes)

Return value	Error codes
GDD_ERR_OK
GDD_ERR_TOUT
GDD_ERR_ILLHNDL
GDD_ERR_BUSY
GDD_ERR_RECOVER
GDD_ERR_NOTREADY
GDD_ERR_MEDIA
GDD_ERR_HWARE
GDD_ERR_ILLREQ
GDD_ERR_UNITATTENT
GDD_ERR_PROTECT
GDD_ERR_ABORT
GDD_ERR_NOREADABLE
GDD_ERR_TRAYOPEND
GDD_ERR_CHECKBUSY

Function	Gets TOC

Usage example	gdFsGetToc(0, &buf);

Note	This is a completion return function.  If this function is executed during interrupt inhabited period, the process gets stuck without being able to time-out.

| Function | gdFsGetDrvStat | Get drive status |
| --- | --- | --- |

Format	Sint32  gdFsGetDrvStat(void)

Input	None

Output	None

Return value	Drive status
If the status cannot be obtained, then -1 is returned.

Function	Gets the drive status.

Supplement	While the drive is being worked on, there will be brief instants when the drive status cannot be obtained.

Usage example	Sint32 dstat;
dstat = gdFsGetDrvStat();

| Function | gdFsReqDrvStat | Renew drive status |
| --- | --- | --- |

Format	Sint32  gdFsReqDrvStat(void)

Input	None

Output	None

Return value	Error code
GDD_ERR_OK
GDD_ERR_BUSY

Function	Renew drive status

Supplement	Issue a dummy command to the drive and let it update the drive status.
After the command is issued, control is returned immediately. However, It takes at least several ms for the command execution to finish.
While the command is running, the drive status cannot be read.
To check whether the command execution is finished, use the system handle.

Usage example	Sint32 dstat;
GDFS sys;

gdFsReqDrvStat();
sys = gdFsGetSysHn();

/* After a while */

if (gdFsGetStat(sys) != GDD_STAT_BUSY) {
  /* Command execution is finished */
  dstat = gdFsGetDrvStat();
} else {
  /* Command execution is not finished */
}

##### Callback Registration Functions

| Function | gdFsEntryRdEndFunc | Enter read end callback function |
| --- | --- | --- |

Format 	void  gdFsEntryRdEndFunc(GDFS gdfs, GDFS_FUNC func, void *obj);

Input	gdfs		File handle
func		Callback function
obj		First argument to hand to callback function

Output	None

Return value	None

Function	Enters the read end callback function.

Usage example	void rdend_callback(void *obj)
{/Processing */
}

gdFsEntryRdEndFunc(gf, rdend_callback, (void *) 0x1234);

| Function | gdFsEntryTrEndFunc | Enter transfer end callback function |
| --- | --- | --- |

Format 	void  gdFsEntryTrEndFunc(GDFS gdfs, GDFS_FUNC func, void *obj)

Input	gdfs		File handle
func		Callback function
obj		First argument to hand to callback function

Output	None

Return value	None

Function	Enters the transfer end callback function.

Usage example	void trend_callback(void *obj)
{/Processing */
}

gdFsEntryTrEndFunc(gf, trend_callback, (void *) 0x1234);

| Function | gdFsEntryErrFunc | Enter error callback function |
| --- | --- | --- |

Format	void  gdFsEntryErrFunc(GDFS gdfs, GDFS_ERRFUNC erfunc, void *obj)

Input	gdfs		File handle
erfunc	Callback function
obj		First argument to hand to callback function

Output	None

Return value	None

Function	Enters the error end callback function.

Usage example	void err_callback(void *obj, Sint32 errcode)
{/Processing */
}

gdFsEntryErrFunc(gf, err_callback, (void *) 0x1234);

| Function | gdFsEntryErrFuncAll | Enter error callback function (All) |
| --- | --- | --- |

Format	void  gdFsEntryErrFuncAll(GDFS_ERRFUNC erfunc, void *obj)

Input	erfunc	Callback function

obj		First argument to hand to callback function

Output	None

Return value	None

Function	Register callback function present when error occurs on all handle

Usage example	void err_callback(void *obj, Sint32 errcode)
{/* Processing */
}

gdFsEntryErrFuncAll(err_callback, (void *) 0x1234);

##### Digital Audio Playback Functions

| Function | gdFsDaPlay | Start track-specified DA playback |
| --- | --- | --- |

Format	Sint32  gdFsDaPlay(Sint32 st_track, Sint32 end_track, Sint32 reptime)

Input	st_track		Start track number
end_track		End track number
reptime		Number of repeats
			(0 - 14: number of repeats, 15: endless loop)

Output	None

Return value	Error codes
GDD_ERR_OK
GDD_ERR_BUSY

Function	Starts DA playback from a specified track.

Usage example	gdFsDaPlay(4, 4, 0);
gdFsDaPlay(4, 4, 0);

/* After a while */

stat = gdFsGetStat(sys);
if (stat == GDD_STAT_COMPLETE) {
  /* Playback is finished */
} else if (stat == GDD_STAT_ERR) {
  /* Error */
}

Note	Single-density track cannot be played back.
To check whether playback is finished, use the system handle.

| Function | gdFsDaPlaySct | Start sector-specified DA playback |
| --- | --- | --- |

Format	Sint32  gdFsDaPlaySct(Sint32 st_sct, Sint32 end_sct, Sint32 reptime)

Input	st_sct		Start sector

end_sct	End sector

reptime	Number of repeats
		(0 - 14: number of repeats, 15: endless loop)

Output	None

Return value	Error codes
GDD_ERR_OK
GDD_ERR_BUSY

Function	Starts DA playback from a specified sector.

Usage example	GDFS sys = gdFsGetSysHn();
gdFsDaPlaySct(0xC000, 0xD000, 2);

/* After a while */

stat = gdFsGetStat(sys);
if (stat == GDD_STAT_COMPLETE) {
  /* Playback is finished */
} else if (stat == GDD_STAT_ERR) {
  /* Error */
}

Note 	Single-density track cannot be played back.
To check whether playback is finished, use the system handle.

| Function | gdFsDaStop | Stop DA playback |
| --- | --- | --- |

Format	Sint32  gdFsDaStop(void)

Input	None

Output	None

Return value	Error codes
GDD_ERR_OK
GDD_ERR_BUSY

Function	Stops DA playback.

Usage example	GDFS sys = gdFsGetSysHn();
gdFsDaStop();
/* After a while */

stat = gdFsGetStat(sys);
if (stat == GDD_STAT_COMPLETE) {
  /* Stop is finished */
} else if (stat == GDD_STAT_ERR) {
  /* Error */
}

Note	To check whether stop is finished, use the system handle.

| Function | gdFsDaPause | Pause DA playback |
| --- | --- | --- |

Format	Sint32  gdFsDaPause(void)

Input	None

Output	None

Return value	Error codes
GDD_ERR_OK
GDD_ERR_BUSY

Function	Pauses DA playback.

Usage example	GDFS sys = gdFsGetSysHn();
gdFsDaPause();
/* After a while */

stat = gdFsGetStat(sys);
if (stat == GDD_STAT_COMPLETE) {
  /* Pause is finished */
} else if (stat == GDD_STAT_ERR) {
  /* Error */
}

Note	To check whether pause is finished, use the system handle.

| Function | gdFsDaRelease | Release DA playback pause |
| --- | --- | --- |

Format	Sint32  gdFsDaRelease(Sint32 reptime)

Input	reptime		Number of repeats

Output	None

Return value	Error codes
GDD_ERR_OK
GDD_ERR_BUSY

Function	Releases DA playback pause

Usage example	GDFS sys = gdFsGetSysHn();
gdFsDaRelease(0);
/* After a while */

stat = gdFsGetStat(sys);
if (stat == GDD_STAT_COMPLETE) {
  /* Release pause is finished */
} else if (stat == GDD_STAT_ERR) {
  /* Error */
}

Remarks	reptime is currently invalid.

Note	To check whether release pause is finished, use the system handle.

| Function | gdFsDaGetInfo | Get DA playback information |
| --- | --- | --- |

Format	Sint32  gdFsDaGetInfo(GDFS_DAINFO *dainfo)

Input	None

Output	dainfo		DA playback information

Return value	Error codes
GDD_ERR_OK
GDD_ERR_BUSY

Function	Get DA playback information

Usage example	GDFS sys = gdFsGetSysHn();
gdFsDaRelease(&dainfo);
/* After a while */

stat = gdFsGetStat(sys);
if (stat == GDD_STAT_COMPLETE) {
  /* OK to get playback information */
} else if (stat == GDD_STAT_ERR) {
  /* Error */
}

Remarks	Be sure to get the system handle and monitor the status.
The information is stored only when it becomes GDD_STAT_COMPLETE.

##### Server function

| Function | gdFsExecServer | Server function |
| --- | --- | --- |

Format	void  gdFsExecServer(void)

Input	None

Output	None

Return value	None

Function	Server function

Usage example	gdFsExecServer();

Remarks	The function is automatically registered in the VblankIn interrupt when the library is initialized.

The main tasks are:

1.	Start the GD server function.
(Perform actual process to drive.)

- Get status.
(At this moment, status of each handle is updated.)

A. Extras

A.1  Description of Error Constants

| Error Constant | Type | Description |
| --- | --- | --- |
| GDD_ERR_OK | N | Normal termination |
| GDD_ERR_INIT | F | Library initialization failed |
| GDD_ERR_RESET | F | Drive initialization failed (chance of recovery) |
| GDD_ERR_LIBOV | F | Overlay initialization failed |
| GDD_ERR_MOUNT | F | Read of PVD or Root Directory failed |
| GDD_ERR_DISC | F | Not appropriate media (when CD-ROM, music CD or wrong GD is inserted) |
| GDD_ERR_DIRREC | F | Incorrect directory record (no longer occurs) |
| GDD_ERR_CANTOPEN | F | File open failed (no longer occurs) |
| GDD_ERR_NOTFOUND | F | File not found |
| GDD_ERR_NOHNDL | F | No free handle available (no longer occurs) |
| GDD_ERR_ILLHNDL | F | Illegal handle being used |
| GDD_ERR_NOTDIR | F | Something different from directory specified |
| GDD_ERR_DIROVER | F | Number of directory entries exceeded |
| GDD_ERR_BUSY | R | Executing other command |
| GDD_ERR_32ALIGN | F | Address specified which is not 32-byte boundary |
| GDD_ERR_SIZE | F | Incorrect transfer size specified |
| GDD_ERR_SEEK | F | Incorrect seek mode specified |
| GDD_ERR_OFS | F | Incorrect position specified |
| GDD_ERR_ILLTMODE | F | Incorrect transfer mode specified |
| GDD_ERR_READ | R | Read failure |
| GDD_ERR_NOTREAD | F | Read not in progress |
| GDD_ERR_TOUT | F | No respond from the drive (for 20 sec.) |
| GDD_ERR_EOF | F | End of file detected (no longer occurs) |
| GDD_ERR_TRAYOPEND | FC | Tray is open |
| GDD_ERR_SIZEOVER | F | Incorrect transfer size specified |
| GDD_ERR_FATAL | F | Fatal error (no longer occurs) |
| GDD_ERR_UNDEF | F | Undefined error (no longer occurs) |

| GDD_ERR_NOERR | NCD | No error to be reported |
| --- | --- | --- |
| GDD_ERR_RECOVER | NCD | Recovery from error detected (Basically, can be recovered to normal) |
| GDD_ERR_NOTREADY | FRCD | Occurs when checking media(R) or when media is not inserted(F) |
| GDD_ERR_MEDIA | FCD | Retry is disable or unrecoverable error occurred Common cause is scratch on the media (chance of recovery) |
| GDD_ERR_HWARE | FCD | Not able to execute normally due to hardware degradation and such like (chance of recovery) |
| GDD_ERR_ILLREQ | FCD | Illegal command issued or drive detects unexpected situation (chance of recovery) |
| GDD_ERR_UNITATTENT | FCD | Change of media detected |
| GDD_ERR_PROTECT | FCD | Detect the problem unable to transact due to stain and such like |
| GDD_ERR_ABORT | FCD | Abortion detected |
| GDD_ERR_NOREADABLE | FC | Not readable media |
| GDD_ERR_CHECKBUSY | RC | Busy status due to checking media (Basically GDD_ERR_UNITATTENT occurs after this error) |

Note : Error Type

N…normal, R…recoverable, C…error callback occurred, D…issued from the drive

(Basically error callback occurs when command to access the drive is issued.)