## PCM Stream Playback Precautions

### Outline

This document contains precautions to be observed when using the SEGA libraries to implement PCM stream playback. These precautions partially augment those given in the Sound Manual. The entire contents of the Sound Manual must be understood before doing developing work.

1.	Data Transfer Timing

2.	Data Buffer Characteristics

3.	Memory Block Release

### 1.	Data Transfer Timing (Precautions for PCM Stream Data Transfer)

PCM stream playback will be programmed in such a way that a suitable amount of data is transferred at each V-int. Normally, the initial data are transferred before playback start, and then additional data are transferred at each V- int. However, it is important to remember the fact that, as shown in the illustration below, V-int transfer data processing may start to run before the initial data playback. This occurs when sound start is slightly delayed in relation to V-int. The sound playback start request is issued using the sdSysServer function or sdSysFlushHostCmd function. These functions send a command request to the sound driver. The command is interpreted by the ARM-CPU which is much slower than the SH-CPU. Because some time is required for interpreting the command, the next waveform data transfer may begin before sound playback starts. As explained also in the reference for sound functions, waveform data before the start of playback are transferred not from the transfer position of the previous data but from the start of the ring buffer.

To prevent this problem, measures should be taken to check the playback condition before data transfer by V-int. Use the SdPstmGetStat function for this purpose, and reference the m_Flg of the SDS_PSTM_CH_STAT structure member.

V-int           V-int           V-int

Application

Initial      V-int           V-int           V-int

data transfer  data transfer   data transfer     data transfer

Sound library                    Problem occurs when playback starts here

Playback

command received

Problem does not occur when playback starts here

### 2.	Data Buffer Characteristics

This section lists precautions regarding data transfer to the ring buffer for playback.

･	Before starting playback of the PCM stream, sizes bigger than 0700H cannot be specified.

･	Before starting playback of the PCM stream, calling this API multiple times will result only in overwriting the start of the ring buffer, not in progressive updating.

･	As shown in the illustration, when the remaining buffer value is smaller than the amount of transfer data, the result of sdPstmIsTransferWaveData will always be FALSE. The remaining data will not be automatically written to the start of the buffer.

Ring buffer (Example A000h)

As shown at left, when the transfer data do not fit fully into the buffer, the remaining data must be given special treatment. In the example, the remaining buffer size is 1000h, therefore 3000h data cannot be sent. Only 1000h data should be sent, and then the transfer target pointer should be reset to the start of the buffer. Data can then be sent again in 3000h chunks.

3000h data

3000h data

3000h data

### 3.	Memory Block Release

When memory has been reserved for data transfer, the following sequence will lead to operation errors and should be avoided.

Data transfer

Reserve memory block (1 channel)

sdMemBlkCreate

Set left channel parameters

sdMemBlkSetPrm

Transfer left channel data

sdPstmTransferWaveData

Set right channel parameters

sdMemBlkSetPrm

Transfer right channel data

sdPstmTransferWaveData

Release memory block

sdMemBlkDestroy

end

The procedure at left shows PCM playback of stereo data. Although the procedure is basically correct, the left channel sound will not be reproduced properly. This is because the memory block is only reserved for one channel, using the memory data area for transfer of both the left and right channel sound. To prevent this problem, reserve a memory block separately for the left and right channel. This will allow normal playback.

Details

Because the sound library operation reference is V-int, setting data for transfer with a function will not result in actual data transfer before the next V-int. This causes the above described problem. When the function is an immediate return function, V-int must be taken into consideration.