KATANA Middleware Library

Specifications

6/30/98	Ver. 0.50

8/03/98	Ver. 0.55

10/02/98	Ver. 1.00

10/15/98	Ver. 1.01

10/29/98               Ver. 1.03

12/08/98               Ver. 1.10

SEGA Enterprises, Ltd.

Revision History

| Date | Version | Description of Changes |
| --- | --- | --- |
| 6/30/98 | 0.50 | Original document |
| 8/03/98 | 0.55 | Modified description mistake. (About the argument of mwPlyStartFname) |
| 10/02/98 | 1.00 | Corrected above error again (parameter of mwPlyStartFname and mwPlyGetStat) Added Wave playback function specification and sample. |
| 10/15/98 | 1.01 | Corrected above error. Added Wave data specification. Added following global functions: mwPlySetOutVol,mwPlyGetOutVol,mwPlySetOutPan, mwPlyGetOutPan,mwPlyPause |
| 10/29/98 12/08/98 | 1.03 1.10 | Modified description mistake. Modified description mistake. |

Contents

1.	Overview	1

1.1	Purpose	1

1.2	Modular Structure	1

2.	Middleware Library Internal Structure	2

3.	Middleware Usage Methods	3

3.1	Initializing the middleware library	3

3.2	How to use the middleware library	3

3.3	Middleware playback handle	3

3.4	Operational states of the middleware playback handle	4

3.5	Playback of MPEG Sofdec	5

3.6	Playback of Wave file	6

4.	Data Specifications	7

4.1	Constants	8

4.2	Data Types	9

5.	Function Specifications	10

5.1	Functions for MPEG Sofdec	11

5.2	Functions for Wave	16

5.3	Global functions	18

# Overview

## Purpose

This library is intended to provide simple play of motion pictures and sound. Files of the following formats can be played back as data stream on the GD-ROM.

- Sofdec data file

This data is created by compressing video using MPEG/Video and audio by Sofdec/Audio and multiplexing them.

- WAVE data file

This is the uncompressed PCM sound data in WAVE format and ADPCM-compressed sound data

## Modular Structure

The modular structure of the Middleware Library is shown in the following diagram.

| Application |  |
| --- | --- |
| Shinobi Library | Middleware Library |
| Hardware |  |

Fig. 1.1  Modular Structure

# Middleware Library Internal Structure

The Middleware Library consists of the following modules. Applications can easily play motion pictures and sounds using the Middleware API.

Table 2.1  Middleware Library Internal Modules

| Module Name | Description |
| --- | --- |
| Module Manager | Assigns decoder CPU time |
| Stream Controller | Loads interleaved video and sound data. |
| Stream Spriter | Separates interleaved motion pictures and sound data, and sends to decoder |
| Video Decoder | Decompresses motion picture data and sends to video renderer |
| Audio Decoder | Decompresses sound data and sends to audio renderer |
| Video Renderer | Outputs decoded moving picture data |
| Audio Renderer | Outputs decoded audio data |

The following figure shows the internal structure of the Middleware Library.

Fig. 2.2  Middleware Library Internal Module

# Middleware Usage Methods

## Initializing the middleware library

To initialize the middleware library, the initialization function for each codec must be called.

<In the case of Sofdec>

mwPlyPreInitSofdec();

sbInitSystem(NJD_RESOLUTION_640x480_NTSCI, NJD_FRAMEBUFFER_MODE_ARGB8888, 1);

mwPlyInitSofdec(NULL);

<In the case of WAV>

sbInitSystem(NJD_RESOLUTION_640x480_NTSCI, NJD_FRAMEBUFFER_MODE_ARGB8888, 1);

mwPlyInitWav(MWD_PLY_MN_EXEC_VSYNC);

## How to use the middleware library

To update the internal status of the middleware library, call the main routine server function (mwPlyExecServer) before calling the njWaitVSync function. Also, because the start of the game frame is notified to the middleware library, the mwPlyStartFrame function must be called immediately after the njWaitVSync function.

whlie (1) {

njWaitVSync();

mwStartFrame();	/* Notify start of game frame */

:

(Game progressing)

:

mwExecServer();	/* Decode processing, etc. */

}

## Middleware playback handle

The audio and video data as compressed by each codec can be played back with the use of the middleware playback handle. First, you need to generate the middleware playback handle for each codec.This can be done by using the appropriate API for the codec.

MWPLY plysfd, plywav;

plysfd = mwPlyCrateSofdec(&cprm);

/* Middleware playback handle for Sofdec data playback */

plywav = mwPlyCrateWav(&cprm);

/* Middleware playback handle for WAVE data playback */

While different API is required for different handle, playback instruction can be done using the same API.

mwPlyStarFname(plysfd, “SAMPLE.SFD”);	/* Playback of Sofdec data */

mwPlyStarFname(plywav, “SAMPLE.WAV”);	/* Playback of WAVE data */

The mwPlyStart and mwPlyStop functions are used to control playback start and stop of the video and audio data.

## Operational states of the middleware playback handle

The operational states of a middleware playback handle is listed below. Immediately after a middleware playback handle is created, it is in the STOP state. When playback starts, the state changes in this sequence: PREP -> PLAYING -> PLAYEND.

Table 3.2  Handle Status

| Status | Description |
| --- | --- |
| STOP | Play is stopped |
| PREP | Preparing to play |
| PLAYING | Playing in progress |
| PLAYEND | Finished playing |
| ERROR | An error has occurred |

The changes of state are shown in the following figure.

Fig. 3.3  State Diagram

## Playback of MPEG Sofdec

The following is a sample program for playing back MPEG Sofdec data.

<Set up interrupt stack, etc.>

/* Application main function */

void main(void)

{

MWPLY	ply;	/* Middleware playback handle */

MWE_PLY_STAT	stat;	/* Handle status */

MWS_PLY_CPRM_SFD	cprm;	/* Handle creation parameter structure */

mwPlyPreInitSofdec();	/* Set up interrupt stack, etc. */

sbInitSystem(NJD_RESOLUTION_640x480_NTSCI, 
					NJD_FRAMEBUFFER_MODE_ARGB8888, 1);

/*

*  Screen and sound initialization

*/

mwPlyInitSofdec(NULL);			/* Library initialization */

cprm.ftype  = MWD_PLY_FTYPE_SFD;

cprm.dtype  = MWD_PLY_DTYPE_AUTO;

cprm.work   = syMalloc(MWD_SFD_SIZE_WORK);

cprm.wksize = MWD_SFD_SIZE_WORK;

ply = mwPlyCreateSofdec(&cprm);		/* Handle creation */

mwPlyStartFname(ply, “SAMPLE.SFD”);	/* Start playing */

for (;;) {

njWaitVSync();				/* Wait for V-Sync */

mwPlyStartFrame();			/* Notify start of game frame */

stat = mwPlyGetStat(ply);		/* Get handle status */

if ( stat == MWE_PLY_STAT_PLAYEND ) {

break;

}

mwPlyExecServer();			/* Run middleware library */

}

mwPlyStop(ply);				/* Stop playback */

mwPlyDestroy(ply);				/* Destroy handle */

syFree(cprm.work);

mwPlyFinishSofdec();				/* Library initialization */

}

## Playback of Wave file

The following is a sample program for playing back Wave file.

<Set up interrupt stack, etc.>

/* Application main function */

void main(void)

{

MWPLY	ply;	/* Middleware playback handle */

MWE_PLY_STAT	stat;	/* Handle status */

MWS_PLY_CPRM_WAV	cprm;	/* Handle creation parameter structure */

/*

*  Screen and sound initialization

*/

mwPlyInitWav(MWD_PLY_MN_EXEC_VSYNC);	/* Library initialization */

cprm.sjbuf = syMalloc(MWD_PLY_ALLOC_SJBUF(48));

cprm.size  = MWD_PLY_ALLOC_SJBUF(48);

ply = mwPlyCreateWav(&cprm);		/* Handle creation */

mwPlyStartFname(ply, “SAMPLE.WAV”);	/* Start playing */

for (;;) {

njWaitVSync();				/* Wait for V-Sync */

mwPlyStartFrame();			/* Notify start of game frame */

stat = mwPlyGetStat(ply);		/* Get handle status */

if ( stat == MWE_PLY_STAT_PLAYEND ) {

break;

}

mwPlyExecServer();			/* Run middleware library */

}

mwPlyStop(ply);				/* Stop playback */

mwPlyDestroy(ply);				/* Destroy handle */

syFree(cprm.sjbuf);

mwPlyFinishWav();				/* Library initialization */

}

# Data Specifications

The Library data list is shown below.

Table 4.3  Data List

| Data Name |  | Function | No. |
| --- | --- | --- | --- |
| Constants |  |  |  |
|  | MWE_PLY_STAT_~ | Handle Status | 1.1 |
|  | MWE_PLY_FTYPE~ | Type of file to play | 1.2 |
|  | MWE_PLY_DTYPTE~ | Display type of video | 1.3 |
| Data Types |  |  |  |
|  | MWPLY | Middleware playback handle | 2.1 |
|  | MWS_PLY_CPRM_SFD | Structure of parameter of handle creation (for MPEG Sofdec) | 2.2 |
|  | MWS_PLY_CPRM_WAV | Structure of parameter of handle creation (for WAVE) | 2.3 |

## Constants

| Title Data | Data Name MWE_PLY_STAT_~ | Data Handle status | No 1.1 |
| --- | --- | --- | --- |

The following constants indicate handle status.

| Constant Name | Description |
| --- | --- |
| MWE_PLY_STAT_STOP | Stopping |
| MWE_PLY_STAT_PREP | Preparing |
| MWE_PLY_STAT_PLAYING | Playing |
| MWE_PLY_STAT_PLAYEND | Finishing playing |
| MWE_PLY_STAT_ERROR | Error state |

| Title Data | Data Name MWE_PLY_FTYPE~ | Data Type of file to play | No 1.2 |
| --- | --- | --- | --- |

The following constants indicate handle status.

| Constant Name | Description |
| --- | --- |
| MWE_PLY_FTYPE_SFD | MPEG Sofdec (Sound + Picture) |
| MWE_PLY_FTYPE_MPV | MPEG Sofdec (Picture only) |

| Title Data | Data Name MWE_PLY_DTYPE～ | Data Display type of video | No 1.3 |
| --- | --- | --- | --- |

The following constants indicate the state of the handle.

| Constant Name | Description |
| --- | --- |
| MWE_PLY_DTYPE_AUTO | Display size of screen is automatically decided. |
| MWE_PLY_DTYPE_FULL | Display is adjusted to fill the full screen. |

## Data Types

| Title Data | Data Name MWPLY | Data Middleware playback handle | No 2.1 |
| --- | --- | --- | --- |

Handle to control playing of motion pictures and sound

| Title Data | Data Name MWS_PLY_CPRM_SFD | Data Handle creation parameter structure | No 2.2 |
| --- | --- | --- | --- |

Parameter structure to set when creating a Middleware playback handle. The members are as follows.

| Member | Type | Description |
| --- | --- | --- |
| ftype | Sint32 | Type of file to play Select from MWE_PLY_FTYPE_~ (*1) |
| dtype | Sint32 | Display type of video. Can be selected from MWE_PLY_DTYPE_~ |
| work | Uint8* | Pointer to the buffer for use |
| wksize | Sint32 | Secured buffer size |

*1: With Ver. 1.10, only MWE_PLY_FTYPE_SFD can be selected.

| Title Data | Data Name MWS_PLY_CPRM_WAV | Data Handle creation parameter structure | No 2.3 |
| --- | --- | --- | --- |

Parameter structure to set when creating a Middleware playback handle. The members are as follows.

| Member | Type | Description |
| --- | --- | --- |
| sjbuf | Uint8* | Pointer to the buffer for use |
| size | Sint32 | Secured buffer size |

# Function Specifications

The library functions are listed below.

Table 5.4  Function List

| Function Name |  | Function | No. |
| --- | --- | --- | --- |
| Functions for MPEG Sofdec |  |  |  |
|  | mwPlyPreInitSofdec | Set up system initialization | 1.1 |
|  | mwPlyInitSofdec | Initialize the Library | 1.2 |
|  | mwPlyFinishSofdec | Finish Library processing | 1.3 |
|  | mwPlyCreateSofdec | Create a handle | 1.4 |
|  | mwPlySetDispPos | Set up the display position | 1.5 |
|  | mwPlySetDispSize | Set up the display size | 1.6 |
|  | mwPlySetBright | Set up the brightness | 1.7 |
|  | mwPlyGetBright | Get the brightness | 1.8 |
|  | mwPlySetBrightOfst | Set the brightness offset | 1.9 |
|  | mwPlyGetBrightOfst | Get the brightness offset | 1.10 |
| Functions for Wave |  |  |  |
|  | mwPlyInitWav | Initialize the Library | 2.1 |
|  | mwPlyFinishWav | Initialize the Library | 2.2 |
|  | mwPlyCreateWav | Create a handle | 2.3 |
| Global functions |  |  |  |
|  | mwPlyDestroy | Destroy a handle | 3.1 |
|  | mwPlyStartFname | Start playing | 3.2 |
|  | mwPlyStop | Stop playing | 3.3 |
|  | mwPlyGetStat | Get the handle status | 3.4 |
|  | mwPlyGetTime | Get the number of the playing sample | 3.5 |
|  | mwPlyStartFrame | Notify start of game frame | 3.6 |
|  | mwWavPause | Perform pause setting | 3.7 |
|  | mwWavSetOutVol | Perform volume setting | 3.8 |
|  | mwWavGetOutVol | Get the volume's value | 3.9 |
|  | mwWavSetOutPan | Set up panpot | 3.10 |
|  | mwWavGetOutPan | Get the panpot value | 3.11 |
|  | mwPlyExecServer | Server function | 3.12 |
|  | mwPlyEntryErrFunc | Register a function to call on error | 3.13 |

## Functions for MPEG Sofdec

This is the function required for playback of MPEG Sofdec.

| Title Function | Function Name mwPlyPreInitSofdec | Function Set up system initialization. | No 1.1 |
| --- | --- | --- | --- |

[Syntax]	void mwPlyPreInitSofdec(void);

[Input]	none

[Output]	none

[Rtn Val]	none

[Purpose]	Set up interrupt stack.

| Title Function | Function Name mwPlyInitSofdec | Function Initializes the Sofdec library. | No 1.2 |
| --- | --- | --- | --- |

[Syntax]	void mwPlyInitSofdec(MWS_PLY_INIT_SFD *iprm);

[Input]	iprm:	Initialization parameter (usually set to NULL)

[Output]	none

[Rtn Val]	none

[Purpose]	Initializes the Library

It sets up the various server functions, which fall into the following types:

(1)	Main server
Executed within the main routine when mwPlyExecServer is called.

(2)	V-Sync server
Executed at V-Sync interrupt.

(3)	WaitVsync server
Executed within njWaitVsync during the wait time for the next game frame.
The Sofdec library runs decode in this server.

| Title Function | Function Name mwPlyFinishSofdec | Function Finish the Sofdec Library processing. | No 1.3 |
| --- | --- | --- | --- |

[Syntax]	void mwPlyFinishSofdec(void);

[Input]	none

[Output]	none

[Rtn Val]	none

[Purpose]	Finishes Library processing

| Title Function | Function Name mwPlyCreateSofdec | Function Create a handle. | No 1.4 |
| --- | --- | --- | --- |

[Syntax]	MWPLY mwPlyCreateSofdec(MWS_PLY_CPRM_SFD *cprm);

[Input]	cprm:	handle creation parameter

[Output]	none

[Rtn Val]	Middleware playback handle

[Purpose]	Creates a handle

By specifying dtype of the creation parameter, the display region can be controlled.

<In the case of automatic adjustment>

dtype = MWD_PLY_DTYPE_AUTO: the display region is adjusted automatically as multiple of integer number.

If the size is smaller than 640 x 480, the display region will be reduced by 1 dot. In the case of 320 x 240, it will become 639 x 479.

<In this case of full screen display>

dtype = MWD_PLY_DTYPE_FULL: full screen display

This is different from the case of automatic adjustment; the display region is not reduced by 1 dot but the display will be set at 640 x 480.

| Title Function | Function Name mwPlySetDispPos | Function Set up the display position. | No 1.5 |
| --- | --- | --- | --- |

[Syntax]	void mwPlySetDispPos(MWPLY ply, float lx, float ly);

[Input]	ply:	Middleware playback handle
lx:	X-coordinate
ly:	Y-coordinate

[Output]	none

[Rtn Val]	none

[Purpose]	Set up the display position.

| Title Function | Function Name mwPlySetDispSize | Function Set up the display size. | No 1.6 |
| --- | --- | --- | --- |

[Syntax]	void mwPlySetDispSize(MWPLY ply, float sx, float sy);

[Input]	ply:	Middleware playback handle
sx:	X-coordinate
sy:	Y-coordinate

[Output]	none

[Rtn Val]	none

[Purpose]	Set up the display size.

| Title Function | Function Name mwPlySetBright | Function Set up the brightness. | No 1.7 |
| --- | --- | --- | --- |

[Syntax]	void mwPlySetBright(MWPLY ply, Sint32 val);

[Input]	ply:	Middleware playback handle
val:	Brightness (0~255)

[Output]	none

[Rtn Val]	none

[Purpose]	Set up the brightness. By default, it is set to 192.

| Title Function | Function Name mwPlyGetBright | Function Get the brightness. | No 1.8 |
| --- | --- | --- | --- |

[Syntax]	Sint32 mwPlyGetBright(MWPLY ply);

[Input]	ply:	Middleware playback handle

[Output]	none

[Rtn Val]	Brightness

[Purpose]	Get the brightness.

| Title Function | Function Name mwPlySetBrightOfst | Function Set the brightness offset. | No 1.9 |
| --- | --- | --- | --- |

[Syntax]	void mwPlySetBrightOfst(MWPLY ply, Sint32 val);

[Input]	ply:	Middleware playback handle
val:	Brightness offset (0~255)

[Output]	none

[Rtn Val]	none

[Purpose]	Set the brightness offset. By default, it is set to 0.
You can create the fade-in effect by reducing the value between 255 and 0.
On the other hand, by increasing the value between 0 and 255, you can create the fade-out effect.

| Title Function | Function Name mwPlyGetBrightOfst | Function Get the brightness offset. | No 1.10 |
| --- | --- | --- | --- |

[Syntax]	Sint32 mwPlyGetBrightOfst(MWPLY ply);

[Input]	ply:	Middleware playback handle

[Output]	none

[Rtn Val]	Brightness offset

[Purpose]	Get the brightness offset.

## Functions for Wave

This are the functions required for playback of Wave.

| Title Function | Function Name mwPlyInitWav | Function Initialize the library. | No 2.1 |
| --- | --- | --- | --- |

[Syntax]	void mwPlyInitWav(Sint32 mode);

[Input]	mode:	Server function calling mode:

MWD_PLY_AT_EXEC_VSYNC:	Execute the server function without V-Sync interrupt.

MWD_PLY_MN_EXEC_VSYNC:	The application calls the server function and performs server processing.

[Output]	none

[Rtn Val]	none

[Purpose]	Initialize the library.

| Title Function | Function Name mwPlyFinishWav | Function Process end of library. | No 2.2 |
| --- | --- | --- | --- |

[Syntax]	void mwPlyFinishWav(void);

[Input]	none

[Output]	none

[Rtn Val]	none

[Purpose]	Process end of library.

| Title Function | Function Name mwPlyCreateWav | Function Create handle. | No 2.3 |
| --- | --- | --- | --- |

[Syntax]	MWPLY mwPlyCreateWav(MWS_PLY_CPRM_WAV *cprm);

[Input]	cprm:	Parameter for handle creation

[Output]	none

[Rtn Val]	Middleware playback handle

[Purpose]	Create handle.

## Global functions

These are functions which are not tied to any codec type.

| Title Function | Function Name mwPlyDestroy | Function Destroy handle. | No 3.1 |
| --- | --- | --- | --- |

[Syntax]	void mwPlyDestroy(MWPLY ply);

[Input]	ply:	Middleware playback handle

[Output]	none

[Rtn Val]	none

[Purpose]	Destroys the handle

| Title Function | Function Name mwPlyStartFname | Function Start playing. | No 3.2 |
| --- | --- | --- | --- |

[Syntax]	void mwPlyStartFname(MWPLY ply, Sint8 *fname);

[Input]	ply:	Middleware playback handle

[Output]	none

[Rtn Val]	none

[Purpose]	Starts playing motion pictures and sound

| Title Function | Function Name mwPlyStop | Function Stop playing. | No 3.3 |
| --- | --- | --- | --- |

[Syntax]	void mwPlyStop(MWPLY ply);

[Input]	ply:	Middleware playback handle

[Output]	none

[Rtn Val]	none

[Purpose]	Stops playing motion pictures and sound

| Title Function | Function Name mwPlyGetStat | Function Get the handle status. | No 3.4 |
| --- | --- | --- | --- |

[Syntax]	MWE_PLY_STAT mwPlyGetStat(MWPLY ply);

[Input]	ply:	Middleware playback handle

[Output]	none

[Rtn Val]	Internal handle status

[Purpose]	Gets the internal status of the Middleware playback handle

| Constant Name | Description |
| --- | --- |
| MWE_PLY_STAT_STOP | Stopping |
| MWE_PLY_STAT_PREP | Preparing |
| MWE_PLY_STAT_PLAYING | Playing |
| MWE_PLY_STAT_PLAYEND | Finished Playing |
| MWE_PLY_STAT_ERROR | Error |

| Title Function | Function Name mwPlyGetTime | Function Get the number of the playing sample. | No 3.5 |
| --- | --- | --- | --- |

[Syntax]	void mwPlyGetTime(MWPLY ply, Sint32 *ncount, Sint32 *tscale);

[Input]	ply:	Middleware playback handle

[Output]	ncount:	Playing sample number(time)
tscale:	Sampling frequency (unit of time) (Hz)

[Rtn Val]	none

[Purpose]	Gets the playing time according to the number of the playing sample

[Remarks]	(a)	Actual time (rtime) can be calculated by:
rtime = ncount / tscale;

| Title Function | Function Name mwPlyStartFrame | Function Notify start of game frame. | No 3.6 |
| --- | --- | --- | --- |

[Syntax]	void mwPlyStartFrame(void);

[Input]	none

[Output]	none

[Rtn Val]	none

[Purpose]	Notify that the game frame has started.

| Title Function | Function Name mwPlyPause | Function Perform pause setting. | No 3.7 |
| --- | --- | --- | --- |

[Syntax]	void mwPlyPause (MWPLY ply, Sint32 sw);

[Input]	ply:	Middleware playback handle
sw:	Pause switch (0: continue, 1:pause)

[Output]	none

[Rtn Val]	none

[Purpose]	Perform switching of pause.
* In ver. 1.10, MPEG Sofdec does not support pause.

| Title Function | Function Name mwPlySetOutVol | Function Perform volume setting. | No 3.8 |
| --- | --- | --- | --- |

[Syntax]	void mwPlySetOutVol (MWPLY ply, Sint32 vol);

[Input]	ply:	Middleware playback handle
vol:	Volume (-999~0)

[Output]	none

[Rtn Val]	none

[Purpose]	Set up the volume. By default, it is set to 0.
You can create the fade-in effect by increasing the value between -999 and 0.
On the other hand, by decreasing the value between 0 and -999 , you can create the fade-out effect.

| Title Function | Function Name mwPlyGetOutVol | Function Get the volume's value. | No 3.9 |
| --- | --- | --- | --- |

[Syntax]	Sint32 mwPlyGetOutVol (MWPLY ply);

[Input]	ply:	Middleware playback handle

[Output]	none

[Rtn Val]	Volume

[Purpose]	Get the value of the volume that is currently set.

| Title Function | Function Name mwPlySetOutPan | Function Set up panpot. | No 3.10 |
| --- | --- | --- | --- |

[Syntax]	void mwPlySetOutPan (MWPLY ply, Sint32 chno, Sint32 pan);

[Input]	ply:	Middleware playback handle

chno:	Channel to set
Monaural/stereo (left): MWD_CH_L
Stereo (right): MWD_CH_R

pan:	Panpot (MWD_PAN_LEFT~MWD_PAN_RIGHT)
Auto mode: MWD_PAN_AUTO

[Output]	none

[Rtn Val]	none

[Purpose]	Set up panpot for each output channel.
By default, it is set to the auto mode. In the case of monaural, the value is MWD_PAN_CENTER and for stereo the left is MWD_PAN_LEFT and the right MWD_PAN_RIGHT.

| Title Function | Function Name mwPlyGetOutPan | Function Get the panpot value. | No 3.11 |
| --- | --- | --- | --- |

[Syntax]	Sint32 mwPlyGetOutPan (MWPLY ply, Sint32 chno);

[Input]	ply:	Middleware playback handle

chno:	Channel to get
Monaural/stereo (left): MWD_CH_L
Stereo (right): MWD_CH_R

[Output]	none

[Rtn Val]	Panpot value of the supported channel

[Purpose]	Get the value of the panpot which is currently set.

| Title Function | Function Name mwPlyExecServer | Function Server function. | No 3.12 |
| --- | --- | --- | --- |

[Syntax]	void mwPlyExecServer(void);

[Input]	none

[Output]	none

[Rtn Val]	none

[Purpose]	Refreshes the internal status of the handle.

[Remarks]	Update the internal state of handle.

| Title Function | Function Name mwPlyEntryErrFunc | Function Register a function to call on error | No 3.13 |
| --- | --- | --- | --- |

[Syntax]	void mwPlyEntryErrFunc (MW_PLY_ERRFN errfn, void *obj);

[Input]	errfn:	Error function
obj:	Error object

[Output]

[Rtn Val]	none

[Purpose]	Registers a function to call on error.

[Remarks]	The registered error function will be called when an error occurs.
A text string is passed as the 2nd parameter of the error function.By using the text string, you can check the content of the error.

/* User error function */

void user_error(void *user_obj, char *msg)

{

for (;;) {

njPrintC(NJM_LOCATION(3, 3), msg);

}

}

void main(void)

{

mwPlyEntryErrFunc(user_error, user_obj);

}