98/08/28

Shinobi Library Additional Reference

Sega Enterprises, Inc.

# Introduction

This document supplies additional reference information regarding the various library functions. Please note that items described in this document may change in future.

# Shinobi Environment CPU Mode

In the Shinobi environment, the SH7091 operates in special privileged mode. During initialization (AIP), SR.MD=1 is set. Changing this bit is prohibited.

# Bank Register Use in the Shinobi Environment

## Introduction

This section describes use of the bank registers when writing user programs partially in Assembler.

## General Registers

1.	Initialization

･	SR.MD=1, SR.RB=1 are set. (These bits may not be changed by the user.)

･	R0 - R7 correspond to R0_BANK1 - R7_BANK1.

･	Bank registers correspond to R0_BANK0 - R7_BANK0.

2.	Hidden Registers

･	R1_BANK0 - R7_BANK0 are not used by the library.

3.	Interrupt Library Store Registers

･	Except for the following registers, all registers are stored:
Hidden registers BANK_R0 - BANK0, DBR, SGR, PC, R15

4.	Bank Register Use

R0_BANK0 - R7_BANK0 are available to the user.

## Floating Point Registers

1.	Initialization

･	fpscr = 0x00040001 (FPSCR.FR = 0, FPSCR.SZ = 0) causes initialization.

･	FR0 - FR15 correspond to FPR0_BANK0 - FPR15_BANK0.

･	The data size of the FMOV instruction is 32 bits.

2.	Library Use

･	The bank floating point register (fprn_bank1) is used as current matrix for Ninja. It may not be destroyed by the user.

･	When calling the library, fpscr.sz and fpscr.fr must be = 0.

3.	Interrupt library store register

･	All floating point registers and fpsrc, fpul are stored. (Reference: General Registers, 3. Interrupt Library (syInt) Store Registers)

4.	Bank Register Use

･	When FPR_BANK1 is used by the user, its contents must be stored before use and restored before calling the library.

## GBR Registers

･	This library does not use the GRB registers.

･	The registers are available to the user.

## Others

･	For details regarding these registers, please refer to the SH7091 Programming Manual.

# Cache Operation

## User Cache Operations

For the following operations, the library user must currently operate the cache of the CPU (SH7091) explicitly, using the cache library.

(1)	Invalidating the instruction cache (ICache) before executing a program (read-in portion) that was read from GD.

Reason: The GD library assures coherency of the operand cache but it cannot distinguish between regular data and program data. Because global ICache invalidation is not appropriate, the user must cover this part.

(2)	Texture area write-back for texture loading immediately after rewriting the texture area in main memory. This is not necessary when reading the texture from file.

Reason: Currently, the njLoadTexture functions do not perform overall write-back for the cache, in order to improve performance.

## Library Cache Operations

The library functions perform cache operations in the following cases.

(1)	Initialization

The entire cache is purged (invalidated after write-back).

Applicable function: syHwInit

(2)	The operand cache (OCache) is invalidated before file read and transfer for the applicable area.

Applicable functions: gdFsread, gdFsTrans32

(3)	Write-back of applicable area after loading a texture from file

# Memory Map

| 8C00:0000 (64K) | BootROM used area Interrupt vector default stack (13 KB) * For details, see separate map |  |  |
| --- | --- | --- | --- |
| 8C01:0000 | Program + data area * For details, see separate document (used sections) |  | ･ Divided into user program and library sections |
| (16M-64K) | Heap area |  | ･ Used as : after program BSG32 section Uses syMalloc and standard library malloc Heap range is specified with syMallocInit |
| 8D00:0000 |  |  |  |

# 8C00:0000 - 8C01:0000 Memory Map

･	Because a part (8C00:C000) of the initialization code area may be destroyed after execution, the default stack is 13K (8c00:c000 - 8c00:f400). This area is filled with "SEGASEGA..." by the startup routine.

| 8C00:0000 | BootROM (KABUTO) dedicated area |  |
| --- | --- | --- |
| 8C00:C000 | Initialization code |  |
| 8C00:E000 | AIP (application initialization program) |  |
|  | Stack area |  |
| 8C00:F400 | Interrupt vector (for applications) (2K) |  |
| 8C00:FC00 | Library management table (1K) |  |
| 8c01:0000 | Application program : |  |

# Usage Sections

･	Section names are listed in order of arrangement.

･	Section names used by the library consist of P (code), C (constant), D (data), B (BSS), followed by SG. 32 is added to 32-byte aligned sections.

･	Shaded parts are used by the library.

| # | Section name | Purpose | Alignment size | Remarks |
| --- | --- | --- | --- | --- |
| 1 | DSGLH | ･ Jump to : start of application ･ Library handle *1 area | 4 | *1 Library overlay information |
| 2 | DSGLE | Library handle end | 4 |  |
| 3 | P | User *2 program area | 4 | *2 Including standard library. Same below. |
| 4 | C | User constant area | 4 |  |
| 5 | C32 |  | 32 |  |
| 6 | D | User initialized data area | 4 |  |
| 7 | D32 |  | 32 |  |
| 8 | D_INIT_ | Global constructor area | 4 | For C++ (not implemented) |
| 9 | D_END_ | Global destructor area | 4 | For C++ (not implemented) |
| 10 | R | (Not used) | 4 |  |
| 11 | R32 |  | 32 |  |
| 12 | PSG | Library program area (excluding Ninja) | 4 |  |
| 13 | Pending |  | 4 |  |
| 14 | CSG | Library definition area | 4 |  |
| 15 | CSG32 |  | 32 |  |
| 16 | DSG | Library initialized data area | 4 |  |
| 17 | DSG32 |  | 32 |  |
| 18 | RSG | (Not used) | 4 |  |
| 19 | RSG32 |  | 32 |  |
| 20 | B | User non-initialized data area | 4 | Cleared to 0 until BSG32 section at startup |
| 21 | B32 |  | 32 |  |
| 22 | BSG | Library non-initialized data area | 4 |  |
| 23 | BSG32 |  | 32 | Heap starts after this name section |

## Revision History

1998/08/27    Document creation