#### Draw Buffer Organization

When creating applications in the SEGA Library environment, the draw model using SH4 and Holly must be understood properly. This document explains the procedure and timing used for drawing.

Sega Enterprises, Inc.

# Draw Data Flow

When drawing a model,the SEGA Library uses a buffer and proceeds according to the following rules.

SH4

Holly

Native buffer 1

Native buffer 2

Frame buffer 1

Frame buffer 2

Texture area

[1]

[1]

[2]

[2]

③

[3]

Semi-transparent

Semi-transparent modifier

Punch-through

Opaque

Main RAM

16M Byte

Texture RAM

8M Byte

Opaque

Vertex processing work buffer

Vertex data

Registration buffer

Fig. 1  Draw Model

As illustrated in Fig. 1, drawing proceeds in 3 steps.

[1] Vertex registration

[2] DMA transfer

[3] Rendering。

Vertex registration is considered complete with the njWaitVSync() function.

# Main RAM Usage Status

For drawing an image, two buffers are required in main RAM, the vertex processing work buffer and the vertex data registration buffer.

## Vertex processing work buffer

This is a temporary buffer which is created with the njInit3D() function.

## Vertex data registration buffer

The vertex data registration buffer consists of the following five buffers:

Opaque modifier vertex buffer

	Semi-transparent polygon vertex buffer

	Semi-transparent modifier vertex buffer

	Punch-through vertex buffer

Punch-through modifiers are stored in the opaque modifier buffer.

The buffer size is set with the njInitVertexBuffer() function.

8Mbyte

Texture buffer

Native buffer 1

Native buffer 2

Frame buffer 1

Frame buffer 2

Texture RAM

Total size of the texture RAM is 8 MByte. It consists of the following three buffers:

	Frame buffer

	Texture buffer

	Native buffer

Because the frame buffer and native buffer are double buffers, two each are required.

The njCalcTexture() function can be used to check the remaining amount of texture RAM.

- Fig. 2  Texture RAM

## Frame buffer

The amount of texture memory occupied by the frame buffer depends on the draw mode and the buffer mode.

When the sbInitSystem() function is executed to set the frame buffer mode, the following sizes are reserved as texture RAM.

| Frame Mode | RGB565,RGB555 ARGB1555(16Bit) | RGB888 (24Bit) | ARGB8888 (32Bit) |
| --- | --- | --- | --- |
| VGA | 0x12C000 | 0x1C2000 | 0x258000 |
| 320X240_NTSCNI | 0x04B000 | 0x070800 | 0x096000 |
| 320X240_NTSCI | 0x04B000 | 0x070800 | 0x096000 |
| 640X240_NTSCNI | 0x096000 | 0x0E1000 | 0x12C000 |
| 640X240_NTSCI | 0x096000 | 0x0E1000 | 0x12C000 |
| 320X480_NTSCNI | 0x096000 | 0x0E1000 | 0x12C000 |
| 320X480_NTSCI | 0x096000 | 0x0E1000 | 0x12C000 |
| 640X480_NTSCNI | 0x12C000 | 0x1C2000 | 0x258000 |
| 640X480_NTSCI | 0x12C000 | 0x1C2000 | 0x258000 |
| 640X480_NTSCNI_FF | 0x096000 | 0x0E1000 | 0x12C000 |

* The figures apply to two frame buffers.

Table 1  Size Determined by Draw Mode and Frame Buffer Mode

## Texture buffer

The amount of texture RAM that can be used for textures can be set by the njSetTextureMemorySize() function.

## Native buffer

The native buffer serves for registering draw commands for the frame buffer. The size allocated to the native buffer is 8 MByte minus the texture buffer and frame buffer size. However, the maximum size for the native buffer is 4 MByte.

# Latency

The SEGA library environment supports two drawing modes called "3V Latency" and "2V Latency". The settings for these modes are made with the njInitVertexBuffer() function.

## 3V Latency

3V Latency is a simple drawing mode which performs vertex registration, DMA transfer, rendering, and display.

Peripheral

Get 1

Vertex registration 1

DMA transfer 1

Rendering 1

Display 1

Peripheral

Get 2

Vertex registration 2

DMA transfer 2

Rendering 2

Display 2

V

V

V

V

V

V

- 図3.　3Ｖ Latency パイプライン

Fig. 3  3V Latency Pipeline

For this mode, a double buffer for registering vertex data is required. The principle of 3V Latency is shown in Fig. 4

Vertex data registration buffer

SH4

HOLLY

Vertex processing work buffer

Fig. 4  3V Latency Model

## 2V Latency

2V Latency is a drawing mode which performs vertex registration + DMA transfer, rendering, and display.

As opposed to 3V Latency, where all data are first stored in the vertex data registration buffer before DMA transfer, out of the five buffers, the one specified by the user as the buffer with the highest access frequency is used not for storing data but for constantly sending them to Holly. The contents of the other buffers are transferred after vertex registration is complete.

Peripheral

Get 1

Rendering 1

Display 1

Display 2

V

V

V

V

V

V

Fig. 5  2V Latency Pipeline

Vertex data

registration buffer

SH4

Holly

Vertex processing

work buffer

Fig. 6  3V Latency Model

Compared to 3V Latency, 2V Latency requires less than half the memory size because the vertex registration buffer is a single buffer. This allows faster handling of signals from peripherals.

Since opaque buffer access frequency is generally high, user-specified through-buffer access will be more common.

The principle of 2V Latency is shown in Fig. 6.