Quick Steps to burning GD-ROMs

(using CD-Craft)

# Sega Europe

## Preparation

### Connection

Please connect the GD Writer to a spare SCSI port of your PC, and refresh your SCSI bus. The rotary switch at the back of the unit can modify the SCSI ID of the burner if a clash occurs.

### Testing

Make sure that you have a working and running emulation of your project first on your development kits. This involves creating a 1ST_READ.BIN file from your project (or converting your .elf using elf2bin converter) and having a correct IP.BIN (can be generated from the shinobi/njsamp/etc/ip directory). The GDROM burning process is slow – so time spent testing at this stage will save you a lot of time later.

### Files

Make sure you have the following directories and files on the PC connected to the burner (the actual filenames can be different).

GDROOT	the source directory for the high-density area containing the directory structure and all game files including 1st_read.bin executable for your product.

SINGLE	the source directory for single-density area. No game files in here. ABSTRACT.txt, BIBLIOGR.txt and COPYRIGH.txt go here plus any additional “goodies” you may wish to include.

GDDA    	any number of *.da (raw redbook) files in the order that they should appear on the GDROM (starting at track 4).

Warning.da 	for track 2 audio in single density session.

IP0000.BIN	system file for single density area

IP.BIN 		system file for high density area

## Using CD-Craft

1) Run CD-Craft from the SDK.

2) Select File / New

3) Select Script file type.

4) Give the script a name and click "browse" to choose a location for it (must NOT be in any of the directories listed above)

5) Turn OFF “define a dummy script for single density area” option.

6) Click set-up for low-density area

7) Use the browser to point to the SINGLE file directory (as described above) and select add.

8) Select the CD-DA tab, select "use CDDA track" and browse for warning.da.

9) Select the System are set-up tab, select "use system area" and browse for IP0000.BIN

10) Click OK

11) Repeat steps 6) to 10) for the High Density area, this time adding your GDROOT directory to source files, your GDDA directory to CDDA, and IP.BIN for system area. MAKE SURE "Place file under source file directories" is OFF.

12) Select CREATE IMAGE FILE and click GO. Image file (.cim) will be created

13) Select Write Master Disk and click the “settings” button (the CD, just to the right of the mode box)

14) Turn write test OFF

15) Click image file tab and Browse to select your new CIM file.

16) Click “OK”

17) Click “GO” with GD-ROM in burner. Make sure that any resident programs are closed first that may interrupt the burning process on your PC. GDROM Burning takes around an hour.