READ ME for the macro assembler/linker user		01.13,1998

06.03,1997

01.27,1997

09.17,1996

08.01,1995

01.23,1995

11.25,1994

09.22,1994

08.23,1994

05.16,1994

03.14,1994

02.01,1994

10.20,1993

10.06,1993

Present assembler name	: M86K.EXE

Old assembler name	: M86xxxx.EXE

Present linkage loader name	: L86K.EXE

Old linkage loader name	: L86xxxx.EXE

Reserved word	: Words for operating assembler.

See "RESERVED WORD" in this document.

ex. ACC, NOP, RADIX, CHIP

Latest version number	:	Ver. 4.0K

Latest RWD file version	:	Ver. 4.09C

## Environment for M86K

M86K requires setting the environment variable as below.

M86KRSVDFILE	specifies our micros, register names and others.

CHIPNAME	specifies terget micro. (definable in the source

file)

ex.	SET M86KRSVDFILE=A:\EXE\M86KRSVD.RWD

SET CHIPNAME=LC86xxxx

M86KRSVD.RWD file is required with M86K. It should be mount on the direc-

tory with the M86K.

When M86K.EXE and the M86KRSVD.RWD is in the same directory, you need not

set the environment variable, M86KRSVDFILE.

## Main memory space

M86K requires 256Kbytes or more on main memory.

## Assemble

M86K assembles the source file(.ASM) to the object file which has source

file name. The extend is '.OBJ'.

M86K requires five parameters;

1st parameter	the source file name

2nd parameter	the object file name (default: the source file name)

3rd parameter	the listing file name (default: null)

4th parameter	the cross reference file name (default: null)

5th parameter	the error file name (default: null)

## Error file

You can get the error file. Its extend is '.ERR'.

See 'Assemble'.

## Error message

You can use the TAG JUMP function.

Use the error file.

## Symbol table

If you need the symbol table, then make the cross reference file.

## Free format

You can write down the instruction from leftmost position of the line.

ex.	AAA:	INC ACC

NOP

## Labels

Labels needs the column ':' after the strings.

ex.	LABEL_1:	inc ACC

## Character data

The character data must be use with two double quotations as below.

ex.	DC   "ABCD"	--> 41424344

DC   "\""	--> 22

DC   "\\"	--> 5C

## Symbol length

255 characters maximum.

## About END pseudo instruction

You can neglect the END description.

When neglecting END, all the source line is assembled.

## About MACRO definition

When the parameter of macros has the expressions, the parameter must be

enclosed by parenthesis () in a macro routine.

ex.        EXAM AA-1

:

EXAM  MACRO PARA

INC   (PARA)

ENDM

## Additional pseudo instructions

CHIP			Set a terget CPU name.    ex.   CHIP LC866032

COMMENT			Output a comment into the object file.

WIDTH			Set the width of the listing file.

BANK			Specify the bank number of DSEG.

BRO,JMPO,CALLO		Optimized branch, optimized jump and optimized

call       These instructions generate the appro-

bzo,bnzo,beo,bneo	priate codes.

dbnzo,bpco,bpo,bno	Use with lower-case letters ; bzo to bno.

When using the above instructions,BZO etc, with

the upper-case letters, you get an 'undefined

symbol' error.

INBLOCK			Assemble based on the 4Kbyte-boundary.

FREE			Assemble not based on the 4Kbyte-boundary.

REMARKS			Define a comment indicator.

ex.   remarks "comments"

REMARKS_OFF		Cancel the REMARKS instruction.

FREE_ON,FREE_OFF	Assembler keywords.

RADIX			Define the base number.

ex.  radix  016.D      ; based on the hexadecimal

CHANGE			For the LC868000 series only.

WORLD			For the LC868000 series only.

OTHER_SIDE_SYMBOL	For the LC868000 series only.

** When using CHANGE,WORLD,OTHER_SIDE_SYMBOL, see the LC868000 users

manual

## Boundary

M86K generates two types of the object file.

One is generated on the 4Kbyte-boundary. The other is generated out of

the 4Kbyte-boundary.

ex.	cseg	inblock		; Assemble based on the boundary.

cseg	free		; Assemble not based on the boundary.

## About DSEG

When using the ORG instruction in the DSEG, the attribute of segments

is absolute.

## Strings

M86K recognizes the upper-/lower-case letters.

## For generating the external ROM table

When generating the external ROM table, the top of the source line must

be defined the following pseudo instruction.

world external_data

## New switches

-P    Get the memory area to job on the main memory.

If you get the 'no more PARAMETER' error message, try to assemble

with -P switch like as -P8192. Set value more than 4096.

ex.     M86K -P8192 xxxxxx

## Operation between the relocatable values

M86K can operate the relocatable values each other.

## Additional pseudo instruction (1)

You can define the comment indicator except ';' with the REMARKS

instruction. Defined strings must be set the leftmost position of a line.

ex.            remarks  "xxx"    ; Define the indicator

"xxx"     inc    acc  ; This line isnot recognized as a instruction.

remarks_off       ; Cancel the definition.

## Additional pseudo instruction (2)

You can define the base number with the RADIX instruction.

Default value is decimal.

ex.      radix  2    ; binaty

## Numeric description

Binary data      : xxxx.B,   xxxxB(decimal only)

Decimal data     : xxxx.D

Hexadecimal data : xxxx.H,   xxxxH

## Reserved symbols or strings (reserved words)

You cannot specify the words the same as these symbols.

.IF	.LIST	.MACRO	.PRINTX	.XIF	.XLIST	.XMACRO

@R0	@R1	@R2	@R3

ADD	ADDC	AND	ASEG

BANK	BE	BN	BNE	BNZ	BP	BPC	BR	BRF

BRO	BZ	BEO	BNO	BNEO	BNZO	BPO	BPCO

BZO

CALL	CALLF	CALLO	CALLR	CHANGE	CHIP	CLR1	COMMENT	CSEG

DB	DBNZ	DC	DEC	DIV	DS	DSEG	DW	DBNZO

ELSE	END	ENDF	ENDIF	ENDM	EQ	EQU	EXITM	EXTERN

EXTERNAL	EXTERNAL_DATA

FREE	FREE_OFF	FREE_ON	FUNCTION

GE	GT

HIGH

IFB	IFBBR	IFDEF	IFDIF	IFDIFBR	IFE	IFIDN	IFIDNBR	IFNB

IFNBBR	IFNDEF	IFNE	INBLOCK	INC	INCLUDE	INTERNAL

IRP	IRPC

JMP	JMPF	JMPO

LAND	LD	LDC	LDX	LE	LOCAL	LOR	LOW	LT

LXOR

MACRO	MOD	MOV	MUL

NE	NOP	NOT	NOT1

OR	ORG	OTHER_SIDE_SYMBOL

PAGE	POP	PUBLIC	PUSH

RADIX	REMARKS	REMARKS_OFF	REPT1	RET	RETI	ROL	ROLC

ROR	RORC

SET	SET1	SHL	SHR	ST	STX	SUB	SUBC

TITLE

WIDTH	WORLD

XCH	XOR

.. Bugs of the Macro instruction are fixed.          #3.0G

.. The LC864300,LC86F8208 available		     #4.0

.. The LC86F3000 available			     #4.0J

.. Bugs of OBJ file when using the MACRO instruction

in the INCLUDEed file are fixed.		     #4.0J

.. M86K creates the OBJ file on network.	     #4.0K

## Current Problems

1. If the source file(.ASM) includes 10000 lines or more, a malfunctional evaluation file may be generated.

2. If making 64KB-packed program in one file for LC868000, a malfunctional evaluation file is generated.

READ ME for the linkage loader user

Latest version number	:	Ver. 5.00C

Latest LIB file version	:	Ver. 4.09

## Link (1)

Link after preparing  the object file(OBJ), the option file(OPT) and the

character generater data file(CGR).

L86K needs the data file L86K.LIB. LC86K.LIB must be mount the directory

with M86K.

When linking the LC86F8208 objects, LIB8200.OPT file is required in the

directory which includes the L86K.exe.			#4.00A

## Link (2)

L86K links the object files(.OBJ) to the evaluation file which has the

first-linked file name. The extend is '.EVA'.

L86K requires five parameters;

1st parameter	the object file name

2nd parameter	the evaluation file name (default: the first object name)

3rd parameter	the library file name (default: null)

4th parameter	the option file name (default: eva file name)

5th parameter	the CGR file name (default: option file name)

L86K generates the evaluation file called 'EVA file'.

The EVA file includes the debug information and source line information.

## DSEG

The symbols in DSEG is relocatable. In case of using ORG, it becomes absolute.

So, symbols assigned by ORG isnot affected the specified offset address

when linking.

Also, symbols in DSEG is linked in order with the first parameter at link.

New option switch:

-E   Ignore duplicated addressing error in DSEG.	#2.08D

## Overflow checking

L86K doesnot check the overflows of the operand data of the instructions.

If you want checking, use -w switch at link. Then you can get the result

of checking as warnings.

ex.	L86L -w ******

## Optimize

See OPTMZ.ENG.

Use -A switch to optimize. You can use -O,-F,-R switches with -A.

## Mapping file

L86K can generates the mapping file, its file name is the same as the

EVA file. And its extend is '.MAP'.

Use -P switch at linking.

ex.	L86K -P ******

Also, L86K can generates the listing of the local symbols in the mapping

file.

Use -L switch with -P switch at linking.

ex.	L86K -P -L ******

<!> You can make the mapping file even if "Jump address out of range"

error is occured.					#2.07

<!> Re-newed a map file form.				#2.08

<!> Re-newed a map file form for the LC868000 only.	#2.08A

<!> Clear the border of linked object number.		#2.08C

<!> The LC864600/4700/4800/4900 available		#4.04

<!> The LC86F3000 available				#4.05

<!> The LC864A00,LC868500 available			#5.00B

## When ordering

For ordering, you must prepare the intellec HEX formatted data of the program.

After finishing the debug and making a complete program, use the file converter

EVA2HEX.exe to convert the EVA file to the HEX file.

ex.	EVA2HEX filename

## External ROM table

L86K can generates the external ROM table file for the LC868000 series

micros.

See the Software manual and the LC868000 series users manual.

ex.	L86K -B=nn ******

The HEX file named ******.Hnn is generated.

nn means the bank number of the external ROM bank.

01 <= nn <= FF, nn<>00

## New option switch

L86K V2.08 or later is added its option switch '-s'.

'-s' option operates to stop the symbol-sorting when total of symbols

exceeds 8K symbols. If linked objects is included over 8K symbols, L86K

outputs message how many symbols they are included.

L86K V2.08D or later is added its option switch '-E'

'-E' ignores duplicated addressing errors in DSEG.        #2.08D

Trouble-shooted

.. When using symbols more than 32K, L86K v2.07C or former had displayed

an error. L86K v2.07E can link it.

.. When reverse-scrolling at mixed display mode in debugging, the Emulator

displayed uncorrectly. L86K v2.08 provides an EVA file to display

correctly.

.. When using the option /I, L86K v2.08 provides uncorrent evaluation file.

L86K v2.08A can do it.

.. When using pseudo instruction ORG, L86K v2.08 also provides uncorrect

evaluation file. L86K v2.08A can do it.

.. L86K v2.08D generates a map file when detecting DSEG size-exceed error.

Also, DSEG error messages renew.

.. L86K v5.00C generates 65280-byte EVA file.

Tips >>>

1. When "Fatal error ; unmatched chipname" is displayed

The chipname of the object file is different from of the option

file or of the other object files.

You must use the same chipname when assembling and specifying

the option data.

2. When "multiple allocation" is displayed

The EVA file isnot created when the address of the object file

is overlapped to other files.

Use -P option to confirm the overlapped address.

3. Whem "RAM address exceeds limit" is displayed

This message may show that no chipname is used.

Comfirm your operation about the chipname.

4. When "Public symbol multiple define: <<LABEL>>" is displayed

One label is specified as public symbol (LABEL) in several

linked-files.

Create the MAP file to confirm the multiple-defined symbol,

and replace to the correct LABEL.

5. Labels in DSEG

You cannot use labels only in the data segment(DSEG).

abc:			(no good)

def:	ds	1	(good)

## Current Problems

1. A malfunctional evaluation file is generated if linking 257-or-more segments.

Thank you.

End of lines